3GPP TSG RAN WG1 Meeting #101                                                               	           R1-200xxxx

e-Meeting, May 25th - June 5th, 2020

Title:	[Draft] LS on TDD configuration indication in PSBCH

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN2

CC:

Contact Person:

Name:	Teng Ma

E-mail Address:	mateng@catt.com

1. Overall Description:

In NR V2X synchronization discussion, RAN1 reached the following agreements:

Agreements: - For indication of TDD configuration: - X=1 bit indicates the number of patterns - Value 0 indicates one pattern is used. - Value 1 indicates two patterns are used. - Y=4 bits indicate the periodicity information - When one pattern is used, Y indicates the periodicity of the pattern. - When two patterns are used, Y jointly indicates the periodicities of the two patterns. - Z=7 bits indicate the UL slots - UL slots are jointly indicated by 7 bits when two patterns are configured. - FFS other details. Agreements: - When transmitting PSBCH, UE derives the values of X/Y/Z; from RRC configuration of TDD-UL-DL-ConfigCommon and maps them to the “indication of TDD configuration” field of PSBCH. Agreements: For indication of the UL slots by Z, - If single pattern is configured, Z bits indicate the UL slot number in the pattern is n. - If two patterns are configured, Z bits indicate the state index derived by the UL slots, n=n2w*P*2μ+1w+n1w Where - n1 is the number of UL slots in the first pattern, - n2n2 is the number of UL slots in the second pattern, - PP is the periodicity in units of ms of the first pattern, - w is the granularity of resource indication, - μ is (working assumption) - 0/1/2/3 corresponds to the 15/30/60/120 kHz SCS for SL respectively.

The corresponding TP of TS 38.213 based on the agreements above is endorsed as in R1-200xxxx.

In RAN1’s perspective, SL-TDD-Config with 12-bit payload size in PSBCH cannot be set to a value with the same meaning of TDD-UL-DL-ConfigCommon. The current definition in TS 38.331 can be changed as:

5.8.9.4.3 Transmission of MasterInformationBlockSidelink message The UE shall set the contents of the MasterInformationBlockSidelink message as follows: 1> if in coverage on the frequency used for the NR sidelink communication as defined in TS 38.304 [20]. 2> set inCoverage to true; 2> if tdd-UL-DL-ConfigurationCommon is included in the received SIB1: 3> set sl-TDD-Config according to the definition in subclause 16.1 of TS 38.213;

2   Actions:

To RAN WG2:

RAN1 respectfully asks RAN2 to take the agreements above into account.

3   Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #102	24 - 28 Aug. 2020, 		Online

TSG-RAN WG1 Meeting #102bis	12 - 16 Oct. 2020, 		China