# [101-e--NR-5G_V2X_NRSL-SYNC-01]

# Email discussion/approval regarding PSBCH content/TDD configuration indication

- Issue 1: Indication of TDD configuration

- Issue 2: SL-TDD-Config determination

- Issue 4: PSBCH contents and payload size (WAs confirmation)

By 5/29, with potential TP till 6/4 – Teng (CATT)

## Issue 1 Indication of TDD configuration

FL Proposal:

Proposal 1: For indication of TDD configuration, the pattern(s) indication (X) and periodicity indication (Y) follows the two tables below:

Table 1. Periodicity indication Y with single TDD pattern (X=0)

| Periodicity indication Y | P (ms) | Single pattern |
| --- | --- | --- |
| 0 | 0.5 | 0.5 |
| 1 | 0.625 | 0.625 |
| 2 | 1 | 1 |
| 3 | 1.25 | 1.25 |
| 4 | 2 | 2 |
| 5 | 2.5 | 2.5 |
| 6 | 4 | 4 |
| 7 | 5 | 5 |
| 8 | 10 | 10 |
| 9-15 | Reserved |  |

Table 2. Periodicity indication Y with two TDD patterns (X=1)

| Periodicity indication Y | P+P2 (ms) | Two patterns |  |
| --- | --- | --- | --- |
|  |  | P | P2 |
| 0 | 1 | 0.5 | 0.5 |
| 1 | 1.25 | 0.625 | 0.625 |
| 2 | 2 | 1 | 1 |
| 3 | 2.5 | 0.5 | 2 |
| 4 | 2.5 | 1.25 | 1.25 |
| 5 | 2.5 | 2 | 0.5 |
| 6 | 4 | 1 | 3 |
| 7 | 4 | 2 | 2 |
| 8 | 4 | 3 | 1 |
| 9 | 5 | 1 | 4 |
| 10 | 5 | 2 | 3 |
| 11 | 5 | 2.5 | 2.5 |
| 12 | 5 | 3 | 2 |
| 13 | 5 | 4 | 1 |
| 14 | 10 | 5 | 5 |
| 15 | 20 | 10 | 10 |

Proposal 2: For indication of the granularity of UL resources,

- If single pattern is configured, the granularity of the number of UL resources indicated by SL-TDD-Config is 1 slot.

- If two patterns are configured, the granularity of the number of UL resources indicated by SL-TDD-Config follows the table below.

| Periodicity indication Y | P+P2 (ms) | Two patterns |  | Granularity in slots with different SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | P | P2 | 15kHz | 30 kHz | 60 kHz | 120 kHz |
| 0 | 1 | 0.5 | 0.5 | 1 |  |  |  |
| 1 | 1.25 | 0.625 | 0.625 |  |  |  |  |
| 2 | 2 | 1 | 1 |  |  |  |  |
| 3 | 2.5 | 0.5 | 2 |  |  |  |  |
| 4 | 2.5 | 1.25 | 1.25 |  |  |  |  |
| 5 | 2.5 | 2 | 0.5 |  |  |  |  |
| 6 | 4 | 1 | 3 | 1 |  |  | 2 |
| 7 | 4 | 2 | 2 |  |  |  |  |
| 8 | 4 | 3 | 1 |  |  |  |  |
| 9 | 5 | 1 | 4 |  |  |  |  |
| 10 | 5 | 2 | 3 |  |  |  |  |
| 11 | 5 | 2.5 | 2.5 |  |  |  |  |
| 12 | 5 | 3 | 2 |  |  |  |  |
| 13 | 5 | 4 | 1 |  |  |  |  |
| 14 | 10 | 5 | 5 | 1 |  | 2 | 4 |
| 15 | 20 | 10 | 10 | 1 | 2 | 4 | 8 |

Proposal 3: For indication of the UL slots by Z,

- If single patter is configured, Z bits indicate the UL slots;

- If two patterns are configured, Z bits indicate the state index derived by the UL slots.

| Company | Views |
| --- | --- |

## Issue 2 SL-TDD-Config determination

FL Proposal: SL-TDD-Config in PSBCH is set to the same values in SIB provided by gNB

| Company | Views |
| --- | --- |

## Issue 4 PSBCH contents and payload size

RAN1#99 meeting

Working assumption: - PSBCH payload size is 56 bits including 24 bits of CRC. Agreements: - Note: “green” already earlier; “blue” new agreements, “brown” working assumption, “change marks” for updates PSBCH contents; Number of bits; Notes. DFN; 10. Indication of TDD configuration; 12; System-wide information, e.g. TDD-UL-DL common configuration and/or potential SL slots. Slot index; 7; Note: Up to 3 bits can be carried in DM-RS or in PBCH payload.. In-coverage indicator; 1. Reserve bits. CRC; 24. Total bits; 56.

FL Proposal: Confirm the working assumptions in RAN1#99 for the PSBCH contents for NR SL Rel-16, and reserve bits are 2

| Company | Views |
| --- | --- |