3GPP TSG RAN WG1 #101-e			R1-2004906

e-Meeting, May 25th – June 5th, 2020

Title:	[Draft] LS reply to RAN2 on Cast type indication and MAC agreements

Response to:	R2-2004082

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	LG Electronics [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Hanbyul Seo

Tel. Number:

E-mail Address:	hanbyul.seo@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 thanks RAN2 for the LS on the cast type indication and MAC agreements. RAN1 would like to inform the following agreements and conclusions that may be related to the LS in R2-2004082:

Agreements: - SCI format 2-A includes a 2-bit information field providing an explicit indication for the cast type Conclusion: - It is feasible from L1 signaling perspective to use Groupcast option 1 (i.e., NACK only feedback) when Zone ID or Communication range requirement is not provided, if RAN2 decides to support this operation. - No action in RAN1 unless RAN2 informs RAN1 about their decision (to support or not) - Note that if RAN2 decides to support it, RAN1 needs to further discuss

For the questions RAN2 asked in the LS, RAN1 answers are as follows:

Q1: Will the corresponding cast type be always indicated by SCI?

A1: Yes. The cast type (broadcast, groupcast, unicast) of a transport block is always indicated via SCI.

Q2: If the cast type is indicated by SCI, how is the corresponding cast type used by an Rx UE?

A2: In RAN1 specifications, the cast type indication is used by an RX UE to identify the location of the associated PSFCH resource. The physical layer mechanism for determining the associated PSFCH resource is different in groupcast option 1 (HARQ-ACK information includes only NACK only), groupcast option 2 (HARQ-ACK information includes ACK and or NACK) and unicast (HARQ-ACK information includes ACK orand NACK).

2. Actions:

To RAN1 group

RAN1 respectfully requests RAN2 to take the above into account in the related work.

3. Date of Next TSG-RAN1 Meetings:

RAN1#102-bis	12 - 16 October 2020   	China, CN

RAN1#103	16 - 20 November 2020   	US, US