3GPP TSG RAN WG1 #101	R1-200XXXX

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (Nokia)

Title:	Thread#2 for AI 7.2.4.6 QoS management

Agenda item:	7.2.4.6

Document for:	Discussion and Decision

Introduction

[101-e-NR-5G_V2X_NRSL-QoS-02] Email discussion/approval regarding the assumption on P_("MAX" ,CBR) when maximumtransmitPower-SL is not provided, by 5/28, with potential TP by 6/3 – Torsten (Nokia)

This email thread includes the following issue:

- Procedures Issue 1.4: P_{MAX,CBR}

TS 38.213 V16.1.0 currently defines the following behaviour for PMAX,CBR in PSSCH power control:

PMAX,CBR is determined by a value of maximumtransmitPower-SL based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if maximumtransmitPower-SL is not provided, PMAX,CBR=0

Notes:

- Parameter maximumtransmitPower-SL corresponds to sl-MaxTxPower in current TS 38.331.

- Current text imposes a rather low upper bound on TX power (0 dBm) when maximumtransmitPower-SL is not provided. This is not based on any agreement and was probably not intended.

Company views

Q1: Should TS 38.213 state explicitly that CBR-based power control does not apply to a PSSCH transmission using resources allocated using mode 1?

| Company | View | Comments |
| --- | --- | --- |
| Nokia, NSB | no | The consequence of not explicitly mentioning the mode in TS 38.213 is that, depending on RAN2 details, it may be possible to support CBR-based power control in mode 1: While the objective “Support for simultaneous configuration of Mode 1 and Mode 2 for a UE” was removed from the WID, as far as I can see nothing currently prevents the gNB from including both sl-ScheduledConfig and sl-UE-SelectedConfig (which indirectly provides sl-MaxTxPower=maximumtransmitPower-SL) in an RRCReconfiguration message and nothing states that the UE is not expected to handle such a configuration. If my understanding is correct, then the gNB can provide a UE in mode 1 with maximumtransmitPower-SL. Moreover, a future release may introduce “Support for simultaneous configuration of Mode 1 and Mode 2 for a UE”, in which case including both the sl-ScheduledConfig and the sl-UE-SelectedConfig IEs in RRCReconfiguration will naturally be supported. |
| Samsung | Yes | Our understanding is that CBR-based sidelink power control is not supported in mode 1 by 38.331 description (maximumtransmitPower-SL is conftgured only for Mode2 (SL-UE-SelectedConfig-r16)). If this is not a common understanding, we prefer to capture this in 38.213. Otherwise, if this is a common understanding, we do not need to capture this in 38.213 additionally. |

Q2: Currently in TS 38.213, PMAX,CBR=0 dBm when maximumtransmitPower-SL is not provided – should this behaviour be changed? If so, how?

| Company | View | Comments |
| --- | --- | --- |
| Nokia, NSB | YesPMAX,CBR= Infinity | The point is that CBR-based power control does not apply if maximumtransmitPower-SL is not provided. This could be captured in several different ways, but setting the value to Infinity looks most straightforward and readable to me. |
| Samsung | Yes, if maximumtransmitPower-SL is provided, PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) else, PPSSCH(i)=minPCMAX,minPPSSCH,Di,PPSSCH,SL(i) | We suggest to follow the LTE approach (Section 14.1.1.5 in 36.213) as shown in the left hand side, rather than to introduce an infinity value. |

Q3: Anything else for this issue?

| Company | Comments |
| --- | --- |

Issues

Topic 1: Sidelink Congestion Control

Issue 1.5: CBR-based sidelink power control in mode 1

Background

For the LTE V2X sidelink, CBR-based sidelink power control was only applied in resource allocation mode 3. The current specification in TS 38.213 on the other hand appears to apply CBR-based sidelink power control regardless of resource allocation mode:

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of maximumtransmitPower-SL based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if maximumtransmitPower-SL is not provided, PMAX,CBR=0

Above I wrote that this appears to apply CBR-based sidelink power control regardless of resource allocation mode: It is not immediately obvious if the intention is that

- CBR-based sidelink power control can actually be applied in mode 1; or

- for mode 1 the parameter maximumtransmitPower-SL cannot be provided, hence CBR-based sidelink power control in mode 1 is actually not supported.

Views expressed in contributions

[Nokia, NSB] propose to make an explicit agreement that CBR-based power control can be applied in both resource allocation modes.

[Apple] propose that PMAX,CBR be set to infinity for mode 1.

Feature lead view

It would be useful to check if there is a common understanding of the intention. If RAN1 does not address this then the decision is de facto up to RAN2 – if maximumtransmitPower-SL can be provided to a mode 1 UE then CBR-based sidelink power control is supported in mode 1.

Procedures Issue 1.4: P_{MAX,CBR}

The following issue was identified in agenda item 7.2.4.5 Physical layer procedures for sidelink [feature lead summary R1-2003569]:

- Issue 1-4: the assumption on PMAX,CBR when maximumtransmitPower-SL is not provided

  - PMAX,CBR is not used for power control

    - Support: [vivo, R1-2003383] [OPPO, R1-2004073] [Apple, R1-2004219]

Vivo: text proposal for the case that maximumtransmitPower-SL is not provided.

OPPO: propose that “For mode 2, PMAX,CBR is set to infinity.”

Apple: propose that “The value P_{max_CBR} in the PSSCH transmit power formula is set to infinite for mode 1 or if the higher layer parameter “maximumtransmitPower-SL” is not provided.”

Current specification in TS 38.213 V16.1.0:

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of maximumtransmitPower-SL based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if maximumtransmitPower-SL is not provided, PMAX,CBR=0

Alternatives proposed as value of  PMAX,CBR when maximumtransmitPower-SL is not provided:

- minPCMAX,minPPSSCH,Di,PPSSCH,SL(i)

- Infinity

References

- RP-200129, “Revised WID: 5G V2X with NR sidelink”

- TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR

- TR 38.885, Study on NR Vehicle-to-Everything (V2X)

# Background

## WI Objectives

At RAN#83, a new work item “5G V2X with NR sidelink” (5G_V2X_NRSL) was approved ‎[1]. Two of the objectives are relevant for the present agenda item:

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage. - … - Congestion control [RAN1, RAN2] 4. Specify support for QoS management [RAN2, RAN3, RAN1]

## Earlier Agreements

The following relevant agreements have been reached in previous meetings:

### QoS

Agreements: From RAN1 perspective, at least the following QoS-related parameters relevant to physical layer studies are considered: - Priority - latency - reliability

Agreements: RAN1 studies further how to use - priority, - latency, - reliability, - minimum required communication range (as defined by higher layers) if agreed to use in the physical layer aspects of at least - resource allocation and - congestion control and - resolution of in-device coexistence issues and - power control

In the Sidelink resource allocation mode 2 agenda item, the following working assumption was reached:

Working assumption: - An indication of a priority of a sidelink transmission is carried by SCI payload - This indication is used for sensing and resource (re)selection procedures - This priority is not necessarily the higher layer priority

Agreements: - For the priority indication in 1st stage SCI: - Up to RAN2 on how to define the mapping between the priority indication and the corresponding QoS - Size is 3 bits (as a working assumption)

### Sidelink Congestion Control

Agreements: - Introduce at least one congestion metric for NR sidelink - FFS details – to be done in WI phase (if included)

Agreements: - Congestion control is supported at least for sidelink mode 2 - Note: details of congestion control can be covered in the work item phase, not in this SI.

Conclusion: - It is deemed beneficial to report Sidelink Congestion Metrics(s) to a gNB - Consequently, it is recommended to specify the corresponding details in the WI phase

Agreements: Support at least NR CBR as congestion metric for NR sidelink congestion control. - LTE CBR is the baseline for defining NR CBR.

Agreements: - LTE V2X sidelink congestion control is the starting point for defining NR sidelink congestion control.

Agreements: - Higher-layer reporting of CBR to the gNB is supported for RRC_CONNECTED UEs.

Agreements: - For PSCCH/PSSCH multiplexing option 3, one CBR measurement over a resource pool is defined. - PSFCH resources, if (pre)configured, are excluded from this CBR measurement.

Agreements: Define NR sidelink Channel Occupancy Ratio (CR) measurement. - LTE CR is the baselines

Agreements: - Congestion control can restrict the values of at least the following PSSCH/PSCCH TX parameters per resource pool: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions – already agreed in mode 2 AI - Upper bound of TX power (including zero TX power) - Congestion control can set an upper bound on channel occupancy ratio (CR), CRlimit. - Ranges/bounds of the transmission parameters and CRlimit are functions of QoS and CBR. - In addition to congestion control (in use or not in use), the above parameters can be restricted by reusing the same mechanism as in LTE - For speed, further discussion on absolute vs. relative speed - FFS other parameter(s) that can be restricted - FFS whether or not to tie the speed with a UE capability

Agreements: Lookup table links CBR range with values of the transmission parameters and CRlimit for each value of the indication of a priority of a sidelink transmission carried by SCI payload (as per WA from RAN1#98), Lookup table is (pre)configured. Details up to RAN2. - Up to 16 (as a working assumption) CBR ranges are supported - The working assumption will be automatically confirmed in RAN1#99 if no further input

Agreements: - Sidelink RSSI (SL-RSSI) measurement is used for CBR estimation

Agreements: A sidelink resource is busy for the purpose of CBR measurement if Sidelink RSSI measured by the UE in that resource exceeds a (pre-)configured threshold.

Agreements: The CBR measurement time window size is 100 ms and 100 slots by (pre-)configuration. CR window size is { 1000 ms, 1000 slots } by (pre)-configuration

Agreement: - The future segment of the CR evaluation window reuses the same behaviour as in the LTE V2X sidelink. - FFS whether additional constraints on UE’s choice of values for a and b are needed

Agreement: For the constraints on past/future window in CR evaluation: - n+b shall not exceed the last transmission opportunity of the grant for the current transmission - b >= 0 - b < (a+b+1)/2 Notes: - in the first bullet point above, LTE’s “should” has been replaced by “shall”

Agreement: - UE evaluates CR and applies CR_limit for every (re)transmission.

Agreement: - The CBR processing time is given by UE capability according to the following table µ; Congestion process time 1 (slots); Congestion processing time 2 (slots). 0; 2; 2. 1; 2; 4. 2; 4; 8. 3; 8; 16. - A UE shall only apply a single CBR/CR processing time capability in SL. - CR processing time is the same as CBR processing time.

Agreement: - The slot index in the definition of CBR is the physical slot index. Agreement: - The slot index in the definition of CR is the physical slot index.

### TX Parameter Restrictions

Agreements: - Only TX parameter restriction based on absolute speed can be (pre)configured in Rel-16.

Agreement: - In addition to congestion control (in use or not in use), the following PSSCH/PSCCH TX parameters per resource pool can be restricted by reusing the same mechanism as in LTE: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions Note: This reverts the agreement made in RAN1#98b, which included “Upper bound of TX power” in the set of TX parameters that can be restricted using this mechanism.

# Appendix: Contributions used as basis for the summary

| R1-2003314 | Remaining details of QoS management for sidelink | Nokia, Nokia Shanghai Bell |
| --- | --- | --- |
| R1-2003384 | Remaining issues on QoS management for sidelink | vivo |
| R1-2003499 | Remaining details of QoS management for NR sidelink | Huawei, HiSilicon |
| R1-2003553 | Remaining issues on QoS | ZTE, Sanechips |
| R1-2003567 | Discussion on QoS management for NR sidelink | LG Electronics |
| R1-2003619 | Remaining issues on QoS management in NR V2X | CATT |
| R1-2003878 | On QoS Management for NR Sidelink | Samsung |
| R1-2004077 | Remaining open issues on QoS | OPPO |
| R1-2004220 | Remaining Issues of Sidelink QoS Management | Apple |
| R1-2004297 | Remaining Issues on Congestion control and QoS Management for NR-V2X | InterDigital, Inc. |
| R1-2004549 | QoS management for NR sidelink | Ericsson |

R1-2004220 Apple

Proposal 2: The value Pmax_CBR in the PSSCH transmit power formula is set to infinite for mode 1.