3GPP TSG RAN WG1 #100b-e	R1-2004920

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (OPPO)

Title:	Summary#2 of email thread [101-e-NR-L1enh_URLLC-UCI_Enh-03]

Agenda Item:	7.2.5.2

Document for:	Discussion and Decision

# Introduction

This paper summarizes the following email discussion in RAN1#101-e meeting:

[101-e-NR-L1enh-URLLC-UCI_Enh-03] Other issues with quick checking and agreement by 5/29 and corresponding TP (if any) by 6/5 (Jia, OPPO) including

- Prioritization of power allocation to transmissions with higher priority index (4.3.2)

- Power control issues with two PUCCH-Configs (3.3.1)

- Max number of PUCCH resources (3.2.2)

# Issue 4.3.2: Prioritization of power allocation to transmissions with higher priority index

## Discussion status

Samsung proposal:

Capture prioritization of power allocation to transmissions with higher priority index as follows.

## 7.5 Prioritizations for transmission power reductions For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion would exceed , where is the linear value of in transmission occasion as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to for that frequency range in every symbol of transmission occasion . When determining a total transmit power for serving cells in a frequency range in a symbol of transmission occasion , the UE does not include power for transmissions starting after the symbol of transmission occasion . The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot. - PRACH transmission on the PCell - Transmissions with higher priority index - PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information - PUCCH transmission with CSI or PUSCH transmission with CSI - PUSCH transmission without HARQ-ACK information or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell - SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell *** Unchanged text is omitted ***

IDC proposal:

If the total UE transmit power on serving cells in a frequency range would exceed Pcmax, the UE allocates power to high-priority PUCCH or PUSCH transmissions before low-priority PUCCH or PUSCH transmissions.

The intention of the proposal is agreeable. But the TP may need to be improved. For the corresponding TP, the questions and comments from companies include:

- Is the intention is to use “Transmissions with higher priority index” to cover all the transmissions with higher priority? Then among the transmissions with higher priority, how to do the power allocation? Our understanding is that it will reuse the same rule in the current specification for the transmissions with higher priority. But the current TP seems not reflect this.  One potential solution is to prioritize the higher priority under each bullet in the existing spec, except for the bullet for PRACH transmission.

- For high priority transmission and low priority transmission, power allocation is prioritized for high priority transmission. And for the same priority transmissions, the existing rule in spec is followed.

| Company | Comments |
| --- | --- |
| CATT | We are fine with the TP from Samsung. However, we are not clear why a new RRC parameter is needed given that the power control parameters can be separately configured for each PUCCH-Config. |
| Huawei/HiSilicon | 1. It seems the Samsung proposal above the table is not relevant to the TP here, it is to address the same issue 3.3.1 below. 2. As to the TP, we are generally fine with the intention to prioritize the transmission with higher priority, but the current TP is not clear. For example, is the intention is to use “Transmissions with higher priority index” to cover all the transmissions with higher priority? Then among the transmissions with higher priority, how to do the power allocation? Our understanding is that it will reuse the same rule in the current specification for the transmissions with higher priority. But the current TP seems not reflect this. One potential solution is to prioritize the higher priority under each bullet in the existing spec, except for the bullet for PRACH transmission. |
| OPPO | 1. We are fine with intention of TP. However, the current TP is not clear. For high priority transmisssion and low priority transmission, power allcoation is prirotized for high priority transmission. And for the same priority transmisions, the existing rule in spec is followed. 2. The motivation of a new RRC parameter is not clear for us. Due to power control parameter is configured in PUCCH-Config separately now. |
| Nokia, NSB | - Intention of the TP is fine (from prioritization point of view) but agree with HW & OPPO, but this is then just between priorities and then with the priority the other channels should be considered (i.e. use Rel-15 rules within the same priority) - no need for RRC parameter, as anyhow separately configured within each PUCCH config (as also pointed out by CATT & OPPO) |
| Samsung | Support the TP. The proposal need not be considered as it is already fulfilled given that power control parameters are configured per PUCCH-Config as CATT pointed out and Ericsson addresses below. For same priority transmissions, Rel-15 applies according to the TP. |
| InterDigital | Fine with the TP. Within “transmissions with higher priority index” one could also specify prioritization according to R15 rules. However, given that the case of simultaneous URLLC transmissions in multiple carriers should be relatively infrequent it may be ok to not detail it. |
| Sharp | Fine with the TP and its intention. Since PUCCH-Config for different priorities can be configured separately including transmit power, no extra RRC is needed. |
| Intel | For the proposed TP, we think this actually is out of scope for the WI. The scenario considered here is essentially same as Scenario 6 of intra-UE prioritization as was triggered by RAN2 LS during the R16 SI phase, and Scenarios 6 and 7 were not agreed to be pursued during the WI phase. From a technical perspective, if a UE is supporting critical urgent communications such as URLLC, gNB may not configure UL CA for the UE in the first place if the UE could be power limited. This scenario can be handled by gNB implementation, i.e., based on PHR information reported by the UE, it may not schedule/configure transmissions in multiple carriers. |
| vivo | We can accept the potential proposal for progress. |
| ZTE | We share CATT's view. The proposal is not needed. And the TP is not relevant to the proposal, we are fine with the intention of the TP. |

## Proposals from the discussion

Potential agreement:

Prioritizing the power allocation to transmissions with higher priority index

- Reuse R15 mechanism for power allocation among different higher priority transmissions.

- TP to be provided accordingly.

- Support in principle: Samsung, CATT, HW, OPPO, Nokia, IDC, Sharp, vivo, ZTE

# Issue 3.3.1:  Power control issues with two PUCCH-Configs

## Discussion status

Ericsson proposal:

- The power control parameter of a PUCCH-Config associated to the PUCCH resource determined for transmission should be used for power adjustment purposes. Note that no specification impact is foreseen.

Samsung proposal:

Introduce an RRC parameter priority-P0-PUCCH to adjust p0-PUCCH for PUCCH transmission with HARQ-ACK/SR having priority 1.

| Company | Comments |
| --- | --- |
| CATT | Agree with the clarification. |
| Huawei/HiSilicon | Fine with the clarification in principle. However, just to see if it is common understanding for the following cases with the current proposal here: Case 1: SR only with higher priority on the PUCCH resource in the first PUCCH-config à power control parameters of the first PUCCH-config will be used Case 2: SR with higher priority multiplexed with HARQ-ACK with higher priority on the PUCCH for HARQ-ACK with higher priority à power control parameter for PUCCH with HARQ-ACK to be used Case 3: SR with higher priority multiplexed with HARQ-ACK with higher priority on the PUCCH for SR à power control parameters in the first PUCCH-config will be used. |
| OPPO | Fine with clarification. |
| Nokia, NSB | Agree with the clarification. |
| Samsung | Fine with the clarification. |
| InterDigital | Fine with the clarification. |
| Sharp | Agree with the clarification. |
| Intel | Fine with the clarification. |
| vivo | Fine with the clarification and the potential conclusion |
| Qualcomm | Fine with the clarification and FL proposed conclusion. |
| Apple | Fine with the proposed conclusion. |
| Ericsson | Agree with the proposed conclusion On HW comments, our understanding is the following: Case 1: Agree. Case 2: When SR overlaps with AN, it would be multiplex on the AN resource. So, whichever PUCCH config the AN PUCCH resource is associated with (in this case the second one) would be used for power control. Case 3: As explained above, it doesn’t happen. But you could have SR with low priority configured in 2nd PUCCH-Config, that would be multiplexed with AN with low priority associated to 1st PUCCH-Config. Then 1st PUCCH-Config power control parameter are used. Chengyan> When PUCCH format 1 SR overlap with PUCCH format 1 HARQ-ACK, PUCCH format 1 SR will be transmitted and implicitly representing the HARQ-ACK state, right? Case 3 I mentioned above is for this case. Not sure if missed anything here.J |
| ZTE | Fine with the intention. But the case 3 mentioned by Chenyan is valid, it need more clarification. |

## Proposals from the discussion

Potential conclusion:

It is clarified that the power control parameter of a PUCCH-Config associated to the PUCCH resource determined for transmission should be used for power adjustment purposes.

- No specification impact is needed.

- CATT, HW, OPPO, Nokia, Samsung, IDC, Sharp, Intel, vivo, QC, Apple, E///, ZTE

# Issue 3.2.2:  Extending number of configured PUCCH resources

## Discussion status

Ericsson proposal:

- Maximum number of configured PUCCH resources for HARQ-ACK per PUCCH-Config in Rel-16 remains the same as in Rel-15.

- Maximum number of configured PUCCH resources for CSI in Rel-16 remains the same as in Rel-15.

- Maximum number of configured PUCCH resources for SR in Rel-16 is increased to [64] as compared to Rel-15.

  - Arguments:

    - 8 resources for SR as in Rel-15 should be increased. 64 could seems excessive, The reason for that was having Rel-15 as based line (max 8 per slot) and scale it with 7-8 due to sub-slot. We think keep it as Rel-15 is too low. At least it should be doubled now that different services are involved. So, we suggest 16 SR resources in total.

    - Maximum 32 for the 1st set, and maximum 8 for 2nd, 3rd and 4th set, each.

| Company | Comments |
| --- | --- |
| CATT | We are fine with the first two bullets. For SR, as far as we know, the maximum number of SR configurations per cell group is remained as 8 as in Rel-15. So the motivation to support more than 8 SR resources for a BWP is not clear to us. |
| Huawei/HiSilicon | Share the similar view as CATT that we do not see the motivation to increase the maximum number of PUCCH resources for SR, further clarification on the motivation is needed. We are fine with the first 2 bullets. |
| OPPO | Share the similar view as CATT. |
| Nokia, NSB | Same view as CATT, HW, OPPO: - first two bullets OK / support - need for more than 8 SR resources unclear |
| Samsung | Agree with CATT. |
| InterDigital | Fine with all bullets. |
| Sharp | Agree with CATT. Agree with the first two bullets. |
| Intel | Agree with CATT. |
| vivo | Agree with CATT. For SR, it is not clear why [64] PUCCH resources are needed. More explanations are welcome. |
| Qualcomm | We agree with the second bullet. For the third bullet, we agree with CATT and others that no need to support more than 8 SR resources. For the first bullet, we need more time to check the related UE implementation issues. |
| Apple | The first bullet is not exactly clear to us.Does it mean the value of maxNrofPUCCH-ResourcesPerSet remains as 32? And with two PUCCH-config, the total max # of resources for HARQ-ACK would double. The second bullet is fine. Agree with others on the 3rd bullet that we do not seem to see the need. |
| Ericsson | We are OK with 1st and 2nd. For SR, we can consider 16 resources. - we think 8 resources for SR as in Rel-15 should be increased. 64 could seems excessive, The reason for that was having Rel-15 as based line (max 8 per slot) and scale it with 7-8 due to sub-slot. We think keep it as Rel-15 is too low. At least it should be doubled now that different services are involved. So, we suggest 16 SR resources in total. Clarification to Apple, Yes. Maximum 32 for the 1st set, and maximum 8 for 2nd, 3rd and 4th set, each. Chengyan> According to the current agreement, same set of PUCCH resource is configured for sub-slot and used for all sub-slots, so even with sub-slot seems the situation is still similar as that in Rel-15? |
| ZTE | Agree with CATT's view. Share the same question with Chengyan, the intention of more PUCCH resoures for SR should be clarified. |

## Proposals from the discussion

Proposed conclusion:

- Maximum number of configured PUCCH resources for HARQ-ACK per PUCCH-Config in Rel-16 remains the same as in Rel-15.

- Maximum number of configured PUCCH resources for CSI in Rel-16 remains the same as in Rel-15.

- Maximum number of configured PUCCH resources for SR in Rel-16 remains the same as in Rel-15.

# Conclusions

For Issue 4.3.2, the followings were agreed:

Agreement:

Prioritizing the power allocation to transmissions with higher priority index

- Reuse R15 mechanism for power allocation among different higher priority transmissions.

- TP to be provided accordingly.

For Issue 3.3.1, the followings were agreed:

Conclusion

It is clarified that the power control parameter of a PUCCH-Config associated to the PUCCH resource determined for transmission should be used for power adjustment purposes.

- No specification impact is needed.

# References

- R1-2004674	Summary on UCI enhancements for R16 URLLC, OPPO

- R1-2003318	Remaining issues on UL control enhancements for NR URLLC	ZTE

- R1-2003388	UCI enhancements for URLLC	vivo

- R1-2003440	Remaining Issue of UCI Enhancements for NR URLLC	Ericsson

- R1-2003528	Corrections on UCI enhancement	Huawei, HiSilicon

- R1-2003578	Maintenance of Rel-16 URLLC UCI enhancements	Nokia, Nokia Shanghai Bell

- R1-2003621	Remaining issues on UCI enhancements	CATT

- R1-2003685	Remaining issues on UCI enhancements	MediaTek Inc.

- R1-2003708	Remaining Issues on UCI Enhancements for URLLC	NEC

- R1-2003738	Remaining details on UCI enhancements for eURLLC	Intel Corporation

- R1-2003814	Remaining issue on UCI enhancement	Panasonic Corporation

- R1-2003866	Remaining issues for UCI enhancements	Samsung

- R1-2003920	Discussion on remaining issues on UCI enhancement for URLLC	Beijing Xiaomi Mobile Software

- R1-2003953	Remaining issues on UCI enhancements for URLLC	CMCC

- R1-2003975	UCI enhancements	ETRI

- R1-2003980	Remaining Issues of UCI Enhancements for URLLC	Spreadtrum Communications

- R1-2004030	Remaining issues of UCI enhancements for NR URLLC	LG Electronics

- R1-2004045	Remaining issues on UCI enhancements for URLLC	Fujitsu

- R1-2004115	UCI enhancements for URLLC	OPPO

- R1-2004222	Remaining Issues on UCI Enhancements for eURLLC	Apple

- R1-2004271	UCI Enhancements for URLLC	InterDigital, Inc.

- R1-2004339	Remaining issue on the HARQ-ACK/PUSCH priority	ITRI

- R1-2004390	Remaining issues for UCI enhancement for Rel-16 URLLC	NTT DOCOMO, INC

- R1-2004458	Remaining issues on UCI Enhancements for URLLC	Qualcomm Incorporated

- R1-2004523	Remaining issues on UCI enhancement for NR URLLC	WILUS Inc.

- R1-2004574	Discussion on multiplexing UCI of same type on a PUSCH	ASUSTeK