3GPP TSG RAN WG1 Meeting #101-e                                                                                                R1-2004735

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (vivo)

Title:	Summary of [101-e-NR-L1enh-URLLC-InterUE-02]

Agenda Item:	7.2.5.5

Document for:	Discussion and Decision

- Introduction

The document provides a summary of RAN1#101-e email discussion thread [101-e-NR-L1enh-URLLC-InterUE-02]

Email discussion outcome

Conclusion

- The 1st editor’s note in section 11.2A of endorsed 38.213CR (R1-2003176) is removed.

Conclusion

- There is no consensus to support UL CI in the scenarios where processing capability#2 is not defined

Agreement

- The following text proposal in R1-200XXXX is endorsed for the editor’s CR on TS38.213.

---------------------------------Text proposal starts for TS 38.213, Section 11.2A -------------------------------- =====omitted text ====== A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or an actual repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, as determined in Clauses 9 and 9.2.5 or in Clause 6.1 in [6, TS 38.214], or an SRS transmission on the serving cell if, respectively, =====omitted text ====== ------------------------------- Text proposal ends for TS 38.213, Section 11.2A -----------------------------------

Reason for the above change:

- To clarify that the cancelation is applied to each actual repetition for PUSCH repetition type B

- To clarify that UL cancellation based on UL CI is applied after resolution of UL CG-DG collision as defined in clause 6.1 of TS38.214

Discussions

- Confirmation of possible conclusion

During the prepare phase, we were able to reach consensus on the following proposed conclusion. Please indicate only if you have any problem to confirm this conclusion

- Possible Conclusion

The 1st editor’s note in section 11.2A of endorsed 38.213CR (R1-2003176) is removed

| Company | Comment |
| --- | --- |
| Nokia, NSB | Support the conclusion |
| HW/HiSi | Confirm the conclusion. |
| Sony | Confirm the conclusion. |
| Samsung | Support |
| Panasonic | Support the conclusion. |
| CATT | Support the conclusion. |
| Qualcomm | Support the conclusion. |
| Intel | Support |
| Ericsson | Support |
| Sharp | Support |
| OPPO | Support |
| DOCOMO | Support |
| Apple | Support |
| LG | Support |

- Conclusion

  - The 1st editor’s note in section 11.2A of endorsed 38.213CR (R1-2003176) is removed

- Issue 2: Support of UL CI in the scenarios where processing capability #2 is not defined. [2[5][9][19]

[2][9][19] proposed that whenever UE processing capability 2 is not defined (for the frequency range, or SCS), use UE processing capability 1 to derive the UL CI processing time. Following TP is provided

TP for section 11.2A of TS38.213

------------------------------------------ Start of proposed change ------------------------------------------

An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by XXX. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. Processing capability 1 should be used if processing capability 2 is not defined for a pair of Frequency Range and SCS in [6, TS 38.214]. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 after a last symbol of a CORESET where the UE detects the DCI format 2_4.

------------------------------------------- end of proposed change   ------------------------------------------

[5] proposed that there is no need to support of UL CI in the scenarios where processing capability #2 is not defined.

Question:

- Q1: Do you agree to support UL CI in the scenarios where processing capability #2 is not defined?

  - If yes, please be specific on the justifications

  - If no, please be specific on the concerns

| Company | Comment |
| --- | --- |
| Nokia, NSB | Yes / Agree to support. There is no reason to not support this for cases with UE processing capability #2. The feature as such should also work for UE processing capability #2 – so no need to restrict by specification such operation. |
| HW/HiSi | No need to support UL CI where processing capability is not defined. This would be the case for high SCS as in FR2. Because of the short slot durations, there is no benefit expected when cancelling the eMBB uplink. It should be better to let the already scheduled transmission finish before sending URLLC from another UE. |
| Sony | Yes. The purpose of UL CI is to be able to cancel low priority traffic especially those with long transmission, which is likely be used for UE with capability 1. |
| Samsung | No need. Similar to not defining (X, Y) combinations for SCS associated with FR2. |
| ZTE | No. There are larger bandwidth and shorter symbol in FR2. A higher probability can be expected to find a suitable resource (without preemption) for UL transmission with higher priority within a required time. So the benefit of supporting UL CI in FR2 is questionable. |
| Panasonic | No. As a higher subcarrier spacing is shorter processing time, we think it is nice to support UL CI but not essential. |
| CATT | No. We share the same views as mentioned by above companies. We don’t think UL CI is a typical case for FR2. |
| Intel | Not needed. Application seems to be targeting FR2 where larger SCS would be used for transmission. However, as HW mentioned, at larger SCS scheduling granularity can be short enough and application of UL CI is expected to be limited. |
| Ericsson | Yes, we agree to support the feature for FR2. It is better to be future proof and do not limit UL CI to FR1 only. Agreed CI monitoring periodicity makes it possible to cancel 10 slots +Tproc,2 in advance. Earlier scheduling is also supported by K2 range 0…32 slots. Thus, we do not see strong reasons to limit UL CI for FR2 in principle. Further, if some companies have concerns, it is possible to differentiate corresponding UE feature for FR1 and FR2. |
| Sharp | No. The usage of UL CI in FR2 is limited, as mentioned by companies above. |
| OPPO | No. No strong reason to support UL CI in FR2 configured with larger SCS. In addition, if only one capability, capability 1, is supported, there is no reason to set different scheduling timeline and no collision. |
| DOCOMO | Yes. Agree to support the feature for FR2. As mentioned by some objecting companies, it would be possible to transmit HP UL transmission after LP UL transmission as symbol duration is short with high SCS of FR2. However, there is no need to put additional restriction for this case. It should be up to gNB whether to transmit UL CI to cancel LP UL transmission in such a case as we do in other cases. |
| Apple | No. Same as some other companies, we do not see much benefit to support UL CI in FR2. |
| LG | No. it is doubtable to use UL CI in FR2. |

- Summary of issue#2

- Do you agree to support UL CI in the scenarios where processing capability #2 is not defined?

  - Yes: (5) Nokia, Sony, Ericsson, DOCMO, vivo

  - No: (10) Huawei, Samsung, ZTE, Panasonic, CATT, Intel, Sharp, OPPO, Apple, LG

- Proposed conclusion

  - There is no consensus to support UL CI in the scenarios where processing capability#2 is not defined

- Issue 4: UE handling of intra-UE data-data collision and UL CI [2]

Discussion point#1

[2] Pointed out no only intra-UE handling of data-control collision, the intra-UE handling of data-data collision in UL should also not be affected by UL CI, according to the following agreement in last meeting.

Agreements:  UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions is not affected by UL CI.

However, the current spec text in 38.213 does not cover the data-data collision case

Proposed text proposal for 38.213

---------------------------------Text proposal #2 starts for TS 38.213, Section 11.2A -------------------------------- =====omitted text ====== A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, as determined in Clauses 9 and 9.2.5 or in Clause 6.1 in [6, TS 38.214], or an SRS transmission on the serving cell if, respectively, =====omitted text ====== ------------------------------- Text proposal #2 ends for TS 38.213, Section 11.2A -----------------------------------

Note there will be some parallel discussion in AI7.2.5.7 about the handling of CG-DG and CG-CG with same or different priority, which should be considered in this discussion. Therefore it is intended to discuss whether and how to clarify that data-data collision procedure is not affected by UL CI (limited to CG-DG collision case with Rel-15 timeline and potential Rel-16 timeline, if agreed in AI7.2.5.7)

Discussion point#2

It has been agreed in RAN1#98bis that for PUSCH repetition type B (i.e. Rel-16 PUSCH repetition), the UL is applied individually to each actual repetition.

Agreements:

- In case of PUSCH repetitions, UL CI is applied to each repetition individually (actual repetition in case of Rel-16 PUSCH repetition) that overlaps with the resource (in time and frequency) indicated by UL CI.

It was commented that the current spec is not clear enough about this aspect, it can be discussed whether any TP is needed.

Question:

- Q1: Do you think text proposal shown above is needed to clarify that intra-UE data-data collision handling is not affected by UL CI.

  - Please take the Rel-15 timeline for CG-DG handling and potential Rel-16 timeline (if agreed in AI 7.2.5.7) into consideration

- Q2: What text proposal is needed to clarify that UL CI is applied to each actual repetition individually in case of PUSCH repetition type B?

| Company | Comment |
| --- | --- |
| Nokia, NSB | Q1: The addition in the TP is not doing any harm but maybe not needed either. 38.214 defines what a PUSCH transmission is overall and this is used here (also in other parts of 213, when talking about a PUSCH transmission we are not refereeing to 38.214 in each clause). Q2: We do not see an imminent need here (should be clear enough already). |
| HW/HiSi | We do not have a strong view but agree with Nokia, we should be consistent how PUSCH transmission are referred to in other parts of the spec. Probably this question can be postponed until we have achieved progress on the other issues. |
| Sony | Not needed. But no strong view against it. |
| Samsung | Q1: No clear need but OK if ambiguity is deemed to exist. Q2: No need. |
| ZTE | For Q1, we think the TP is needed. For Q2, we have the following suggestion, <---------------------------Other parts are omitted -------------------------------> A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, as determined in Clauses 9 and 9.2.5, or an SRS transmission on the serving cell if, respectively, - the transmission is PUSCH with priority 0, if the UE is provided applicabilityforCI, - a group of symbols, from the TCI symbols, has at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and - a group of PRBs, from the BCI PRBs, has a corresponding bit value of '1' in the set of bits corresponding to the group of symbols in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission, where - the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (actual repetition of the) PUSCH transmission that is in a group of symbols having corresponding bit values of '1' in the DCI format 2_4; - the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of '1' in the DCI format 2_4. <---------------------------Other parts are omitted -------------------------------> |
| Panasonic | Q1: We think the TP is needed. |
| CATT | Q1: better to have the TP Q2: no need |
| Qualcomm | Q1: we are wondering what is the consequence if nothing is agreed for Q1? Is there any issue of the current spec behaviour? Q2: no strong view. |
| Intel | Q1. The addition in TP to cover the cases in Section 6.1 of 38.214 is needed just to avoid future confusion on whether CG-DG overwriting, at least from Rel-15 cases, may be impacted by UL CI (the answer being “no” as implied by the TP). Q2. If TP is captured wrt Q1, we don’t think any further consideration is needed. PUSCH with repetitions is already there and seems to include R16 repetition as well. |
| Ericsson | Q1. Ok to clarify this. The clearer the better. Since we have mentioned Clause 9 referring to control-data collisions, we should also include other parts which can impact. Q2. Do not see a need to clarify it further. It is already clear that PUSCH itself is actual transmission. |
| Sharp | Q1: We are OK to have the clarification. Q2: No need. |
| OPPO | Q1: No strong view but [6, TS 38.214] has been included in the first sentence, there is no reason to double quote. |
| DOCOMO | Q1: we are ok to clarify it. Q2: no strong view |
| Apple | Q1: no strong view but agree with OPPO that [6, TS 38.214] has been quoted already. Not sure adding it one more time actually changes anything. Q2: prefer to have a clarification. For the companies that think there is no need, it would be good if they can explain from where in the specs it is clear that it is an actual repetition but not a nominal repetition. We have done similar type of clarification in quite a few other places, and I wonder why this case is different. |
| LG | Q1: good to have Q2: PUSCH repetitions means repeated PUSCH with one scheduling. Current spec is clear. |

- Summary

  - For Q1:

    - TP is needed

      - (8) ZTE, Panasonic, CATT, Intel, Ericsson, Sharp, DOCOMO, LG

    - No strong view

      - (6)Nokia, Huawei, Sony, Samsung, OPPO, Apple

    - Not support the TP

      - Qualcomm

  - For Q2:

    - TP needed

      - Apple, ZTE

    - TP not needed

      - Nokia, Samsung, CATT, Intel, Ericsson, Sharp,

    - No strong view

      - Qualcomm, DOCOMO,

- Proposed agreement

  - To adopt the following TP (as resolution for both Q1 and Q2)

| ---------------------------------Text proposal #2 starts for TS 38.213, Section 11.2A -------------------------------- =====omitted text ====== A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or an actual repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, as determined in Clauses 9 and 9.2.5 or in Clause 6.1 in [6, TS 38.214], or an SRS transmission on the serving cell if, respectively, =====omitted text ====== ------------------------------- Text proposal #2 ends for TS 38.213, Section 11.2A ----------------------------------- |  |
| --- | --- |

- Issue 6: Earlier cancellation relaxation for UL CI [20]

[20] As has been discussed for cancellation under intra-UE prioritization, enforcing an exact cancellation time incurs significant implementation complexity for the UE. Instead, if an uplink transmission needs to be cancelled, the UE should be allowed to start the cancellation even before the first indicated symbol.

Proposal: If a UE is indicated by DCI 2_4 to cancel a PUSCH transmission, the UE can cancel the transmission before the first indicated symbol group. In other words, the first indicated symbol using DCI format 2_4 for cancellation is the latest time that the UE should start the cancellation of PUSCH

Question:

Q1:  Do you agree the above proposal and why?

| Company | Comment |
| --- | --- |
| Nokia, NSB | We are fine with the intention of the proposal – but at least the first DM-RS should be transmitted. In principle, there is no reason to force the UE to cancel at the latest point (if easier for the UE implementation to cancel earlier, why not allowing it). But then if a MAC PDU has been delivered and the UE would not even start transmitting (incl. the front-loaded DMRS), the gNB would not know if there is data in the HARQ buffer or not (due to UL skipping). Therefore, at least the first symbol with DM-RS should be retained. |
| HW/HiSi | We do not agree with the proposal. It should be controllable by the gNB when the UE is cancelling. For example, in case the eMBB UE is configured with CBG based transmission, it could be very harmful for the eMBB performance if the eMBB UE could cancel at will. The CBGs that are not affected by UL CI should still be expected to be transmitted. |
| Sony | Our initial view is that the entire PUSCH should be dropped and so it doesn’t matter if it is dropped earlier than the indicated symbols. However, Huawei’s comment on CBG based transmission do make sense. Hence, we have no strong view on this. |
| Samsung | Agree with the proposal (but also no agreement is OK). There is no need to restrict the UE implementation to stop transmission at an exact time instance (no benefit exists in general and, in particular, considering that such cancellation will be infrequent). |
| ZTE | No. Firstly, we don’t think it is essential to modify as the current description in 38.213 is unambiguous. Further, the gNB may want the UE to transmit earlier parts in some cases, such as, the UE is configured with CBG transmission or UCI is piggybacked on the first several symbols of the canceled UL transmission. So it is reasonable to allow the gNB to know which part of UL transmission is still valid after cancelation. Otherwise, unnecessary blind detection will then be required for the gNB. |
| Panasonic | Agree with the proposal. Same view as Samsung. |
| CATT | No. Same view as HW and ZTE. |
| Qualcomm | We support the proposal. As has been discussed for cancellation under intra-UE prioritization, enforcing an exact cancellation time incurs significant implementation complexity for the UE. Furthermore, we think it is beneficial for both UE and gNB implementation as well as specification to have a unified cancelation behaviour for intra- and inter-UE cancellation. |
| Intel | We acknowledge that the intention of the proposal is to allow for flexible UE implementation by not mandating UE to cancel starting at an exact point in time. The main benefit we see from such flexibility is when the UE may be able to cancel the UL transmission in entirety (i.e., avoid partial cancelation). However, here, it should be noted that such benefit to UE implementation will only be opportunistic in nature since there will certainly be cases wherein the UE may not be able to avoid partial cancelation events. Further, as long as cancelation timeline is not an issue, intra-UE cancelation involves replacing the cancelled transmission with another UL transmission. Such is not necessary for the inter-UE case (assuming we do not allow scheduling another UL transmission in cancelled symbols of the affected UL transmission). On the other hand, we also see the point from Huawei that use of CBG-based retransmissions becomes less effective in such a case. Evaluations during the Rel-16 SI had shown the significant benefits from combining CBG-based (re)transmissions with UL CI-based cancelations for long PUSCHs with large TB sizes. Thus, similar to Sony, we also do not see a clear winner amongst the two options. |
| Ericsson | Do not agree. It is beneficial to transmit PUSCH as long as possible until the latest cancellation point. If reception of a part of transmission is enough for PUSCH decoding, then re-Tx might not be needed. => spectral efficiency gain. |
| Sharp | No. We share the same view with HW and ZTE. |
| OPPO | We support proposal. Enforcing an exact cancellation time leads implementation complexity. |
| DOCOMO | We slightly prefer not to have this proposal. Share the same view with HW and Ericsson. |
| Apple | We support the proposal. Share the same view as Samsung and Qualcomm. |
| LG | We do not support proposal. Motivation seems understandable but not essential. |

- Summary of issue#6

  - Alt 1: The first indicated symbol by DCI format 2_4 for cancellation is the latest time that the UE should start the cancellation of PUSCH and UE is allowed to cancel before that symbol

    - Supported by: (8) Samsung (also fine with no agreement), Panasonic, Qualcomm, OPPO, Apple, MediaTek, Motorola Mobility/Lenovo, Ericsson

  - Alt 2: The first indicated symbol by DCI format 2_4 for cancellation is exact the time that the UE starts the cancellation

    - Supported by:  (7) Huawei, ZTE, CATT, Sharp, DOCOMO, LG, Nokia/NSB

  - Neutrual

    - Sony, Intel

- Possible compromise 1

  - Introduce a UE capability signalling for alt 1, and alt 2 is expected if not indicated by the UE

- Possible compromise 2

  - If UE is configured with CBG based transmission for UL, alt 2 is expected

  - Otherwise, alt 1 is allowed.

Previous agreements

- RAN1#96bis

Working assumption:

- PDCCH is used for UL cancelation indication

  - The Working assumption can be revisited if the DCI for cancelation indication only carry very small number of information bits, e.g. 1 bit.

Agreements:

- Upon detecting an UL cancelation indication, at least stop without resuming is supported

  - FFS whether and how to support stop with resume

Agreements:

- Further discuss which UL transmissions that can potentially be cancelled by the UL cancelation indication, including

  - Dynamic scheduled UL transmissions, including PUSCH, PUCCH, SRS

  - Semi-persistent UL transmissions, including PUSCH, PUCCH, SRS

  - Periodic UL transmissions, including configured grant PUSCH, PUCCH, SRS

  - PRACH

Agreements:

- Further discuss, aiming for down-selection, the group common DCI and UE-specific DCI for UL cancelation indication

  - For group common DCI (different from Rel-15 SFI)

    - UE is configured to monitor a group common DCI which indicates the time/frequency region on which the UL cancellation indication applies

  - For UE specific-DCI

    - When applicable, UE is configured to monitor a second UL grant for the same TB as an earlier PUSCH indicating UL cancellation before the end of the earlier PUSCH transmission. In this case, the UE follows the UL cancellation indication.

Conclusion:

- Further discuss the following power control enhancements

  - Increased TPC range

    - FFS details, e.g. supported value range, number of TPC bits, accumulated and/or absolute TPC, configurability of the TPC tables, applicability to SRS/PUCCH.

  - Indication of open-loop parameter sets based on scheduling DCI without using SRI

  - Indication of open-loop parameter sets based on GC-PDCCH

- RAN1#97

Agreements:

- Support at least group common DCI for cancelation indication

  - FFS whether or not to additionally support UE-specific DCI for cancelation indication

Conclusion:

To down-select from the following options for enhanced power control

- Option 1: Indication of open-loop parameter sets by DCI

- For DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI without using SRI is applied to the scheduled transmission

- FFS At least for single active CG-PUSCH, an open-loop parameter set is indicated to the UE by a UE-specific field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

- FFS For a UE, the open-loop parameter sets for DG-PUSCH and CG-PUSCH may be same or different

- Option 2: Indication of TPC with increased range by DCI

- For DG-PUSCH, a TPC with increased range is indicated to the UE by the TPC field in scheduling DCI

- FFS At least for single active CG-PUSCH (and potentially also for DG-PUSCH), a TPC with increased range is indicated to the UE by a UE-specific TPC field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

- At least for DG-PUSCH, for a UE, the number of TPC entries (4 or 8) and power adjustment value for each entry is higher layer configured

- FFS For a UE, the TPC configuration for DG-PUSCH and CG-PUSCH may be same or different

- Option 3:

- For DG-PUSCH, use either the solution from option 1 or option 2 for DG-PUSCH as above

    - To down-select from option 1 and 2

- FFS At least for single active CG-PUSCH, UE derives the transmissions power based on the time/frequency resource indicated by a group common DCI

    - If a CG-PUSCH transmission overlaps with the indicated time/frequency resource, UE use one open-loop parameter set with higher power for the transmission

    - If a CG-PUSCH transmission does NOT overlap with the indicated time/frequency resource, UE use another open-loop parameter set with lower power for the transmission

    - FFS for the case of multiple active CG-PUSCH

    - Note: some companies have concern that this was not captured in the TR as one potential solutions

- RAN1#98

Agreements:

- Reuse the existing methods for search space configuration to support UL CI monitoring

  - FFS possible restrictions

  - Note: this means both symbol level and slot level monitoring periodicities are possible from specification perspective

Agreements:

- The UE DCI size budget is not increased by UL CI monitoring

- Further discuss methods to reduce the UE monitoring for UL CI, e.g.

  - The number of aggregation levels and/or candidates for the UL CI monitoring should be limited

  - Conditions for eMBB UE UL CI monitoring:

    - For UL transmission with associated PDCCH,

      - Option 1: UE starts UL CI monitoring after the PDCCH is decoded

      - Option 2: UE monitors UL CI at least at the latest monitoring occasion ending no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - For UL transmission without associated PDCCH, UE monitors UL CI at least at the latest monitoring occasion that ends no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - Other conditions?

  - Others?

- FFS the enhancement of UE capability (number of non-overlapping CCE and/or blind decodes) for UL CI monitoring

Agreements:

- Upon detecting an UL cancelation indication, for the transmission of UL signal/channels, “stop with resuming” is not supported

  - Except:

    - SRS can still be transmitted on the non-cancelled symbols (conditioned on if SRS can be pre-empted)

    - FFS for the PUSCH repetition (Rel-15 & Rel-16) case

    - FFS for the PUCCH repetition case (conditioned on if PUCCH can be pre-empted)

  - FFS whether another PUSCH can be scheduled in non-pre-empted resource

  - FFS impact (e.g. phase continuity issue) to a different carrier due to UL cancelation

R1-1909774

Agreements:

- The following UL channel/signals can be cancelled by UL cancelation indication

  - PUSCH (including DG-, CG- and SP-)

  - FFS for SRS

  - FFS for PUCCH

    - Option 1: PUCCH (all types) can be cancelled

    - Option 2: Some PUCCH can be cancelled, e.g. PUCCH carrying CSI

    - Option 3: PUCCH cannot be cancelled

  - FFS for PRACH (preamble and/or MSG 3 PUSCH)

Agreements:

- The UE processing time requirement for UL cancelation indication based on N2 defined in Rel-15 UE cap#2 is supported

  - FFS whether the processing time requirement for UL cancelation indication larger than N2 as defined in Rel-15 UE cap#2 can also be supported as an UE capability

  - FFS whether the processing time requirement for UL cancelation indication shorter than N2 as defined in Rel-15 UE cap#2 as can also be supported an UE capability

Agreements:

- For a DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI using a separate field than SRI is supported.

  - FFS number of bits for the indication

- RAN1#98bis

Agreements:

- Regarding UL CI monitoring, support the following:

  - A new RNTI (e.g. CI-RNTI) is used for UL CI

  - FFS: Monitoring periodicity larger than [5] slot is not supported for UL CI

  - The aggregation level(s) and the number of PDCCH candidates configured by RRC

    - FFS possible restrictions, e.g., the ones associated with SFI

  - The DCI payload size for UL CI  is configured by RRC

    - FFS possible values

Agreements:

- SRS can be cancelled by UL CI

- PUCCH cannot be cancelled by UL CI

- RACH related UL transmissions cannot be cancelled by UL CI, including MSG 1/3 in case of 4-step RACH, MSG A in case of 2-step RACH.

Agreements:

- Cross-carrier UL cancelation indication is supported using the same way as Rel-15 SFI/DL PI

  - The indication field position in DCI for each cross-carrier indicated serving cell is configured by RRC

Agreements:

- Different UE processing time capability for UL CI (i.e. shorter or longer than T_proc2 for cap#2 UE) is not considered in Rel-16

  - d2,1=0 also when DMRS and UL-SCH (for the PUSCH to be cancelled) are multiplexed in the 1st symbol

Agreements:

- In case of PUSCH repetitions, UL CI is applied to each repetition individually (actual repetition in case of Rel-16 PUSCH repetition) that overlaps with the resource (in time and frequency) indicated by UL CI.

Agreements:

  - The reference time region where a detected UL CI is applicable is determined by the following:

      - The reference time region starts from X symbols after the ending symbol of the PDCCH CORESET carrying the UL CI, where X is at least equal to the minimum processing time for UL cancelation

        - FFS X can be configured to be larger than the minimum processing time for UL cancelation

      - The duration of the reference time region is configured by RRC

        - FFS Possible values (e.g. 2OS, 4OS, 7OS, 14OS, 28OS?)

      - FFS DL symbols are excluded from the reference time region

Agreements:

    - The reference frequency region where a detected UL CI is applicable is configured by RRC

Agreements:

Support the following for UL CI

  - Each UL cancelation indicator per serving cell has a RRC configurable field size of  X bits

      - One value of X is 14

      - FFS other values (e.g. X can be N (N>0) times of 7)

  - The time domain granularity for the reference time region is configured by RRC

      - FFS the possible values  (e.g. the time region can be divided into [1],[2],[4],[7],[14],…portions)

      - FFS valid configurations according to the duration of the time reference region

  - The frequency domain granularity is determined based on the configured time domain granularity and the configured bit field size of each indicator

  - The time and frequency resource for cancellation is jointly indicated by a 2D-bitmap (i.e. similar as DL PI) over the time and frequency partitions within the reference region

      - FFS dynamic 2D-bitmap

Agreements:

- For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set

  - Introduce one new RRC parameter that contains one additional P0-PUSCH-Set per SRI

  - The one bit indication is present in the UL grant when the above new RRC parameter is configured

  - If present, the one bit in the DCI is used to switch between the P0 value from the existing P0-PUSCH-AlphaSet and the P0 value from the newly configured P0-PUSCH-Set

Conclusion:

No enhancement for CG-PUSCH power control in Rel-16 for inter-UE multiplexing

- RAN1#99

Agreements:

- There is no enhancement to PDCCH monitoring capability (number of BD and non-overlapping CCEs) specifically for UL CI monitoring purpose

Agreements:

- The maximum monitoring periodicity for UL CI is [5] slots

Agreements:

- Up to X BDs can be configured for UL CI

  - FFS per UL CI monitoring occasion or per span

  - The value of X is to be concluded during this week

  - Note: UE is not expected to be configured with search space configuration for UL CI with AL and number of candidates exceeding X BDs

Agreements:

- The maximum size for dci-PayloadSize-forCI is 126

Agreements:

- Possible values for RRC parameter timedurationforCI can be:

  - If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion

    - At least the same as the configured UL CI monitoring periodicity

      - FFS whether or not to additionally support multiple of UL CI monitoring periodicity

  - Otherwise (i.e., >1 monitoring occasion within 1 slot when 1-slot is the configured UL CI monitoring periodicity)

    - {2, 4, 7, [14]} OS, which SCS is used when determine the time duration

      - SCS for the DL BWP carrying UL CI

    - FFS The UE is not expected to be configured with a time duration for CI less than the time different (in symbols) between any adjacent monitoring occasions in a slot

Agreements:

  - Possible values (16 values) for RRC parameter CI-PayloadSize are

    - {[1],2,4,[5],7,8,[10],14,16,[20],[25],28,32,[35],56,112}

  - timeGranularityforCI is defined as number of partitions within the time region, and possible values are

    - {1,2,4,7,14,28}

  - The configured value of CI-PayloadSize shall be a multiple integer of the configured value of timeGranularityforCI

Agreements:

  - The frequency region for UL CI is derived by the following

    - A RIV indication configured by RRC within value range of (0..37949) (i.e. the same way as IE “locationAndBandwidth” for BWP configuration ), the configuration is per serving cell specific

      - The reference point is derived based on the RRC parameter offsetToCarrier (existing parameter, same way as BWP configuration)

    - A reference SCS (no RRC configuration) for a serving cell (to handle the case where a UE is configured with multiple BWPs using different SCSs on the serving cell),

      - Use the SCS for the DL BWP carrying UL CI as the reference SCS

Agreements:

- Support per serving cell configuration for the following parameters

- CI-PayloadSize

- timedurationforCI

- timeGranularityforCI

- frequencyRegionforCI

Agreements:

- If a serving cell is configured with SUL, each UL carrier (SUL and non-SUL) can be configured with different positionInDCI.

Agreements:

- The DL symbols indicated by tdd-UL-DL-ConfigurationCommon are excluded from the reference time region for UL CI

  - The partition of reference time region is done after excluding the DL symbols

  - The symbols used for SSB are also excluded

Agreements:

- Clarification of 2D-bitmap

  - 2D-bitmap is to use X bits for bitmap indication over a time/frequency region with M partitions in time and N partitions in frequency, and X=M x N

Agreements:

Regarding “FFS whether or not to additionally support multiple of UL CI monitoring periodicity”

- If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion, no additionally support that the time duration to be multiple of UL CI monitoring periodicity

Agreement

To determine the P0 value in case SRI is not configured in the DCI

- Option 1A: The open-loop power control parameter set indication field in the DCI can be configurable to be 1 or 2bits

  - P0-PUSCH-Set can provide up to two P0 values

    - UE uses the P0 values according to open loop power control indication field in DCI

    - UE use P0 from P0-PUSCH-AlphaSet when

      - open-loop power control parameter set indication field is 1bit and “0” is indicated, or

      - open-loop power control parameter set indication field is 2bits and “00” is indicated

  - Open-loop power control parameter set indication field can be separately configurable for DCI format 0_1 and DCI format 0_2

    - If open-loop power control parameter set indication field is not present for a DCI format, use P0 from P0-PUSCH-AlphaSet

  - A single configuration of P0-PUSCH-Set applies to both DCI format 0_1 and DCI format 0_2

- RAN1#100-e

Agreements:

- Confirm that 14OS can be configured for timedurationforCI (when 1-slot is the configured UL CI monitoring periodicity with more than one monitoring occasions within 1 slot)

- The possible values for CI-PayloadSize, are {1,2,4,5,7,8,10,14,16,20, 28,32,35,42,56,112}

The following TP is endorsed

------------------------------------ Start of TP for 38.213 --------------------------------------------

11.2A	Cancellation indication

< Unchanged parts are omitted >

For a group of symbols,  bits from each set of bits have a one-to-one mapping with  groups of PRBs where each of the first  groups includes  PRBs and each of the remaining  groups includes  PRBs. A UE determines a first PRB index as  and a number of contiguous RBs as  from frequencyRegionforCI that indicates an offset  and a length  as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB that indicates  for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

< Unchanged parts are omitted >

------------------------------------ End of TP for 38.213 --------------------------------------------

Agreements:

- The maximum UL CI monitoring periodicity is 10 slots.

- Up to X BDs can be configured per UL CI monitoring occasion, X to be decided between X=1 or X=2 in RAN1#100bis.

No RAN1 spec impact - RRC parameter update only.

Agreements:

- UE derives the RUR start based on “logical time” (i.e. assuming DL timing difference is 0 and TA=0) and the actual cancellation symbol based on “actual time” (i.e. assuming actual DL timing difference, actual TA)

- A new RRC parameter delta_offset d having possible values {0, 1, 2} OFDM symbols is introduced, update the spec as the following

For the serving cell, the UE determines the first symbol of the symbols to be the first symbol that is after + d from the end of a PDCCH reception where the UE detects the DCI format 2_4.corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming with being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell.

- Clarify the following by a RAN1 spec update (see below)

  - UE is not expected to cancel the transmission of SRS or PUSCH before the first symbol that is T_proc,2 after the end of the reception of the last symbol of the PDCCH carrying the ULCI including the effect of the timing advance.

38.213 Text proposal (maybe further refined by spec editor)

An indication by a DCI format 2_4 for a serving cell is applicable to PUSCH or SRS transmissions on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer parameter [xxxx]. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. UE is not expected to cancel the transmission of SRS or PUSCH before the first symbol that is Tproc,2 after the end of the reception of the last symbol of the PDCCH carrying the ULCI including the effect of the timing advance.

Agreements:

- UE performs the UL cancellation based on any detected UL CI, no additional specification for the case of overlapping reference time region for multiple UL CI occasions.

- A cancelled PUSCH transmission by a UE is counted towards the number of PUSCH that a UE can support per slot

Conclusion:

- It is possible for a UE to indicate both  pa-PhaseDiscontinuityImpacts  (i.e. 6-23) and the support of UL CI for intra-band UL CA

- For a UE indicates a capability to cancel overlapping PUSCHs on different intra-band serving cells (if any), and the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE cancels the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to.

- RAN1#100bis-e

Agreements:

  UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions is not affected by UL CI.

Agreeement:

- If both UL CI and intra-UE priority indicator are configured for a given UE, support a new RRC parameter to configure Behavior #1

  - Behaviour #1: UL CI is only applicable to the UL transmissions indicated/configured as low priority level

- When the RRC parameter is not provided to the UE, behaviour #2 is used

  - Behaviour #2: UL CI is applicable to UL transmission irrespective of its priority level

- Note: the RRC signaling details will be decided by RAN2

Agreement:

- Up to X BDs can be configured per UL CI monitoring occasion

  - For ULCI monitoring occasion determination, search space sets start at a same OFDM symbol correspond to a same monitoring occasion

  - X=1

Agreement:

- UE uses the smallest SCS configuration between the SCS configurations of the PDCCH for DCI format 2_4 detection and the SCS configurations in scs-SpecificCarrierList of UL carrier to determine the RUR starting symbol.

- UE uses the smallest SCS configurations in scs-SpecificCarrierList of UL carrier to determine offset d.

- Adopt the TP below for 38.213 section 11.2A

TP for 38.213 section 11.2A

11.2A Cancellation indication ---------------------------Other parts are omitted ------------------------------- An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the symbols to be the first symbol that is after from the end of a PDCCH reception where the UE detects the DCI format 2_4, where is provided by XXX with the smallest SCS configuration between the SCS configurations of the PDCCH and the SCS configurations provided in scs-SpecificCarrierList of UL carrier. corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming with being the smallest SCS configuration between the SCS configurations of the PDCCH and the SCS configurations provided in scs-SpecificCarrierList of UL carrier of a PUSCH transmission or of an SRS transmission on the serving cell. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is after a last symbol of a CORESET where the UE detects the DCI format 2_4. <---------------------------Other parts are omitted ------------------------------->

Agreements:

To adopt the following TP for 38.213

- 11.2A Cancellation indication =====omitted text ====== For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated bytdd-UL-DL-ConfigurationCommon, from the time duration provided by timeDurationforCI in timeFrequencyRegion if the configured UL CI monitoring periodicity is 1 slot with more than one monitoring occasions. Otherwise, the time duration is equal to the PDCCH monitoring periodicity provided by the value of monitoringSlotPeriodicityAndOffset, as described in Clause 10.1. - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion =====omitted text ======

Agreement: Adopt the following text proposal for TS38.213 section 11.2A

----------------------------- Text proposal starts for TS 38.213, v16.1.0, Section 11.2A ----------- A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission, or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, or an SRS transmission on the serving cell if, respectively, - a group of symbols, from the TCI symbols, has a corresponding at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and - a group of PRBs, from the BCI PRBs, has a corresponding bit value of '1' in the set of bits corresponding to the group of symbols in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission, where - the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that are is in one or more a groups of symbols having corresponding bit values of '1' in the DCI format 2_4; - the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of '1' in the DCI format 2_4. ----------------------------- Text proposal ends for TS 38.213, v16.1.0, Section 11.2A -------------

Agreement

•          When UE is configured with both DCI format 0_1 and 0_2 with SRI presents in only one of the DCI formats, then for the DCI format without SRI field

  For 1 bit OLPC parameter indication, if OLPC parameter set indication in DCI is set to ‘1’

  P0-PUSCH-Set having the lowest p0-PUSCH-SetId is used.

  For 2 bit OLPC parameter indication, if OLPC parameter set indication in DCI is set to ‘01’ or ‘10’

  P0-PUSCH-Set having the lowest p0-PUSCH-SetId is used.

Agreement: Adopt the following text proposal for TS38.213 section 7.1.1

TP for 38.213 16.1.0 Section 7.1.1 <Unchanged text is omitted> - If the PUSCH transmission is scheduled by a DCI format that does not include a SRI field, or if SRI-PUSCHPowerControl is not provided to the UE, , - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of from - a first P0-PUSCH-AlphaSet in p0-AlphaSets if a value of the open-loop power control parameter set indication field is '0' or '00' - a first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetId value if a value of the open-loop power control parameter set indication field is '1' or '01' - a second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetId value if a value of the open-loop power control parameter set indication field is '10' - else, the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets <Unchanged text is omitted>

Agreement: Adopt the following text proposal for 38.213 section 11.2A

- 11.2A Cancellation indication If a UE is provided UplinkCancellation, the UE is provided a CI-RNTI by ci-RNTI for monitoring PDCCH candidates for a DCI format 2_4 [5, TS 38.212]. UplinkCancellation additionally provides to the UE - a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI - a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier for a SUL carrier, if the serving cell is configured with a SUL carrier for SUL of a serving cell if the serving cell configured with SUL - an information payload size for DCI format 2_4 by dci-PayloadSize-forCI - an indication for time-frequency resources by timeFrequencyRegion For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon,provided by timeDurationforCI in timeFrequencyRegion - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion GCI sets of bits from the MSB of the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. For a group of symbols, NBI=NCIGCI bits from MSB of each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB BCI=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by XXX higher layer parameter delta_offset_d. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 after a last symbol of a CORESET where the UE detects the DCI format 2_4. A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission, or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, or an SRS transmission on the serving cell if, respectively, - a group of symbols, from the TCI symbols, has a corresponding bit value of ‘1’ in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and - a group of PRBs, from the BCI PRBs, has a corresponding bit value of ‘1’ in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission, where - the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that are in one or more groups of symbols having corresponding bit values of ‘1’ in the DCI format 2_4; - the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of ‘1’ in the DCI format 2_4.

Agreement: Adopt the following text proposal for 38.212 section 7.3.1.3.5

- 7.3.1.3.5 Format 2_4 DCI format 2_4 is used for notifying the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE according to Clause 11.2A5 of [5, TS 38.213]. The following information is transmitted by means of the DCI format 2_4 with CRC scrambled by CI-RNTI: - Cancellation indication 1, Cancellation indication 2, …, Cancellation indication indication N. The size of DCI format 2_4 is configurable by higher layers parameter dci-PayloadSize-forCI up to 126 bits, according to Clause 11.2A5 of [5, TS 38.213]. The number of bits for each cancellation indication is configurable by higher layer parameter CI-PayloadSize. For a UE, there is at most one cancellation indication for an UL carrier. <Unchanged text is omitted>

- TR 38.824

- 7.2 Potential enhancements In the following sub-sections, potential enhancements for UL inter UE Tx prioritization/multiplexing are presented. It is recommended to specify both UL cancelation scheme and enhanced UL power control scheme in the work item phase. - 7.2.1 UE UL cancelation mechanisms UE UL cancelation mechanisms are considered as one potential enhancement for UL inter-UE Tx prioritization/multiplexing and are studied from several aspects, including the potential mechanisms (e.g. UE UL cancelation/pausing indication, UL continuation indication, UL re-scheduling indication), physical channel/signal used for the UL cancelation indication, UE processing timeline for the UL cancelation indication, UE monitoring behaviours for the UL cancelation indication, UE PDCCH monitoring capability if the UL cancelation indication is by PDCCH, methods to ensure the reliability of the indication for UE UL cancelation. Either PDCCH or sequence can be considered as potential options for the UL cancelation indication. If PDCCH is used, either group common DCI or UE-specific DCI can be considered as potential options. If sequence is used, either group common sequence or UE-specific sequence can be considered. The monitoring periodicity for the UL cancelation indication should be configurable by the gNB and UE supporting UL cancelation indication should be able to support more than one monitoring occasions for the UL cancelation indication in a slot. If PDCCH is used, whether the UE PDCCH monitoring capability (number of CCEs/BDs per slot) should be increased is to be further investigated. The UE processing time for UL cancelation indication should be equal or shorter than N2 defined in Rel-15 UE capability#2. Upon detecting an UL cancelation indication, UE cancels the corresponding UL transmission. The corresponding UL transmission may include an on-going UL transmission, or an UL transmission that has not been started. After cancelation, the UE may resume the transmission afterwards as one option, or may not resume the transmission afterwards as another option. - 7.2.2 Enhanced UL power control Enhanced UL power control is considered as one potential enhancement for UL inter-UE Tx prioritization/multiplexing and the study mainly focuses on enhanced dynamic power boost for URLLC UE, including dynamic change of power control parameters (e.g. P0 and alpha without SRI configured) and enhanced TPC (e.g. increased TPC range and finer granularity). The need of URLLC UE power change during one transmission instance is not envisioned. It is assumed that there is no change of eMBB UE power control scheme in this study item. Enhanced dynamic power boost for URLLC UE are studied from several aspects, including feasibility of boosting UE power in power limited or interference limited scenarios, physical channel/signal used for the signalling, UE processing timeline for the signalling, UE monitoring behaviours for the signalling, UE PDCCH monitoring capability if the signalling is by PDCCH and methods to ensure the reliability of the signalling. It is concluded that the potential enhanced UL power control may include UE determining the power control parameter set (e.g. P0, alpha) based on scheduling DCI indication without using SRI, or based on group-common DCI indication. Increased TPC range compared to Rel-15 may also be considered. Power boosting is not applicable to power limited UEs.

List of contributions and proposals

| 1 | R1-2003321 | Remaining issues on UL inter-UE multiplexing between eMBB and URLLC | ZTE |
| --- | --- | --- | --- |
| 2 | R1-2003391 | UL inter UE Tx prioritization for URLLC | vivo |
| 3 | R1-2003394 | Summary of UL inter UE Tx prioritization | Moderator (vivo) |
| 4 | R1-2003443 | Remaining Issue of Inter-UE Prioritization and Multiplexing of UL Transmissions | Ericsson |
| 5 | R1-2003527 | Corrections on UL inter-UE multiplexing | Huawei, HiSilicon |
| 6 | R1-2003581 | Maintenance of Rel-16 URLLC Enhanced inter UE Tx prioritization/multiplexing | Nokia, Nokia Shanghai Bell |
| 7 | R1-2003623 | Remaining issues on inter-UE UL multiplexing | CATT |
| 8 | R1-2003686 | Remaining issues on enhanced inter-UE Tx prioritization/multiplexing | MediaTek Inc. |
| 9 | R1-2003709 | Remaining Issues on Enhanced Inter-UE Tx Prioritisaion / Multiplexing | NEC |
| 10 | R1-2003740 | Remaining issues on enhanced inter-UE multiplexing | Intel Corporation |
| 11 | R1-2003868 | Remaining issues for inter-UE multiplexing | Samsung |
| 12 | R1-2003981 | Remaining issues of enhanced inter UE Tx prioritization/multiplexing | Spreadtrum Communications |
| 13 | R1-2004032 | Remaining issues of UL inter UE Tx prioritization | LG Electronics |
| 14 | R1-2004118 | Inter UE Tx prioritization and multiplexing | OPPO |
| 15 | R1-2004185 | Remaining issues on Inter-UE Multiplexing for eURLLC | Sony |
| 16 | R1-2004225 | Remaining Issues on Inter-UE Cancellation for eURLLC | Apple |
| 17 | R1-2004272 | Inter-UE prioritization/multiplexing | InterDigital, Inc. |
| 18 | R1-2004371 | Remaining issues of enhanced inter UE Tx prioritization/multiplexing | Motorola Mobility, Lenovo |
| 19 | R1-2004393 | Remaining issue on inter-UE Tx multiplexing/prioritization | NTT DOCOMO, INC |
| 20 | R1-2004460 | Remaining issues on uplink Inter-UE Tx Multiplexing and Prioritization | Qualcomm Incorporated |
| 21 | R1-2004525 | Remaining issues on inter-UE multiplexing for NR URLLC | WILUS Inc. |
| 22 | R1-2004390 | Remaining issues for UCI enhancement for Rel-16 URLLC | NTT DOCOMO, INC |