# 3GPP TSG RAN WG1 #101                                                                               R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Source:	Email discussion #01 for enhanced configured grant transmission

Title:	Moderator (vivo)

Agenda Item:	7.2.5.6

Document for: 	Discussion and Decision

- Introduction

Based on the phase 1 discussions and suggestions, Chairman allocates following email discussion for eCG for URLLC. It is noted that the deadline for agreements/conclusions is 5/29, in order to have some time to make the acceptable proposals based on companies’ input, please share your views by 12:00 noon time of UTC on 5/27.

[101-e-NR-L1enh-URLLC-eCG-01] Possible RAN1 conclusion on per PUSCH repetition cancellation and CG-CG/DG with different priorities by 5/29 – Lihui (vivo)

- Issue#1: Discuss and draw RAN1 conclusion on per PUSCH repetition cancellation. The following proposal for conclusion is to be used as a starting point for discussions but can be revised further

  - In Rel.15, for CG PUSCH configured with repetition factor K>1, in case there is collision between DG PUSCH and CG PUSCH, the timeline is defined by the starting symbol of a CG-PUSCH repetition that overlaps with the DG PUSCH within a bundle (i.e., DG-PUSCH overrides CG-PUSCH is per repetition).

    - If the HARQ process is the same between CG and DG, UE terminates all remaining repetitions.

    - Otherwise, only overlapped repetitions are terminated.

  - In Rel.15, for DG and CG with the same HARQ process and without resource collision, DG overrides CG under the timeline defined in TS 38.214 section 6.1.

- Issue#2: Discussion on CG-CG/DG with different priorities

- Discussion on per PUSCH repetition cancellation

Regarding issue 3.3 in R1-2003395, based on the comments received, following was proposed to be captured in chairman notes.

Conclusion

In Rel.15, for a DG PUSCH overriding a CG PUSCH configured with repetition factor K>1,

- If the HARQ process is the same between the DG and the CG, DG overrides all remaining repetition occasions after the end of PDCCH reception, under the timeline specified in TS 38.214 section 6.1.

- Otherwise, DG overrides only the CG repetition overlapped with DG, under the timeline specified in TS 38.214 section 6.1.

Any comments?

| Company | View |
| --- | --- |

- Discussion on CG-CG/DG collisions with different priorities

Below are the current PHY layer specifications regarding to the collision handling for consideration.

## TS 38.213 section 9: A PUSCH or a PUCCH, including repetitions if any, can be of priority index 0 or of priority index 1. If a priority index is not provided for a PUSCH or a PUCCH, the priority index is 0. If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. If, after resolving overlapping for PUCCH and/or PUSCH transmissions of a same priority index, a UE determines to transmit - a first PUCCH of larger priority index, a PUSCH or a second PUCCH of smaller priority index, and a transmission of the first PUCCH would overlap in time with a transmission of the PUSCH or the second PUCCH, the UE does not transmit the PUSCH or the second PUCCH - a PUSCH of larger priority index, a PUCCH of smaller priority index, and a transmission of the PUSCH would overlap in time with a transmission of the PUCCH, the UE does not transmit the PUCCH - a first PUSCH of larger priority index on a serving cell, a second PUSCH of smaller priority index on the serving cell, and a transmission of the first PUSCH would overlap in time with a transmission of the second PUSCH, the UE does not transmit the second PUSCH, where at least one of the two PUSCH is not scheduled by a DCI format [Irrelevant text is omitted]

Furthermore, the details of when the UE may interrupt an ongoing transmission PHY-layer prioritization is captured in TS 38.214 in the following way:

## 38.214 section 6.1 UE procedure for transmitting the physical uplink shared channel [Irrelevant text is omitted] [If [a UE reports the capability of intra-UE prioritization], and if a PUSCH corresponding to a configured grant and a PUSCH scheduled by a PDCCH on a serving cell are partially or fully overlapping in time, - If the PUSCH corresponding to the configured grant has priority in configuredGrantConfig set to 1 (i.e., high priority), and the PUSCH scheduled by the PDCCH is indicated as low priority by having the [priority indicator] field in the scheduling DCI set to 0 or by not having the [priority indicator] field present in the scheduling DCI, the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the PUSCH transmission scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant. - Otherwise, the UE shall cancel the PUSCH transmission corresponding to the configured grant at latest starting M symbols after the end of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, and transmit the PUSCH scheduled by the PDCCH, where - M = Tproc,2 +d1, where Tproc,2 is given by clause 6.4 for the corresponding PUSCH timing capability assuming d2,1 = 0 and d1 is determined by the reported UE capability [XXXXX], - In this case, the UE is not expected to be scheduled for the PUSCH by the PDCCH where the PUSCH starts earlier than N symbols after the end of the last symbol of the PDCCH, where - N = Tproc,2 + d2, where Tproc,2 is the PUSCH preparation time of the PUSCH scheduled by the PDCCH using the associated PUSCH timing capability according to clause 6.4 and d2 is determined by the reported UE capability [YYYYY]. - In case of PUSCH repetitions, the overlapping handling is performed for each PUSCH repetition separately. - The UE is not expected to be scheduled for another PUSCH by a PDCCH where this PUSCH starts no earlier than the end of the prioritized transmitted PUSCH and before the end of the time domain allocation of the cancelled PUSCH.] [Irrelevant text is omitted]

In the following, the 1st CG vs. 2nd CG, 1st CG vs. 2nd DG and 1st DG vs. 2nd CG will be discussed assuming that the MAC PDU corresponding to the second grant delivered from MAC to PHY is later than the MAC PDU corresponding to the first grant and the second grant is with higher priority, in other words, the latest agreed MAC spec R2-2004289 (see appendix) allows MAC to generate two MAC PDUs with resource overlapping where the 1st MAC PDU is with low priority and 2nd MAC PDU is with high priority. (It is reasonable that if the 1st grant is with higher priority and delivered from the MAC to PHY, then MAC shall not generate the 2nd grant with lower priority). Note that this document, the high and low priority is in terms of PHY layer priority.

In addition, if your answers are different depending on whether the 1st grant has already started transmission or not, please provide your views separately and clearly for them.

- Case 1: 1st DG vs. 2nd CG

  - Q1-1: whether to support the collision case that the 2nd CG with higher priority cancels the 1st DG with lower priority?

    - Option 1: Yes

    - Option 2: No

| Company | View |
| --- | --- |

  - Q1-2: if your answer to Q1-1 is Yes, what is the expected physical layer handling behavior?

    - e.g. as specified in 38.214 that “the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the PUSCH transmission scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant”?

| Company | View |
| --- | --- |

  - Q1-3: if your answer to Q1-1 is No, what is the expected physical layer handling behavior if MAC layer delivers the CG with higher priority?

| Company | View |
| --- | --- |

  - Q1-4: is there any discrepancy foreseen between PHY and MAC for option 1 and/or option 2, and how to resolve it?

| Company | View |
| --- | --- |

[Summary and proposals: to be added based on companies’ views]

- Case 2: 1st CG vs. 2nd DG

  - Q2-1: whether to support the collision case that the 2nd DG with higher priority cancels the 1st CG with lower priority?

    - Option 1: Yes

    - Option 2: No

| Company | View |
| --- | --- |

  - Q2-2: if your answer to Q2-1 is Yes, what is the expected physical layer handling behavior?

| Company | View |
| --- | --- |

  - Q2-3: if your answer to Q2-1 is No, what is the expected physical layer handling behavior if MAC layer delivers the DG with higher priority?

| Company | View |
| --- | --- |

  - Q2-4: is there any discrepancy foreseen between PHY and MAC for option 1 and/or option 2, and how to resolve it?

| Company | View |
| --- | --- |

[Summary and proposals: to be added based on companies’ views]

- Case 3: 1st CG vs. 2ndCG

  - Q3-1: whether to support the collision case that the 2nd CG with higher priority cancels the 1st CG with lower priority?

    - Option 1: Yes

    - Option 2: No

| Company | View |
| --- | --- |

  - Q3-2: if your answer to Q3-1 is Yes, what is the expected physical layer handling behavior?

| Company | View |
| --- | --- |

  - Q3-3: if your answer to Q3-1 is No, what is the expected physical layer handling behavior if MAC layer delivers the DG with higher priority?

| Company | View |
| --- | --- |

  - Q3-4: is there any discrepancy foreseen between PHY and MAC for option 1 and/or option 2, and how to resolve it?

| Company | View |
| --- | --- |

[Summary and proposals: to be added based on companies’ views]

In addition, there is one more issue that deserves discussion. It is noted that in RAN2 #109bis e-meeting, following agreements were made for URLLC/IIoT:

R2 assumes that PHY-based prioritization and LCH-based prioritization are configured independently and one can be configured without the other (assumption may be modified when LS reply from R1 is received)

Based on above agreement, it is implied that RAN2 assumes MAC prioritization can be configured regardless the PHY priority. Companies are encouraged to provide your views on above RAN2 assumptions.

- Q4: what is your views about the relations between the PHY-based prioritization and LCH-based prioritization?

  - Option 1: they can be configured independently;

  - Option 2: they should be configured jointly;

  - Option 3: PHY-based prioritization should be the prerequisite UE feature for the LCH-based prioritization.

  - Option 4: LCH-based prioritization should be the prerequisite UE feature for PHY-based prioritization.

  - Other options?

| Company | View |
| --- | --- |

[Summary and proposals: to be added based on companies’ views]

- Appendix

## 5.4	UL-SCH data transfer

### 5.4.1	UL Grant reception

[Irrelevant text is omitted]

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels with data available that are multiplexed or can be multiplexed in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU.

When the MAC entity is configured, with lch-basedPrioritization, for each uplink grant which is not already a de-prioritized uplink grant, the MAC entity shall:

1>	if this uplink grant is addressed to CS-RNTI with NDI = 1 or C-RNTI:

2>	if there is no overlapping PUSCH duration of a configured uplink grant which was not already de-prioritized, in the same BWP whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission where the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s).

1>	else if this uplink grant is a configured uplink grant:

2>	if there is no overlapping PUSCH duration of another configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUSCH duration of an uplink grant addressed to CS-RNTI with NDI = 1 or C-RNTI which was not already de-prioritized, in the same BWP, whose priority is higher than or equal to the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission where the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s).

NOTE 6:	If there is overlapping PUSCH duration of at least two configured uplink grants whose priorities are equal, the prioritized uplink grant is determined by UE implementation.

### 5.4.2	HARQ operation

#### 5.4.2.1	HARQ Entity

[Irrelevant text is omitted]

For each uplink grant, the HARQ entity shall:

1>	identify the HARQ process associated with this grant, and for each identified HARQ process:

2>	if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or

2>	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or

2>	if the uplink grant was received in a Random Access Response (i.e. in a MAC RAR or a fallback RAR); or

2>	if the uplink grant was determined as specified in clause 5.1.2a for the transmission of the MSGA payload; or

2>	if the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery; or

2>	if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle:

3>	if there is a MAC PDU in the MSGA buffer and the uplink grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload was selected:

4>	obtain the MAC PDU to transmit from the MsgA buffer.

3>	else if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a fallbackRAR:

4>	obtain the MAC PDU to transmit from the Msg3 buffer.

3>	else if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a MAC RAR; or:

3>	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery:

4>	obtain the MAC PDU to transmit from the Msg3 buffer.

4>	if the uplink grant size does not match with size of the obtained MAC PDU; and

4>	if the Random Access procedure was successfully completed upon receiving the uplink grant:

5>	indicate to the Multiplexing and assembly entity to include MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in the subsequent uplink transmission;

5>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity.

3>	else if this uplink grant is a configured grant configured with autonomousTx; and

3>	if the previous configured uplink grant for this HARQ process was de-prioritized; and

3>	if a MAC PDU had already been obtained for this HARQ process; and

3>	if the uplink grant size matches with size of the obtained MAC PDU; and

3>	if a transmission of the obtained MAC PDU has not been performed:

4>	consider the MAC PDU has been obtained.

3>	else if the MAC entity is not configured with lch-basedPrioritization; or

3>	if this uplink grant is a prioritized uplink grant:

4>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

3>	if a MAC PDU to transmit has been obtained:

4>	if the uplink grant is not a configured grant configured with autonomousTx; or

4>	if the uplink grant is a prioritized uplink grant:

5>	deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process;

5>	instruct the identified HARQ process to trigger a new transmission;

5>	if the uplink grant is a configured uplink grant:

6>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed;

6>	start or restart the cg-RetransmissionTimer, if configured, for the corresponding HARQ process when the transmission is performed.

5>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

6>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed.

5>	if cg-RetransmissionTimer is configured for the identified HARQ process:

6>	if the transmission is performed:

7>	consider the identified HARQ process as not pending.

6>	else:

7>	consider the identified HARQ process as pending.

3>	else:

4>	flush the HARQ buffer of the identified HARQ process.

2>	else (i.e. retransmission):

3>	if the uplink grant received on PDCCH was addressed to CS-RNTI and if the HARQ buffer of the identified process is empty; or

3>	if the uplink grant is part of a bundle and if no MAC PDU has been obtained for this bundle; or

3>	if the uplink grant is part of a bundle of the configured uplink grant, and the PUSCH duration of the uplink grant overlaps with a PUSCH duration of another uplink grant received on the PDCCH or an uplink grant received in a Random Access Response (i.e. MAC RAR or fallbackRAR) or an uplink grant determined as specified in clause 5.1.2a for MSGA payload for this Serving Cell; or:

3>	if the MAC entity is configured with lch-basedPrioritization and this uplink grant is not a prioritized uplink grant:

4>	ignore the uplink grant.

3>	else:

4>	deliver the uplink grant and the HARQ information (redundancy version) of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a retransmission;

4>	if the uplink grant is addressed to CS-RNTI; or

4>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

5>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed.

4>	if the uplink grant is a configured uplink grant:

5>	if the identified HARQ process is pending:

6>	start or restart the configuredGrantTimer for the corresponding HARQ process when the transmission is performed;

5>	start or restart the cg-RetransmissionTimer, if configured, for the corresponding HARQ process when the transmission is performed.

4>	if the identified HARQ process is pending and the transmission is performed:

5>	consider the identified HARQ process as not pending.

When determining if NDI has been toggled compared to the value in the previous transmission the MAC entity shall ignore NDI received in all uplink grants on PDCCH for its Temporary C-RNTI.