Rel.17 FeMIMO EVM Offline Discussion

Phase 1 - High Level

Please share your view on the questions below in the provided tables.

- Q1: If the respective item (cf. WID) requires some discussion on evaluation methodology in addition to what we already agreed in Rel.16 eMIMO

- Q2: If the answer to Q1 is yes, potential aspects of evaluation methodology (initial and high-level assessment, i.e. tentative)

Table 1 Item 1 – Multi-beam

| Company | Q1 (Y/N) | Q2 |
| --- | --- | --- |
| Samsung | Yes | Item 1a specific: - SLS is the primary tool for evaluation at least for intra-cell mobility (typical BM). - Inter-cell mobility may consider other (simpler) metrics such as dropped-call rate - SLS layout to evaluate high-mobility scenarios @FR2 (e.g. highway, intra/inter-cell, HST cf. 2d): Reuse 38.913 and 38.802 as much as possible. - Note: For HST, use the same model as item 2d Item 1b specific: - SLS is the primary tool for evaluation for evaluating MPE issue - MPE-specific assumptions need to be discussed: 1) panel blocking model, 2) EIRP Common (1a and 1b): - UE multi-panel assumptions: - 3-panel UE as a baseline should be evaluated (left, right, top), each panel is 1x4x2 - Beam direction per panel needs to be aligned or at least elaborated |

Table 2 Item 2a – mTRP PDCCH/PUSCH/PUCCH

| Company | Q1 (Y/N) | Q2 |
| --- | --- | --- |
| Samsung | Yes | PDCCH: LLS as primary tool for evaluation - Scenario and channel model: reuse 38.913 as much as possible. (@FR2, focus on IIoT scenarios with a certain blockage probability) - PDCCH configurations: AL, interleaving, DCI format/size, # of symbols - Repetition method, e.g., TDM as a starting point - Decoding assumptions, e.g., whether to allow soft combining - Reliability target for PDCCH PUCCH and PUSCH: LLS as primary tool for evaluation - Scenario and channel model: reuse 38.913 as much as possible. (@FR2, focus on IIoT scenarios with a certain blockage probability) - Resource configuration, e.g., targeted PUCCH format, # of RBs/symbols - Repetition method, e.g., TDM only (intra/inter-slot, same/different resource configuration) - Decoding assumptions, e.g., whether to allow soft combining for PUCCH - Tx power control, e.g., per TRP - Reliability target for PUCCH, PUSCH |

Table 3 Item 2b – inter-cell mTRP

| Company | Q1 (Y/N) | Q2 |
| --- | --- | --- |
| Samsung | No | -- |

Table 4 Item 2c – mTRP beam management

| Company | Q1 (Y/N) | Q2 |
| --- | --- | --- |
| Samsung | Yes | LLS as primary tool for evaluation We think the following assumptions, which had been agreed in Rel-16 can be reused: - Channel model (CDL-A) - BS antenna configuration, i.e., (M, N, P, Mg, Ng) = {(4,8,2,1,1), (4,8,2,2,2)} The followings should be further discussed in Rel-17 - UE antenna configuration e.g., for UE with 2-panels (M, N, P, Mg, Ng) = (2,4,2,1,2) and Ω0,1=Ω0,0+180° FFS, other practical UE implementation with >2 panels (e.g. from item 3) - Pre-determined SNR offset across each TRP-UE channel |

Table 5 Item 2d – HST

| Company | Q1 (Y/N) | Q2 |
| --- | --- | --- |
| Samsung | Yes | LLS as primary tool for evaluation - Deployment scenarios: for both FR1 and FR2, reuse 38.913 as much as possible - HST-SFN channel models: reuse the HST-SFN channel model agreed in RAN4 as much as possible - Time-varying Doppler frequency modeling, e.g., as defined in RAN4 |

Table 6 Item 3 – SRS

| Company | Q1 (Y/N) | Q2 |
| --- | --- | --- |
| Samsung | Yes | LLS as primary tool for evaluation - UL channel estimation and error modeling + UL Tx power - SRS configuration (UL carrier frequency or UL-DL duplex distance, SRS BW, #symbols, comb, time-bundling and antenna switching) - Practical UE implementation on antenna switching - Omni-directional UE antennas for FR1 - Directional UE antennas for FR2, FFS angle between panels for UEs with >2 panels - Performance metric: BLER of PDSCH and/or PUSCH, MSE of channel |

Table 7 Item 4 – CSI

| Company | Q1 (Y/N) | Q2 |
| --- | --- | --- |
| Samsung | Yes | Item 4a: SLS as primary tool for evaluation - Deployment scenarios - Rel-16 MTRP assumption as baseline - Large # of TP antennas for both UMa and InH, e.g., Up to 256 antenna elements assumed in 38.913 - Focus on ideal backhaul assumption - CSI report configuration and overhead: # of reports, periodicity, feedback delay - IMR assumptions for NC-JT and inter-cell interference calculation, e.g., IMR for inter-cell interference only Item 4b: modeling realistic system aspects such as (at least some of) the followings - UL channel estimation and error modeling + UL Tx power (e.g. based on UL PC) - SRS configuration (UL carrier frequency or UL-DL duplex distance, SRS BW, #symbols, comb, time-bundling and antenna switching) - Frequency offset modeling (e.g. according to the FDD reciprocity model in 36.897 - PAPR issue modeling for >=8 CSI-RS ports - UL/DL reciprocity errors (due to different Tx-Rx RF circuitry different UL-DL interference profile) |