3GPP TSG RAN WG1 #101			R1-200nnn

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.6.5

Source: 	Moderator (Ericsson)

Title: 	TP for eMIMO lowPAPR thread #1

Document for:	Discussion and Decision

- Analysis

| # Reasons for change | # In 38.214, there is a missing case for determining the nIDRS: “the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is not used for PUSCH”. |
| --- | --- |
| # Summary of changes | # Introduce the missing case in specifications |
| # Specs/Sections impacted | # TS 38.211 V16.1.0, section 6.4.1.1.1.2 |
| # Consequences if not approved | # The spec is either ambiguous or erroneous |

- Text proposal

TP for TS 38.211 V16.1.0

< Start TP for TS 38.211 V16.1.0> =========================================================================== 6.4.1.1.1.2 Sequence generation when transform precoding is enabled <unchanged text omitted> The sequence group u=fgh+nIDRS mod 30, where nIDRS is given by - nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE and, - the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is not configured or the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is not used for PUSCH, and - the PUSCH is neither scheduled by RAR UL grant nor scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI according to clause 8.3 in [5, TS 38.213]; - nIDRS=NIDnSCID if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space; - nIDRS=NIDcell otherwise <unchanged text omitted> =========================================================================== < End TP for TS 38.211 V16.1.0>