3GPP TSG RAN WG1 #101			R1-200nnn

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.6.5

Source: 	Moderator (Ericsson)

Title: 	TP for eMIMO lowPAPR thread #2

Document for:	Discussion and Decision

- Analysis

| # Reasons for change | # A sentence that the low-PAPR sequence of type 2 changes for different values of α and δ in Section 5.2.3 is incorrect and should be removed/replaced. |
| --- | --- |
| # Summary of changes | # Remove the incorrect sentence and introduce a clarification on fixed alpha and delta values. |
| # Specs/Sections impacted | # TS 38.211 V16.1.0, section 5.2.3 |
| # Consequences if not approved | # The spec is erroneous as multiple sequences are not defined for type 2 |

- Text proposal

TP for TS 38.211 V16.1.0

< Start TP for TS 38.211 V16.1.0> =========================================================================== ### 5.2.3 Low-PAPR sequence generation type 2 The low-PAPR sequence ru,v(α,δ)n is defined by a base sequence ru,vn according to ru,v(α,δ)n=ru,vn, 0≤n<M where M=mNscRB2δ is the length of the sequence, α=0, δ=0 . Multiple sequences are defined from a single base sequence through different values of α and δ. Base sequences ru,vn are divided into groups, where u∈0,1,…,29 is the group number and v is the base sequence number within the group, such that each group contains one base sequence (v=0) of length M=mNscRB2δ, 12≤m2δ. The sequence ru,v0,…,ru,vM-1 is defined by ru,vn=1Mi=0M-1ru,vie-j2πinM n=0,…,M-1 where the definition of ru,vi depends on the sequence length. =========================================================================== < End TP for TS 38.211 V16.1.0>