Title:	Draft reply LS on RAN2 DCP Open Issues				R1-200xxxx

Response to:	R1-2003260 (R2-2004175)

Release:	Rel-16

Study Item:	NR_UE_pow_sav-Core

Source:	CATT (RAN1)

To:	RAN2

CC: 	RAN4

Contact Person:

#### Name:	Fang-Chen Cheng

E-mail Address:	fcc@catt.cn

1. Overall Description:

RAN1 would like to thank RAN2 on DCP open issues.   RAN1 discussed the issue on the collision of DCP and RAR when a DCP monitoring occasion overlaps with the ra-ResponseWindow or msgB-ResponseWindow.   Regarding RAN2’s understanding on RAR impact with the collision of DCP and RAR, RAN1’s view is:

- RAR addressed to C-RNTI would not be impacted if the DCI scheduling RAR addressed to C-RNTI is configured on type-3 CSS with lower index than that of DCP, according to current TS 38.213;

- RAR addressed to C-RNTI would not be prioritized over DCP if it is configured on USS or on type-3 CSS with higher index than that of DCP;

- A continuous collision during the RAR window is not expected between the monitoring occasions of DCP and RAR address to C-RNTI considering the DCP monitoring duration is expected to be much shorter than a RAR window for power saving.

RAN1 also discussed whether there is any need for a new prioritization rule in the specification for RAR with CRC addressed to C-RNTI.    However, RAN1 could not reach an agreement on whether specification change is needed to prioritize RAR addressed to C-RNTI configured on USS (or on type-3 CSS with higher index than that of DCP) over DCP on Type-3 CSS.

RAN1 also agree to have the CR to align the updated parameter ps-TransmitOtherPeriodicCSI in RAN1 specification.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take into account consideration of RAN1’s agreements.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #102   	24th - 28th  August, 2020     e-Meeting

TSG-RAN WG1 Meeting #103    	12th -16th October, 2020     China