3GPP TSG RAN WG1 Meeting #101-e	R1- 200xxxx

e-Meeting, 25th May – 5th June 2020

Title:	Summary of PDCCH based Power Saving Signal/Channel

Agenda item:	7.2.7.1

Source:	CATT

Document for:	Discussion

Email Discussions

Preparation

Issue 1:  Confirm Working Assumption of minimum time gap values (section 3.1)

Issue 2:  RAR is prioritize over DCP during RAR monitoring window (section 3.2)

Issue 3: 	Specification alignment and text proposals (Section 3.3)

Issue 4:  DCI size budget for DCI format 2_6 (Section 3.4)

| Company | Supporting Issues | Comments |
| --- | --- | --- |
| Samsung | Issue 1: OK to discuss. Support confirming WA Issue 2: OK to discuss. Do not support the proposal from FL. Issue 3: OK to discuss. | Issue 4: No need to discuss. No need for any proposal. The arguments in Section 3.4 are incorrect due to the following: - There is no ambiguity on how to account for the total budget of DCI format sizes. UE determines the DCI format size budget based on the search space sets the UE is configured to monitor PDCCH, as described in TS 38.213. - TS 38.213 states that “The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321]”. So, no need for proposal in counting DCI format 2_6 outside Active Time sepeartely. - “NOT be counted” has no meaning for a UE implementation. We cannot say that DCI format 2_6 is an invisible DCI format for the UE with respect to the number of DCI format sizes the UE has to monitor. |
| ZTE | Issue 1,2,3,4 | We are okay to discuss issue 1,2,3,4 in this meeting. Noted that for issue 2, the discussion is triggered by the LS in AI 5, and we are okay to discuss it in AI 5 or AI 7.2.7.1. In section 3.2, our view is added. |
| Ericsson | Issues 1,2,3,4 |  |
| Intel | Issues 1,2,3,4 | As discussed over email, we believe the combination of RNTIs monitored outside active time can be discussed as part of issue 2. In case issue 2 is discussed in AI 5, we suggest to just add a separate issue in 7.2.7.1 targeting that matter. - Issue 5: Combinations of RNTIs monitored outside active time (can be combined with Issue 2 or discussed separately) |
| CMCC | Issues 1,2,3,4 | For issue 2, we also support RAR is prioritize over DCP during RAR monitoring window. In adddtion, we also want to clarify the DCI format 2_6 transimmion behaviour in multiple monitoring occasions. - DCI format 2_6 should be transmitted in only one monitoring occasion or all monitoring occasions. |
| ASUSTeK | Issue 1,2,3,4 | It’s fine to discuss issue 1,2,3,4. Though we are open to discuss issue 4, we think the current text may be sufficient (or with some minor clarification, e.g. DCI size budget only applied for the case within Active Time). |
| Nokia | Issues 1,2,3 and 4 are OK for us. | We agree with Intel, that it would good to clarify 38.202. Current text seems to be contradiction with MAC spesification. |
| MediaTek | Issues 1,2,3,4 | We are also fine to discuss the issue mentioned by Intel to clarify the RNTIs combination monitored outside active time in 38.202. |
| Huawei, HiSilicon | We are fine to discuss Issues 1,2,3,4. | - We think Issue#2 can be discussed in the LS AI. For the combinations with other RNTIs, we think it should be discussed. But we suggest to focus on the case of collision between DCP and RAR addressed to C-RNTI, which is raised in RAN2 LS. For issues newly raised during the preparation phase, it is better to note them this time and give companies time until the next meeting to look into the issue properly. - A number of companies raised the issue to clarify the BWP switching timing considering the DCI format 2_6 triggered dormancy adaptation is different from the BWP switching triggered by scheduling DCI. The same BWP switching due to dormancy adaptation would be transmitted in the DCI format 2_6 which may be transmitted on any one of the multiple occasions. There is no HARQ-ACK transmission for the DCI format 2_6, and therefore the UE and gNB may have different understanding regarding the timing of BWP switching, which means different understanding regarding the interruption time etc. We propose to clearly define/clarify the timing of BWP switching triggered by the DCI format 2_6. Issue#5: clarify/define the start of the BWP swiching triggered by DCI format 2_6. |
| OPPO | Issues 1,2,3,4 | Fine to discuss these issue. For issue 1, we shall discuss how to UE and gNB align BWP switching. For issue 3, we can discuss this issue in RAN 1 but it would be better make the change in RAN2. |
| vivo | Issues 1,2,3,4 We propose also to discuss the starting time of BWP switching after dormancy indication received from DCI format 2_6 | We are fine to discuss the issues 1,2,3,4. For issue 2, (collisoin of DCP and RAR), we see some contributions in AI 5 as well and is not captured in the supporting companies list. It seems some companies think it works well by gNB implementation and some thinks spec impact is needed (with different spec impact consideration). Therefore we kindly suggest to have more discussions during this and next week. In additional , we propose also to discuss the starting time of BWP switching after dormancy indication received from DCI format 2_6 |

Summary from contributions reviews

Minimum time gap – values

RAN1#99 agreements Agreements:s The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to [3]ms - FFS impact of dormancy/non-dormancy transition RAN1#100-e agreements Agreements PS_offset range from {0.125ms to 15 ms} for all SCS. Agreements The PS_offset resolution is 0.125 ms. Agreements Candidate values for the minimum time gap are specified by RAN1 and shared with RAN4 · Minimum time gap is no more than 3 ms for all SCSs · Two values of minimum time gap for each SCS are proposed as - SCS 15kHz: {TBD, TBD} slots - SCS 30kHz {TBD, TBD} slots - SCS 60kHz {TBD, TBD} slots - SCS 120kHz {TBD, TBD} slots RAN1#100bis-e agreements Proposal 1: - The value of minimum time gap is decoupled with SCell dormancy indication. - Working assumption: Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24.

During RAN1#100bis-e email discussion,  working assumption was made to have the value of minimum time gap is decoupled with SCell dormancy indication and two values of minimum time gap.

Proposals from companies

- Confirmation of working assumptions – CATT, MediaTek, Samsung, CMCC, OPPO, Ericsson, Nokia

- New values – Huawei, Qualcomm, DoCoMo

  - Huawei

|  | NR Slot length (ms) | Capability Type 1 |  | Capability Type 2 |
| --- | --- | --- | --- | --- |
|  |  | UE not report pdcch-MonitoringAnyOccasions | UE reports pdcch-MonitoringAnyOccasions |  |
| 0 | 1 | 0 | 1 | 1 |
| 1 | 0.5 | 0 | 1 | 2 |
| 2 | 0.25 | 1 |  | 4 |
| 3 | 0.125 | 2 |  | 8 |

  - Qualcomm

    - SCS 15kHz: {1, 3} slots

    - SCS 30kHz: {2, 6} slots

    - SCS 60kHz: {3, 12} slots

    - SCS 120kHz: {6, 24} slots

  - DoCoMo

| SCS (kHz) | Minimum Time Gap TminimumTimeGap(slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 5 |
| 60 | 1 | 9 |
| 120 | 2 | 18 |

Proposal:  Confirm the working assumptions

- The value of minimum time gap is decoupled with SCell dormancy indication.

- Working assumption: Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows,

| SCS (kHz) | Minimum Time Gap TminimumTimeGap (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

Collisoin of DCP and RAR

RAN2 LS in [18] would like to ask RAN1 how to resolve the collision between the DCP and the RAR addressed to C-RNTI with different quasi-collocated property during BFR.

1. Collision between DCP and RAR addressed to C-RNTI RAN2 has discussed UE behavior when a DCP monitoring occasion overlaps with the ra-ResponseWindow or msgB-ResponseWindow. RAN2 understanding is that according to current TS 38.213 prioritization rules, if DCP collides with RAR addressed to C-RNTI (e.g. during BFR) and the search spaces are not quasi-collocated, DCP will be prioritized as it is type-3 CSS and thus impacting legacy RAR behavior. RAN2 would like to ask RAN1 the following: - To confirm RAN2 understanding that if DCP and RAR search spaces are not quasi-collocated, a collision between DCP and RAR addressed to C-RNTI will impact legacy RAR handling. - From RAN2 point of view, the understanding is that RAR addressed all RNTIs should be prioritized over DCP by the UE. - RAN2 would like to ask if RAN1 has any concerns with the understanding above? If RAN1 doesn’t have any concerns, what is RAN1 preference on where to capture this behavior e.g. TS 38.213 or in TS 38.321 via a DCP monitoring exception rule similar to overlap with DRX Active time?

The collision of DCP and RAR monitoring were discussed with proposals as follows,

- RAR is prioritized over DCP –

  - gNB implementation with current specification -  vivo, Huawei, Samsung

  - RAR with CRC scrambled by C-RNTI over DCP – CATT, Intel, LG, Ericsson, Nokia,ZTE,CMCC

Proposal:

RAR is prioritize over DCP during RAR monitoring window.  Discuss further

- RAN2 LS reply

- Any RAN1 specification change

Spcification Alignment - Clarification the interaction between PHY and MAC layers

RAN1#100bis-e agreements Proposal 1: L1 procedure of DCI format 2_6 detection - Successful decoding of DCI format 2_6 - L1 sends a positive indication to MAC when the value of wakeup indication bit is “1” - L1 sends a negative indication to MAC when the value of wakeup indication bit is “0” - Miss-detection - all CRC checks fails on DCI format 2_6 - L1 does not send any indication to MAC - All invalid monitoring occasions – following legacy behavior to wakeup - L1 sends a positive indication to MAC TP was approved and captured in [19]

The proposed TP was agreed to capture the general behavior.  However, the editor of 38.213 deos not capture the TP exactly as those were agreed in [19].  There were discussions (ZTE, CATT, AsusTek, Spreadtrum) to clarify the exact text of the decoding bit value ‘1’ and ‘0’ from DCI format 2_6 associated with Wake-up and no-Wake-up indication.  In addition, RRC parameters ps-PositionDCI-2-6 and sizeDCI-2-6  are updated.

Proposal:

TP to capture value ‘1’ and ‘0’ from DCI format 2_6 associated with Wake-up and no-Wake-up indication

-----------------------------------------------------Start of TP of 38.213----------------------------------------------------------

10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]

-	a PS-RNTI for DCI format 2_6 by ps-RNTI

-	a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1

-	a payload size for DCI format 2_6 by SizeDCI_2-6 sizeDCI_2-6

-	a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where

-	the UE may not start the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '0', and

- a value ‘0’ of Wake-up indication bit is the no-Wake-up indication

- a value ‘1’ of Wake-up indication bit is the Wake-up indication

- the UE starts the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '1'

-----------------------------------------------------End of TP of 38.213---------------------------------------------------------------

RAN2 also reach agreements on the update parameters name in [18]

Agreements

1	RAN2 confirms that the flags ps-TransmitPeriodicL1-RSRP and ps-TransmitPeriodicCSI are defined per cell group

2	The flags ps-TransmitPeriodicCSI and ps-TransmitPeriodicL1-RSRP are independent, and it is possible to control UE to report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

3	The flag ps-TransmitPeriodicCSI is renamed to ps-TransmitOtherPeriodicCSI

TP for Clause 5.1.6.1 and 5.2.2.5 of TS 38.214.

5.1.6.1 CSI-RS reception procedure <omitted text> If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. <omitted text>

5.2.2.5 CSI reference resource definition <omitted text> When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to ‘cri-RSRP’ or ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to ‘cri-RSRP’ if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <omitted text>

DCI size budget for DCI format 2_6

Currently the DCI format size budget per UE is determined without separation for the capability between CONNECTED or IDLE/INACTIVE, nor between outside active time and inside active time.  The only separation is done via defining the RNTIs for UE monitoring. There are RNTIs (C-RNTI, P-RNTI, SI-RNTI, RA-RNTI) that UE may need to monitor outside active time which the corresponding DCI formats are accounted in DCI-format size budget.  The intent of the DCI format 2_6 outside active time use would NOT be counted in the total budget of DCI format sizes.   Several companies (vivo, CATT, MediaTek, Ericsson, Qualcomm) have proposals in counting DCI format 2_6 outside Active Time sepeartely and how to account for the total budget of DCI format sizes.

- Proposal: DCI format 2-6 size is aligned to the DCI  size budget outside Active Time.

Others

- The starting time of BWP switching after dormancy indication received from DCI format 2_6  –

  - Inconsistent power saving information (vivo) – no-Wakeup and non-dormant SCell indications for a UE

  - More than one DCI format 2_6 are received (vivo, Huawei) –

  - No DCI format 2_6 monitoring during BWP switching

- Valid moniotoring occasion when more than one avalailable moniotoring occasions in a search space set (LG)

- No restriction on minimum time gap without UE capability feedback (Qualcomm)

Contributions summary and proposals

| vivo[1] | - Proposal 1: PDCCH monitoring for RAR dropping due to different QCL properties can be avoided by proper network implementation. No additional UE behavior need to be specified. - Send LS to RAN2 to inform above decisions. - Proposal 2: UE is not expected to be indicated by PDCCH WUS not to wake up while SCell group is indicated to non-dormancy state. Capture TP in Appendix 1 in R1-2003403 for TS38.213. - Proposal 3: The starting point of BWP switching of Scell dormancy should be defined from the following alternatives, - Alt 1: the starting of BWP switching of Scell dormancy is after the last valid monitoring occasion for DCI format 2-6 - Alt 2: the starting of BWP switching time of Scell dormancy is n slot prior to DRX ON, where n is the Scell dormancy/non-dormancy switching time. - Proposal 4: The size of DCI format 2-6 is not restricted by the existing DCI size budget (3+1) in Rel-15 which is used in Active Time. Capture TP in Appendix 2 in R1-2003043 for TS38.212. |
| --- | --- |
| ZTE [2] | - Proposal 1: The following TP of L1 procedure of the detection of DCI format 2_6 is proposed (wakeup bit). - Proposal 2: Adopt the following TP for Clause 5.1.6.1 and 5.2.2.5 of TS 38.214. (ps-TransmitOtherPeriodicCSI) |
| Huawei, HiSilicon [3] | - Proposal 1: Confirm the working assumption made in RAN1#100b-e that the value of minimum time gap is decoupled with SCell dormancy indication. - Proposal 2: The values of minimum time gap during which the detection of DCI format 2_6 is not required prior to the start of ON Duration Timer are determined per SCS according to the reported capability type of UE as the following table. NR Slot length (ms); Capability Type 1; Capability Type 2. UE not report pdcch-MonitoringAnyOccasions; UE reports pdcch-MonitoringAnyOccasions. 0; 1; 0; 1; 1. 1; 0.5; 0; 1; 2. 2; 0.25; 1; 4. 3; 0.125; 2; 8. - Proposal 3: Further discuss and narrow down between Alt.1, Alt.2 and Alt. 3 for UE behaviour when dormancy indication is configured. - Proposal 4: Adopt TP1 in TS 38.214 to clarify UE behaviour of RRM measurement when DCI format 2_6 is configured. |
| CATT [4] | - Proposal 1: The working assumption is confirmed. Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. - Proposal 2: RAR indicated by PDCCH with CRC scrambled by C-RNTI or MCS-C-RNTI should be prioritized over DCP by the UE outside Active Time, especially when they have different QCL-TypeD properties. - Proposal 3: The detected value of the Wake-up indication bit from DCI format 2_6 needs to be defined clearly to have ‘1’ associated with Wake-up indication and ‘0’ associated with no-Wake-up indication. - Proposal 4: DCI format 2_6 is not counted in the DCI size alignment. |
| MediaTek[5] | - Proposal 1: Confirm the working assumption for minimum time gap in RAN1 #100b. - The value of minimum time gap is decoupled with SCell dormancy indication. - Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. - Proposal 2: DCI format 2_6 is not counted in the DCI size budget. - Proposal 3: Adopt the TP in TS 38.213 as follows (prior to). |
| Intel [6] | - Proposal 1: If the DCI format 2_6 monitoring occasion overlaps with ra-ResponseWindow or msgB-ResponseWindow, the UE does not monitor for DCI format 2_6. - Proposal 2: Adopt the following TP |
| Samsung [7] | - Proposal #1: Confirm the working assumption for the values of the minimum time gap - Observation #1: The specifications with respect to the size of DCI format 2_6 are complete. The Rel-15 DCI format size budget is not exceeded. The size of DCI format 2_6 should be counted towards the maximum number of DCI format sizes as for every other DCI format. - Observation #2: There is no need for any specification impact with respect to monitor ‘RAR’ scheduled by a DCI format with a C-RNTI or an MCS-C-RNTI. |
| NEC [8] | - Using the unified wording “PCell or PSCell” is also recommended instead of “PCell or SpCell”. |
| CMCC [9] | - Proposal 1. Support to confirm the working assumption - The value of minimum time gap is decoupled with SCell dormancy indication. - Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. - Proposal 2. DCI format 2_6 should be transmitted in only one monitoring occasion or all monitoring occasions. |
| Spreadstrum[10] | - Proposal 1: For P-CSI/L1-RSRP measurement/report, consider to adopt TP in Appendix 5.1. - Proposal 2: To clarify the real starting of monitoring is the beginning of the 1st full “duration”, consider to adopt TP in Appendix 5.2. - Proposal 3: To align parameters in RAN2, such as ps-Wakeup, ps-PositionDCI-2-6 and sizeDCI-2-6, consider to adopt TP in Appendix 5.2 and 5.3. |
| LG [11] | - Proposal 1: At least for CFRA, the SS set related to BFR procedure has a higher priority than SS set to be monitored for DCI format 2_6. - Proposal 2: The monitoring occasion which has at least one actually monitored candidate is regarded as a valid monitoring occasion. - Proposal 3: A UE monitors DCI format 2_6 in the first available full duration within a monitoring window. |
| OPPO [12] | - Proposal 1: Confirm the working assumption. - The value of minimum time gap is decoupled with SCell dormancy indication. - Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. - Proposal 2: when the UE is configured scell dormancy operation, the UE doesn’t need to monitor WUS during its BWP switching delay before DRX ON. |
| AsusTek[13] | - Proposal: RAN1 adopts either TP1 or TP2 to avoid discrepancy on interaction between PHY and MAC on wake up indication |
| Ericsson [14] | - Proposal 1 UE is not required to monitor DCI 2-6 in monitoring occasions that overlaps with the ra-ResponseWindow or beam-failure recovery procedure. - Proposal 2 RAN1 to provide the following response to RAN2 LS (R1-2003260) - RAN1 does not have a concern with the RAN2 understanding and asks RAN2 to capture in 38.321 that the UE wakes up in an upcoming ON duration when a DCI 2-6 monitoring occasion overlaps the ra-ResponseWindow or beam-failure recovery procedure. - Proposal 3 RAN1 to confirm the minimum time gap values in the WA as well as the decoupling of minimum time gap values from Scell dormancy indication. - Proposal 4 DCI sizes in the budget calculation are counted separately for the cases of within Active time and outside Active time. - Proposal 5 Adopt TP1 for 38.212 subclause 7.3.1.0 DCI sizes in the budget calculation are counted separately for the cases of within Active time and outside Active time. |
| NTT DoCoMo [15] | - Proposal 1: Two candidate values of UE reported minimum time gap for each SCS are: SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 5. 60; 1; 9. 120; 2; 18. |
| Qualcomm[16] | - Proposal 1: For consistency, the lower set of values for the minimum time gap should not be smaller than the Z values (i.e., {1, 1, 2, 2}) for the application delay of the minimum scheduling offset restriction. - Proposal 2: For the reported UE capability on the minimum time gap, the following sets of values can be considered: - SCS 15kHz: {1, 3} slots - SCS 30kHz: {2, 6} slots - SCS 60kHz: {3, 12} slots - SCS 120kHz: {6, 24} slots - Proposal 3: For the reported UE capability on the minimum time gap, adding a third option of ‘No restriction’ can be considered. The third option can be implicitly signaled by omitting the capability report for the minimum time gap. - Proposal 4: For the aggregation level and the number of PDCCH candidates for DCI format 2_6, reuse those for DCI format 2_0. - Proposal 5: Separate sets of DCI format sizes are accounted for within and outside the Active Time. The size of DCI format 2_6 is counted in the DCI size budget for “outside Active Time”. |
| Nokia, NSB [17] | - Proposal 1: When monitoring occasions of DCI format 2_6 overlaps with PDCCH monitoring in search space given recoverySearchSpaceId as described in Section 6 of 38.213, UE should follow the legacy DRX operation. - Proposal 2: Send a LS to RAN2 indicating that RAN1 agrees that there could be impact to the legacy RAR monitoring configuration and that the behaviour related to DCP and RAR monitoring could be captured to RAN2 specification. - Proposal 3: Confirm the working assumption take in the RAN1#100bis-e: working assumption: - The value of minimum time gap is decoupled with SCell dormancy indication. - Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. |

Reference

- R1-2003403		Maintenance of PDCCH-based power saving signal	vivo

- R1-2003486		Remaining issues on WUS PDCCH	ZTE

- R1-2003518		Remaining issues on PDCCH based power saving	Huawei, HiSilicon

- R1-2003630		Remaining issues on the Power Saving Signals/Channels	CATT

- R1-2003664		Remaining issues on PDCCH-based power saving signal	MediaTek Inc.

- R1-2003745		Remaining details of PDCCH-based power saving signal/channel	Intel Corporation

- R1-2003884		Remaining issues for PDCCH-based power saving signal	Samsung

- R1-2003924		TP for further alignment with RAN2 specifications	NEC

- R1-2003957		Remaining issues on power saving signal/channel	CMCC

- R1-2003999		Clarification on power saving signal	Spreadtrum Communications

- R1-2004025		Remaining issues on PDCCH-based power saving signal/channel	LG Electronics

- R1-2004101		Remaining issues for Power saving signal	OPPO

- R1-2004320		Wake up indication for ON duration timer	ASUSTeK

- R1-2004357		Remaining issues for WUS	Ericsson

- R1-2004398		Maintenance for PDCCH-based power saving signal/channel	NTT DOCOMO, INC.

- R1-2004467		Remainign issues in power saving signal/channel	Qualcomm Incorporated

- R1-2004577		On open issues related to DCI format 2_6	Nokia, Nokia Shanghai Bell

- R1-2003260		LS on RAN2 DCP Open Issues, RAN2		 InterDigital.

- R1-2003177		TR38.213 CR 0105 Corrections on UE power savings	 Samsung