3GPP TSG RAN meeting #88e										       RP-20xxxx

Electronic Meeting, June 29 - July 3, 2020

Status Report to TSG

Agenda item:			9.9.4

| WI / SI Name | Study on NR coverage enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_cov_enh |  |  |  |  |
| Unique ID | 860036 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-193240 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2020 (was 06/2020) | Core part: NA | Performance part: NA | Testing part: NA |  |
| Overall Completion level | Study Item: 20% | Core part: NA | Performance Part: NA | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Jianchi Zhu |
|  | Company | China Telecom |
|  | Email | zhujc@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #101-e

- TR skeleton was endorsed in R1-2004753.

Agreements:

- Adopt the following target data rates for eMBB performance evaluation for FR1.

- Urban scenario: DL 10Mbps, UL 1Mbps

- Rural scenario: DL 1Mbps, UL 100kbps

- Rural with long distance scenario: DL 1Mbps, UL 100kbps, 30kbps (optional)

Agreements:

- For VoIP performance evaluation based on link-level simulation for FR1.

- A packet size of [320] bits with 20ms data arriving interval is adopted.

- TBD: TBS for SIP invite message. Payload of 1500 bytes can be a starting point.

Agreements:

- The basic evaluation methodology is based on link-level simulation for FR1.

- Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

- Step 2: Obtain the baseline performance based on required SINR and link budget template.

- Note: aspects related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately

- The evaluation methodology based on system-level simulation is optional for FR1.

- Note: The simulation assumptions for SLS are up to companies’ reports.

Agreements:

- For link level simulation, adopt the following table for PUSCH and PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz Other frame structures can be reported by companies. |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS Rural: NLoS and LoS |
| BWP | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. [CDL] |
| UE velocity | Urban: 3km/h for indoor Rural: 3km/h for indoor, 120km/h (optional 30km/h) for outdoor |
| Frequency hopping | w/ or w/o frequency hopping for PUSCH w/ frequency hopping for PUCCH. |

- FFS whether there are any additional simulation considerations for the extreme coverage scenarios (e.g., rural)

Agreements:

- Down selection on the following options for the link budget template for FR1 in next meeting.

- Option 1: Adopt single link budget template based on IMT-2020 self-evaluation with necessary revisions, including adding/removing/revising some parameters.

  - FFS: The template provided by FL in Tdoc R1-2005005.

- Option 2: Adopt both templates, i.e. link budget template in IMT-2020 self-evaluation and link budget template in TR 36.824.

- Option 3: Adopt single link budget template in TR 36.824 with necessary revisions, including adding/revising some parameters.

Agreements:

Down selection on the following options for antenna array gain for LLS based methodology for FR1 in next meeting.

- Option 1: Antenna array gain is included in the link budget template.

- FFS: array gain = 10 * 1og10 (number of antenna elements/number of TxRUs)

- FFS: For TDL channel model

- FFS: Values reflective of realistic implementation and network operation.

- Option 2: Antenna array gain is included in LLS.

- FFS: For CDL channel model

Agreements:

- For link level simulation, adopt the following table for PDSCH for FR1.

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS |
| Other parameters | FFS |

Agreements:

- For link level simulation, adopt following TBS for Msg3 for FR1

- 56 bits

Agreements:

- For link level simulation, the packet size of VoIP for FR2 is the same as FR1.

Agreements:

- For link level simulation, TBS of Msg3 for FR2 is the same as FR1.

Agreements:

- The evaluation methodology for FR2 is the same as FR1.

Agreements:

- The link budget template for FR2 is the same as FR1.

Agreements:

- For link level simulation, adopt the following table for PUSCH and PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 28GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) Other frame structures can be reported by companies. |
| Subcarrier Space | 120kHz |
| UE velocity | Indoor scenario:3km/h Urban scenario: 3km/h for indoor, 30km/h for outdoor. Suburban scenario: 3km/h for indoor, 30km/h, (optional: 120km/h) for outdoor. |
| Occupied channel bandwidth for | 100MHz, [400MHz] |
| Frequency hopping for PUSCH | w/ or w/o frequency hopping |

Agreements:

- For link level simulation, adopt the following table for PUSCH for eMBB data or VoIP for FR1.

| Parameters | Values |
| --- | --- |
| BLER for PUSCH | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains for PUSCH | 1，2 (optional) |
| DMRS configuration for PUSCH | For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| Waveform for PUSCH | DFT-s-OFDM, CP-OFDM (optional) |
| Repetitions for PUSCH | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ repetition. The actual number of repetitions is reported by companies. FFS: Repetition type B |
| HARQ configuration for PUSCH | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| Latency requirements for voice | 50ms/100ms |
| PUSCH duration | 14 OS |

Agreements:

- For link level simulation, adopt the following table for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| PUCCH format type | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. For PUCCH format 3: BLER for Ack/Nack, SR: 1% FFS: BLER for CSI (10% or 1%) |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE transmit chains for PUCCH | 1 |
| Number of repetitions for PUCCH | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| DMRS configuration for PUCCH | FFS: number of DMRS symbols for PUCCH Format 3. |

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Number of receive antenna elements for BS | Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB receive chains in LLS. FFS: correlation - Option 2: Number of gNB receive chains = number of TXRUs in LLS. FFS: correlation [gNB architectures to study for CDL: - Urban: 64 receive chains for 2.6 and 4 GHz in LLS - Rural: 8 receive chains for 4GHz and 2.6GHz in LLS - 4 receive chains for 2GHz and 700MHz in LLS.] [gNB modeling in LLS for CDL: Number of gNB receive chains = number of TXRUs in LLS.] |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |
| PRBs/TBS/MCS for eMBB for PUSCH | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Note: For TDL models, companies report whether antenna array gain is included in LLS or link budget template. Array gain calculation method and how channel estimation is accounted for is reported by companies

Agreements:

- Adopt the following target data rates for eMBB performance evaluation for FR2.

- Indoor: DL: 25Mbps, UL:5Mbps

- Urban: DL: 25Mbps, UL: 5Mbps

- Suburban: FFS: (DL: 1Mbps, UL: 50kbps)

Agreements:

- For link level simulation, adopt the following table for Msg.3 for FR1.

| Parameters | Values |
| --- | --- |
| Number of PRBs | 2 |
| Waveform | DFT-s-OFDM |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| PUSCH duration | 14 OS |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, adopt the following table for PDCCH for FR1.

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER for PDCCH | 1% BLER FFS: 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreements:

- For link level simulation, adopt the following table for SSB for FR1.

| Parameters | Values |
| --- | --- |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, adopt the following table for PRACH for FR1.

| Parameters | Values |
| --- | --- |
| Format | Format 0, Format B4, or Format C2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability FFS: 10% missed detection. |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, for PDSCH of Msg.4 for FR1.

  - Reuse the following simulation assumption for PDSCH

    - Waveform, [PDSCH duration]

  - FFS: Payload size: [3000bits].

  - Other parameters: Reported by companies.

Agreements:

- For link level simulation, for SSB, PDCCH, PDSCH and PDCCH of Msg.2, PDSCH of Msg.4 and PDSCH for FR1.

  - Reuse following simulation assumptions agreed for PUSCH.

    - Scenario and frequency, frame structure, SCS, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS.

  - The number of UE receive chains:

    - 4 for 4GHz/2.6GHz

    - 2 or 4 for 2GHz

    - 2 for 700MHz

  - For PDSCH, reuse BLER, HARQ, Latency requirements for voice agreed for PUSCH.

    - Reuse DM-RS configuration agreed for PUSCH except that 3 DMRS symbols is used for Msg2.

- For link level simulation, for PRACH and Msg.3 for FR1.

  - Reuse following simulation assumptions agreed for PUSCH

    - Scenario and frequency, frame structure, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS and Number of UE transmit chains.

  - For Msg.3, reuse SCS, HARQ configuration, frequency hopping agreed for PUSCH.

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| BLER | For eMBB, w/ HARQ, 10% iBLER, Optional: companies report rBLER. w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| DMRS configuration | For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping for PUSCH: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH/PDSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM for PUSCH, CP-OFDM for PDSCH FFS: CP-OFDM for PUSCH |
| Repetitions for PUSCH/PDSCH | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ repetition. The actual number of repetitions is reported by companies. FFS: Repetition type B for PUSCH. |
| HARQ configuration for PUSCH/PDSCH | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PUSCH/PDSCH duration | 14 OS for PUSCH, 12 OS for PDSCH |

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Optional: 512, (M,N,P,Mg,Ng) = (8,8,2,2,2) |
| Number of TxRUs for BS | 2 Note: Analog beamforming is assumed. |
| Number of UE Tx/Rx chains | 1T2R, 2T2R |
| Channel model for link-level simulation | CDL- A, TDL-A, [urban/suburban: TDL-C] Note: company can provide simulation results based on either TDL channel or CDL model |
| Delay spread | Indoor scenario: 30ns Urban scenario: 100ns Suburban scenario: 100ns |
| Latency requirements for voice | 50ms/100ms |
| PRBs/TBS/MCS for eMBB for PUSCH/PDSCH | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use [30] PRBs for 5Mbps for PUSCH and full bandwidth for 25Mbps for PDSCH as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH/PDSCH | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK for PDSCH/PUSCH Optional: pi/2 BPSK for PUSCH |

Agreements:

- For link level simulation, adopt the following simulation assumption for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Number of UE antenna elements | 8, one panel:(M, N, P) = (2,2,2), FFS: Two panels in link budget, one panel in LLS, 16 for each panel: (M, N, P) = (4,2,2) |

Agreements:

- For link level simulation, adopt the following table for PUCCH for FR2.

| Parameters | Values |
| --- | --- |
| Format | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI FFS: Format 0, 2 |
| BLER for PUCCH | The same as FR1 |
| Number of PRBs for PUCCH | The same as FR1 |
| Number of UE transmit chains for PUCCH | The same as FR1 |
| Number of repetitions for PUCCH | The same as FR1 |
| PUCCH duration | 14 OFDM symbols FFS: 4 OFDM symbols |
| DMRS configuration for PUCCH | FFS: [4] DMRS symbols for PUCCH Format 3. |

Agreements:

- For link level simulation, adopt the following table for PDCCH for FR2.

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48PRBs |
| Tx Diversity | Reported by companies |
| BLER for PDCCH | 1% BLER. FFS: 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreements:

- For link level simulation, adopt the following table for PRACH for FR2.

| Parameters | Values |
| --- | --- |
| Format | Format B4, (Optional: Format C2) |
| SCS | Reported by companies. |
| Performance metric | 0.1% false alarm, 1% miss-detection FFS: 10% missed detection. |
| Number of SSB beams | Reported by companies |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, for SSB, PDCCH, PDSCH and PDCCH of Msg.2, PDSCH of Msg.4 for FR2.

  - Reuse following simulation assumptions for PDSCH

    - Scenario and frequency, frame structure, SCS, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS, number of UE Tx/Rx chains and UE antenna elements.

- For link level simulation, for PUCCH, PRACH and Msg.3 for FR2.

  - Reuse following simulation assumptions for PUSCH

    - Scenario and frequency, frame structure, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS, number of UE antenna elements for PUSCH.

  - For PRACH and Msg.3, reuse number of UE Tx chains for PUSCH.

  - For PUCCH, reuse SCS for PUSCH.

  - For Msg.3, reuse SCS, HARQ configuration, frequency hopping for PUSCH.

2.1.2	Remaining Open issues

- Remaining issues on evaluation methodology and simulation assumptions for FR1 and FR2.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation.

  - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services.

  - The target channels include at least PUSCH/PUCCH.

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions).

  - Study the additional enhanced solutions for FR2 if any.

  - Evaluate the performance of the potential solutions based on link level simulation.

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #101e:

- R1-2003833, Draft skeleton of TR 38.830 Study on NR coverage enhancements, v0.0.0, China Telecom

- R1-2004753, Draft skeleton of TR 38.830 Study on NR coverage enhancements, v0.0.1, China Telecom

- R1-2003832, Work plan for Study on NR coverage enhancements, China Telecom

- R1-2005004, [101-e-NR-Cov-Enh] Email discussion on evaluation methodology and simulation assumptions for NR coverage enhancements, Moderator (China Telecom)

- R1-2005192, [101-e-Post-NR-Cov-Enh] Email discussion/approval focusing on remaining  evaluation assumptions, Moderator (China Telecom)

- R1-2004155, Overview of coverage enhancement: scenarios, channels, services and potential solutions, Huawei, HiSilicon

- R1-2004631, General Considerations for the Coverage Enhancement Study, Ericsson

- R1-2003648, Discussion on the methodology for coverage enhancement, CATT

- R1-2003919, Assumptions for NR coverage evaluation, vivo

- R1-2004027, Discussion on evaluation for coverage enhancement, LG Electronics

- R1-2004377, Considerations for Coverage Enhancement, Indian Institute of Tech (H)

- R1-2004632, Evaluation Methodology for Coverage Enhancements, Ericsson

- R1-2003298, Baseline coverage performance for FR1	Huawei, HiSilicon

- R1-2004716, Baseline coverage performance for FR1	Huawei, HiSilicon

- R1-2003338, Discussion on baseline coverage performance for FR1, ZTE

- R1-2003342, NR Coverage requirements and simulation assumption for FR1, Sierra Wireless, S.A.

- R1-2003435, Evaluation on NR coverage performance for FR1, vivo

- R1-2003464, Requirements for Voice coverage enhancements with FR1, SoftBank Corp.

- R1-2003649, Discussion on the baseline performance and simulation  assumptions of coverage enhancement for FR1, CATT

- R1-2003683, Discussion on scenarios for FR1 baseline performance evaluation, MediaTek Inc.

- R1-2003773, Discussion on baseline coverage performance for FR1, Intel Corporation

- R1-2003778, Downlink coverage in FR1, Charter Communications, Inc

- R1-2003816, Discussion on evaluation methodologies for baseline coverage performance analysis, Panasonic Corporation

- R1-2003821, Baseline evaluation for NR UL coverage enhancement	Lenovo, Motorola Mobility

- R1-2003834, Evaluation methodology and preliminary baseline performance for NR coverage enhancements, China Telecom

- R1-2003914, Scenarios and simulation assumptions for coverage enhancement in FR1, Samsung

- R1-2003940, Simulation Assumptions and Baseline Coverage for FR1, Nomor Research GmbH, Facebook

- R1-2003970, Discussion on coverage enhancements in FR1, CMCC

- R1-2004108, NR coverage performance for FR1, OPPO

- R1-2004178, Baseline coverage evaluation of UL and DL channels – FR1, Nokia, Nokia Shanghai Bell

- R1-2004196, On NR coverage analysis in FR1, Sony

- R1-2004249, On baseline coverage performance for FR1, Apple

- R1-2004304, Simulation assumptions and throughput performance for UL in FR1, InterDigital, Inc.

- R1-2004338, Preliminary evaluation for FR1 Urban scenario, Sharp

- R1-2004352, Simulation Parameters and Initial Results for FR1, Ericsson

- R1-2004424, Baseline coverage performance for FR1, NTT DOCOMO, INC

- R1-2004497, Baseline FR1 coverage performance, Qualcomm Incorporated

- R1-2004540, Baseline coverage performance for FR1, xiaomi

- R1-2003299, Baseline coverage performance for FR2, Huawei, HiSilicon

- R1-2003339, Discussion on baseline coverage performance for FR2, ZTE

- R1-2003436, Evaluation on NR coverage performance for FR2, vivo

- R1-2003650, Discussion on the baseline performance and simulation assumptions of coverage enhancement for FR2, CATT

- R1-2003774, Discussion on baseline coverage performance for FR2, Intel Corporation

- R1-2003779, Downlink coverage in FR2, Charter Communications, Inc

- R1-2003915, Scenarios and simulation assumptions for coverage enhancement in FR2, Samsung

- R1-2003971, Discussion on coverage enhancements in FR2, CMCC

- R1-2004179, Baseline coverage evaluation of UL and DL channels – FR2, Nokia, Nokia Shanghai Bell

- R1-2004197, Considerations on Simulation Assumptions for Coverage Enhancements for FR2, Sony

- R1-2004305, Simulation assumptions for UL in FR2, InterDigital, Inc.

- R1-2004353, Simulation Parameters and Initial Results for FR2, Ericsson

- R1-2004425, Baseline coverage performance for FR2, NTT DOCOMO, INC

- R1-2004498, Baseline FR2 coverage performance, Qualcomm Incorporated

- R1-2003300, Potential solutions for coverage enhancement, Huawei, HiSilicon

- R1-2003340, Discussion on potential techniques for coverage enhancements, ZTE

- R1-2003343, Potential techniques for NR coverage enhancements, Sierra Wireless, S.A.

- R1-2003437, Discussion on potential techniques for coverage enhancements, vivo

- R1-2003651, Discussion on the method for coverage enhancement, CATT

- R1-2003775, Discussion on potential techniques for NR coverage enhancement, Intel Corporation

- R1-2003817, Discussion on potential techniques for coverage enhancements, Panasonic Corporation

- R1-2003835, Potential solutions for PUSCH coverage enhancements	, China Telecom

- R1-2003916, Considerations on potential techniques for coverage enhancement, Samsung

- R1-2003938, Potential Solutions on Coverage Enhancement for FR1, Nomor Research GmbH, Facebook

- R1-2004028, Discussion on potential techniques for coverage enhancement, LG Electronics

- R1-2004109, The potential solutions to enhance NR coverage, OPPO

- R1-2004180, Discussion on possible approaches and solutions for NR coverage enhancement, Nokia, Nokia Shanghai Bell

- R1-2004198, Techniques for NR coverage enhancement, Sony

- R1-2004250, On potential techniques for coverage enhancement, Apple

- R1-2004273, Potential techniques for coverage enhancements, InterDigital, Inc.

- R1-2004354, Potential Areas for Coverage Enhancement, Ericsson

- R1-2004426, Potential techniques for coverage enhancements, NTT DOCOMO, INC

- R1-2004499, Potential techniques for coverage enhancement, Qualcomm Incorporated

- R1-2003341, Discussion on evaluation methodology for NR coverage, ZTE

- R1-2003438, Discussion on coverage evaluation assumptions and performance  for RedCap UE, vivo

- R1-2003652, Preliminary simulation results for different channels, CATT

- R1-2003776, Discussion on simulation assumption for NR coverage enhancement, Intel Corporation

- R1-2003917, Preliminary results for VoIP service in TDD, Samsung

- R1-2004181, Evaluation assumptions for NR coverage enhancement evaluation, Nokia, Nokia Shanghai Bell

- R1-2004355, Coverage Enhancements: Four BS Antennas at Low Band, Ericsson

- R1-2004610, System-level simulation assumptions and evaluation metrics for coverage enhancement, Huawei, HiSilicon

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template