3GPP TSG-RAN WG1 Meeting #101-e	R1-2003289

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	8.3.1

Source:	Ericsson

Title:	Potential UE complexity reduction features for Redcap

Document for:	Discussion and Decision

# Introduction

In RAN#86, a new study item on the support of reduced capability NR devices for use cases such as industrial wireless sensors, video surveillance, and wearables was approved [1]. One of the requirements for these three use cases, as described in [1] is lower device cost and complexity as compared to high-end eMBB and URLLC devices of Rel-15/Rel-16. According to [1], these three use cases also have specific requirements regarding data rates, latency, battery lifetime, availability and reliability such as

Industrial wireless sensors:

- Communication service availability is 99.99%.

- End-to-end latency is less than 100 ms; but for safety related sensors, latency requirement is lower, 5-10 ms.

- Reference bit rate is less than 2 Mbps (potentially asymmetric e.g. UL heavy traffic).

- The battery should last at least few years.

Video Surveillance:

- Reference economic video would require bitrates 2-4 Mbps, latency < 500 ms, reliability 99%-99.9%.

- High-end video, e.g. for farming, would require bitrates 7.5-25 Mbps.

- Traffic pattern is dominated by UL transmissions.

Wearables:

- Reference bitrate for smart wearable application can be 10-50 Mbps in DL and minimum 5 Mbps in UL.

- Peak bit rate of the device 150 Mbps for downlink and 50 Mbps for uplink.

- Battery of the device should last multiple days (up to 1-2 weeks).

In this contribution, we discuss UE complexity reduction suitable for these three use cases considering the use case specific requirements such as data rates, latency, battery lifetime, availability, and reliability described above. More specifically, we consider UE bandwidth reduction, reduced number of UE antennas and supported transmission modes, half-duplex FDD, and relaxed UE processing time and capability as potential means for achieving UE complexity and/or cost reduction in both FR1 and FR2. These aspects are elaborated more in the sections that follow. The objective of UE complexity reduction will likely result in a set of Redcap UE capabilities that are below the set of minimum capabilities that can be expected from a legacy NR UE. For the convenience of referring to the minimum set of Redcap UE capabilities, we will use the term Redcap UE type. How a Redcap UE conveys to the network that it supports this set of minimum capabilities is a separate discussion and is not a focus of this contribution.

The main conclusions based on the discussions on various UE complexity reduction aspects are summarized in overview tables in the Conclusions section.

# UE bandwidth reduction

## Description

During the previous studies in [2] and in the study item description (SID) [1], UE bandwidth reduction is identified as one way to reduce the UE complexity. However, since the current NR system targets UE with wide bandwidth, i.e., 100 MHz in FR1 and 200 MHz in FR2, reduced UE bandwidth may have impact on some of the L1 channels. Therefore, in the SID, it is noted that “Rel-15 SSB bandwidth should be reused and L1 changes minimized”. In the frequency domain, an SSB consists of 240 contiguous subcarriers, and the SSB supports 15 kHz and 30 kHz subcarrier spacing in FR1 and 120 kHz and 240 kHz subcarrier spacing in FR2. In this section, we discuss the UE complexity reduction due to UE bandwidth reduction and the potential impacts.

In the SID, the required bitrate ranges from 2 Mbps to 150 Mbps. Certainly, the device that supports higher data rate can be used for the low data rate services. However, device complexity can be further reduced for the lower data rate services to reduce the device cost and power consumption. In FR1, it is preferred that all the Redcap UEs share the same minimum capability in terms of what bandwidth needs to be supported. This is beneficial in terms of initial access design and to achieve a unified framework to understand the coverage impacts and device costs aspects. One set of essential minimum capabilities can be defined for all the Redcap UEs. This minimum set of essential capabilities is driven for supporting a majority of Rel-17 Redcap use cases, e.g., sensors, video surveillance, and wearables that do not require very high bit rates. An optional set of capabilities in terms of number of MIMO layers, number of receive branches, higher order of modulations, to cater the higher data rate requirements can be further added. In this way, economies of scale can be achieved, and at the same time various form factors can also be supported. In FR2, a similar approach can be used.

In FR1, the SSB supports 15 kHz and 30 kHz subcarrier spacing, which corresponds to 3.6 MHz and 7.2 MHz bandwidth, respectively. Therefore, from receiving the SSB point of view, a minimum of 10 MHz bandwidth at the UE side is enough to receive the SSB in FR1. However, in order to support all the configurations of CORESET for Type0-PDCCH, a minimum of 20 MHz bandwidth is preferred (around 110 RBs in terms of 15 kHz SCS). We discuss this issue more in section 2.4.1.

In FR1, as described in section 5.3 in TS 38.101, the support of 5 MHz, 10 MHz, and 15 MHz is necessary for the operation of NR UEs. Therefore, with a UE bandwidth of 20 MHz, it is still necessary for the UE to support smaller bandwidths in order for the UEs to operate on smaller bandwidth part (BWP) or carrier bandwidth than 20 MHz.

Support of 20 MHz maximum bandwidth is preferred for the Redcap UEs in FR1. The Redcap UEs are also expected to operate on BWP or carrier bandwidth that is smaller than 20 MHz in FR1.

In FR2, the SSB supports 120 kHz and 240 kHz subcarrier spacing, which corresponds to 28.8 MHz and 57.6 MHz bandwidth, respectively. Notice that 240 kHz subcarrier spacing is only used for SSB transmission, not for other DL transmissions. In addition, it is also possible to configure both 120 kHz and 240 kHz SSB in the same cell. Therefore, from receiving the SSB point of view, a Rx bandwidth of 50 MHz at the UE side may be enough to receive the SSB in FR2. More discussion on this in section 2.4.1.

It is expected that the Redcap UEs will operate also in the NR TDD band, and hence the reduction of the maximum bandwidth should be applied to both the downlink and uplink

## Analysis of performance against performance requirements

### Impact on Coverage

Reduction of the maximum bandwidth may result in some degradation in the coverage due to less frequency diversity gain and potentially due to the reduced received energy because of the reduced bandwidth. However, this needs to be considered jointly with other cost reduction techniques, e.g., reduced number of TX and RX antennas. Therefore, it is important to first identify how much coverage loss is expected compared to the baseline case for various channels and what are the bottleneck channels. Some channels may have reduced coverage due to reduced bandwidth without impacting the overall coverage of the entire system. Therefore, to conduct proper studies on the impacts on coverage, evaluation methodologies and baseline performance should be established first. We discuss these in detail in our companion paper [10].

### Data rate

The use cases with data rate, latency and battery life requirements are listed in section 1. Based on experience from LTE-M, we assume up to 64QAM in DL and 16QAM UL can be supported. In FR1, from a data rate point of view, 20 MHz bandwidth with one MIMO layer is enough to support the industrial sensor, the reference economic video and the high-end video use case as well as most of the wearable use cases that do not require a peak data rate of 150 Mbps in the DL. In addition, for the cost sensitive use cases, half-duplex UEs can be considered, which is discussed in section 4. For high-end wearables requiring a peak rate of 150 Mbps, a 20 MHz bandwidth with two MIMO layers at least in the DL are necessary, especially considering some of the asymmetric TDD configurations. Since there are no significant differences in terms of UE bandwidth for various use cases, one single device type with different capabilities is enough in FR1. More discussion on UE support for higher order modulation and MIMO is provided in section 6.

In FR2, a legacy UE is required to support 200 MHz carrier bandwidth option. From a data rate point of view and in accordance with the SSB bandwidth, a 50 MHz bandwidth may be enough to cover all the use cases.

It is expected that the minimum supported data rate is not affected when the maximum UE bandwidth is reduced to 20 MHz in FR1 and 50 MHz in FR2. However, a peak data rate reduction can be expected due to the reduced bandwidth and number of MIMO layers.

### Power consumption

From the previous studies in [2], bandwidth reduction can help reduce the power consumption. To be more specific, it is concluded that:

“Reducing the maximum bandwidth provides a reduction in power consumption due to the lower baseband processing requirements in some of the components, possibly including ADC/DAC, FFT, buffering and DL/UL processing blocks. Exactly which components are affected depends on the options being chosen”.

However, if the transmission and/or reception time becomes longer for some channels due to reduced data rates, this may contribute to an increased power consumption as the UE receiver/transmitter need to stay on for a longer time.

The studies in [2] applies to low band, i.e., the LTE bands. In FR2, due to the properties of the RF components, the power consumption properties may not be the same as in the low bands, see chapter 19 in [9]. In FR2, significant power consumption reduction is expected for mixers, ADC and DAC, especially when the UE bandwidth is smaller than 100 MHz. However, depending on the implementation, the insertion loss (IL) for the filters can increase when the passband bandwidth is reduced and may impact the power consumption in the RF frontend. The impact is highly dependent on the RF design and therefore, further studies are required to understand the power consumption profile for UEs with reduced bandwidth in FR2. For FR1, notice that the transmission time for the channels mostly depends on the data rate requirement of the service. Therefore, for sensor type of services with low data rate requirement, the transmission time is not expected to be prolonged due to the reduced bandwidth. However, for some channels, e.g. PDCCH, the reception time may be longer due to reduced bandwidth, coverage recovery compensation and relaxed UE processing time. Therefore, the trade-off between power consumption and UE bandwidth reduction should be considered for the lower data rate services. For wearables with peak rate requirements of 150 Mbps, the expected battery life is multiple days [1]. If 20 MHz and two MIMO layers are applied for these high-end devices, we do not foresee any prolonged transmissions and receptions given the relatively high expected processing capabilities of these UEs.

## Analysis of complexity or cost reduction

According to [1], “The lowest capability considered should be no less than an LTE Category 1bis modem”. Therefore, we can use NR Rel-15 and LTE Cat-1bis as reference points, at least for FR1. In FR1, as the maximum bandwidth is expected to be reduced from 100 MHz to 20 MHz, there can be a cost reduction for RF and/or baseband components. For the RF part, we can expect cost reductions for the power amplifiers and RF transceiver in FR1. From the previous studies in [2], we may expect similar cost saving for the RF part in FR1.

For the baseband part, cost reduction can be expected in, FFT/IFFT, post-FFT data buffering, receiver processing block, HARQ buffer, UL processing block, MIMO specific processing blocks. Due to the reduced bandwidth and the expected data rate, following the methodology in [2], we can assume a cost reduction potential of 50% or more. However, studies may be necessary in order to quantify more accurate numbers, especially considering the advancement in technology and manufacturing compared to the study done in [2].

In FR2, if a maximum 50 MHz is considered, cost savings can be expected for mixers, ADC and DAC as well as FFT/IFFT, post-FFT data buffering, receiver processing block, HARQ buffer, UL processing block. However, for some other components, e.g., RF filters and MIMO specific processing blocks, the cost needs to be further studied. For example, as stated in [9], depending on the implementations, smaller bandwidth in FR2 may give various challenges in the filter designs.

## Impact on specifications

### Cell search and system information acquisition

As discussed above, an SSB consists of 240 contiguous subcarriers. If a minimum of 20 MHz bandwidth is used in FR1 and a minimum of 50 MHz bandwidth is used in FR2, the specification for SSB is not expected to be impacted. Although the SSB bandwidth is 57.6 MHz in FR2 when the 240 kHz subcarrier spacing is used, the synchronization signal (SS) only spans 127 subcarriers and therefore has a bandwidth much less than 50 MHz. The PBCH can span over the entire 57.6 MHz SSB bandwidth. In this case, a Redcap UE with 50 MHz bandwidth can simply skip receiving the PBCH subcarriers outside of its receive bandwidth. This will have some impact on PBCH coverage, though, which needs to be studied.

After acquiring the SSB, the next step is to acquire other SIBs. In NR, other SIBs are scheduled by the Type0-PDCCH CSS. To be more specific, according to TS 38.213,

“Upon detection of a SS/PBCH block, the UE determines from MIB that a CORESET for Type0-PDCCH CSS set, as described in Subclause 13, is present if kSSB <= 23 [4, TS 38.211] for FR1 or if kSSB > 11 for FR2; the CORESET for Type0-PDCCH CSS set may be provided by PDCCH-ConfigCommon.”

The UE procedure for monitoring Type0-PDCCH CSS Sets is given in Subclause 13 of TS 38.213. In Subclause 13 of TS 38.213, we can see that for FR1, CORESET for Type0-PDCCH can be configured as large as 17.28 MHz in frequency domain and up to 3 OFDM symbols. For FR2 assuming 120 kHz subcarrier spacing is used for PDCCH, CORESET for Type0-PDCCH can be configured as large as 69.12 MHz in frequency domain and up to 2 OFDM symbols.

Therefore, for FR1, if a minimum of 20 MHz UE bandwidth is assumed, all the current configurations of CORESET for Type0-PDCCH can be supported. However, for FR2, if a minimum bandwidth of 50 MHz UE bandwidth is assumed, backward-compatible enhancements to the CORESET for Type0-PDCCH may need to be considered for the Redcap UEs to access the system. For example, if the CORESET for Type0-PDCCH is configured to 69 MHz, the UE can simply skip the subcarriers that are outside of its Rx bandwidth. This will result in coverage loss for the Redcap UE, which need to be analysed.

In general, from a UE battery consumption point of view, it is better for the UE to quickly identify whether it can camp on a cell. Therefore, it is beneficial for the UE to as early as possible identify whether the cell supports Redcap. For SI blocks, the reduced UE bandwidth will have no impact on the SI information acquisition if we introduce new SI blocks only for Redcap UEs. However, if the NR SI blocks need to be shared with Redcap UEs, then the NR SI blocks need to be transmitted within the bandwidth of the Redcap UEs. As some of the SI blocks in NR can be configured to be transmitted based on the UE’s demand, the additional overhead from introducing some new sets of SI blocks only for Redcap UEs is expected to be small.

In terms of system overhead and scheduling flexibility, there is a trade-off between introducing new sets of SIs for Redcap UEs or (partly) reuse the existing NR SI blocks, which requires further studies.

If 20 MHz UE bandwidth is assumed in FR1 and 50 MHz UE bandwidth is assumed in FR2, there is little impact on the cell search and system information acquisition for Redcap UEs.

Support of 50 MHz maximum bandwidth is preferred for the Redcap UEs in FR2.

### Random access procedure

This includes the preamble transmission on PRACH, Message 3 transmission on PUSCH, Message 2/4 transmissions on PDSCH, and the corresponding signaling (e.g. grants, HARQ-ACK).

NR defines two kinds of PRACH preambles, i.e., short preambles and long preambles. They differ in lengths and numerologies. Long preambles have two numerologies, i.e., 1.25 kHz and 5 kHz, and can only be used in FR1. If we use the 15 kHz frame structure as reference, a preamble with 1.25 kHz numerology occupies 6 RBs and a preamble with 5 kHz numerology occupies 24 RBs. For short preambles, 15 kHz or 30 kHz subcarrier spacing is used for FR1 and 60 kHz or 120 kHz subcarrier spacing is used for FR2. A short preamble always occupies 12 RBs in the frequency domain regardless of the preamble numerology. Therefore, the PRACH bandwidth both for FR1 and FR2 is within the reduced bandwidth discussed above and we can conclude that the reduction in UE bandwidth has no impact on the transmission of the PRACH preambles.

However, when it comes to message 2/3/4 transmission, there may be some challenge. In NR, message 2 and message 4 are transmitted within the CORESET#0 bandwidth. Notice that the scheduling information of message 3 is carried in the random-access response rather than scheduled by PDCCH. Therefore, if the CORESET#0 bandwidth is larger than the bandwidth supported by the UE, the gNB has to identify that a UE is a Redcap UE and allocate the resources of message 2/3/4 correspondingly. For FR1 this is not a problem, if a maximum of 20 MHz bandwidth is assumed to be supported by the UE. But for FR2, this can be potentially an issue if a maximum of 50 MHz bandwidth is assumed to be supported by the UE and CORESET#0 bandwidth is configured to 69 MHz. However, it may be possible to use a UE based implementation solution to make the system work without specification change. For example, the UE can simply skip the subcarriers that are outside of its Rx bandwidth. It is understood that this will result in certain coverage loss for Redcap UEs. Additionally, there might be a cost of the random-access capacity of the NR cell, as all the UEs (both NR and Redcap UEs) need to be treated in the same way until the gNB knows their capabilities.

### Channels in connected mode

After finishing the random-access procedures, the UE enters the connected mode. In connected mode, once the gNB knows the UE capabilities, it can configure the DL and UL channels, e.g. PDSCH, PDCCH, PUCCH and PUSCH, accordingly. Some impacts on the specifications are expected for these channels in connected mode. The expected impacts are because of the potential coverage loss and the reduced scheduling flexibility due to bandwidth reduction. Notice that repetitions of various channels are supported by NR and some of the coverage losses can be recovered by HARQ retransmission or automatic repetition (e.g. slot aggregation) also. But the coverage loss due the UE complexity reduction needs to be studied further.

The reduced bandwidth also reduces scheduling flexibility both for UL and DL. For PDCCH, the capacity may be reduced due to the reduced bandwidth, especially for high aggregation levels, e.g., AL = 16. There may also be restrictions on the interleaved CCE-to-REG mapping due to the reduced bandwidth. For PDSCH, no impact is foreseen, as the current scheduling granularity can be as small as 1 RB and repetition over multiple slots is supported. Similarly, no spec impacts are expected for PUSCH and PUCCH.

For FR1, a 20 MHz UE maximum bandwidth should be supported. One single type of Redcap UE with at least two different capabilities is considered, i.e., one low data rate services and another for high data rate services.

For FR2, a 50 MHz UE maximum bandwidth should be supported.

# Reduced number of UE antennas

## Scope

3GPP has traditionally limited the specifications to performance requirements defined at the antenna connector. For LTE and NR FR1, the antenna connector concept provides a clear distinction between a receiver (or transmitter) branch and the radiating antenna. With the introduction of NR FR2, the scope of the specifications has grown to include radiated measurements, and therefore naturally puts requirements on the actual antenna implementation. The distinction between the receiver (or transmitter) and the antenna has also been loosened due to advanced antenna functionality required for beamforming, and the tight integration of the RFIC and the antenna at mm-wave bands. A physical antenna connector may in practice no longer exist in FR2 devices.

For the sake of simplicity and for the purpose of facilitating an efficient discussion, we will however in this paper assume that:

- A distinct interface between antenna and transmitter/receiver branch is supported both in FR1 and FR2. Note that RAN4 often uses the term antenna port in its specifications to denote this interface. This is not the same as the RAN1 antenna port concept. We therefore prefer to use the term antenna connector to refer to this interface, which can also be found in RAN4 specifications.

- An antenna is defined by one or more antenna panels.

- Each antenna panel contains an array of one or more radiating antenna elements.

- Each antenna element supports single or dual polarization.

The below figure presents a high level illustration to exemplify the above assumptions. The figure shows an implementation containing two transceivers, each comprising a transmitter and receiver branch. The implementation hence supports two receiver and two transmitter branches. Each set of transmit and receive branches is connected to the antenna via the antenna connectors. The antenna includes two antenna panels, which may e.g. be mounted on each side of a UE. The antenna panels supports V- and H-polarizations, and each transceiver is connected to one of the two polarizations. The antennas are fed by an analogue power distribution network, which in the context of the transmit operation may support a switch for selecting one of the two panels, while it for receive operation may comprise a combiner. The antenna is furthermore supported by an analogue beamforming network implementing tapering for each of the radiating antenna elements in the antenna panels. The intention here is illustrate the separation between receive/transmit branches and the radiated antenna and its related functionality (e.g. the beamforming network), and not to accurately depict a state-of-the-art implementation. Note that TR 38.803 in some details presents the UE RF reference architecture assumed for NR FR2.

Figure 1 Illustrating a pair of RF transceivers, a pair of antenna panels, and their interface, i.e. the antenna connectors.

The above discussions make a clear distinction between the transmit/receive branches and the antenna.  Despite this it is our assumption that the objective to study a “Reduced number of UE RX/TX antennas“ [1] should include a study of a reduction in number of receiver and transmitter branches as well, as such a reduction may also reduce the number of RX/TX antennas.

The scope of the objective to study a “Reduced number of UE RX/TX antennas“ includes a study of the reduction in number of receiver and transmitter branches.

## Release 15 antenna and receiver/transmitter branch support

The UE requirements in TS 38.101-1/2 give a good understanding of the assumed UE capability in terms of number of receiver and transmitter branches, number of antenna panels, number of antenna elements per panel and their polarization support. This capability can be regarded as the baseline in the study on antenna complexity reduction stemming from a reduced number of UE RX/TX antennas. It needs to be emphasized that the discussion in the next two sections present an indication of a possible NR UE antenna configuration and does not preclude that UE vendors may use other implementations that fulfils the RAN4 requirements.

### FR1

In FR1 all requirements are based on conducted measurements, and the antenna implementation is left up to implementation. The specification requires support for four receiver branches in bands n7, n38, n41, n77, n78, n79, and otherwise support for two receiver branches (see TR 38.817 for the detailed background). An exception exists for automotive applications, which may use 2 receiver branches also in the bands originally intended for 4 receiver branches (see TS 38.300). It can be noted that DL MIMO support can be signalled for up to 8 layers (see TS 38.331), meaning that FR1 UEs in principle may support up to 8 receiver branches.

Support for a single transmitter branch is enough unless advanced transmission schemes such as UL-MIMO are supported. UL-MIMO may be supported for rank 2 or 4 indicating support for 2 or 4 transmitter branches.

The below table summarizes these observations to indicate a NR FR1 UE implementation that can be used as reference in the complexity reduction discussions.

Table 1 Summary of Release 15 UE reference for FR1.

| Frequency range | Release 15 RX/TX branch reference | Release 15 RX/TX antenna reference | Comments |
| --- | --- | --- | --- |
| FR1 | 2 or 4 RX branches depending on band. 1 TX branch | N/A | 3GPP does not specify antenna aspects for FR1. An exception allowing 2 instead of 4 RX branches is approved for automotive applications. Optional support for MIMO requires higher number of RX/TX branches. |

### FR2

In FR2 the requirements do not explicitly indicate the antenna configuration used. However, the RAN4 agreements indicate that power class 3 definitions, e.g. min peak EIRP (Equivalent Isotopically Radiated Power) and spherical coverage, are based on a de facto standard implementation using a single transmitter connected to a pair of antenna arrays, with each array supporting dual polarized antenna elements. TR 38.817 indicates that transmission on a single antenna panel is enough for the min peak EIRP measurement, and that a dual antenna panel configuration is needed for meeting the spherical coverage requirements.

The receiver side reference sensitivity requirements requires the use of two receiver branches (see R4-1709140). Since FR2 supports only TDD bands, the receiver branches are expected to be connected to the same antenna configuration as the transmitter branches (see Figure 1). This is also confirmed by e.g. R4-1800289, which indicates that each receiver is connected to an antenna panel comprising four antenna elements. The reference sensitivity requirement requires that each supported polarization is measured independently (see TR 38.101-2) and was defined under the assumption of no receiver diversity gain (see e.g. R4-1800337). This indicates that the two receiver panels may be assumed to be associated with orthogonal polarizations.

The EIS (Effective Isotropic Sensitivity) spherical coverage was defined based on the EIRP spherical coverage requirement, implying that it was based on two antenna panels, each supporting 4 antenna elements.

The below table summarizes these observations to indicate a NR FR2 UE implementation that can be used as reference in the complexity reduction discussions.

Table 2 Summary of Release 15 UE baseline for FR2.

| Frequency range | Release 15 RX/TX branch reference | Release 15 RX/TX antenna reference | Comments |
| --- | --- | --- | --- |
| FR2 | 2 RX branches 1 TX branch | 2 antenna panels, each panel supporting 4 dual polarized antenna elements | Antenna baseline is inferred from the RAN4 radiated requirements. The UE needs to support one panel on the front of the UE, and one on the back of the UE. TX and RX branches can be assumed to make use of the same antenna configuration. Optional support for MIMO requires higher number of RX/TX branches. |

## Suitable UE antenna and receiver/transmitter branch configuration for Redcap UEs

### FR1

To accommodate a lower UE complexity for IoT use cases, a reduced number of transmit and receive branches and antennas can be considered. In FR1, the antenna implementation is outside of 3GPP’s scope and the focus should be on studying a suitable number of transmit and receive branches. If support for UL MIMO is not required, then it is enough to support a single transmitter branch already today.

Redcap support for a single transmitter branch in FR1 is a suitable baseline assumption.

For the receiver side, two or four receiver branches are supported today. The reference sensitivity diversity gain assumed by RAN4 is 3 dB when going from 1 to 2 receive branches, and above 2 dB when going from 2 to 4 receive branches (see TS 38.101). It is proposed to first consider if it is suitable to limit the receiver diversity to support 2 receiver branches for all bands. This is expected to offer a significant complexity reduction at the cost of losing some receiver diversity gain. The step of going from 2 to 1 receiver is more difficult to justify as it will eliminate any receiver combining schemes completely. Low-complexity alternatives supporting a single receiver are already available in LTE. It should also be noticed that a reduced number of receive antennas does not only impact performance requirements, it also impacts RRM accuracy and capability.

Redcap support for 2 receiver branches in FR1 is a suitable baseline assumption.

With that said, it’s clear that 2 receive branches demand support for 2 antennas which in FR1 may impact the supported device form factor. RAN1 should therefore discuss the need to allow support of just a single receive branch in FR1.

Support for a single receiver branch in FR1 for facilitating compact form factors deserves further discussion.

Table 3 Summary of suggested Release 17 baseline for Redcap in FR1.

| Frequency range | Redcap RX/TX branch baseline | Redcap RX/TX antenna baseline | Comments |
| --- | --- | --- | --- |
| FR1 | 2 RX branches 1 TX branch | N/A | Support for a single RX branch in FR1 deserves further discussion. |

### FR2

For FR2, just as for FR1, it is enough to support a single transmitter branch if more advanced features such as MIMO are neglected.

Redcap support for a single transmitter branch in FR2 is a suitable baseline assumption.

For the receiver side, again we see clear benefits for supporting two receiver branches. In FR2, there should not be an issue with this with regards to the form factor.

Redcap support for 2 receiver branches in FR2 is a suitable baseline assumption.

In FR2, the antenna configuration also needs to be considered. The number of panels and the number of elements per panel define the power class capabilities, the reference sensitivity, and the beam correspondence. R4-1707330 provides a relevant investigation of the spherical coverage achievable for a UE using 1, 2, 3 or 4 antenna panels. It is shown that the incremental loss in spherical coverage for a handheld UE is largest when going from 2 to 1 antenna panel. In some directions, it is as large as ~7 dB. In general cases this appears to be a high cost. However, for some IoT applications it can be expected that the device is fixed to a wall, and then the spherical coverage difference between 1 and 2 panels is expected to be lower which may motivate a simplified RF architecture comprising a single antenna panel. This observation applies equally well both for the TX and the RX.

Redcap support for radiated requirements corresponding to at least two antenna panels (i.e. same as reference implementation) is a suitable baseline assumption.

For some stationary IoT devices radiated requirements corresponding to one antenna panel may be justified and further discussion on this relaxation is required in RAN1.

The number of antenna elements per panel is decisive for the EIRP and spherical coverage requirements. In R4-1714445 it is shown that the TX chain implementation loss in mm-bands may range up to ~11.5 dB. To compensate for this loss and guarantee that the currently specified min peak EIRP levels are maintained then each panel needs to support a gain corresponding to that provided by at least 4 cross-polarized antenna elements, if assuming a 4-dB element gain per polarization dimension (see R4-1714445).

To compensate for the potential losses in a handheld UE an antenna capability equivalent to at least 4 cross polarized antenna elements are expected to be required per panel.

In R4-1712319 the losses are explained, and its seen that in some cases they can be reduced e.g. if limiting the band support to a single band. On the other hand, it appears reasonable to assume that the form factor integration loss may increase in Redcap. RAN1 needs to study the expected losses in a Redcap UE to determine a suitable antenna gain per panel which can be associated to a certain number of antenna elements per panel. It also needs to be considered whether a reduced EIRP as a result of antenna gain reduction is within the scope of the study. If not then it will be challenging, or even impossible, to reduce the antenna gain and the number of elements per panel, or to allow for larger implementation margins.

RAN1 needs to study the expected losses in a Redcap UE to determine a suitable antenna panel gain, which can be associated to a number of antenna elements per panel to facilitate a discussion on antenna complexity reduction.

RAN1 needs to consider if a reduced EIRP as a result of antenna gain reduction is in the scope of the Redcap SI, since EIRP is closely coupled to the implementation margins and the antenna design.

Table 4 Summary of suggested Release 17 baseline for Redcap in FR2.

| Frequency range | Redcap RX/TX branch baseline | Redcap RX/TX antenna baseline | Comments |
| --- | --- | --- | --- |
| FR2 | 2 RX branches 1 TX branch | 2 antenna panels | Support for a single antenna panel in FR2 deserves further discussion. The number of antenna elements per panel should be determined based on expected Redcap UE RF losses, and level of permitted EIRP relaxation. |

## Analysis of performance against performance requirements

### FR1

A reduction in the number of receive branches will affect the demodulation performance and the ability to support higher channel rank (i.e. MIMO). TS 38.101-4 presents demodulation performance for various channels for a UE using 2 or 4 receiver branches, and it is seen that the difference is roughly 2.5 dB for AWGN channel conditions. For fading conditions, the difference is expected to increase.

In FR1 the impact on performance from supporting 2 instead of 4 receiver branches is expected to be around 2.5 dB in AWGN conditions.

### FR2

A reduction in the antenna configuration is expected to directly impact the coverage supported by Redcap UEs. The receiver power Prx in a wireless system operating in line-of-sight conditions can be determined as:

Prx = Ptx*Gtx*(1/4πd2)*Arx = EIRP*(1/4πd2)*Grx*λ2/4π	(Eq. 1)

Where Ptx is the conducted transmitted power, Gtx is the transmitter antenna gain, d is the transmitter-to-receiver distance, Arx is the receiver antenna aperture, and λ is the wavelength. If we use the fact that that Arx = Grx*λ2/4π then Eq. 1 can rewritten:

Prx = Ptx*Gtx*Grx*λ2/(4πd)2	(Eq. 2)

Eq. 2 clearly shows that the attenuation between transmitting and receiving nodes is directly dependent on the receiver and transmitter antenna gains. For Redcap this means that any reduction in the antenna configuration will impact both uplink and downlink coverage. The reduced peak antenna gain, in the boresight of the antenna beam, due to going from 4 to 2 elements per panels is 3 dB which impacts UL and DL performance equally. In the transmitter it’s expected that each antenna element is associated with a PA, so this reduction will also reduce the number of PAs which means that the aggregated conducted power, i.e. the total radiated power (TRP), is reduced by an additional 3 dB.

The UL loss in peak EIRP is anticipated to be 6 dB when reducing the number of antenna elements (and PAs) from 4 to 2.

The impact on DL performance is expected to be 3 dB in the DL due to the reduced peak antenna gain when reducing the number of antenna elements from 4 to 2.

In the uplink the impact can be compensated for by an increase in the power amplifier output power. In R4-1712319 its shown that the conducted power for the supported mm-bands is in the range 12-14 dBm. For Redcap we do however assume that it is not wanted to increase the PA operating point.

In FR2 it’s assumed that a Redcap UE should not require an increased PA operating power compared to Rel-15/16 UEs.

Beyond the impact stemming from a reduced peak antenna gain it’s of high relevance to assess the more general impact on spherical coverage when going from 4 to 2 antenna elements per antenna panel. Figure 2 shows the simulated spherical coverage for a reference UE and an envisioned Redcap UE. The reference UE supports 2 antenna panels (on the UEs front and back), each comprising four λ/2-spaced λ/2-dipole antenna elements. It supports a TRP of 15 dBm and a peak EIRP of 23 dBm and meets the RAN4 UE power class requirements in TS 38.101-2. The suggested Redcap UE supports 2 antenna panels, each comprising two λ/2-dipole antenna elements. It supports a TRP of 15 dBm and a peak EIRP of 17 dBm. The results show that the loss in spherical coverage never exceeds the 6-dB loss observed at the peak EIRP. It’s also shown that the performance loss can be reduced to 3 dB if the PA operating level is increased by 3 dB to maintain the TRP at 15 dBm.

The loss in spherical coverage when reducing the number of antenna elements from 4 to 2 is expected to be limited by the loss observed at the peak EIRP.

Figure 2 Impact on spherical coverage when reducing the number of antenna elements from 4 to 2 per panel.

## Analysis of complexity or cost reduction

The complexity reduction when going from 4 to 2 receiver branches in FR1 is expected to be significant. The number of antennas, the number of RF chains, and the number of baseband receiver chains can be reduced by a factor of 2. It should be noticed that this relaxation is already agreed for automotive applications.

A reduction in the antenna configuration is also expected to bring down complexity. In FR2 each antenna element is expected to be associated with one TX PA (possibly two if one PA per polarisation is assumed), one RX LNA and the beamforming network needed for analog beam forming. To mitigate the increase in phase noise expected in FR2 each antenna element may also be associated with its own frequency reference (e.g. LO or PLL). A reduced antenna gain and TRP stemming from a reduced number of PAs is also expected to simplify the overall design including how to manage heating and formfactor related aspects. It is also a tedious task to perform linearization over multiple parallel PAs and to tune the RF circuits e.g. to achieve high beam forming gains, so a reduced antenna capability is expected to reduce the efforts required for design, implementation and testing.

Depending on the implementation, parts of the implementation such as the beamforming network may also be band-specific, meaning that the complexity reduction may scale with the number of supported frequency bands.

A reduction in antenna configuration is expected to bring observable benefits in terms of HW complexity reductions and in reduced design and testing effort.

## Impact on specifications

A reduced number of receiver branches in FR1 is expected to impact several aspects of the RAN4 specifications including demodulation, RF and RRM. In FR2 a new antenna configuration for Redcap will directly impact at least the RF parts in the RAN4 requirements. A preliminary study on the impact can be made once RAN1’s view on a suitable antenna configuration for Redcap becomes clear. An in-depth study of the impact requires RAN4’s participation, which can wait until a potential WI on Redcap.

# Half-duplex FDD

## Description

Half-duplex FDD (HD-FDD) is a technique adopted in LTE-M and NB-IoT for lowering the device cost. With HD-FDD, the device does not need to simultaneously transmit and receive at the same time. This allows the device to use a switch in place of one or more duplex filters. However, this requires the gNB scheduler to make proper scheduling decisions, and to be aware that the UE is in fact a HD-FDD device, so that the device is not scheduled to transmit in the uplink and receive in the downlink at the same time. Furthermore, a guard period may be needed when the device transitions from receive to transmit and vice versa. This will need to be considered by the scheduler as well.

As of Rel-16, since the FDD bands are all in FR1, HD-FDD is only pertinent to FR1.

## Analysis of performance against performance requirements

### Data rate requirements

For wireless sensor and video surveillance use cases, it is expected that the traffic is more uplink oriented. In this case, the scheduler has great flexibility to schedule the device for uplink transmissions to ensure the data rate requirements can be met. For example, given a device UE bandwidth of 20 MHz and QPSK modulation, the device in theory can support up to approximately 30 Mbps after accounting for all the overheads, e.g. DMRS, PDCCH, etc. This is enough for meeting the 2 Mbps data rate requirement for sensors, 2-4 Mbps for economic video, or 7.5-25 Mbps for high-end video cameras.

For wearables, the reference data rate requirements are 10-50 Mbps in DL and minimum 5 Mbps in UL. This can be supported with, for example, 20 MHz device bandwidth and 64QAM. This would allow enough margin to account for the throughput lost due to transitioning from Rx to Tx, or from Tx to Rx, should the reference data rate requirements need to be achieved simultaneously for both DL and UL.

The peak data rates of wearable use cases are 150 Mbps for the DL and 50 Mbps for the UL, according to [1]. These data rates can be achieved for an HD-FDD device provided that the device bandwidth is 20 MHz and support 64QAM and up to 2 MIMO layers.

An HD-FDD device supporting 20 MHz device bandwidth can fulfil all the Rel-17 Redcap data rate requirements.

### Latency requirements

Rel-17 latency requirements for a Redcap device is very relaxed for all use cases, e.g., 100 ms for non-safety related sensors and 500 ms for surveillance cameras. The only exception is for safety related sensors, which require 5-10 ms latency. In our view, a HD-FDD Redcap device would have no problem meeting the 100 ms latency requirement. However, whether a HD-FDD device can meet the 5-10 ms latency requirement for safety related sensors requires further study.

It requires further study to determine whether a HD-FDD device can meet the 5-10 ms latency requirement for safety related sensors

### Coverage

In [2], it was concluded that HD-FDD will not result in a loss in coverage for delay-tolerant traffic in the LTE-M study. We do not see any reason for drawing a different conclusion in NR.

HD-FDD will not results in a loss in coverage.

### Power consumption and energy efficiency

In [2], the following findings are captured:

“The insertion loss of the switch in the HD-FDD UE is less than in the duplexer of an FD-FDD UE: reducing the electrical power required to produce a certain amount of radiated RF power. Half duplex operation means some components can be put in a reduced power state until required. It is recognised that RF and baseband power consumption is often dictated by implementation.”

These findings also apply to NR. However, we note that an HD-FDD device may stay ON for a longer time to finish a data session compared to an FD-HDD device that is capable for transmit and receive at the same time. A longer data session has a negative impact on energy efficiency. The overall impact of HD-FDD on energy efficiency needs to be studied.

## Analysis of complexity or cost reduction

In [2], an analysis on the cost reduction achieved by HD-FDD is included. The cost saving factors include:

- Use of a switch instead of a duplexer filter. A switch represents a small percentage of the cost of the duplexer filter.

- Reduced complexity and memory in the baseband module because there is no need to provision processing power for concurrent downlink and uplink operations.

Additionally, certain type of HD-FDD operation, e.g. Type-B discussed below, enables the UE to operate using only one single oscillator. This contributes to cost saving.

The analysis in [2] uses LTE FD-FDD as a baseline, and the cost reduction achieved by HD-FDD is quantified. It was concluded in [2] that for a single-band device, using a switch instead of a duplexer filter provides an overall cost saving of 4%-8%. It was further noted that the potential relative cost reduction may be even larger for multi-band devices (that may have multiple duplexer filters).

Regarding cost saving from reduced computational requirements, the results in [2] were mixed. Some sources indicate no benefit at all, whereas some sources indicate 5%-10.5% saving in the baseband module.

In terms of total cost saving, the results captured in [2] range from 4%-19%. As most of the NR FDD bands are also LTE bands, we expect the findings in [2] to be valid for NR.

## Impact on specifications

There is already a notion of non-full-duplex device according to TS 38.211:

“A UE not capable of full-duplex communication and not supporting simultaneous transmission and reception as defined by parameter simultaneousRxTxInterBandENDC, simultaneousRxTxInterBandCA or simultaneousRxTxSUL [10, TS 38.306] among all cells within a group of cells is not expected to transmit in the uplink in one cell within the group of cells earlier than NRx-TxTc after the end of the last received downlink symbol in the same or different cell within the group of cells where NRx-Tx is given by Table 4.3.2-3

A UE not capable of full-duplex communication and not supporting simultaneous transmission and reception as defined by parameter simultaneousRxTxInterBandENDC, simultaneousRxTxInterBandCA or simultaneousRxTxSUL [10, TS 38.306] among all cells within a group of cells is not expected to receive in the downlink in one cell within the group of cells earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same or different cell within the group of cells where NTx-Rx is given by Table 4.3.2-3”

As described above, a transition time has been defined to facilitate the transition from Tx and Rx, or vice versa, for a non-full-duplex device. However, the transition time specified in the mentioned table requires a device to have one frequency generator each for Tx and Rx. For an LTE FDD device, such a half-duplex mode operation is referred to as Type-A half-duplex. We will use the same terminology in the discussion below. To support Type-A HD-FDD operation, a simple extension in the TS 38.211 specifications is to broaden the notion of non-full-duplex device to further include Type-A HD-FDD devices.

NR specifications can be easily extended to support Type-A half-duplex FDD operation by broadening the notion of non-full-duplex device to further include Type-A HD-FDD devices.

In the LTE specifications, a second type of HD-FDD operation is defined, namely Type-B HD-FDD, which is intended for additional device cost reduction. Type-B HD-FDD operation allows for a longer transition time between Tx and Rx, which makes possible for the device to operate with a single frequency generator. LTE Cat-0, Cat-M and Cat-NB UEs only need to support Type-B HD-FDD operation, in which case a guard subframe is inserted at every switch from downlink to uplink and from uplink to downlink, giving the device time to retune its carrier frequency. If there is a foreseen benefit in UE cost reduction by enabling a single frequency generator Redcap device, a longer transition time can be defined to support Type-B HD-FDD operation.

Discuss further whether there is enough cost reduction benefit by enabling a single frequency generator HD-FDD device operating in an FDD band to motivate the associated performance loss and changes to specifications and scheduler implementations.

To support a longer transition time beyond currently defined in Table 4.3.2-3 in TS 38.211, there are potential impacts on DCI and scheduler implementation. Regarding DCI, the impact potentially could be on TDRA (time domain resource assignment), and PDSCH-to-HARQ_feedback timing indicator. However, in our view these fields are already sufficiently flexible in dealing with longer gaps between DL and UL, and thus the impact will mainly land on the scheduler implementation. If Type-B HD-FDD is introduced, it is desired to aim to minimize the impact on scheduler implementations.

If Type-B HD-FDD is introduced, it is desired to aim to minimize the impact on scheduler implementations.

Another potential specification impact might be on TS 38.101 considering that HD-FDD may result in a lower noise figure.

Introducing HD-FDD operation might have impact on TS 38.101 due to a reduced device noise figure.

# Relaxed UE processing time

## Description

Another potential UE complexity reduction aspect is related to UE processing time, e.g. PDSCH processing or PUSCH preparation procedure time. In this section, we discuss existing UE processing time capabilities and outline potential impacts if such processing time is relaxed further for Redcap UE. It should be noted that the focus here is on processing that is related to data transmission. There are other parameters related to UE reporting such as CSI, SRS, etc. which are not discussed here.

In NR, there are currently two existing UE processing time capabilities (1 and 2) specified for PDSCH processing and PUSCH preparation time, where capability 2 is a more aggressive version of capability 1. The specified values are sub-carrier spacing dependent and are provided in Table 5 to Table 8 as a reference. More details can be found in Section 5.3 and 6.4 of TS 38.214.

Table 5 PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | Only front-loaded DMRS | Additional DMRS symbol configured or no higher layer parameter configured |
| 0 | 8 | 13 or 14 depending on the position of additional DMRS |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 6 PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |
| --- | --- |
|  | Only front-loaded DMRS |
| 0 | 3 |
| 1 | 4.5 |
| 2 | 9 for frequency range 1 |

Table 7 PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Table 8 PUSCH preparation time for PUSCH timing capability 2

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 5 |
| 1 | 5.5 |
| 2 | 11 for frequency range 1 |

According to the specification, UE PDSCH processing time (N1) is the major part of the minimum time (Tproc,1) between the end of PDSCH and the start of PUCCH carrying the corresponding HARQ-ACK, while UE PUSCH preparation time (N2) is a major part of the minimum time (Tproc,2) between the end of the scheduling PDCCH and the start of the scheduled PUSCH. We note that Tproc,1 includes the time for processing PDCCH, PDSCH, and preparing for PUCCH, while Tproc,2 includes the time for processing PDCCH and preparing for PUSCH. Since the UE processing time capability has a direct impact on achievable transmission latency, we investigate how average latency for different number of HARQ retransmissions would be impacted if the UE processing time were to be relaxed for Redcap.

## Analysis of performance against latency requirements

Most of the relevant use cases for Redcap have rather relaxed latency requirements, e.g. <100 ms for non-safety related industrial wireless sensors, or <500 ms for video surveillance. For such requirements, it can be reasonable to relax UE processing time capability further if the complexity reduction gain is found to be meaningful. However, for some use cases with relatively strict latency requirement, UE processing time plays a crucial role in achieving the performance requirement. The industrial wireless sensors for safety related applications [1] have a latency requirement of 5-10 ms. This level of requirement can be considered rather strict and should be considered when any potential complexity reduction related to UE processing time is discussed.

One example of latency impact analysis is provided below where the UE processing time for PDSCH processing and PUSCH preparation are assumed to be doubled compared to those for Capability 1. Note that this assumption is not based on a thorough analysis and is only used for the purpose of discussion.

The average user-plane one-way UL achievable latencies for different numbers of HARQ retransmissions are given in Table 9 where the latency analysis includes different components of the overall transmission latency as illustrated in Figure 3. We assume TDD pattern = DDSU, i.e. two DL slots followed by a special slot and one UL slot, where the special slot S is assumed to consist of 10 DL symbols, 2 guard symbols, and 2 UL symbols, respectively. Other assumptions include SCS = 30 kHz, PDCCH monitoring occasion periodicity = 1 slot, SR periodicity = 4 slots, SR duration = 14os, PUSCH duration = 14os. For definition of user plane one-way latency, please see [6].

To understand how the numbers in Table 9 are obtained, consider e.g. the latency of initial UL transmission for UE capability #1. On average, there is a waiting time of roughly 2 slots for the SR opportunity. Then SR transmission itself takes 1 slot, followed by some processing at gNB and waiting time for PDCCH occasion which together contribute another 1 slot. The UL grant transmission, UE PUSCH preparation time (dependent on N2), and waiting time for UL symbols according to the assumed TDD pattern for the PUSCH transmission together takes another 2 slots. Lastly, the PUSCH transmission takes 1 slot, followed by some gNB processing time of PUSCH is assumed to be N1/2. That is, in total, the initial UL transmission takes roughly 7.4 slots on average.

Figure 3 Illustration of the analysis of one-way latency considering different components of the overall transmission latency.

Table 9 Average one-way UL latency for different number of HARQ retransmissions

| (SR-based) UL average one-way achievable RAN latency (ms) | UE capability #1 (Rel-15) | Assumed relaxed UE capability (i.e. doubled compared to Cap #1) |
| --- | --- | --- |
| Initial transmission | 3.68 | 5.86 |
| 1 HARQ retransmission | 5.68 | 9.86 |
| 2 HARQ retransmissions | 7.68 | 13.86 |
| 3 HARQ retransmissions | 9.68 | 17.86 |

As seen in Table 9, already for the initial transmission, the relaxed UE processing time capability can impact the average one-way UL latency such that a 5-ms requirement cannot be fulfilled. If a 10-ms latency is considered, then up to 1 HARQ retransmission can be supported. Moreover, other complexity reduction aspects such as reduced number of TX/RX antennas at the UE can impact the performance such as coverage. To recover such potential loss, a larger number of retransmissions may be considered which contributes to overall transmission latency. If UE processing time is relaxed, the latency impact would be amplified even further.

## Analysis of complexity or cost reduction

It may be natural to argue that the timing for PDSCH processing and PUSCH preparation are related to UE complexity, i.e., if the minimum processing time is extended/relaxed, UE complexity would be reduced. For example, with a longer PDSCH processing time, a UE may process PDSCH in a less complex manner without needing to transmit HARQ-ACK in a relatively fast timeframe. This may lead to some baseband cost/complexity reduction and potentially less energy consumption. However, this is unclear and may be implementation dependent. Moreover, relaxed UE processing time could lead to an overall longer period of time for which the UE needs to stay active which can impact the battery life. Therefore, further analysis of the actual complexity/cost reduction gain and other impacts is needed before any conclusion can be made.

As mentioned in Section 5.1 that N1 and N2 are major parts of Tproc,1 and Tproc,2 which both include time for PDCCH processing. If PDCCH monitoring limit is reduced for Redcap, implying that PDCCH monitoring complexity is reduced, the existing UE capability #1 might already be sufficient. Thus, it is unclear if further relaxation on N1 and N2 from the existing UE capability #1 is really needed.

We also note that in general there is a trade-off between complexity reduction, processing latency, and scheduling flexibility. Although relaxing UE processing time may help to reduce UE complexity or cost, it can have an impact on the latency performance which is important for some use cases as shown earlier.

Relaxed UE processing time for PDSCH processing and PUSCH preparation can have significant impact on average transmission latency and scheduling flexibility.

The impact on UE complexity and cost reduction needs to be further investigated as it is unclear and may be implementation dependent.

UE processing time includes time for PDCCH processing. Whether further relaxed UE processing time is required depends on other aspects of complexity reduction, e.g., those related to PDCCH monitoring limits.

Latency requirement and scheduling flexibility should be considered when discussing any potential relaxed UE processing time for PDSCH processing and PUSCH preparation.

Actual cost reduction from a UE perspective needs to be considered to investigate whether the gain from more relaxed UE processing time is justified.

## Impact on specifications

As discussed above, relaxed UE processing time has potential impact on latency performance, scheduling flexibility, and UE complexity/cost. It requires careful investigation whether the gain from more relaxed UE processing time is justified. If a new relaxed UE processing time is introduced, RAN1 specification impact can be expected where another UE processing time capability is introduced in TS 38.214.

# Relaxed UE processing capability for PDSCH/PUSCH

## Description

As mentioned in section 1, industrial wireless sensors, video surveillance, and wearables are the identified use cases for Redcap devices. The expected UL and DL data rates and latencies for the targeted use cases are significantly relaxed compared to the existing requirements for eMBB and URLLC scenarios. With these significantly less stringent requirements on data rates and latencies compared to the existing Rel-16 NR requirements, relaxation in UE processing capability is naturally expected and could be achieved via UE bandwidth reduction as discussed in section 2 and also through the following other identified potential aspects for reduced peak rate for the data channels:

- Reduction of maximum transport block sizes for DL and UL

- Restriction on the maximum modulation order

- Support on DL and UL carrier aggregation is not required

- Support on UL MIMO and DL MIMO may not be required depending on Redcap use cases

In the following sub-sections, the above-mentioned aspects will be analysed and the applicability of each one of them for Redcap devices will be discussed.

## Analysis of performance against performance requirements

### Maximum data rate

In [1], it is stated that the minimum UE capability for Redcap devices should not be lower than that currently supported in LTE Cat-1bis modems which have the following requirements [3]:

Table 10 LTE Cat-1bis basedband and RF requirements.

|  | DL Category 1bis | UL Category 1bis |
| --- | --- | --- |
| Maximum number of DL/UL-SCH transport bits received within a TTI | 10296 | 5160 |
| Maximum physical layer peak data rates | 10.296 Mbps | 5.16 Mbps |
| Supported Modulation schemes | QPSK, 16QAM, 64QAM | QPSK, 16QAM |
| UE bandwidth | 20 MHz | 20 MHz |
| Receive antenna | 1 | NA |
| Maximum number of supported MIMO layer in DL/UL | 1 | 1 |

In section 2, we discussed that a minimum 20-MHz bandwidth should be supported by Redcap devices in FR1. This minimum bandwidth requirement implies that at least 64QAM modulation in DL and 16QAM modulation in UL are required for Redcap UEs in order to meet at least the same performance as that for LTE Cat-1bis modems. In theory, this allows maximum ~100 Mbps and ~70 Mbps bit rates to be achieved in DL and UL respectively if operating in an FDD band. For operation in a TDD band, with widely used TDD patterns, e.g., 3:1 DL:UL, an Redcap device of 20 MHz UE bandwidth can fulfil the data rate requirements of industrial sensor, video surveillance, and typical wearable use cases without MIMO and carrier aggregation.

A Redcap device operating on 20 MHz UE bandwidth can fulfil the data rate requirements of industrial sensor, video surveillance, and typical wearable use cases without MIMO and carrier aggregation.

For high-end wearable use cases, the DL peak rate is as high as 150 Mbps and UL peak rate is 50 Mbps. To achieve these peak rates, a higher UE capability than discussed above is needed. If a Redcap device is limited to 20 MHz UE bandwidth, it would require higher modulation schemes, MIMO and/or CA support as shown in the table below to achieve the expected peak rate:

Table 11 MIMO and CA requirements for FR1 high-end wearables with different maximum modulation orders in a FDD band operation.

| options | UL (50 Mbps bit rates) | DL (150 Mbps bit rates) |
| --- | --- | --- |
| (1) | QPSK + MIMO | QPSK + MIMO + CA |
| (2) | 16QAM and no MIMO or CA | 16QAM + MIMO + CA |
| (3) | 64QAM and no MIMO or CA | 64QAM + MIMO |
| (4) | 256QAM and no MIMO or CA | 256 QAM and no MIMO or CA |

It should be noted that carrier aggregation and higher modulation scheme such as 256QAM would impose more stringent RF requirements on the device (more details on the RF requirements due to CA and 256QAM are presented in section 6.3.) Thus, transmission/reception schemes requiring either the support of 256QAM or carrier aggregation (i.e. such as options (1), (2), (4) for DL and option (4) for UL) may not be preferable, which makes option (1), (2) and (3) for UL and option (3) for DL preferable candidates for high-end wearable devices if operating in a FDD band. For operation in a TDD band with a typical 3:1 DL:UL TDD pattern, 256QAM (without MIMO) or 64QAM+MMIO support would become necessary for high-end wearables for UL transmissions.

Taking the possible constraint that the Redcap devices may have the preference of only 1 TX antenna (as discussed in section 3), imposing MIMO support on UL would increase the hardware cost significantly and may not be desirable.

256QAM modulation in combination with carrier aggregation would impose stringent RF and complexity requirements and are not preferable for Redcap devices.

Modulation orders and transmission schemes with 64QAM + MIMO for DL and 16QAM for UL are preferable for FR1 high-end wearable devices that requires 150 Mbps peak bit rates for DL and 50 Mbps peak bit rates for UL operating in an FDD band.

Similarly, the same exercise above could be applied to FR2 with the bandwidth (i.e. 50 MHz) proposed in section 2. As observed from the discussion for FR1, higher peak bit rates for high-end wearables use cases could be easily achieved with higher modulation order and/or with MIMO. For FR2, as larger bandwidth is proposed, the requirements on the support of highest modulation order and the multi-layer transmission scheme should be in the same order or relaxed. For this reason, we don’t see there would be a need for a special technique to achieve the peak bit rate requirement for FR2 use cases. Thus, only the peak bit rates requirements for high-end wearables are discussed below to illustrate the highest modulation order and multi-layer support required in FR2. .

In FR2, 150 Mbps in DL is also expected. Assuming a UE bandwidth of 50 MHz, peak data rates of ~200Mbps, in FDD bands, and ~150 Mbps, in a TDD band with TDD pattern such as 3:1 DL:UL, could be achieved with 64QAM modulation (without MIMO). Nevertheless, in FR2, the phase noise impact could be significant with 64QAM; and Phase Tracking Reference Signal (PT-RS) for phase noise compensation would be necessary [7]. Alternatively, 16QAM with MIMO could be considered to avoid phase noise impact. Note that with 2RX as baseline (as proposed in section 3), using MIMO scheme would not impose significant cost increase in HW requirements.

In FR2, the required bit rates for Redcap devices operating in an FDD or a typical TDD band could be fulfilled with 64QAM (and no MIMO) which would however require PT-RS for phase noise compensation. Alternatively, 16QAM and MIMO could be considered to avoid phase noise impact.

### Coverage

In [2], it stated that reducing the peak data rate in general does not make the coverage worse. The same should be applicable to NR.

Reducing the peak data rate in general does not make the coverage worse.

### Power consumption

Restricting the peak data rate would not worsen power consumption as stated in [2] unless longer transmission or reception times are required.

In general, processing of smaller TB size and restricting the maximum modulation order may reduce power consumption in baseband processing block and in RF ADC/DAC (as less bit-resolution design could be used at lower modulation order), respectively [2].

Power consumption would not be made worse by restricting the peak data rate.

## Analysis of complexity or cost reduction

In this section, complexity/cost reductions from baseband implementation and RF requirements aspects are discussed.

By restricting TBS, PRBs, and modulation orders in general different cost saving aspects could be achieved due to less restrictive requirements on UL processing blocks, DL channel decoding, HARQ buffering, post-FFT/IFFT data buffering, less precision on ADC/DAC, less restrictive power amplifier, LO phase noise and EVM requirements [2].

The EVM requirements, specified in RAN4 as shown in Table 12, define the modulation or signal quality that the transmitters and receivers shall meet. As shown, as the modulation order increases, the error tolerance decreases which implies higher quality device is required if it is to support higher modulation orders. The main contributions to the modulation inaccuracy are the impairments of RF components such as power amplifier nonlinearity, untracked phase noise and imperfection in IQ modulator.

Table 12 Minimum requirements for error vector magnitude [6].

| Parameter | Unit | Average EVM level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30.0 |
| QPSK | % | 17.5 |
| 16QAM | % | 12.5 |
| 64QAM | % | 8.0 |
| 256QAM | % | 3.5 |

If the Redcap UEs allow two antennas, it may be preferable to avoid high modulation order such as 256QAM, but carrier aggregation or MIMO would become necessary in order to meet the peak data rate requirements for high-end wearable use cases as discussed in section 6.2.1. However, carrier aggregation imposes different RF challenges such as cross- and in-band isolations between aggregated bands and attenuation of harmonic interferences generated by nonlinear components in the RF chains [5]. To achieve acceptable performance, it is necessary to take RF considerations mentioned above into accounts when designing mobile devices that are CA capable. Moreover, with carrier aggregation, the device will be required to monitor and blindly decode information broadcasted in each component carrier that belongs to the band combination that the device supports. This would further increase the implementation complexity and power consumption.

With MIMO, there could be cross-layer interference which would be significantly easier to solve compared to the RF issue that would be encountered with CA. Moreover, if the device would already require multiple antennas as for DL as the baseline requirement discussed in section 3, multiple RF chains with necessary RF components (e.g. antenna, ADC, low-noise amplifier and filter) would already be required to be part of the RX RF design. The additional cost for using MIMO would only then be contributed by additional baseband processing which is relatively small. This makes MIMO more favourable for meeting peak data rate compared to the techniques with high modulation order (i.e. 256QAM) or carrier aggregation provided more than 1 antenna is already required for Redcap devices.

However, if the Redcap devices are not equipped with multiple antennas (hence multiple RF chains), 256 QAM may not be avoided in certain TDD deployment scenarios to meet the required peak bit rates.

If Redcap UEs allow multiple antennas, data reception scheme with MIMO to achieve peak data rate should be considered as it would be more cost effective compared to that with 256QAM or carrier aggregation.

## Impact on specifications

By restricting the maximum data rate, there could be potential impacts on:

- RAN1 specification if optimisation on DCI messages should be considered to remove parameters that will not be assigned.

# Relaxed UE processing capability for PDCCH

## Description

Another aspect that contributes to the complexity of UE processing capability is decoding PDCCH that carries DCI. For a UE to successfully decode DCI, it must perform trial-and-error decoding based on all possible combinations with different parameters such as number of DCI sizes, CCE indexes, aggregation levels and RNTIs, and the amount of processing required on blind decoding can be relatively high and contribute to significant part of the UE power consumption. Thus, consideration of the relaxation on the current maximum number of blind decoding per slot (i.e. PDCCH candidates processing), as shown in Table 13, could be beneficial for low complexity Redcap devices. Furthermore, if the PDCCH decoding process could be completed faster (as the result of blind decoding reduction), UE’s microsleep duration can be increased thus reducing UE power consumption. However, this may increase scheduling inflexibility and potentially increase latency and blocking probability.

Relaxation on the maximum number of blind decoding attempts, and/or the PDCCH decoding processing time, could be beneficial for Redcap devices. However, the impact on the scheduling flexibility and PDCCH capacity needs to be evaluated.

Table 13 Blind decoding and CCE limits in NR.

| SCS [kHz] | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| Max # BD per slot (in NR) | 44 | 36 | 22 | 20 |
| Max # CCEs which require channel estimation per slot (in NR) | 56 | 56 | 48 | 32 |

Potential techniques as discussed in our companion contribution [4] could be considered to reduce the maximum number of blind decoding:

- restricting the PDCCH aggregation levels, and/or

- limiting the number of PDCCH candidates, and/or

- limiting the number of different DCI sizes to be monitored

Note that in LTE, cell-specific reference signals are used for PDCCH demodulation. In NR, each PDCCH candidate has self-contained DM-RS associated with it and the UE needs to estimate the channel using these DM-RS signals. The processing load on channel estimation for PDCCH in NR thus scales with the number of CCEs (i.e. aggregation levels) being monitored.

Restricting the PDCCH aggregation levels (consequently, CCEs for channel estimation), the number of PDCCH candidates and the number of different DCI sizes could be potential techniques in processing capability/complexity relaxation for the UE.

In the following sub-sections, the above-mentioned aspects will be analysed and the applicability of each one of them for Redcap devices will be discussed.

## Analysis of performance against performance requirements

### Coverage

In NR, there are {1, 2, 4, 8, 16} PDCCH aggregation levels (i.e. number of CCEs) and with each aggregation level it can be allocated with 1, 2 or 3 OFDM symbols with different minimum CORESET bandwidth requirements. As discussed in section 2, we considered UE bandwidth with 20 MHz for FR1 and 50 MHz for FR2 may be sufficient for Redcap use cases. From our analysis discussed in our companion contribution on PDCCH consideration [4], we showed that for Redcap UEs with 20 MHz BW in FR1, AL 16 for one-symbol CORESET duration may not be supported. For Redcap UEs with 50 MHz BW in FR2, the CORESET (other than CORESET#0) BW in the following configurations exceeds the UE BW:

- AL 16 for 120 kHz SCS, regardless CORESET duration

- AL 8 for 120 kHz SCS, one-symbol CORESET duration

- AL 16 for 60 kHz SCS, one-symbol CORESET duration

In general, in larger cells and extreme coverage scenarios, higher AL is required. In FR1, coverage loss is not expected due to reducing the maximum UE bandwidth to 20 MHz. However, more than one-symbol CORESET duration will be required to achieve highest aggregation level in extreme coverage scenarios. Whereas in FR2, techniques such as time repetition and/or reducing DCI sizes may be required for coverage recovery as the result of limited UE bandwidth and thus limited supported ALs.

Reducing the Redcap UE bandwidth may restrict the highest supported aggregation levels for the UE which can reduce the coverage.

As discussed in [4], with 20 MHz BW in FR1, all CORESET #0 configurations can be fully supported. However, for an FR2 Redcap UE with e.g., 50 MHz bandwidth, the CORESET #0 BW may exceed the UE BW in the following configurations:

- 120 kHz SCS:

  - CORESET with 48 RBs and one OFDM symbol

  - CORESET with 48 RBs and two OFDM symbols

- 60 kHz SCS:

  - CORESET with 96 RBs and one OFDM symbol

  - CORESET with 96 RBs and two OFDM symbols

Therefore, in the above CORESET #0 configurations, the potential impact on the PDCCH decoding performance needs to be evaluated.

With 20 MHz BW in FR1, all CORESET #0 configurations can be fully supported.

Reducing the Redcap UE bandwidth may impact the PDCCH performance in some CORESET #0 configurations (particularly in FR2). The potential impact needs to be further studied.

### Power consumption

The overall PDCCH power saving due to the blind decoding reduction depends on the scenario and the contribution of the PDCCH monitoring part. As from the example given in our companion contribution [4], assume PDCCH monitoring corresponds to 60% of the total UE power consumption, reducing the number of BDs by half can provide 9% power saving. However, this could have an impact on the scheduling flexibility and blocking probability.

It should be noted that according to the study in [8], the UE power saving from increasing the PDCCH monitoring period (and thus relaxing processing load on channel estimation) is expected to be higher than that archived by blind decoding reduction.

Power saving could be achieved by relaxing the number of blind decoding per slot and/or CCEs for channel estimation. However, by increasing the PDCCH monitoring period, the power saving gain achieved can be higher and may not be necessary to reduce the number of blind decoding limits.

## Analysis of complexity or cost reduction

In this section, complexity/cost reductions from baseband implementation and RF requirements aspects are discussed.

Reducing the maximum number of PDCCH blind decodes would mainly relax the amount of baseband processing that the device must perform for PDCCH. The degree of complexity relaxation on PDCCH would depend on the techniques.

Note that relaxation of PDCCH blind decoding or CCEs could potentially reduce scheduling flexibility and the impact on blocking probability need to be considered.

When considering cost reduction by reducing the number of PDCCH blind decoding per slot and/or CCEs for channel estimation, the impacts on scheduling flexibility and blocking probability should also be considered.

## Impact on specifications

Potential techniques for reducing the number of PDCCH blind decoding and CCEs for channel estimations could have impacts on RAN1, RAN2 and RAN4 specifications.

# Conclusion

Considering the complexity and cost reduction aspects discussed in this contribution, a Redcap UE type in FR1 and FR2 can be defined according to Table 14 and Table 15 shown below. As mentioned in the Introduction section, the notion of “Redcap UE type” refers to a minimum set of UE capabilities that are common to all Redcap UEs. This however does not prohibit a UE to support additional capability. The additional UE capability can be signalled to the network.

In FR1, we propose the minimum set of Redcap UE capabilities includes the support of 20 MHz bandwidth. In our view, the UE ecosystem is well positioned to optimize the implementation of a 20 MHz UE platform, leveraging the know-how and investment in developing LTE UE product over many years.

Table 14: Redcap UE Type in FR1

| Capability | Baseline | Comments |
| --- | --- | --- |
| Channel bandwidth | 20 MHz | Includes support of smaller channel bandwidths |
| Number of Tx branches | 1 |  |
| Number of Rx branches | [2] | Support for a single RX branch in FR1 deserves further discussion. |
| Duplex mode | HD-FDD, TDD | Support of FD-FDD may be indicated through UE capacity signaling |
| UE processing capability (N1, N2 values) | TBD | Need further discussion to assess - Whether relaxation relative to Capability 1 has meaningful cost reduction benefit - Scheduler impact - Latency impact |
| # active BWP | TBD | Need to assess whether relaxation relative to Rel-15 baseline has meaningful benefit in cost reduction |
| # of CORESET per BWP | TBD | Need to assess whether relaxation relative to Rel-15 baseline has meaningful benefit in cost reduction |
| # of BD and CCE limits per slot | TBD | Need further discussion to asses - Whether relaxation relative to Release 15 baseline 1 has meaningful cost reduction benefit - Scheduler impact |
| CA support | No |  |
| DL MIMO support* | No | For achieving the peak DL bit rate for high-end wearable use cases, 2-layer MIMO is needed. The additional UE capability beyond the baseline can be handled through UE capability signaling. |
| DL modulation* | QPSK, 16QAM, 64QAM | Additional HoM support can be provided through UE capability signaling |
| UL modulation* | QPSK, 16QAM | Additional HoM support can be provided through UE capability signaling |
| Items with asterisk mark (*) may be left out and handled through UE capability signaling |  |  |

Table 15: Redcap UE Type in FR2

| Capability | Baseline | Comments |
| --- | --- | --- |
| Channel bandwidth | 50 MHz |  |
| Number of Tx branches | 1 |  |
| Number of Rx branches | 2 |  |
| Duplex mode | TDD |  |
| Antenna configuration (reflected in RAN4 requirements) | [2] antenna panels | Antenna baseline is reflected in the RAN4 radiated requirements. Support for a single antenna panel deserves further discussion. The number of antenna elements per panel should be determined based on expected Redcap UE RF losses, and level of permitted EIRP relaxation. |
| UE processing capability (N1, N2 values) | TBD | Need further discussion to assess - Whether relaxation relative to Capability 1 has meaningful cost reduction benefit - Scheduler impact - Latency impact |
| # active BWP | TBD | Need to assess whether relaxation relative to Rel-15 baseline has meaningful benefit in cost reduction |
| # of CORESET per BWP | TBD | Need to assess whether relaxation relative to Rel-15 baseline has meaningful benefit in cost reduction |
| # of BD and CCE limits per slot | TBD | Need further discussion to assess - Whether relaxation relative to Release 15 baseline 1 has meaningful cost reduction benefit - Scheduler impact |
| CA support | No |  |
| DL MIMO support | 2-layer |  |
| DL modulation* | QPSK, 16QAM | Additional HoM support can be provided through UE capability signaling |
| UL modulation* | QPSK, 16QAM | Additional HoM support can be provided through UE capability signaling |
| Items with asterisk mark (*) may be left out and handled through UE capability signaling |  |  |

Observations made throughout this contribution are collected below.

Observation 1	Support of 20 MHz maximum bandwidth is preferred for the Redcap UEs in FR1. The Redcap UEs are also expected to operate on BWP or carrier bandwidth that is smaller than 20 MHz in FR1.

Observation 2	In terms of system overhead and scheduling flexibility, there is a trade-off between introducing new sets of SIs for Redcap UEs or (partly) reuse the existing NR SI blocks, which requires further studies.

Observation 3	If 20 MHz UE bandwidth is assumed in FR1 and 50 MHz UE bandwidth is assumed in FR2, there is little impact on the cell search and system information acquisition for Redcap UEs.

Observation 4	Support of 50 MHz maximum bandwidth is preferred for the Redcap UEs in FR2.

Observation 5	To compensate for the potential losses in a handheld UE an antenna capability equivalent to at least 4 cross polarized antenna elements are expected to be required per panel.

Observation 6	In FR1 the impact on performance from supporting 2 instead of 4 receiver branches is expected to be around 2.5 dB in AWGN conditions.

Observation 7	The UL loss in peak EIRP is anticipated to be 6 dB when reducing the number of antenna elements (and PAs) from 4 to 2.

Observation 8	The impact on DL performance is expected to be 3 dB in the DL due to the reduced peak antenna gain when reducing the number of antenna elements from 4 to 2.

Observation 9	In FR2 it’s assumed that a Redcap UE should not require an increased PA operating power compared to Rel-15/16 UEs.

Observation 10	The loss in spherical coverage when reducing the number of antenna elements from 4 to 2 is expected to be limited by the loss observed at the peak EIRP.

Observation 11	A reduction in antenna configuration is expected to bring observable benefits in terms of HW complexity reductions and in reduced design and testing effort.

Observation 12	An HD-FDD device supporting 20 MHz device bandwidth can fulfil all the Rel-17 Redcap data rate requirements.

Observation 13	It requires further study to determine whether a HD-FDD device can meet the 5-10 ms latency requirement for safety related sensors

Observation 14	HD-FDD will not results in a loss in coverage.

Observation 15	NR specifications can be easily extended to support Type-A half-duplex FDD operation by broadening the notion of non-full-duplex device to further include Type-A HD-FDD devices.

Observation 16	Introducing HD-FDD operation might have impact on TS 38.101 due to a reduced device noise figure.

Observation 17	Relaxed UE processing time for PDSCH processing and PUSCH preparation can have significant impact on average transmission latency and scheduling flexibility.

Observation 18	The impact on UE complexity and cost reduction needs to be further investigated as it is unclear and may be implementation dependent.

Observation 19	UE processing time includes time for PDCCH processing. Whether further relaxed UE processing time is required depends on other aspects of complexity reduction, e.g., those related to PDCCH monitoring limits.

Observation 20	An Redcap device operating on 20 MHz UE bandwidth can fulfil the data rate requirements of industrial sensor, video surveillance, and typical wearable use cases without MIMO and carrier aggregation.

Observation 21	256QAM modulation in combination with carrier aggregation would impose stringent RF and complexity requirements and are not preferable for Redcap devices.

Observation 22	Modulation orders and transmission schemes with 64QAM + MIMO for DL and 16QAM for UL are preferable for FR1 high-end wearable devices that requires 150 Mbps peak bit rates for DL and 50 Mbps peak bit rates for UL operating in an FDD band.

Observation 23	In FR2, the required bit rates for Redcap devices operating in an FDD or a typical TDD band could be fulfilled with 64QAM (and no MIMO) which would however require PT-RS for phase noise compensation. Alternatively, 16QAM and MIMO could be considered to avoid phase noise impact.

Observation 24	Reducing the peak data rate in general does not make the coverage worse.

Observation 25	Power consumption would not be made worse by restricting the peak data rate.

Observation 26	Relaxation on the maximum number of blind decoding attempts, and/or the PDCCH decoding processing time, could be beneficial for Redcap devices. However, the impact on the scheduling flexibility and PDCCH capacity needs to be evaluated.

Observation 27	Restricting the PDCCH aggregation levels (consequently, CCEs for channel estimation), the number of PDCCH candidates and the number of different DCI sizes could be potential techniques in processing capability/complexity relaxation for the UE.

Observation 28	Reducing the Redcap UE bandwidth may restrict the highest supported aggregation levels for the UE which can reduce the coverage.

Observation 29	With 20 MHz BW in FR1, all CORESET #0 configurations can be fully supported.

Observation 30	Reducing the Redcap UE bandwidth may impact the PDCCH performance in some CORESET #0 configurations (particularly in FR2). The potential impact needs to be further studied.

Observation 31	Power saving could be achieved by relaxing the number of blind decoding per slot and/or CCEs for channel estimation. However, by increasing the PDCCH monitoring period, the power saving gain achieved can be higher and may not be necessary to reduce the number of blind decoding limits.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For FR1, a 20 MHz UE maximum bandwidth should be supported. One single type of Redcap UE with at least two different capabilities is considered, i.e., one low data rate services and another for high data rate services.

Proposal 2	For FR2, a 50 MHz UE maximum bandwidth should be supported.

Proposal 3	The scope of the objective to study a “Reduced number of UE RX/TX antennas“ includes a study of the reduction in number of receiver and transmitter branches.

Proposal 4	Redcap support for a single transmitter branch in FR1 is a suitable baseline assumption.

Proposal 5	Redcap support for 2 receiver branches in FR1 is a suitable baseline assumption.

Proposal 6	Support for a single receiver branch in FR1 for facilitating compact form factors deserves further discussion.

Proposal 7	Redcap support for a single transmitter branch in FR2 is a suitable baseline assumption.

Proposal 8	Redcap support for 2 receiver branches in FR2 is a suitable baseline assumption.

Proposal 9	Redcap support for radiated requirements corresponding to at least two antenna panels (i.e. same as reference implementation) is a suitable baseline assumption.

Proposal 10	For some stationary IoT devices radiated requirements corresponding to one antenna panel may be justified and further discussion on this relaxation is required in RAN1.

Proposal 11	RAN1 needs to study the expected losses in a Redcap UE to determine a suitable antenna panel gain, which can be associated to a number of antenna elements per panel to facilitate a discussion on antenna complexity reduction.

Proposal 12	RAN1 needs to consider if a reduced EIRP as a result of antenna gain reduction is in the scope of the Redcap SI, since EIRP is closely coupled to the implementation margins and the antenna design.

Proposal 13	Discuss further whether there is enough cost reduction benefit by enabling a single frequency generator HD-FDD device operating in an FDD band to motivate the associated performance loss and changes to specifications and scheduler implementations.

Proposal 14	If Type-B HD-FDD is introduced, it is desired to aim to minimize the impact on scheduler implementations.

Proposal 15	Latency requirement and scheduling flexibility should be considered when discussing any potential relaxed UE processing time for PDSCH processing and PUSCH preparation.

Proposal 16	Actual cost reduction from a UE perspective needs to be considered to investigate whether the gain from more relaxed UE processing time is justified.

Proposal 17	If Redcap UEs allow multiple antennas, data reception scheme with MIMO to achieve peak data rate should be considered as it would be more cost effective compared to that with 256QAM or carrier aggregation.

Proposal 18	When considering cost reduction by reducing the number of PDCCH blind decoding per slot and/or CCEs for channel estimation, the impacts on scheduling flexibility and blocking probability should also be considered.

# References

RP-193238, New SID on support of reduced capability NR devices, Ericsson, RAN#86, Sitges, Spain, December 2019.

TR 36.888, Machine-Type Communications (MTC) User Equipments (UEs) based on LTE, v12.0.0, June 2013.

TS 36.306, User Equipment (UE) radio access capabilities, v16.0.0, March 2020.

R1-2003290, Reduced PDCCH monitoring for Redcap, RAN1#101-e, e-Meeting, May 25th – June 5th, 2020.

TS 38.101-1, User Equipment (UE) radio transmission and reception, V16.3.0, March 2020.

Report M.2410-0 (11/2017), Minimum requirements related to technical performance for IMT-2020 radio interface(s).

R4-1703086, On phase noise and feasible sub-carrier spacing for mm-wave bands, Ericsson.

TR 38.840, Study on User Equipment (UE) power saving in NR, V16.0.0, June 2019.

5G NR: The Next Generation Wireless Access Technology, Erik Dahlman, Stefan Parkvall, Johan Sköld, Elsevier, ISBN: 978-0-12-814323-0.

R1-2003291, Functionality for coverage recovery for Redcap, RAN1#101-e, e-Meeting, May 25th – June 5th, 2020.