3GPP TSG RAN WG1 Meeting #101-e	                                                                               R1-2003299

E-meeting, 25 May – 5 June, 2020

Agenda Item:	8.4.1.2

Source:	Huawei, HiSilicon

Title:	Baseline coverage performance for FR2

Document for:	Discussion and Decision

Introduction

The study item of NR coverage enhancement was approved at RAN#86 meeting [1]. The main objective is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. For FR2, the detailed objectives include:

- Target scenarios: indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs).

- Target services: eMBB service as first priority and VoIP as second priority.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services.

- Study the additional enhanced solutions if any.

In this contribution, we discuss the FR2 baseline coverage evaluation method and configurations.

Link budget template for FR2 coverage evaluation

The link budget calculation which was adopted in IMT-2020 self-evaluation for 3GPP submission [2][3] could be a start for coverage evaluation.

Proposal 1: Reuse the evaluation methodology of link budget in IMT-2020 self-evaluation for Rel-17 NR coverage enhancement for FR2.

Discussions for FR2 coverage evaluation

Consideration on focusing scenarios

In order to identify whether the coverage requirement is satisfied in the SID, we should perform initial link budget and coverage analysis of NR for the following scenarios:

- Scenario 1: indoor gNB serving indoor UEs,

- Scenario 2: outdoor gNB serving outdoor UEs,

  - Scenario 2a: Urban macro

  - Scenario 2b: Rural macro

- Scenario 3: outdoor gNB serving indoor UEs,

- Scenario 3a: Urban macro

- Scenario 3b: Rural macro

Consideration on focusing channels

For FR2, the carrier frequency (CF) is much higher than that of FR1, and CF impacts on the overall pathloss calculation a lot. Hence, all the downlink and uplink channels should be evaluated, including initial access channels (IAM), control channels, and data channels.

Proposal 2: All the physical channels can be evaluated for FR2.

Consideration on configuration

For FR2 coverage evaluation, it is considered to focus on a subset of evaluation configurations and technical features. This could facilitate the preparation of the baseline coverage performance. The proposed focused evaluation configuration is shown in Table 1. Link-level simulation should be carried to get the required SNR based on the assumptions, which follow [2] and [4].

Proposal 3: Focused on the configurations in Table 1 for FR2 evaluation.

Table 1 Focused evaluation configurations

| Parameters | Indoor hotspot – eMBB | Urban Macro– eMBB | Rural – eMBB |
| --- | --- | --- | --- |
| System configuration | NR 30 GHz TDD DL/UL: DDDSU | NR 30 GHz TDD DL/UL: DDDSU | NR 30 GHz TDD DL/UL: DDDSU |
| Physical channel | IAM: SS/PBCH, PRACH, Msg3 DL: PDCCH, PDSCH UL: PUCCH, PUSCH | IAM: SS/PBCH, PRACH, Msg3 DL: PDCCH, PDSCH UL: PUCCH, PUSCH | IAM: SS/PBCH, PRACH, Msg3 DL: PDCCH, PDSCH UL: PUCCH, PUSCH |
| Channel state | NLOS | NLOS NLOS O-to-I | NLOS NLOS O-to-I |
| Channel model | CDL-C 30 ns, ZSA = 5 degree, ZSD = 1 degree The CDL table is translated so that the strongest cluster’s AoD and AoA occur at a random angle for both the antenna panels of TRP and UE in the local coordinate. The value of the random angle is selected to be uniformly distributed from +30 to -30 degree. The random value is chosen independently for both AoD and AoA |  |  |
| Bandwidth and MCS | Data channels: required bandwidth and MCS satisfying target data rate, 25 Mbps and 5 Mbps for downlink and uplink respectively; PRACH: format C2 Msg3: payload of 56 bits with MCS |  |  |
| SCS | 120 kHz | 120 kHz | 120 kHz |
| Antennas settings | TRP: (M,N,P) = (8,16,2). (dV, dH) = (0.5, 0.5)λ. UE: (2,4,2), with directional antenna element (HPBW=90º, directivity 5dB) |  |  |
| Initial timing and frequency Offset | Timing: 0 us Frequency: 0 ppm at both TRP and UE |  |  |
| UE speed | 3km/h | 3km/h | 3km/h |

Consideration on link budget evaluation

For link budget evaluation, the target transmission bit rate, occupied bandwidth, and antenna configuration needs to be considered, which have impact on the link level simulation that derives the required SINR. In addition, the Feeder loss, Cable loss, Receiver interference density, etc. can follow the value used in IMT-2020 evaluation. The consideration is listed in Table 2 according to [2] and [3].

Proposal 4: FR2 link budget should be evaluated based on the parameters listed in Table 2.

Table 2 evaluation parameters for link budget

|  | IAM channels | DL control channel | DL data channel | UL control channel | UL data channel |
| --- | --- | --- | --- | --- | --- |
| Transmission bit rate (bit/s) | PRACH: format C2 Msg3: 56 bit SS/PBCH: 120 kHz | NR PDCCH: DCI format 1-0; DCI size = 64 bit; QPSK, aggregation level = 16 CCE | 25 Mbps | NR PUCCH: Format 1 (long PUCCH with 14 OFDM symbols), 1 bit | 5 Mbps |
| Number of transmit antennas | DL: 256 UL: 16 | 256 | 256 | 16 | 16 |
| Number of TXU | 2 | 2 | 2 | 2 | 2 |
| Number of receive antennas | DL: 16 UL: 256 | 16 | 16 | 256 | 256 |
| Number of RXU | 2 | 2 | 2 | 2 | 2 |
| Occupied channel bandwidth | Data channels: the maximum bandwidth is of 100 MHz (66 RB); IAM channels: according to NR |  |  |  |  |
| Required SINR via link level simulation | Conduct link level simulation with a 2x2 system; Link level simulation based on a 2x1 precoder for control and IAM channels. |  |  |  |  |
| Channel EIRP (dBm) | Indoor: DL 46, UL 26 Urban and rural: DL 66, UL 26 |  |  |  |  |
| Feeder loss, Cable loss, Receiver interference density | Follow 3GPP’s ITU submission for IMT-2020 |  |  |  |  |
| Penetration margin | For O-to-I: outdoor-to-indoor penetration loss model is used. - High penetration is considered. For NLOS (O-to-O): in-car penetration loss model is used. |  |  |  |  |

Conclusions

The contribution provides our considerations on FR2 coverage evaluation, and the proposals are listed as following:

Proposal 1: Reuse the evaluation methodology of link budget in IMT-2020 self-evaluation for Rel-17 NR coverage enhancement for FR2.

Proposal 2: All the physical channels can be evaluated for FR2.

Proposal 3: Focused on the configurations in Table 1 for FR2 evaluation.

Proposal 4: FR2 link budget should be evaluated based on the parameters listed in Table 2.

- References

RP-193240, “New SID on NR coverage enhancement”, RAN#86, Sitges, Spain, December 9 – 12, 2019.

Report ITU-R M.2412-0, Guidelines for evaluation of radio interface technologies for IMT-2020, 10/2017.

R1-1809511, “Consideration on self evaluation of IMT-2020 for link budget,” Huawei, HiSilicon, RAN1#94, Aug 2018.

R1-1710007, “DMRS for NR-PBCH,” Huawei, HiSilicon, NR AH#2, June 2017.