3GPP TSG RAN WG1 #101		R1-2003355

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Discussion on NRU SSB monitoring capabilities

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the issue of NRU SSB monitoring capabilities, as raised in LS [1] from RAN4.

- Discussion

In [1], RAN4 asks RAN1 to feedback on whether UEs need to monitor all candidate SSB/PBCH positions within a given discovery burst transmission window and related issues on agreed UE capability. The questions are based on the agreement in RAN4 below.

Agreement: Define the following UE capabilities: - For RLM/BFD/CBD UE is required to monitor at least N1 candidate SSB positions from the set of SSBs that are QCLed with each other within the set of configured resources - For intra and inter-frequency measurements UE is required to monitor at least N2 candidate SSB positions from the set of SSBs that are QCLed with each other within discovery burst transmission window - FFS for the case Q is not provided to the UE - FFS how to handle IDLE mode capabilities - Candidate N1 and N2 values are [1, 2, …] - FFS whether N1 = N2 - FFS whether to have different capabilities for FBE and LBE modes

In RAN1, in order to increase the transmission possibility of SSB, the total number of candidate SSB positions in one period are extended from 8 to 20 and QCL relationship between them could be configured by Q. In addition, the discovery burst transmission window is defined to indicate the maximum number of valid candidate SSB positions. One example is shown in Figure 1 with DRS window length is 3ms and Q=4. Due to LBT requirement, gNB will transmit the SSBs when LBT succeeds, which means the actually transmission SSB positions is changed period by period as Figure 1 shows. Thus from UE side, it is preferred to monitor all the candidate SSB positions with the configured SSB index so that it won’t miss any SSB sample when measurement.

Figure 1 Example for SSB transmission in NRU

As discussed above, the maximum number of all valid candidate SSBs corresponding to the same SSB index depends on the configuration of discovery burst transmission window length and QCL parameter Q, which is summarized in the Table 1. As we mentioned, it is preferred to monitor all candidate SSBs from RAN1 perspective but the number seems fairly large especially for some configurations. So the limitation of UE monitoring capability is needed to cover the typical case in real deployment, e.g. Q=4.

Observation 1: It is preferred to monitor all the candidate SSBs but the number seems fairly large especially for some configurations.

Table 1       Maximum number of all valid candidate SSBs

| DRS window | Q=1 | Q=2 | Q=4 | Q=8 |
| --- | --- | --- | --- | --- |
| 1ms | 4 | 2 | 1 | - |
| 2ms | 8 | 4 | 2 | 1 |
| 3ms | 12 | 6 | 3 | 1 or 2 |
| 4ms | 16 | 8 | 4 | 2 |
| 5ms | 20 | 10 | 5 | 2 or 3 |

So regarding the questions from RAN4, the proposed answers are listed below:

Q1: Provide feedback whether monitoring within a given discovery burst transmission window all candidate SS/PBCH block indexes corresponding to the same SS/PBCH block index is mandatory for UEs

A1: It is not mandatory but preferred from RAN1 perspective. Otherwise, there will be some performance degradation.

Q2: Provide feedback on the values of N1 and N2, considering the impact on the network performance if UEs are not monitoring all candidate positions.

A2: The value of N1 and N2 should cover the typical configuration case in real deployment.

Q3: Provide feedback on whether differentiation is needed for UEs operating in FBE and LBE modes

A3: No strong motivation to differentiate the value in FBE and LBE mode.

Q4: Provide feedback for the case when Q is not provided to the UE

A4: RAN1 has agreed that UE will be always provided with Q.

Proposal 1: Send LS to RAN4 to inform the above feedback on the questions.

- Conclusion

In this contribution, we discussed NRU SSB monitoring capabilities, and have following observation and proposal.

Observation 1: It is preferred to monitor all the candidate SSBs but the number seems fairly large especially for some configurations.

Proposal 1: Send LS to RAN4 to inform the above feedback on the questions.

- References

- R1-2003274, “LS on NRU SSB monitoring capabilities”, RAN1 #101-e.