3GPP TSG RAN WG1 #101				                                               R1-2003385

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Remaining issues on support of NR Uu controlling LTE sidelink

Agenda Item:	7.2.4.7

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our view on the remaining issues of the DCI for NR Uu controlling LTE sidelink.

- Discussion

For inter-RAT scheduling LTE mode-3, only SPS based scheduling is supported. Therefore, LTE V2X SPS related fields such as SPS configuration index and activation/release indication should be mandatorily present in DCI format 3_1 if UE is configured to monitor DCI format 3_1.

However, the corresponding field description in 38.212 simply refers to clause 5.3.3.1.9A of DCI format 5A in which the SPS configuration index and activation/release indication exist only if an LTE V2X SPS RNTI (i.e., SL-SPS-V-RNTI) is used to scramble to the DCI. Consequently, these fields will be absent in DCI format 3_1 as there is no such RNTI specified in NR. Moreover, the meaning of each value of the activation/release bit is not defined.

38.212 7.3.1.4.2 Format 3_1 The following information is transmitted by means of the DCI format 3_1 with CRC scrambled by SL-L-CS-RNTI: **********omitted********** - SL SPS configuration index – 3 bits as defined in clause 5.3.3.1.9A of [11, TS 36.212]. - Activation/release indication – 1 bit as defined in clause 5.3.3.1.9A of [11, TS 36.212]. If the UE is configured to monitor DCI format 3_0 and the number of information bits in DCI format 3_1 is less than the payload of DCI format 3_0, zeros shall be appended to DCI format 3_1 until the payload size equals that of DCI format 3_0.

36.212 5.3.3.1.9A Format 5A DCI format 5A is used for the scheduling of PSCCH, and also contains several SCI format 1 fields used for the scheduling of PSSCH. **********omitted********** When the format 5A CRC is scrambled with SL-SPS-V-RNTI, the following fields are present: - SL SPS configuration index – 3 bits as defined in subclause 14.2.1 of [3]. - Activation/release indication – 1 bit as defined in subclause 14.2.1 of [3].

Observation 1. In current spec, SPS configuration index and activation/release indication in 5.3.3.1.9A of DCI format 5A are present only when the DCI format 5A CRC is scrambled with SL-SPS-V-RNTI. However, these two fields should be mandatorily present in DCI format 3_1 given that only SPS based LTE V2X is supported for inter-RAT mode-3 scheduling.

To avoid such ambiguity, we propose the following TP:

TP:

------------------------------------------- Start of Draft TP of 212 -------------------------------------

7.3.1.4.2	Format 3_1

DCI format 3_1 is used for scheduling of LTE PSCCH and LTE PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_1 with CRC scrambled by SL-L-CS-RNTI:

-	Timing offset – 3 bits determined by higher layer parameter sl-TimeOffsetEUTRA, as defined in clause 16.6 of [5, TS 38.213]

-	Carrier indicator –3 bits as defined in 5.3.3.1.9A of [11, TS 36.212].

-	Lowest index of the subchannel allocation to the initial transmission -  bits as defined in 5.3.3.1.9A of [11, TS 36.212].

-	Frequency resource location of initial transmission and retransmission, as defined in 5.3.3.1.9A of [11, TS 36.212]

-	Time gap between initial transmission and retransmission, as defined in 5.3.3.1.9A of [11, TS 36.212]

-	SL index – 2 bits as defined in 5.3.3.1.9A of [11, TS 36.212]

-	SL SPS configuration index – 3 bits, as defined in 16.6 of [5, TS 38.213]  as defined in clause 5.3.3.1.9A of [11, TS 36.212].

-	Activation/release indication – 1 bit, as defined in Table 7.3.1.4.2-1 as defined in clause 5.3.3.1.9A of [11, TS 36.212].

If the UE is configured to monitor DCI format 3_0 and the number of information bits in DCI format 3_1 is less than the payload of DCI format 3_0, zeros shall be appended to DCI format 3_1 until the payload size equals that of DCI format 3_0.

Table 7.3.1.4.2-1: Activation/release indication

| Bit field | Activation/release indication |
| --- | --- |
| 0 | release |
| 1 | activate |

---------------------------------------------- End of Draft TP ---------------------------------------------

Proposal 1. When the activation/release indication=1, the corresponding SPS configuration is activated, otherwise it is released.

Proposal 2. Adopt the TP on SPS configuration index and activation/release indication for TS. 38.212.

- Conclusion

In the contribution, we provide our views on the support for NR Uu controlling LTE sidelink, and observe that,

Observation 1. In current spec, SPS configuration index and activation/release indication in 5.3.3.1.9A of DCI format 5A are present only when the DCI format 5A CRC is scrambled with SL-SPS-V-RNTI. However, these two fields should be mandatorily present in DCI format 3_1 given that only SPS based LTE V2X is supported for inter-RAT mode-3 scheduling.

Based on the observations, we propose that:

Proposal 1. When the activation/release indication=1, the corresponding SPS configuration is activated, otherwise it is released.

Proposal 2. Adopt the TP on SPS configuration index and activation/release indication for TS. 38.212.

- Reference

- 3GPP TS 38.212, “NR; Multiplexing and channel coding”, v16.1.0 (2020-03).