3GPP TSG RAN WG1 #101		    R1-2003392

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Enhanced UL configured grant transmissions for URLLC

Agenda Item:	7.2.5.6

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss one remaining issue on enhanced UL configured grant for URLLC.

- Missed agreement in specification

In RAN1 #AH 1901 meeting, following agreement was made for UL configured grant.

Agreement: - In Rel-16, for both Type 1 and Type 2 configured grant and when multiple active configurations are configured in a BWP, transmission of a TB based on the configured grant is associated with a single active configuration, even if the transmission is repeated

In current 38.214 [1], it seems the above agreement is not clearly captured in section 6.1.2.3. Therefore, we suggest to capture the following text in 38.214 to reflect the above agreement.

Proposal:  adopt the following TP in 38.214.

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant <Unchanged parts are omitted> For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberofrepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. If a UE is provided with more than one configuration for Type 1 and/or Type 2 configured grant, for a PUSCH transmissions with a configured grant configuration, when K ≥ 1, the UE shall transmit the TB across the transmission occasion candidates within the same configured grant configuration. <Unchanged parts are omitted>

- Conclusion

In this contribution, we provide our views on one missed agreement that has not yet been captured in the specification. Therefore, following TP is proposed:

Proposal:  adopt the following TP in 38.214.

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant <Unchanged parts are omitted> For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberofrepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. If a UE is provided with more than one configuration for Type 1 and/or Type 2 configured grant, for a PUSCH transmissions with a configured grant configuration, when K ≥ 1, the UE shall transmit the TB across the transmission occasion candidates within the same configured grant configuration. <Unchanged parts are omitted>

- References

- 3GPP, TS 38.214, v.16.1.0, ‘Physical layer procedures for data’