3GPP TSG RAN WG1 #101		R1-2003436

e-meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Evaluation on NR coverage performance for FR2

Agenda Item:	8.4.1.2

Document for:	Discussion and Decision

- Introduction

In previous RAN 86 meetings, NR coverage enhancements study item is agreed, and the scope of this SI is given by [1].

The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows.

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - TDD and FDD for FR1.

  - VoIP and eMBB service for FR1.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

To identify the bottlenecks of Rel-15/Rel-16 physical channels, we provide coverage performance of different channels in Urban scenario and Indoor hotspot scenario for NR FR2 in this contribution. Link level simulation and link budget calculation used in [2] is reused. The detailed simulation assumptions are analyzed and provided in [3], also listed in the Appendix.

- Evaluation assumptions

In IMT-2020, evaluation methodologies have been well discussed for self-evaluation, and the coupling loss and coverage evaluation model can also be reused for this study item to identify the physical channels requiring enhancement. For link level simulation, the detailed parameters are shown in Appendix Table 3, Table 4 and Table 5.

Besides, target ISD in each scenario are provided in [4]. As guided by [1], Urban and Indoor hotspot scenarios are evaluated, the target ISD and corresponding coverage range are given in the following table.

Table 1 target ISD and coverage range

| scenarios | Urban(macro) | Indoor hotspot |
| --- | --- | --- |
| target ISD | 500m | 20m |

Beamforming gain should be considered in link budget calculation. For DL transmission and receptions channels, ideal beamforming gain can be assumed at gNB. For UL channels, modelling of beamforming gain is not realistic, we use average peak EIRP to describe output power of the UE, which is shown in the Appendix B. For DL broadcast channels, the beamforming gain loss due to fixed and wider beam for broadcast channels should be modeled, the detailed model is given in [3], also given in the Appendix C.

It is noteworthy that the evaluation methodologies and assumptions used in coverage enhancement SI can also be used as a baseline for other device types, e.g. for Redcap UE.

Proposal 1: The coverage evaluation methodology based on link budget calculation and link level simulation in ITU self-evaluation can be considered as baseline in NR coverage enhancement SI, and for other device types, e.g. Redcap UEs.

- Evaluation results

In this section, the evaluation results for different scenarios is given, including isotropic coupling loss and max coverage range for both TDD and FDD. Channel model A is considered as the passloss model.

- Urban

The target ISD of urban scenarios is 500m for 30GHz carrier frequency, which means the target cell radius should be greater than 289m. NLOS outdoor-to-outdoor and outdoor to indoor path loss models are considered in the evaluation.

Figure 1 isotropic coupling loss in urban scenario, NLOS O-to-O

Figure 2 max coverage range in urban scenario, NLOS O-to-O

Figure 3 isotropic coupling loss in urban scenario, NLOS O-to-I

Figure 4 max coverage range in urban scenario, NLOS O-to-I

Observation 1: In urban O-to-O scenario, all downlink channels can achieve target coverage range, while all uplink channels need to be enhanced. But in O-to-I scenario, all channels cannot reach target performance of ISD = 500m.

Proposal 2: the target ISD for outdoor to indoor pathloss model in urban scenario should be further discussed.

- Indoor hotspot

The target ISD of indoor hotspot is 20m for 30GHz carrier frequency, so the target cell radius should be longer than 12m. In this scenario, only indoor serves indoor pathloss model is considered.

Figure 5 isotropic coupling loss in indoor hotspot, NLOS I-to-I

Figure 6 max coverage range in indoor hotspot, NLOS I-to-I

Observation 2: In indoor hotspot scenario, all channels can meet the target coverage requirement of red line. While uplink channels, except PUCCH, cannot satisfy the coverage baseline of ISD = 200m.

- Bottleneck of the physical channels

Based on the evaluation results provided in section 3, the performance gap between the target MCL based on evaluation and the target coverage requirements in terms of isotropic loss are summarized in the following table.

Table 2 performance gap between isotropic loss and target coverage requirements

| Gap in dB | PDSCH eMBB | PDSCH voip | PDCCH UE | PDCCH BC | SSB | PUSCH eMBB | PUSCH voip | PUSCH56bit | PUSCH144bit | PRACH | PUCCHformat1 | PUCCHformat3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban O-to-O | -9.13 | -8 | -8.85 | -2.9 | -9.16 | 15.06 | 8.46 | 9.95 | 12.61 | 13.46 | -0.2 | 2.18 |
| Urban O-to-I | 6.7 | 7.73 | 7.5 | 13.46 | 6.92 | 30.79 | 24.17 | 24.42 | 26.9 | 29.93 | 17.28 | 18.49 |
| Indoor hotspot | -43.14 | -53.2 | -43 | -46.01 | -53.52 | -26.19 | -32.81 | -38.83 | -36.21 | -33.57 | -40.1 | -38.88 |

The negative values in green means the coverage of the physical channel can meet the target requirement in corresponding scenario, and the positive values in red shows performance gap compared with the target coupling loss. Meanwhile, the values which are close to the target threshold with limited performance margin are marked with yellow. Based on the above tables, we have following proposals:

Proposal 3: All downlink channels need to be enhanced in the following scenarios

- Urban O-to-I

Proposal 4: All uplink channels need to be enhanced in the following scenarios

- Urban O-to-O and Urban O-to-I

- Conclusion

In this contribution, we provide our evaluation results in urban scenario Indoor hotspot scenario with the following observations and proposals:

Observation 1: In urban O-to-O scenario, all downlink channels can achieve target coverage range, while all uplink channels need to be enhanced. But in O-to-I scenario, all channels cannot reach target performance of ISD = 500m.

Observation 2: In indoor hotspot scenario, all channels can meet the target coverage requirement of red line. While uplink channels, except PUCCH, cannot satisfy the coverage baseline of ISD = 200m..

Proposal 1: The coverage evaluation methodology based on link budget calculation and link level simulation in ITU self-evaluation can be considered as baseline in NR coverage enhancement SI, and for other device types, e.g. Redcap UEs.

Proposal 2: the target ISD for outdoor to indoor pathloss model in urban scenario should be further discussed.

Proposal 3 : All downlink channels need to be enhanced in the following scenarios

- Urban O-to-I

Proposal 4: All uplink channels need to be enhanced in the following scenarios

- Urban O-to-O and Urban O-to-I

- Appendix A: Evaluation assumptions for physical channels

.Table 3 Evaluation parameters for downlink data channels

| Parameters | PDCSH 25Mbps | PDSCH voip |
| --- | --- | --- |
| Frame structure for TDD | DDDSU DDSUU |  |
| Occupied channel bandwidth | 51RB for 120kHz | 4RB for 120kHz |
| TBs (coderate around 1/3) | 5248 for 120kHz | 320 for 120Hz |
| Target BLER | 0.1 |  |
| Channel model | TDL-A |  |
| Delay spread | 30ns |  |
| DMRS Overhead assumption | Type I, two DMRS symbol, no multiplexing with data |  |
| Slot configuration | All DL slots allocated for PDSCH | One slot every 2 frames |

Table 4 Evaluation parameters for uplink data channels

| Parameters | PUCSH 5Mbps | PUSCH voip | PUSCH MSG3 56bit | PUSCH MSG3 144bit |
| --- | --- | --- | --- | --- |
| Frame structure | DDDSUDDSUU |  |  |  |
| Occupied channel bandwidth | 20RB for 120kHz | 4RB for 120kHz | 4RB for 120kHz | 4RB for 120kHz |
| TBs (coderate around 1/3) | 2088 for 120kHz | 320 for 120kHz | 56 for 120kHz | 144 for 120kHz |
| Target BLER | 0.1 |  |  |  |
| Channel model | TDL-A |  |  |  |
| Delay spread | 30ns |  |  |  |
| DMRS Overhead assumption | Type I, two DMRS symbol, no multiplexing with data |  |  |  |
| Slot configuration | All UL slots allocated for PUSCH |  |  |  |

Table 5 Evaluation parameters for control channels

| Parameters | PDCCH (UE/broadcast) | PBCH | PRACH | PUCCH |
| --- | --- | --- | --- | --- |
| Format type | CCE0 AL=8 DCI size = 39bits | 4-shot combining | Format B4 | Format 1, 2bit UCI using 14 OFDM symbols Format 3, 2bit UCI using 14 OFDM symbols |
| Occupied channel bandwidth | 48RB | 20RB | 12RB | 1RB |
| Target BLER | 0.01 |  |  |  |
| Channel model | TDL-A |  |  |  |
| Delay spread | 30ns |  |  |  |

Table 6 Evaluation parameters for detail scenario assumptions

| Parameters | Values |  |
| --- | --- | --- |
| Scenario | Urban | Indoor hotspot |
| frequency | 30GHz |  |
| Subcarrier spacing | 120kHz |  |
| Pathloss model (select from LoS or NLoS) | NLos |  |
| UE velocity | 30 km/h for outdoor 3km/h for indoor | 3 km/h |
| Tx Power | gNB: 47dBm for BW = 400MHz UE: EIRP = 22.4+4dB | gNB:30dBm for BW = 400MHz UE: EIRP = 22.4+4dB |
| Number of BS antennas & antenna configuration | 256 | 32 |
| Number of TXRU for BS | 2 |  |
| Number of UE antennas | 16 |  |
| Number of TRXU for UE | 1 for uplink 2 for downlink |  |

- Appendix B: Minimum Peak EIRP

Multiple samples of minimum peak EIRP measurement data are provided in TS 38.817 for band n257, on top of the measurement data we also consider additional 4dB as average peak EIRP of the UE.

Table 7.2.1.4.1-2: UE Minimum Peak EIRP for Power class 3

| Operating Band | Min Peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: minimum peak EIRP is defined as the lower limit without tolerance |  |

- Appendix C: beamforming gain model for broadcast channels

A simplified broadcast beamforming model are given as follows. If there is only 1 SSB covering the whole cell, no beamforming gain (0dB) will be achieved, and 3dB BF gain can be assumed if 2 SSB is used, which means the BF gain of the broadcast channel is linearly increased with the number of SSBs.

gainmaxbroadcast=10*log10number of SSBs

Besides, the beamforming gain caused by the deviation from the normal direction are illustrated in Figure 13. Assuming the antenna panel at gNB is down tilt with 0 degree (other assumptions are not precluded), and UE is assumed to be located at the cell edge assuming the target ISD is met, which means the ‘d_2D’ is the cell radius, the propagation path ’d_3D’ is deviated from the center of center of the broadcast beam and leads to beamforming gain loss. The degree of deviation can be calculated first, and then obtain the gain loss using the radiation pattern model provided in [5], given as follows,

gain loss=-min12φdevφ3dB2,Amax, Amax=30dB

where the φdev is the deviation angle in degree, φ3dB is the beam width in degree with in which the gain loss is with in 3dB. Hence, the actual BF again for broadcast channels are given by

BF gain broadcast= gainmaxbroadcast- gain loss

Figure 7 typical propagation model and radiation pattern for broadcast beams

- References

- RP-193147, “New SID on NR coverage enhancement”, 2019/12

- Link budget template for 3GPP 5G NR RIT - Channel Model A

- R1-2003919, “Assumptions for NR coverage evaluation”, 2020/5.

- TR 38.913, “Study on scenarios and requirements for next generation access technologies”, 2018/7.

- TR 38.901, “Study on channel model for frequency from 0.5G to 100GHz”, 2019/12.