3GPP TSG RAN WG1 Meeting #101                                                               	           R1-2003589

e-Meeting, May 25th - June 5th, 2020

Title:	[Draft] LS reply on intra-UE prioritization

Response to: 	R1-2003259 (R2-2004121)

Release:	Rel-16

Work Item:	NR_IIOT-Core

Source:	RAN1

To:	RAN2

CC:

Contact Person:

Name:	Yanping Xing

E-mail Address:	xingyanping@catt.cn

1. Overall Description:

RAN1 would like to thank RAN2 for the LS R2-2004121 on intra-UE prioritization.

In R2-2004121, RAN2 provided two possible options to address the gap between RAN1 and RAN2 in case MAC delivers two MAC PDUs for the two conflicting grants to PHY sequentially when the conflicting grants have the same L1 priority as follows.

- RAN2 changes MAC specification to accommodate current PHY behaviour. With this option, MAC will avoid providing second MAC PDU with the same L1 priority to PHY, meaning that PHY would transmit the packet with lower LCH priority data.

- RAN1 changes PHY specification to accommodate current MAC behaviour of prioritizing the second MAC PDU provided from MAC.

RAN1 discussed the two options in RAN1#101-e, and achieved the agreement that Option 2 is preferred. In addition, in order to avoid UCI dropping due to same PHY priority, the second MAC PDU provided from MAC is prioritized in PHY only if there is no UCI multiplexed on the first PUSCH or PUCCH with SR. It is up to RAN2 to decide whether to deliver the second MAC PDU to PHY in this case.

2   Actions:

To RAN2

ACTION: RAN1 would like to respectfully ask RAN2 to take the above information into account.

3   Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #102	24 - 28 Aug. 2020, 		Online

TSG-RAN WG1 Meeting #102bis                                       12 – 16 Oct. 2020                       China