3GPP TSG RAN WG1 Meeting #100bis		                                    R1-2003620

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	Remaining issues on PDCCH enhancements

Agenda Item:	7.2.5.1

Document for:	Discussion and Decision

Introduction

At RAN1#100bis e-meeting, we had extensive discussions on the remaining issues. It’s good to see most of the issues related to RRC parameter and PDCCH monitoring capability have been fixed. In this contribution, we provide our views on the remaining issues which are still open.

Discussion

Alignment for bit fields based on different HARQ-ACK codebooks

Based on the working assumption achieved at RAN1#99 meeting, separate configurations corresponding to different codebooks may result in different bit widths for some bit fields related to codebook, i.e. PDSCH-to-HARQ_feedback timing, beta-offset, DAI, CBGTI and CBGFI.  When two HARQ-ACK codebooks are configured to a UE, different HARQ-ACK codebooks have different priorities. In order to avoid DCI payload size change during priority change, the larger bit width should be assumed and zero padding to the MSB for the smaller size is necessary for the bit widths of bit fields related to HARQ-ACK codebook. However, this is only needed when priority indicator field is configured in the DCI, which may result in priority change. When priority indicator field is not configured in the DCI, the PUSCH or HARQ-ACK of the PDSCH scheduled or indicated by a DCI format has a specific priority which relates to one codebook. The configurations of the aforementioned bit fields are related to one of the two configured HARQ-ACK codebooks without any change and there is no need to align the bit width based on the other one.

Furthermore, it should be also noted that the bit width of DAI bit field in DCI format 1_2 is configured directly by higher layer parameter ‘downlinkAssignmentIndexForDCI-Format1-2-r16’, despite of which HARQ-ACK codebook will be used for the HARQ-ACK transmission. In addition, the CBG configuration for UL transmission is independent with HARQ-ACK codebook. Hence there is no need to capture the alignment operation in the specification for the aforementioned bit fields.

Last but not least, the HARQ-ACK codebook list is configured by ‘PhysicalCellGroupConfig’. In the other word, it is configured per cell group instead of a serving cell.

In order to fix the above issues, we propose the following text proposal.

Text proposal#1 - 7.3.1.1.2 Format 0_1 < --------------------omitted text-------------------------> - 1st downlink assignment index – 1, 2 or 4 bits: - 1 bit for semi-static HARQ-ACK codebook; - 2 bits for dynamic HARQ-ACK codebook, or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included-r16 configured; - 4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included-r16 = "enable". - 2nd downlink assignment index – 0, 2 or 4 bits: - 2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks, or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included-r16 configured; - 4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included-r16 = "enable"; - 0 bit otherwise. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format0-1-r16 is configured and if the bit width of the downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller downlink assignment index until the bit width of the downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format0-1-r16 is not configured, the bit width of the downlink assignment index indicator in DCI format 0_1 is determined according to the HARQ-ACK codebook with the same priority as PUSCH scheduled by DCI format 0_1. < --------------------omitted text-------------------------> - CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PUSCH is not configured or if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1; otherwise, 2, 4, 6, or 8 bits determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for PUSCH. < --------------------omitted text-------------------------> - beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213]. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format0-1-r16 is configured and if the bit width of the beta_offset indicator in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for the two HARQ-ACK codebooks are the same. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format0-1-r16 is not configured, the bit width of the beta_offset indicator in DCI format 0_1 is determined according to the HARQ-ACK codebook with the same priority as for PUSCH scheduled by DCI format 0_1. < --------------------omitted text-------------------------> - 7.3.1.1.3 Format 0_2 < --------------------omitted text-------------------------> - Downlink assignment index – 0, 1, 2 or 4 bits - 0 bit if the higher layer parameter Downlinkassignmentindex-ForDCIFormat0_2 is not configured; - 1, 2 or 4 bits otherwise, - 1st downlink assignment index – 1 or 2 bits: - 1 bit for semi-static HARQ-ACK codebook; - 2 bits for dynamic HARQ-ACK codebook. - 2nd downlink assignment index – 0 or 2 bits - 2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks; - 0 bit otherwise. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format0-2-r16 is configured and if the bit width of the Downlink assignment index in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 0_2 for the two HARQ-ACK codebooks are the same. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format0-2-r16 is not configured, the bit width of the downlink assignment index indicator in DCI format 0_2 is determined according to the HARQ-ACK codebook with the same priority as PUSCH scheduled by DCI format 0_2. < --------------------omitted text-------------------------> - beta_offset indicator – 0 bit if the higher layer parameter betaOffsets = semiStatic; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamic-ForDCIFormat0_2 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamic-ForDCIFormat0_2 as defined by Table 9.3-3 in [5, TS 38.213]. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format0-1-r16 is configured and if the bit width of the beta_offset indicator in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for the two HARQ-ACK codebooks are the same. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format0-2-r16 is not configured, the bit width of the beta_offset indicator in DCI format 0_2 is determined according to the HARQ-ACK codebook with the same priority as for PUSCH scheduled by DCI format 0_2. < --------------------omitted text-------------------------> - 7.3.1.2.2 Format 1_1 < --------------------omitted text-------------------------> Downlink assignment index – number of bits as defined in the following < --------------------omitted text-------------------------> - 0 bits otherwise. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format1-1-r16 is configured and if the bit width of the Downlink assignment index in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 1_1 for the two HARQ-ACK codebooks are the same. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format1-1-r16 is not configured, the bit width of the Downlink assignment index indicator in DCI format 1_1 is determined according to the HARQ-ACK codebook associated with DCI format 1_1. < --------------------omitted text-------------------------> - PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK. - When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format1-1-r16 is configured and if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the two HARQ-ACK codebooks are the same. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format1-1-r16 is not configured, the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 is determined according to the HARQ-ACK codebook associated with DCI format 1_1. < --------------------omitted text-------------------------> - CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PDSCH is not configured, otherwise, 2, 4, 6, or 8 bits as defined in Clause 5.1.7 of [6, TS38.214], determined by the higher layer parameters maxCodeBlockGroupsPerTransportBlock and maxNrofCodeWordsScheduledByDCI for the PDSCH. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format1-1-r16 is configured and if the bit width of the CBG transmission information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG transmission information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG transmission information until the bit width of the CBG transmission information in DCI format 1_1 for the two HARQ-ACK codebooks are the same. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format1-1-r16 is not configured, the bit width of the CBG transmission information in DCI format 1_1 is determined according to the HARQ-ACK codebook associated with DCI format 1_1. - CBG flushing out information (CBGFI) – 1 bit if higher layer parameter codeBlockGroupFlushIndicator is configured as "TRUE", 0 bit otherwise. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format1-1-r16 is configured and if the bit width of the CBG flushing out information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG flushing out information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG flushing out information until the bit width of the CBG flushing out information in DCI format 1_1 for the two HARQ-ACK codebooks are the same. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format1-1-r16 is not configured, the bit width of the CBG flushing out information in DCI format 1_1 is determined according to the HARQ-ACK codebook associated with DCI format 1_1. - 7.3.1.2.3 Format 1_2 < --------------------omitted text-------------------------> - Downlink assignment index – 0, 1, 2 or 4 bits - 0 bit if the higher layer parameter Downlinkassignmentindex-ForDCIFormat1_2 is not configured; - 1, 2 or 4 bits determined by higher layer parameter Downlinkassignmentindex-ForDCIFormat1_2 otherwise, - 4 bits if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI - 1 or 2 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 1 bit or 2 bits are the counter DAI. < --------------------omitted text-------------------------> - PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK-ForDCIFormat1_2. When two HARQ-ACK codebooks are configured , if higher layer parameter priorityIndicatorForDCI-Format1-2-r16 is configured and if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for the two HARQ-ACK codebooks are the same. When two HARQ-ACK codebooks are configured, if higher layer parameter priorityIndicatorForDCI-Format1-2-r16 is not configured, the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 is determined according to the HARQ-ACK codebook associated with DCI format 1_2.

Proposal 1:  Endorse the above text proposal#1 to correct the procedure of bit width which is only necessary when two HARQ-ACK codebooks and priority indicator are both configured.

- 2.2 Alignment for RRC parameter definition

Furthermore, we realize that the RRC parameter names denoted in TS 38.212 are not aligned with the current TS 38.331. We provide the text proposal to fix this issue.

Text proposal#2 - 7.3.1.1.3 Format 0_2 DCI format 0_2 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Carrier indicator – 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeForDCI-format0-2-r16, as defined in Clause 10.1 of [5, TS38.213]. < ------------omitted text------------> Frequency domain resource assignment – number of bits determined by the following: - NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 6.1.2.2.1 of [6, TS 38.214] - log2NRBG, K1NRBG, K1+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K1NRBG, K1+1/2, NRBG+1 bits if both resource allocation type 0 and 1 are configured, where NRBG, K1=NRBUL, BWP+NUL, BWPstartmodK1/K1, NRBUL, BWP is the size of the active UL bandwidth part, NUL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K1 is given by higher layer parameter resourceAllocationType1GranularityForDCI-Format1-2-r16. If the higher layer parameter resourceAllocationType1GranularityForDCI-Format1-2-r16 is not configured, K1 is equal to 1. < ------------omitted text------------> - For resource allocation type 1, the log2NRBG, K1NRBG, K1+1/2 LSBs provide the resource allocation as follows: - For PUSCH hopping with resource allocation type 1: - NUL_hop MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where NUL_hop=1 if the higher layer parameter frequencyHoppingOffsetListsForDCI-Format0-2-r16 contains two offset values and NUL_hop=2 if the higher layer parameter frequencyHoppingOffsetListsForDCI-Format0-2-r16 contains four offset values - log2NRBG, K1NRBG, K1+1/2-NUL_hop bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] < ------------omitted text------------> - Time domain resource assignment – 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationListForDCI-Format0-2-r16 if the higher layer parameter is configured, or I is the number of entries in the higher layer parameter PUSCH-TimeDomainResourceAllocationList if the higher layer parameter PUSCH-TimeDomainResourceAllocationList is configured and the higher layer parameter pusch-TimeDomainAllocationListForDCI-Format0-2-r16 is not configured; otherwise I is the number of entries in the default table. - Frequency hopping flag – 0 or 1 bit: - 0 bit if the higher layer parameter frequencyHoppingForDCI-Format0-2-r16 is not configured; - 1 bit according to Table 7.3.1.1.1-3 otherwise, as defined in Clause 6.3 of [6, TS 38.214]. - Modulation and coding scheme –5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit - Redundancy version – 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-ForDCI-Format0-2-r16 - If 0 bit is configured, rvid to be applied is 0; - 1 bit according to Table 7.3.1.2.3-1; - 2 bits according to Table 7.3.1.1.1-2. - HARQ process number – 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeForDCI-Format0-2-r16 - Downlink assignment index – 0, 1, 2 or 4 bits - 0 bit if the higher layer parameter downlinkAssignmentIndexForDCI-Format0-2-r16 is not configured; - 1, 2 or 4 bits otherwise, - 1st downlink assignment index – 1 or 2 bits: - 1 bit for semi-static HARQ-ACK codebook; - 2 bits for dynamic HARQ-ACK codebook. - 2nd downlink assignment index – 0 or 2 bits - 2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks; - 0 bit otherwise. < ------------omitted text------------> SRS resource indicator – or log2NSRS bits, where NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListForDCI-Format0-2-r16, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', - bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListForDCI-Format0-2-r16 , and associated with < ------------omitted text------------> - log2NSRS bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListForDCI-Format0-2-r16 , and associated with the higher layer parameter usage of value 'codeBook'. < ------------omitted text------------> - Precoding information and number of layers – number of bits determined by the following: - 0 bits if the higher layer parameter txConfig = nonCodeBook; - 0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook; - 4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankForDCI-Format0-2-r16 , and codebookSubsetForDCI-Format0-2-r16 ; - 2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankForDCI-Format0-2-r16 , and codebookSubsetForDCI-Format0-2-r16 ; - 2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankForDCI-Format0-2-r16 and codebookSubsetForDCI-Format0-2-r16 ; - 1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankForDCI-Format0-2-r16 and codebookSubsetForDCI-Format0-2-r16 . Antenna ports – number of bits determined by the following: - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured; - 2, 3, 4, or 5 bits otherwise, < ------------omitted text------------> If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 and is configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCI-Format0-2-r16 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCI-Format0-2-r16 2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. - SRS request – 0, 1, 2 or 3 bits - 0 bit if the higher layer parameter SRSRequest-ForDCIFormat0_2 is not configured; - 1 bit as defined by Table 7.3.1.1.3-1 if higher layer parameter srs-RequestForDCI-Format0-2-r16 = 1 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; - 2 bits if higher layer parameter srs-RequestForDCI-Format0-2-r16 = 1 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second bit is defined by Table 7.3.1.1.3-1; < ------------omitted text------------> - CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSizeForDCI-Format0-2-r16. - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured and transform precoder is disabled, or if transform precoder is enabled, or if maxRankForDCI-Format0-2-r16 =1; - 2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. - beta_offset indicator – 0 bit if the higher layer parameter betaOffsets = semiStatic; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamicForDCI-Format0-2-r16 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamicForDCI-Format0-2-r16 as defined by Table 9.3-3 in [5, TS 38.213]. < ------------omitted text------------> - DMRS sequence initialization – 0 or 1 bit - 0 bit if the higher layer parameter dmrs-SequenceInitializationForDCI-Format0-2-r16 is not configured or if transform precoder is enabled; - 1 bit if transform precoder is disabled and the higher layer parameter dmrs-SequenceInitializationForDCI-Format0-2-r16 is configured. - UL-SCH indicator – 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. [Except for DCI format 0_2 with CRC scrambled by SP-CSI-RNTI,] a UE is not expected to receive a DCI format 0_2 with UL-SCH indicator of "0" and CSI request of all zero(s). Open-loop power control parameter set indication – 0 or 1 or 2 bits. - 0 bit if the higher layer parameter p0-PUSCH-SetList-r16 is not configured; - 1 or 2 bits otherwise, - 1 bit if SRS resource indicator is present in the DCI format 0_2; - 1 or 2 bits as determined by higher layer parameter olpc-ParameterSetForDCI-Format0-2-r16 if SRS resource indicator is not present in the DCI format 0_2; - Priority indicator – 0 bit if higher layer parameter priorityIndicatorForDCI-Format0-2-r16 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213]. - Invalid symbol pattern indicator – 0 bit if higher layer parameter invalidSymbolPatternIndicatorForDCI-Format0-2-r16 is not configured; otherwise 1 bit as defined in Clause 6.1.2.1 in [6, TS 38.214]. < -----------omitted text------> - Format 1_2 DCI format 1_2 is used for the scheduling of PDSCH in one cell. The following information is transmitted by means of the DCI format 1_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format. - Carrier indicator – 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeForDCI-Format1-2-r16, as defined in Clause 10.1 of [5, TS38.213]. < ---------------------omitted text----------------- > Frequency domain resource assignment – number of bits determined by the following: - NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS 38.214]; - log2NRBG, K2NRBG, K2+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K2NRBG, K2+1/2, NRBG+1 bits if both resource allocation type 0 and 1 are configured, where NRBG, K2=NRBDL, BWP+NDL, BWPstartmodK2/K2, NRBDL, BWP is the size of the active DL bandwidth part, NDL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K2 is determined by higher layer parameter ResourceAllocationType1GranularityForDCI-Format1-2-r16 . If the higher layer parameter ResourceAllocationType1GranularityForDCI-Format1-2-r16 is not configured, K2 is equal to 1. - If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1. - For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214]. - For resource allocation type 1, the log2NRBG, K2NRBG, K2+1/2 LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214] If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part. - Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 if the higher layer parameter is configured, or I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationList if the higher layer parameter pdsch-TimeDomainAllocationList is configured when the higher layer parameter pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 is not configured; otherwise I is the number of entries in the default table. VRB-to-PRB mapping – 0 or 1 bit: - 0 bit if the higher layer parameter vrb-ToPRB-InterleaverForDCI-Format1-2-r16 is not configured; - 1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6 of [4, TS 38.211]. PRB bundling size indicator – 0 bit if the higher layer parameter prb-BundlingTypeForDCI-Format1-2-r16 is not configured or is set to 'static', or 1 bit if the higher layer parameter prb-BundlingTypeForDCI-Format1-2-r16 is set to 'dynamic' according to Clause 5.1.2.3 of [6, TS 38.214]. - Rate matching indicator – 0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1ForDCI-Format1-2-r16 and rateMatchPatternGroup2ForDCI-Format1-2-r16, where the MSB is used to indicate rateMatchPatternGroup1ForDCI-Format1-2-r16 and the LSB is used to indicate rateMatchPatternGroup2ForDCI-Format1-2-r16 when there are two groups. - ZP CSI-RS trigger – 0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as log2(nZP+1) bits, where nZP is the number of aperiodic ZP CSI-RS resource sets configured by higher layer parameter aperiodicZP-CSI-RS-ResourceSetsToAddModListForDCI-Format1-2-r16. < ---------------------omitted text----------------- > - HARQ process number – 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeForDCI-Format1-2-r16 Downlink assignment index – 0, 1, 2 or 4 bits - 0 bit if the higher layer parameter DownlinkAssignmentIndexForDCI-Format1-2-r16 is not configured; - 1, 2 or 4 bits determined by higher layer parameter DownlinkAssignmentIndexForDCI-Format1-2-r16 otherwise, - 4 bits if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI - 1 or 2 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 1 bit or 2 bits are the counter DAI. < ---------------------omitted text----------------- > - PUCCH resource indicator – 0 or 1 or 2 or 3 bits determined by higher layer parameter numberOfBitsForPUCCH-ResourceIndicatorForDCI-Format1-2-r16 - PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK-ForDCI-Format1-2-r16. When two HARQ-ACK codebooks are configured for the same serving cell, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for the two HARQ-ACK codebooks are the same. Antenna port(s) – 0, 4, 5, or 6 bits - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 is not configured; - Otherwise 4, 5 or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4. If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeAForDCI-Format1-2-r16 and dmrs-DownlinkForPDSCH-MappingTypeBForDCI-Format1-2-r16 and is configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeAForDCI-Format1-2-r16 and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeBForDCI-Format1-2-r16 . A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 but configured with one or more of dmrs-DownlinkForPDSCH-MappingTypeAForDCI-Format1-2-r16 and dmrs-DownlinkForPDSCH-MappingTypeBForDCI-Format1-2-r16 , antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.2.2-1/2/3/4. - Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI-ForDCI-Format1-2-r16 is not enabled; otherwise 1 or 2 or 3 bits determined by higher layer parameter tci-PresentInDCI-ForDCI-Format1-2-r16 as defined in Clause 5.1.5 of [6, TS38.214]. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, - if the higher layer parameter tci-PresentInDCI-ForDCI-Format1-2-r16 is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_2, - the UE assumes tci-PresentInDCI-ForDCI-Format1-2-r16 is not enabled for all CORESETs in the indicated bandwidth part; - otherwise, - the UE assumes tci-PresentInDCI-ForDCI-Format1-2-r16 is enabled for all CORESETs in the indicated bandwidth part. - SRS request – 0, 1, 2 or 3 bits - 0 bit if the higher layer parameter SRSRequest-ForDCIFormat1_2 is not configured; - 1 bit as defined by Table 7.3.1.1.3-1 if the higher layer parameter srs-RequestForDCI-Format1-2-r16 = 1 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; < ---------------------omitted text----------------- > - DMRS sequence initialization – 0 or 1 bit - 0 bit if the higher layer parameter dmrs-SequenceInitializationForDCI-Format1-2-r16 is not configured; - 1 bit otherwise. - Priority indicator – 0 bit if higher layer parameter priorityIndicatorForDCI-Format1-2-r16 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

Proposal 2: Adopt the text proposal#2 to align the RRC parameter names between TS 38.331 and TS 38.212.

Differentiation for different DCI formats

As agreed during the previous meetings, most of the bit fields included in a new DCI format are configurable. In the other words, the payload size of a new DCI format can be smaller than or larger than or equal to that of a fallback DCI format. Although there is an identifier in each DCI format, it can only be used to indicate whether this DCI format is scheduling UL data or DL data.  Once the fallback DCI format and new DCI format has the same payload size, UE cannot differentiate two different DCI formats and misunderstanding happens between gNB and UE. The same issue also exists in the case new DCI format and the non-fallback DCI format has the same payload size. In order to guarantee a common understanding between gNB and UE, a padding bit can be appended to the new DCI format once its payload size is equal to that of a legacy fallback DCI format. Similarly, a padding bit can also be appended to the non-fallback DCI format once its payload size is equal to that of a new DCI format.

It was argued that both non-fallback DCI formats and new DCI formats have full configurability. gNB could guarantee different DCI formats have different payload sizes and it is totally implementation issue.

However, it should be noticed that we have specify the behavior in case non-fallback DCIs have same size as that of fallback DCI, i.e. one zero bit will be appended to the non-fallback DCI. The same logic can be straightly applied to the case we encounter in Rel-16. Secondly, the only thing gNB should consider when configures parameters to a UE is functionality. If gNB has to take the DCI payload alignment issue into account, it will significantly complicate gNB implementation as most of the bit fields in a DCI are configurable and related to a RRC parameter. The TP proposed in [1] could be a starting point.

Proposal 3: If a new DCI format has the same payload size as that of a fallback DCI format, one bit is appended to the new DCI format. If a non-fallback DCI format has the same payload size as that of a new DCI format, one bit is appended to the non-fallback DCI format.

Conclusion

This contribution discussed the remaining issues on PDCCH enhancements to adequately support Rel-16 URLLC use cases. Based on the discussion we have the following proposals:

Proposal 1:  Endorse the above text proposal#1 to correct the procedure of bit width which is only necessary when two HARQ-ACK codebooks and priority indicator are both configured.

Proposal 2: Adopt the text proposal#2 to align the RRC parameter names between TS 38.331 and TS 38.212.

Proposal 3: If a new DCI format has the same payload size as that of a fallback DCI format, one bit is appended to the new DCI format. If a non-fallback DCI format has the same payload size as that of a new DCI format, one bit is appended to the non-fallback DCI format.

References

  - R1-2001404, ‘Summary of email discussion [100e-NR-L1enh_URLLC_PDCCH-01] on remaining issues on DCI format’, Huawei