3GPP TSG RAN WG1 Meeting #101   	  		                                   R1-2003641

e-meeting, 25th May – 5th June 2020

Source:	CATT

Title:	Discussion of evaluation of NR positioning performance

Agenda item:	8.2.2

Document for:	Discussion and Decision

Introduction

At RAN#86 meeting, the study item on NR Positioning Enhancements was approved [1]. From RAN1’s perspective, the SI includes the following objectives:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

This contribution discusses the evaluation of NR positioning performance, including impact of clutter parameters on LOS probability and the absolute time of arrival model on positioning performance with initial simulation results in InF scenarios.

Initial simulation evaluation of NR positioning performance in InF scenarios

This section discusses initial simulation evaluation of NR positioning performance in InF scenarios. Section 2.1 analyzes the impact of clutter parameters on LOS probability. In section 2.2, the impact of absolute time of arrival model on positioning performance is analyzed. The details of simulation assumptions are given in Appendix, where the key simulation parameters are given in Table 1.

Table 1: Key simulation parameters for InF scenarios

| Parameters | Values |
| --- | --- |
| Bandwidth for DL PRS | 100MHz |
| Subcarrier Space | 30KHz |
| Carrier frequency | 4GHz |
| Comb size of DL PRS | 6 |
| Measurement Algorithm | MUSIC based TOA estimation |
| Positioning Algorithm | Chan based DL-TDOA[5] |
| gNB Sync error | Perfect Sync |
| InF scenarios | InF-SH (sparse clutter, high BS), InF-DH (dense clutter, high BS) |

Impact of clutter parameters on LOS probability

According to Section 7.4.2 in TR 38.901, the LOS probability of 4 InF scenarios is modelled as follows:

| InF-SL InF-SH InF-DL InF-DH | PrLOS,subsced2D=exp-d2Dksubsce where ksubsce=-dclutterln1-rfor InF-SL and InF-DL-dclutterln1-r∙hBS-hUThc-hUTfor InF-SH and InF-DH The parameters dclutter, r, and hc are defined in Table 7.2-4 |
| --- | --- |

In the evaluation of the LOS probability, we simulate three sets of clutter parameters {density r, height hc,size dclutter} as shown in the following:

| Clutter parameters: {density r, height hc,size dclutter} | CASE 1：Sparse clutter : {20%, 2m, 10m} CASE 2：Dense clutter : {60%, 6m, 2m} CASE 3：Middle clutter : {40%, 2m, 2m} |
| --- | --- |

CASE 1 and CASE 2 correspond, respectively, the spares clutter and dense clutter scenarios, as described in section 7.8.4 of TR 38.901, while CASE 3 with middle clutter was proposed in email discussion.

Assuming BS antenna height = 8 m and UE antenna height = 1.5m, Figure 1 shows the LOS probability PrLOS,subsce vs. d2D for the three CASEs, where d2D represents 2-D distance between UE and TRP in horizontal dimension in unit of meter. As shown in the Figure 1, the LOS probability of CASE 1 (SH) is the highest (more than 90% @d2D = 50m), the LOS probability of CASE 2 (DH) is the lowest (less than 10% @d2D = 10m, and 0% @d2D = 50m), and the LOS probability of CASE 3 is in the middle (about 35% @d2D = 50m).

Figure 1: LOS probability PrLOS,subsce vs 2D-distance d2D

According to fast fading channel coefficient generation procedure model in TR 38.901, when propagation condition (LOS/NLOS) is uncorrelated for different BS-UT links and assigned according to Table 7.4.2-1 of TR 38.901, the probability of number of LOS links per UE can be calculated for different CASEs. Figure 2 illustrates the probability of number of LOS links per UE for three CASEs. As shown in the Figure 2, the number of LOS links per UE is from 13 to 18 for CASE 1, fewer than 2 for CASE 2, and between 1 and 8 for CASE 3.

Figure 2: Probability of number of LOS links per UE

The impact of the probability vs. number of LOS links per UE on TOA measurement error is shown in Figure 3. As shown in the Figure 3, at 90% CDF point, the TOA error is 0.107 meter for CASE1 (InF-SH) and 19.58 meter for CASE 2 (InF-DH). The reason lies in the facts that the number of LOS links per UE for CASE2 (InF-DH) is much fewer than that for CASE 1 (InF-SH), and the absolute time of arrival model for NLOS link includes a random delay, which results in much bigger TOA error for NLOS link than that for LOS link.

Figure 3: Impact of probability of number of LOS links per UE on TOA error

Observation 1: Configuration of clutter parameters have great impact on the LOS probability, where sparse clutter brings in high LOS probability and dense clutter has low LOS probability. Random delays in NLOS links, as described in absolute time of arrival model in TR 38.901, may result in large TOA measurement error. The following three CASEs of clutter parameters should be considered for the evaluation of positioning performance in InF scenarios in Rel-17 positioning SI.

| Clutter parameters: {density r, height hc,size dclutter} | CASE 1：Sparse clutter: {20%, 2m, 10m} CASE 2：Dense clutter: {60%, 6m, 2m} CASE 3：Middle clutter: {40%, 2m, 2m} |
| --- | --- |

Impact of absolute time of arrival model on positioning performance

In this section, we analyze the impact of absolute time of arrival model on positioning performance for InF-DH and InF-SH scenarios. To support the simulations, we have considered the propagation time delay due to the total path length in step 11 of fast fading model as defined in TR38.901.

The impact of the absolute time of arrival model on positioning performance for InF-DH is shown in Figure 4 and Figure 5. Figure 4 shows the simulation results when the absolute time of arrival is not considered in the simulation. At 90% CDF point, the horizontal positioning error is only 0.57 meter. Figure 5 presents the simulation results when the absolute time of arrival is considered in the simulation. At 90% CDF point, the horizontal positioning error is increased to 41.64 meters. The reason that the absolute time of arrival has such big impact on the positioning error lies in that nearly all selected TRPs (E.g., 6~10) used for positioning per UE are NLOS links for InF-DH scenario, where the random delay Δτ introduces significant TOA error for NLOS link.

Figure 6 and Figure 7 illustrate the impact of the absolute time of arrival model on positioning performance for InF-SH. Figure 6 shows the simulation results when the absolute time of arrival is not considered, while Figure 7 presents the simulation results when the absolute time of arrival is considered. As shown in Figure 6 and Figure 7, at 90% CDF point, the horizontal positioning errors are the same for two CASEs with and without the random delay Δτ. The reason lies in that for InF-SH, the probability of number of LOS links per UE is much higher than that in InF -DH and all the selected TRPs (E.g., 6~10) used for positioning per UE are LOS links, where the random delay Δτ introduces no TOA error for LOS link.

Observation 2: The impact of absolute time of arrival model on the positioning performance is significantly for InF-DH, and negligible for InF-SH. The absolute time of arrival model should be included in the evaluation of positioning performance in Rel-17 positioning SI.

Figure 4: Positioning error for InF-DH without modeling absolute time of arrival

Figure 5: Positioning error for InF-DH with modeling absolute time of arrival

Figure 6: Positioning error for InF-SH without modeling absolute time of arrival

Figure 7: Positioning error for InF-SH with modeling absolute time of arrival

Conclusions

In this contribution, we discuss evaluation of NR positioning performance, and give the following observations:

Observation 1: Configuration of clutter parameters have great impact on the LOS probability, where sparse clutter brings in high LOS probability and dense clutter has low LOS probability. Random delays in NLOS links, as described in absolute time of arrival model in TR 38.901, may result in large TOA measurement error. The following three CASEs of clutter parameters should be considered for the evaluation of positioning performance in InF scenarios in Rel-17 positioning SI.

| Clutter parameters: {density r, height hc,size dclutter} | CASE 1：Sparse clutter: {20%, 2m, 10m} CASE 2：Dense clutter: {60%, 6m, 2m} CASE 3：Middle clutter: {40%, 2m, 2m} |
| --- | --- |

Observation 2: The impact of absolute time of arrival model on the positioning performance is significantly for InF-DH, and negligible for InF-SH. The absolute time of arrival model should be included in the evaluation of positioning performance in Rel-17 positioning SI.

References

- RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, 9th – 12th December, 2019.

- 3GPP TR 38.855, “Study on NR positioning support”,  V16.0.0 (2019-03)

- R1- 2003639, Summary of discussion on IIoT Scenarios for NR Positioning Enhancements, CATT, RAN1#101e

- R1- 2003640, IIoT use cases and scenarios for evaluation of NR Positioning Enhancements, CATT, RAN1#101e

- Y. T. Chan, and K. C. Ho, “A Simple and Efficient Estimator for Hyperbolic Location”, IEEE TRANSACTIONS ON SIGNAL PROCESSING, VOL. 42, NO. 8, AUGUST 1994

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V16.1.0 (2019-12)

Appendix

Common scenario parameters applicable for all scenarios (TR 38.855)

Table 5-1: Common scenario parameters applicable for all InF scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| Total gNB TX Power, dBm | 24dBm | 24 dBm, EIRP should not exceed 58 dBm |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-1: Common scenario parameters applicable for all scenarios (TR 38.855)

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-2: UE antenna radiation pattern model 1 (FR2) (TR 38.855)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
|  | Note: are in local coordinate system. |

Simulation assumptions for calibration of the indoor factory scenario (TR 38.901)

Table 5-2 Parameters common to InF scenario(s)

|  |  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- | --- |
| Channel model |  | [InF-SL, InF-DL, InF-SH, InF-DH and/or InF-HH] | [InF-SL, InF-DL, InF-SH, InF-DHand/or InF-HH] |
| Layout | Hall size | InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m InF-HH: 300x150 m |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |
|  | Room height | 10m |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Peneteration loss |  | 0dB |  |
| Number of floors |  | 1 |  |
| UE drop procedure |  | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility |  | 3km/h |  |
| UE antenna height |  | 1.5m |  |
| Min gNB-UE distance (2D), m |  | 0m |  |
| gNB antenna height |  | BS height = 1.5 m for InF-SL and InF-DL BS-height = 8 m for InF-SH and InF-DH |  |
| Clutter parameters: {density r, height hc,size dclutter} |  | Sparse clutter: {20%, 2m, 10m} Dense clutter: {60%, 6m, 2m} Middle clutter: {40%, 2m, 2m} |  |
| LOS probability |  | LOS probability for InF scenarios is modelled according to Section 7.4.2 in TR 38.901 |  |
| Absolute time of arrival |  | Absolute time of arrival for InF scenarios is modelled according to Section 7.6.9 in TR 38.901 |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |  |