3GPP TSG RAN WG1 Meeting #101                                                                	   R1- 2003650

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	Discussion on the baseline performance and simulation assumptions of

coverage enhancement for FR2

Agenda Item:	8.4.1.2

Document for:	Discussion and Decision

Introduction

At RAN#86 meeting, a study item was achieved as a new Rel-17 SID on NR coverage enhancement [1]. For FR2, the related objectives are as follows

- The target scenarios and services include

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

In this contribution, we provide preliminary thoughts on the baseline per performance and simulation assumptions for FR2.

Discussion

The baseline performance for FR2

Similar to FR1 [2], the baseline performance in FR2 can be defined as the MCL from the link budget evaluation based on the given target data rate. The related target data rates for downlink in Indoor and Urban/ suburban scenario are provided in Table 1 which refers to the Rel-17 SID on NR coverage enhancement [1]. The target performance is the MPL (Maximum Path Loss) at the cell edge corresponding to the target ISD also shown in table 1 which reuses the assumptions in ITU [3].

Table 1: the target data rate and ISD requirements for different scenarios

| Frequency band | Scenarios | Duplexing scheme | Service | Target data rate | Target ISD |
| --- | --- | --- | --- | --- | --- |
| FR2 | Indoor | TDD | eMBB as first priority | DL:25Mbps UL:5Mbps | 20m |
|  | Urban |  |  | DL: [25Mbps] UL: 5Mbps | ISD 1: 200m ISD 2: 500m |
|  | suburban |  |  | DL: [1Mbps] UL: 50Kbps | N/A |

Proposal 1: The baseline performance is the MCL from link budget evaluation based on target data rate.

Proposal 2: The target performance is the MPL at the cell edge corresponding to the target ISD.

The simulation assumptions for PDCCH/PDSCH/PUCCH/PUSCH

To identify the bottleneck channel, we provide the detailed link level simulation assumptions in order to obtain the required SNR. As a starting point, it is straightforward to reuse most of simulation assumptions for FR1, assumed parameters for PDCCH/PDSCH/PUCCH/PUSCH with updating some parameters according the requirements of FR2, which are provided in Table 2~5. .

Table 2: the link level simulation assumptions for PDCCH on FR2

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenarios | Indoor | Urban | suburban |
| System Bandwidth | 50MHz | 50MHz | 50MHz |
| Carrier | 28G | 28G | 28G |
| SCS | 60KHz | 60KHz | 60KHz |
| Antenna configuration | 2T2R | 2T2R | 2T2R |
| Payload size (include 24 bits CRC): | 64bits | 64bits | 64bits |
| AL | 16 | 16 | 16 |
| Length of PDCCH | 2OS | 2OS | 2OS |
| Channel Coding | POLAR | POLAR | POLAR |
| Channel Model： | TDL-A, 26ns for NLOS; TDL-D, 20ns for LOS | TDL-C, 266 ns for NLOS; TDL-C, 234ns for O2I TDL-E, 80ns for LOS | N/A |
| UE Mobility: | 3km/h | 3km/h | 3km/h |
| Channel estimation: | True | True | True |
| Target BLER | 10^-2 | 10^-2 | 10^-2 |

Table 3: the link level simulation assumptions for PDSCH on FR2

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenarios | Indoor | Urban | Suburban |
| system Bandwidth | 50MHz | 50MHz | 50MHz |
| Carrier: | 28G | 28G | 28G |
| SCS | 60KHz | 60KHz | 60KHz |
| Antenna configuration | 2T2R | 2T2R | 2T2R |
| Length of TTI | 14 OS | 14 OS | 14 OS |
| Duration of PDCCH | 3 OS | 3 OS | 3 OS |
| Number of DMRS | 1 OS | 1 OS | 1 OS |
| Occupied channel bandwidth | 66PRBs | 66PRBs | 66PRBs |
| MCS [Note1] | MCS14 (602/1024, QPSK) | MCS14 (602/1024, QPSK) | MCS 0 (30/1024,QPSK) |
| Channel Coding | LDPC | LDPC | LDPC |
| Channel Model： | TDL-A, 26ns for NLOS; TDL-D, 20ns for LOS | TDL-C, 266 ns for NLOS; TDL-E, 80ns for LOS | N/A |
| UE Mobility: | 3km/h | 3km/h | 3km/h |
| Channel estimation | True | True | True |
| Target BLER | 10^-1 | 10^-1 | 10^-1 |
| Maximum number of transmission | 1 (No HARQ) | 1 (No HARQ) | 1 (No HARQ) |
| UL DL configuration | DDDSUDDSUU | DDDSUDDSUU | DDDSUDDSUU |
| Note 1: The code rate and modulation order is calculated by the target data rate and the available resource for PDSCH excluding the PDCCH and DMRS resource. |  |  |  |

Table 4: the link level simulation assumptions for PUCCH on FR2

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Indoor | Urban | Suburban |
| system Bandwidth | 50MHz | 50MHz | 50MHz |
| Carrier | 28G | 28G | 28G |
| SCS | 60KHz | 60KHz | 60KHz |
| Antenna configuration | 1T2R | 1T2R | 1T2R |
| Format type | Format 1 | Format 1 | Format 1 |
| Length of PUCCH | 14 OS | 14 OS | 14 OS |
| Number of UCI bits | 1 bit | 1 bit | 1 bit |
| Occupied channel bandwidth | 1 PRB | 1 PRB | 1 PRB |
| Channel Mode | TDL-A, 26ns for NLOS; TDL-D, 20ns for LOS | TDL-C, 266 ns for NLOS; TDL-E, 80ns for LOS | N/A |
| UE Mobility: | 3km/h | 3km/h | 3km/h |
| Channel estimation | True | True | True |
| Frequency hopping | On | On | On |
| PUCCH repetition | Yes | Yes | Yes |

Table 5: the link level simulation assumptions for PUSCH on FR2

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Indoor | Urban | Suburban |
| system Bandwidth | 50MHz | 50MHz | 50MHz |
| Carrier | 28G | 28G | 28G |
| SCS | 60KHz | 60KHz | 60KHz |
| Antenna configuration | 2T2R | 2T2R | 2T2R |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| Number of DMRS | 1 OS | 1 OS | 1 OS |
| Occupied channel bandwidth | 30PRBs | 30PRBs | 4PRBs |
| MCS [Note2] | CR=449/1024, QPSK | CR=449/1024, QPSK | CR=120/1024, QPSK |
| Channel Coding | LDPC | LDPC | LDPC |
| Channel Mode | TDL-A, 26ns for NLOS; TDL-D, 20ns for LOS | TDL-C, 266 ns for NLOS; TDL-E, 80ns for LOS | N/A |
| UE Mobility: | 3km/h | 3km/h | 3km/h |
| Channel estimation | True | True | True |
| Target BLER | 10^-1 | 10^-1 | 10^-1 |
| Maximum number of transmission | 1 (No HARQ) | 1 (No HARQ) | 1 (No HARQ) |
| UL DL configuration | DDDSUDDSUU | DDDSUDDSUU | DDDSUDDSUU |
| Note 2: The code rate and modulation order is calculated by the target data rate and the available resource for PUSCH excluding the DMRS resource. |  |  |  |

Proposal 3: The parameters provided in Table 2 - Table 5 can be used as starting point for LLS.

As analyzed in our companion contribution [2], the following issues should be also clarified for FR2 in order to achieve a common understanding:

- How to understand the target data rate

- Whether to define different target coverage for TDD and FDD separately

- How to understand the target data rate of VoIP

Proposal 4: Clarify the following issues for the common understanding:

- How to understand the target data rate

- Whether to define different target coverage for TDD and FDD separately

- How to understand the target data rate of VoIP

Conclusion

This contribution discussed the baseline performance and simulation assumptions for FR2, we have following proposals:

Proposal 1: The baseline performance is the MCL from link budget evaluation based on target data rate.

Proposal 2: The target performance is the MPL at the cell edge corresponding to the target ISD.

Proposal 3: The parameters provided in Table 2 - Table 5 can be used as starting point for LLS of FR2.

Proposal 4: Clarify the following issues for the common understanding:

- How to understand the target data rate

- Whether to define different target coverage for TDD and FDD separately

- How to understand the target data rate of VoIP

References

  - RP-193240, New SID on NR coverage enhancement, China Telecom, RAN Meeting #86.

  - R1-2003649, Discussion on the baseline performance and simulation assumptions of coverage enhancement for FR1, CATT, RAN1#101.

  - 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies.

  - Report  ITU-R  M.2412-0, Guidelines for evaluation of radio interface technologies for IMT-2020