3GPP TSG RAN WG1 Meeting #101                                                                	   R1-2003652

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	Preliminary simulation results for different channels

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

At RAN#86 meeting, the SID on NR Rel-17 coverage enhancement was approved [1]. The detailed objectives are as follows:

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - TDD and FDD for FR1.

  - VoIP and eMBB service for FR1.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

In this contribution, we provide preliminary simulation results of PDCCH/PDSCH/PUCCH/PUSCH for FR1.

Discussion

As discussed in the companion contribution [2], it is proposed to reuse the simulation assumptions and link budget methodology employed in IMT-2020 self-evaluation for 3GPP submission as much as possible, performance gap is evaluated according to the methodology. The preliminary simulation results in terms of performance gap between target coverage and baseline coverage are provided as following. More detailed results for different channels can be found in Appendix and the detailed link level assumption is provided in companion contribution [3].

Figure 1: The coverage performance for different channels in urban scenario

Figure 2: The coverage performance for different channels in rural scenario with 4GHz

Figure 3: The coverage performance for different channels in rural scenario with 2GHz

From the Figure 1-Figure 3, we have the following observations:

- For PDCCH, the coverage can meet the requirement in majority scenarios. Even in the case where ISD equals to 6000m, only 1 dB gap between the target coverage and baseline coverage. It should be noted the coverage can be further enhanced if wide-band RS is configured.

- For PDSCH, the coverage is not the bottleneck in all the simulated scenarios.

- For PUCCH, the coverage enhancement is only necessary when large ISD, e.g. 6000m, is assumed. It can satisfy the coverage requirement with ISD=1732m without repetition.

- For PUSCH, the coverage is more challenging.

  - PUSCH coverage can’t meet the requirement of target ISD=400m in Urban scenario with 4GHz

  - PUSCH coverage can’t meet the requirement of target ISD=1732m in Rural scenario with 4GHz

  - PUSCH coverage can’t meet the requirement of target ISD=6000m in Rural scenario with 2GHz

  - PUSCH coverage can’t meet the requirement of target ISD=30 km in Rural with long distance scenario with 700MHz

Observation 1: The coverage of PDCCH and PDSCH is not the bottleneck at least in FR1.

Observation 2: The coverage of PUCCH is the bottleneck only in limited scenarios.

Observation 3: The coverage of PUSCH is bottleneck in majority scenarios.

Conclusion

This contribution discussed the preliminary simulation results for PDCCH/PDSCH/PUCCH/PUSCH in FR1, we have the following observations.

Observation 1: The coverage of PDCCH and PDSCH is not the bottleneck at least in FR1.

Observation 2: The coverage of PUCCH is the bottleneck only in limited scenarios.

Observation 3: The coverage of PUSCH is bottleneck in majority scenarios.

References

  - RP-193240, New SID on NR coverage enhancement, China Telecom, RAN Meeting #86.

  - R1-2003648, Discussion about baseline coverage performance, , CATT, RAN1#101

  - R1-2003649, Discussion on the baseline performance and simulation  assumptions of coverage enhancement for FR1, CATT, RAN1#101

Appendix 1: Link level simulation results

Table 1: The performance gap for PDCCH in Urban with 4GHz

| Scenario and Carrier | PDCCH length | Payload size | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=400m | Gap | ISD=500m | Gap |
| Urban 4GHz | 2OS | 64 | NLOS (O-to-I) | 51 | -7.66 | 135.07 | 117.97 | -17.10 | 121.75 | -13.31 |

Table 2: The performance gap for PDCCH in Rural with 4GHz

| Scenario and Carrier | PDCCH length | Payload size | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=1732m | Gap |
| Rural 4GHz | 2OS | 64 | NLOS (O-to-I) | 51 | -7.66 | 145.17 | 131.57 | -13.60 |
|  |  |  | NLOS (O-to-O) |  | -7.71 | 146.72 |  | -15.15 |

Table 3: The performance gap for PDCCH in Rural with 2GHz

| Scenario and Carrier | PDCCH length | Payload size | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=1732m | Gap | ISD=6000m | Gap |
| Rural 2GHz | 2OS | 64 | NLOS (O-to-I) | 51 | -7.57 | 145.08 | 125.55 | -19.53 | 146.40 | 1.32 |
|  |  |  | NLOS (O-to-O) |  | -7.76 | 146.77 |  | -21.22 |  | -0.37 |

Table 4: The performance gap for PDSCH in Urban with 4GHz

| Scenario and Carrier | Service and data rate | Frame structure | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=400m | Gap | ISD=500m | Gap |
| Urban 4GHz | 10 Mbps for eMBB | DDDSUDDSUU | NLOS (O-to-I) | 51 | 4.9 | 126.09 | 117.97 | -8.12 | 121.75 | -4.33 |
|  | 12.2Kbps for VoIP | - | NLOS (O-to-I) | 51 | -9.07 | 140.06 |  | -22.09 |  | -18.30 |

Table 5: The performance gap for PDSCH in Rural with 4GHz

| Scenario and Carrier | Service and data rate | Frame structure | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=1732m | Gap |
| Rural 4GHz | 1 Mbps for eMBB | DDDSUDDSUU | NLOS (O-to-I) | 51 | -6.6 | 147.93 | 131.57 | -16.36 |
|  |  |  | NLOS (O-to-O) | 51 | -5.7 | 149.05 |  | -17.48 |
|  | 12.2Kbps for VoIP | - | NLOS (O-to-I) | 51 | -8.7 | 150.03 |  | -18.46 |
|  |  |  | NLOS (O-to-O) | 51 | -8 | 151.35 |  | -19.78 |

Table 6: The performance gap for Rural PDSCH in with 2GHz

| Scenario and Carrier | Service and data rate | Frame structure | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=1732m | Gap | ISD=6000m | Gap |
| Rural 2GHz | 1Mbps for eMBB | DDDSUDDSUU | NLOS (O-to-I) | 51 | -8 | 149.33 | 125.55 | -23.78 | 146.40 | -2.93 |
|  |  |  | NLOS (O-to-O) | 51 | -8 | 151.35 |  | -25.80 |  | -4.95 |
|  | 12.2Kbps for VoIP | - | NLOS (O-to-I) | 51 | -9.6 | 150.93 |  | -25.38 |  | -4.53 |
|  |  |  | NLOS (O-to-O) | 51 | -8.66 | 152.01 |  | -26.46 |  | -5.61 |

Table 7: Performance gap for PUCCH for Urban with 4GHz

| Carrier | Pathloss model | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ISD = 400m | Gap | ISD = 500m | Gap |
| 4GHz | NLOS (O-to-I) | -6.9 | 123.32 | 118 | -5.32 | 121.81 | -1.51 |

Table 8: Performance gap for PUCCH for Rural with 4GHz

| Carrier | Pathloss model | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ISD = 1732m | Gap |
| 4GHz | NLOS (O-to-I) | -6.9 | 131.40 | 131.58 | 0.18 |
| 4GHz | NLOS (O-to-O) | -7 | 133 | 131.58 | -1.42 |

Table 9: Performance gap for PUSCH for Rural with 2GHz

| Carrier | Pathloss model | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ISD = 1732m | Gap | ISD = 6000m | Gap |
| 2GHz | NLOS (O-to-I) | -7.3 | 131.80 | 125.56 | -6.24 | 146.40 | 14.6 |
| 2GHz | NLOS (O-to-O) | -7.2 | 133.2 | 125.56 | -7.64 | 146.40 | 13.2 |

Table 10: Performance gap for PUSCH for Urban with 4GHz

| Carrier | Service and data rate | Frame structure | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD = 400m | Gap | ISD = 500m | Gap |
| 4GHz | 1Mbps for eMBB | DDDSUDDSUU | NLOS (O-to-I) | 30 | -4.3 | 112.6 | 117.97 | 5.37 | 121.75 | 9.15 |
| 4GHz | 12.2kbps for VoIP | - | NLOS (O-to-O) | 4 | 0.7 | 116.35 | 117.97 | 1.62 | 121.75 | 5.4 |

Table 11: Performance gap for PUSCH for Rural with 4GHz

| Carrier | Service and data rate | Frame structure | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD = 1732m | Gap |
| 4GHz | 100kbps for eMBB | DDDSUDDSUU | NLOS (O-to-I) | 4 | -0.6 | 125.97 | 131.57 | 5.6 |
| 4GHz | 100kbps for eMBB | DDDSUDDSUU | NLOS (O-to-O) | 4 | -1.6 | 128.99 | 131.57 | 2.58 |
| 4GHz | 12.2kbps for VoIP | - | NLOS (O-to-I) | 4 | 1.2 | 124.17 | 131.57 | 7.4 |
| 4GHz | 12.2kbps for VoIP | - | NLOS (O-to-O) | 4 | 0.6 | 126.79 | 131.57 | 4.78 |

Table 12: Performance gap for PUSCH for Rural with 2GHz

| Carrier | Service and data rate | Frame structure | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD = 1732m | Gap | ISD = 6000m | Gap |
| 2GHz | 100kbps for eMBB | UUUUU | NLOS (O-to-I) | 4 | -2 | 130.38 | 125.55 | -4.83 | 146.4 | 16.02 |
| 2GHz | 100kbps for eMBB | UUUUU | NLOS (O-to-O) | 4 | -2.2 | 132.6 | 125.55 | -7.05 | 146.4 | 13.8 |
| 2GHz | 12.2kbps for VoIP | - | NLOS (O-to-I) | 4 | 1 | 127.38 | 125.55 | -1.83 | 146.4 | 19.02 |
| 2GHz | 12.2kbps for VoIP | - | NLOS (O-to-O) | 4 | 1.1 | 129.3 | 125.55 | -3.75 | 146.4 | 17.1 |

Table 13: Performance gap for PUSCH for Rural with long distance scenario

| Carrier | Service and data rate | Frame structure | Pathloss model | The number of PRBs | Required SNR (dB) | Baseline performance(dB) | Target performance (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD = 12km | Gap | ISD = 30km | Gap |
| 700MHz | 100kbps for eMBB | UUUUU | LOS (O-to-O) | 4 | -9.8 | 140.20 | 127 | -13.2 | 142.91 | 2.71 |
| 700MHz | 12.2kbps for VoIP | - | LOS (O-to-O) | 4 | -7.4 | 137.8 | 127 | -10.8 | 142.91 | 5.11 |