3GPP TSG RAN WG1 Meeting #101	R1-2003671

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.4.2.2

Source: 	MediaTek Inc.

Title: 	Sidelink mode-2 resource allocation

Document for:	Discussion/Decision

# Introduction

In this paper, we discuss the remaining issues on sidelink resource allocation mode 2.

# Discussion

In the RAN1#100bis-e meeting [1], the following agreements were made on resource selection aspects:

Agreements: - In Step 2, a UE should/shall select resources so that HARQ retransmission resources can be reserved by a prior SCI, except that - In case no resource can be found for reservation (e.g., based on the identified candidate set after Step 1) for a retransmission of a TB, the re-transmission can be transmitted on a resource that is not reserved - After the resource selection is performed, HARQ retransmission on a resource not reserved by a prior SCI is allowed due to transmission dropping caused by prioritization, pre-emption and congestion control - To discuss and conclude “should vs. shall” in RAN1#101 Working assumption: - The UE should/shall indicate min(Nselected, N) first-in-time resources when setting the values of frequency resource assignment and time resource assignment in SCI format 0_1, where - Nselected is the number of resources selected by MAC within 32 slots (including the current one) - N is the maximum number of resources that can be signalled in one SCI - To discuss and conclude “should vs. shall” in RAN1#101

We have the following proposal on “should vs. shall” issues:

Proposal 1: The following behavior is preferred:

- UE should select resources so that HARQ retransmission resources can be reserved by a prior SCI.

- The UE should indicate min(Nselected,N) first-in-time resources when setting the values of frequency resource assignment and time resource assignment in SCI format 0_1

# Conclusion

The following proposals are made in this contribution.

Proposal 1: The following behavior is preferred:

- UE should select resources so that HARQ retransmission resources can be reserved by a prior SCI.

- The UE should indicate min(Nselected,N) first-in-time resources when setting the values of frequency resource assignment and time resource assignment in SCI format 0_1

# References

- Chairman’s notes, RAN1#100bis-e, April 2020.