3GPP TSG RAN WG1 Meeting #101-e 	R1-2003678

e-Meeting, May 25th – June 05th, 2020

Agenda Item: 7.2.11.9

Source: MediaTek Inc.

Title: 	Views on Rel-16 UE features for NR mobility enhancements

Document for: Discussion

- Introduction

This paper provides our views on Rel-16 UE features for NR mobility enhancements. Detailed proposals are described in the following sections.

- Views on UE features for NR mobility enhancements

For [FG 21-2d UL transmission cancellation], we support to keep it. The reason is that the current working assumption of UL cancellation spec seems to imply a symbol-based cancellation, which can impose high implementation complexity from UE’s perspective.

In the FL summary of 7.2.9.1 [1] and the corresponding draft CR on UL transmission cancellation [2], the current working assumption for 38.213 Clause 15 is:

Basically, the additional spec (in red text) says that a symbol-based cancellation is applied:

- A UE does not expect to cancel a transmission symbol on the source cell that occur, relative to a last symbol of a detected DCI on target cell, after a number of symbols smaller than Tproc,2.

An illustrative figure is shown below:

Agreement:

- Confirm WA from RAN1 #98bis on UL transmission of signals/channels for DAPS HO with the following changes:

  - Collision (in above) is defined for the following cases:

    - physical time resources for UL channel/signals partially or fully overlap for the intra-frequency intra-band scenario.

    - physical time and frequency resources for UL channel/signals partially or fully overlap in time and frequency for any other scenario.

  - Note: Cases when UE realizes UL transmission collides after transmission to the source/target cell is ongoing can be discussed during the CR review.

It can be seen from the figure above that, UE may need to do symbol-based UL cancellation after transmission to the source/target cell is ongoing. According to the RAN1 #99 agreement:

It is mentioned that whether to do cancellation when UE realizes UL transmission collides after transmission to the source/target cell is ongoing should be discussed further.

Besides, in the RAN1 #99 agreement, the collision is defined when there is partially or fully overlap for time resources, while partially overlap seems to be indication for “UL transmission based” cancellation, not for “symbol based” cancellation. Therefore, current working assumption of UL cancellation spec seems to imply a symbol-based cancellation and imposes high complexity to UE implementation. Hence, we support to keep [FG 21-2d UL transmission cancellation].

Proposal 1: Keep FG 21-2d UL transmission cancellation].

Reference

R1-2002806, “Summary of email discussions for [100b-e-NR-Mob-Enh-01]”, Intel, RAN1 #100-bis-e

R1-2003137, “Draft CR on UL transmission cancellation for UL DAPS HO”, Intel, RAN1 #100-bis-e

# Proposed changes on current RAN1 UE features table

Mobility Enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21. Mobility Enhancement | 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network cannot configure UE with DAPS HO | Per Band | No | N/A | N/A |  | [Optional with capability signalling] |
|  | 21-1b | Inter-frequency DAPS HO | Support of inter-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network cannot configure UE with DAPS HO | Per BC | No | N/A | N/A |  | [Optional with capability signalling] |
|  | 21-2 | Basic UE power sharing for DAPS HO | Support of semi-static power sharing mode1 |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | FFS: The UE is only able to drop the transmission to the source. | Per BC | No | N/A | N/A |  | Optional with capability signalling |
|  | 21-2a | Semi-static UL power sharing mode 2 for DAPS HO | Support of semi-static power sharing mode 2 |  | 21-2 |  | Yes | N/A |  | Per BC | No | N/A | N/A |  | Optional with capability signalling |
|  | 21-2b | Dynamic UL power sharing for DAPS HO | Support of dynamic power sharing |  | 21-2 |  | Yes | N/A |  | Per BC | No | N/A | N/A |  | Optional with capability signalling |
|  | 21-2d | [UL transmission cancellation] | [Indicates support of cancelling UL transmission to the source cell] |  | DAPS (Note: RAN2 feature) |  | Yes | N/A |  | Per Band and Per BC | No | N/A | N/A |  | Optional with capability signalling |