3GPP TSG RAN WG1#101-e			R1-2003879

e-Meeting, 25th May – 5th June, 2020

Agenda item:	7.2.6.1

Source: 	Samsung

Title: 	On maintenance of Rel.16 MU CSI enhancements

Document for:	Discussion and Decision

# Introduction

The contribution discusses our view about some of the UE capability related features.

# UE capability

The outcome of UE feature discussion during RAN1#100-b-e [1] includes the following codebook-related feature groups (FGs).

| 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - FFS: CBSR with hard amplitude restriction |
| --- | --- | --- |
| 16-3a-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=2 |
| 16-3a-2 | Support of parameter combinations 7-8 | Support of parameter combinations 7-8 |
| 16-3a-3 | Support of rank 3,4 | Support of rank 3,4 |
| 16-3a-4 | CBSR | ALT 1) CBSR with soft amplitude restriction (capture consequence if not supported hard amplitude restriction is supported) ALT 2) CBSR |
| 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 |
| 16-3b-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 |
| 16-3b-2 | Support of rank 3,4 | Support of rank 3,4 |
| 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | FFS: the detailed component design |

In this section, two remaining FFS items are discussed.

The first item is regarding codebook subset restriction (CBSR) for regular eTypeII codebook. The following agreement was made in RAN#98b.

Agreement For amplitude restriction mechanism: - Alt 0. Analogous to Rel.15 Type II (SD beam group restriction + per coefficient hard amplitude restriction) - Four beam groups are selected via higher-layer configured bitmap B1 - For each spatial beam in each of the four beam groups, hard restriction (maximum amplitude of 0 or 1) is applied to any of the coefficients associated with the beam (the restriction is applied for both polarizations of the beam). This maximum amplitude restriction is higher-layer configured with four bitmaps - Alt 3A from RAN1#98 (soft with sum-power-ratio constraint), simplified to 1KNZ(λ,k,l0)×m=0M-1pres,l02λ,m,k≤γl0 for each (λ,k,l0) where KNZ(λ,k,l0) denotes the number of NZCs associated with (λ,k,l0). - The value of γl0 is configured from the Rel.15 2-bit amplitude restriction table - The number of beam-groups is the same as Rel.15 Type II CBSR Support Alt0 as mandatory and Alt3A (described above) as optional analogous to Rel.15 Type II codebook. - Cf. Notes in UE capability agreement in RAN1#98bis regarding “mandatory” and “optional”

As highlighted, the soft restriction (Alt 0 in agreement) is mandatory for a UE supporting CBSR and soft restriction (Alt 3A in agreement) is optional. Based on this agreement and two alternatives (ALT 1 and ALT 2) according to FG 16-3a-4, there are two possible options for capability signalling:

- Option 1: if ALT 1 of FG 16-3a-4 is agreed, then FFS in FG 16-3a should be included (not removed). Hence,

  - FG 16-3a includes #4 basis component - CBSR with hard amplitude restriction

  - FG 16-3a-4 includes CBSR with soft amplitude restriction

- Option 2: if ALT 2 of FG 16-3a-4, then entire CBSR (both hard as well as soft amplitude restrictions) is a UE optional feature. Hence,

  - Item #4 basis component from FG 16-3a is removed

  - FG 16-3a-4 includes CBSR. Since soft restriction was agreed to UE optional, the capability signalling according to FG 16-3a-4 should include explicit indication whether the UE is capable of soft amplitude restriction.

In our view, support of CBSR is a basic feature of a codebook since it provides more flexibility to the network. Therefore, Option 1 is preferable. Note that there is a similar capability signalling of amplitudeSubsetRestriction for “soft” amplitude restriction in Rel. 15 TypeII CBSR, the same capability signalling can be reused for Rel. 16 eTypeII.

Proposal 1: Support Option 1 for Rel. 16 eTypeII CBSR, i.e.,

- FG 16-3a includes #4 basis component - CBSR with hard amplitude restriction

- FG 16-3a-4 includes CBSR with soft amplitude restriction

- Reuse parameter Rel.15 amplitudeSubsetRestriction for soft amplitude restriction

The second item is regarding FG 16-8 (mixed codebook types or concurrent codebooks). In our view, the likelihood of and need for network configuring two Type II codebooks (Rel. 15/16 regular and port selection) to a UE is unclear. In practice, processing more than one TypeII CSI reports simultaneously is very challenging (nor desired) in terms of UE’s CPU processing capabilities. Therefore, the details of FG 16-8 should be discussed based on the assumption that a mixed codebook type shall not include more than one TypeII codebooks.

Proposal 2: the details of FG 16-8 should be discussed based on the assumption that a mixed codebook type shall not include more than one TypeII codebooks.

Conclusion

In this contribution, the following proposals are given:

Proposal 1: Support Option 1 for Rel. 16 eTypeII CBSR, i.e.,

- FG 16-3a includes #4 basis component - CBSR with hard amplitude restriction

- FG 16-3a-4 includes CBSR with soft amplitude restriction

- Reuse parameter Rel.15 amplitudeSubsetRestriction for soft amplitude restriction

Proposal 2: the details of FG 16-8 should be discussed based on the assumption that a mixed codebook type shall not include more than one TypeII codebooks.

# References

- R1-2003073, “RAN1 UE features list for Rel-16 NR after RAN1#100bis-E,” AT&T/NTT DOCOMO, INC.