3GPP TSG RAN WG1#101-e			R1-2003881

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.6.2

Source: 	Samsung

Title: 	On Rel.16 multi-TRP/panel transmission

Document for:	Discussion and Decision

Introduction

This contribution provides Samsung’s views on Rel.16 multi-TRP/Panel Transmission.

Default QCL enhancements for multi-TRP/Panel

- Issue 1. Condition to support enhanced default beam for PDSCH

In RAN1#99, two agreements have been made that the support of enhanced default beams for both multi-DCI based and single-DCI based multi-TRP are up to UE capability as follows.

Agreement For multi-DCI based multi-TRP/panel transmission, if CORESETPoolIndex is configured, - If the time offset between the reception of the PDCCH and the corresponding PDSCH is less than a threshold, UE could assume that the DM-RS ports of PDSCH are QCL-ed with the RS(s) with respect to the QCL parameter(s) used for PDCCH of the lowest CORESET index among CORESETs configured with the same value of CORESETPoolIndex, - in the respective latest slot in which one or more CORESETs associated with each of CORESETPoolIndex within the active BWP of the serving cell are monitored by the UE - The support of this feature is indicated by UE capability - If the UE does not support the above feature, Rel-15 behavior is reused regardless of CORESETPoolIndex Agreement For single-DCI based Multi-TRP/panel transmission with at least one configured TCI states for the serving cell of scheduled PDSCH containing 'QCL-TypeD', - If the offset between the reception of the PDCCH and the corresponding PDSCH is less than timeDurationForQCL and after the reception of activation command of TCI states for UE specific PDSCH, the UE may assume that DMRS ports of PDSCH follows QCL parameters indicated by default TCI state(s) as following: - Use the TCI-states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states which are activated for PDSCH. - If all the TCI codepoints are mapped to a single TCI state, then Rel-15 behavior is followed The support of this feature is part of UE capability.

However, current RAN1 specification misses the highlighted part of agreements which has to be fixed. In addressing the agreements, we have to follow the RAN2 guidance [1] which avoids defining a functionality implicitly by reported UE capabilities, but guides gNB to configure the functionality explicitly by DL RRC signalling. In this sense, we need to introduce two new RRC parameters, each of which indicates enabling enhanced default beam for multi-DCI and single-DCI, respectively. The RRC parameters are incorporated in the following TP:

Proposal 1: Introduce new RRC parameters indicating UE to enable enhanced default beam for multi-DCI and single-DCI, respectively.

Proposal 2: On enhanced default beam for multi-DCI and single-DCI, adopt the following TP in TS 38.214 V16.1.0:

5.1.5 Antenna ports quasi co-location --- start of TP --- Independent of the configuration of tci-PresentInDCI and tci-PresentInDCI-ForFormat1_2 in RRC connected mode, if no TCI codepoints are mapped to two different TCI states and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If a UE configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, for both cases, when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and the UE is configured with [enableEnhDefaultBeamMDCI], the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. If the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states and the UE is configured with [enableEnhDefaultBeamSDCI], the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. --- end of TP ---

- Issue 2. Default beam for cross-carrier scheduling in single-PDCCH based NC-JT

In Rel. 16, PDSCH default beam behaviors have been enhanced for two different operation scenarios, single-PDCCH based NC-JT and cross carrier scheduling in MR-DC, as captured in the table below.

| Multi-TRP Agreement For single-DCI based Multi-TRP/panel transmission with at least one configured TCI states for the serving cell of scheduled PDSCH containing 'QCL-TypeD', - If the offset between the reception of the PDCCH and the corresponding PDSCH is less than timeDurationForQCL and after the reception of activation command of TCI states for UE specific PDSCH, the UE may assume that DMRS ports of PDSCH follows QCL parameters indicated by default TCI state(s) as following: - Use the TCI-states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states which are activated for PDSCH. - If all the TCI codepoints are mapped to a single TCI state, then Rel-15 behavior is followed The support of this feature is part of UE capability. MR-DC Agreements: When PDSCH and its scheduling PDCCH are in the different CCs, if the PDCCH-to-PDSCH delay < Threshold-Sched-Offset timeDurationForQCL or if the TCI information is absent from the DCI, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell Agreement: Adopt a new RRC configuration parameter with which the network selects either Rel-15 or Rel-16 behaviour for the default beam selection | [TS 38.214 V16.1.0] 5.1.5 Antenna ports quasi co-location ... Independent of the configuration of tci-PresentInDCI and tci-PresentInDCI-ForFormat1_2 in RRC connected mode, if no TCI codepoints are mapped to two different TCI states and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If a UE configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, for both cases, when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. If the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enabledDefaultBeamForCCS]: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell. … |
| --- | --- |

Although there has been no intention to make above two features be mutually exclusive to each other from RAN1 discussions so far, it seems that the current specification does not capture any UE behavior when single PDCCH based NC-JT operation is scheduled by cross carrier scheduling. To address this issue, the following TP can be considered:

Proposal 3: On PDSCH default beam for single-PDCCH based NC-JT, adopt the following TP in TS 38.214 V16.1.0:

5.1.5 Antenna ports quasi co-location --- start of TP --- If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enabledDefaultBeamForCCS]: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell; - When tci-PresentInDCI is set to 'enabled' and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and if at least one TCI codepoint indicates two TCI states for the component carrier of scheduled PDSCH and the UE is configured with [enableEnhDefaultBeamSDCI], the UE may assume that the DM-RS ports of PDSCH of the component carrier are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. --- end of TP ---

- Issue 3. Default beam for AP-CSI-RS in NC-JT

In current specification, default beam operation is defined to receive AP-CSI-RS with triggering offset smaller than beamSwitchTiming as follows: If AP-CSI-RS is not overlapped with any DL signal, UE applies the default beam assumption of PDSCH in Rel-15. Otherwise, UE applies the QCL assumption of the other DL signal. Such operation is designed in Rel-15 with single TRP assumption and directly applying it to Rel-16 multi-TRP causes some serious problems which should be corrected as stated below.

First, consider the case that AP-CSI-RS is not overlapped with any DL signals. Here, the term ‘overlap’ means time domain overlapping with the DL signal in the same CC, or different CC in the same band. In this case, if enhanced default beam for PDSCH is enabled, UE has to perform not only enhanced default beam operation to receive PDSCH but also Rel-15 default beam operation to receive AP-CSI-RS. Such fragmented default beam operation complicates UE behavior with no clear benefit. To fix the problem, default beam of PDSCH and that of AP-CSI-RS need to be unified. In other words, when enhanced default beam is configured, it is applied to AP-CSI-RS as well as PDSCH. In multi-DCI based multi-TRP, AP-CSI-RS triggered by an UL DCI associated with specific CORESETPoolIndex can be received via the default beam for that CORESETPoolIndex. In single-DCI based multi-TRP, AP-CSI-RS can be received via any of the two enhanced default beams. For simplicity, we can apply the first among the two for AP-CSI-RS reception. If enhanced default beam for PDSCH is not enabled, Rel-15 default beam can be applied.

Proposal 4: When AP-CSI-RS is not overlapped with any DL signals, apply default beam to receive AP-CSI-RS as

- Default beam of the CORESETPoolIndex associated with the triggering DCI, if enhanced default beam for M-DCI M-TRP is enabled

- First among the two default beams, if enhanced default beam for S-DCI M-TRP is enabled

- Rel-15 default beam, if enhanced default beam is not enabled

Note: The DL signal could be in the same CC, or different CC in the same band.

Second, consider the case that AP-CSI-RS is overlapped with DL signals. For multi-DCI based multi-TRP, AP-CSI-RS and an overlapping DL signal shall be transmitted from the same TRP because the same Rx beam would be applied to them. In other words, according to current spec AP-CSI-RS should not be overlapped with DL signal from other TRP which needs dynamic coordination across TRPs to meet such restriction. Since dynamic coordination is challenging especially in non-ideal backhaul scenario, which is a main scenario for multi-DCI based multi-TRP, such restriction needs to be relaxed. A simple solution is letting AP-CSI-RS to follow QCL assumption of the overlapped DL signal when AP-CSI-RS and the DL signal are associated with the same CORESETPoolIndex. When AP-CSI-RS and overlapped DL signal are associated with different CORESETPoolIndex, default beam associated with AP-CSI-RS would apply in receiving the AP-CSI-RS. For the case when AP-CSI-RS is overlapped with DL signal with no CORESETPoolIndex association, e.g., P-CSI-RS, existing Rel-15 rule can be applied.

For single-DCI based multi-TRP, AP-CSI-RS can be overlapped with a DL signal having two TCI states. In this case, it is unclear which of the two TCI states would be applied for AP-CSI-RS reception. For simplicity, we can apply the first one among the two for AP-CSI-RS reception. When AP-CSI-RS is overlapped with a DL signal with single TCI state, existing Rel-15 rule can be applied.

Proposal 5: When AP-CSI-RS is overlapped with DL signal in a same band,

- For multi-DCI based multi-TRP

  - When AP-CSI-RS is overlapped with a DL signal associated with the same CORESETPoolIndex, apply QCL assumption of the DL signal in receiving the AP-CSI-RS

  - When AP-CSI-RS is overlapped with a DL signal associated with different CORESETPoolIndex, apply default beam associated with the AP-CSI-RS

  - When AP-CSI-RS is overlapped with DL signal with no CORESETPoolIndex association, reuse Rel-15 rule

- For single-DCI based multi-TRP

  - When AP-CSI-RS is overlapped with a DL signal having two TCI states, apply the first indicated TCI state of the DL signal in receiving the AP-CSI-RS

  - When AP-CSI-RS is overlapped with a DL signal having single TCI state, reuse Rel-15 rule

Note: The DL signal could be in the same CC, or different CC in the same band.

- Issue 4. Default beam for scheme 3/4

For the default beam behaviour of Rel-15 PDSCH slot aggregation, the default beam behaviour for a single PDSCH is applied by treating multiple PDSCHs based on slot aggregation as a single PDSCH. However, for the Rel-16 scheme 3 and 4, there are two different behaviours in TS 38.214 as follows:

- In Clause 5.1.2.1, it is stated that if two TCI states are indicated by the DCI field 'Transmission Configuration Indication' for scheme 3 and 4, the UE is expected to receive two PDSCH transmission occasions, where the first and second TCI states are applied to the first and second PDSCH transmission occasions, respectively. This operation is applied regardless of the offset between the reception of the DL DCI and the corresponding PDSCH and the threshold timeDurationForQCL.

- In Clause 5.1.5, it is also stated that if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE may assume that the enhanced default beams for PDSCH are corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states.

Then, when the offset between the DCI and the corresponding PDSCH transmission occasion is less than the threshold, the UE is expected to buffer two PDSCHs based on the enhanced default beams, but after the threshold, the UE needs to apply the indicated TCI states. This may be a problem since the enhanced default beams and indicated TCI states are not always same. Also, the enhanced default beam can be supported based on the reported UE capability. If the UE capability is not reported, only the Rel-15 default beam is used. Considering both the consistency with Rel-15 default beam behaviour and the UE capability for enhanced default beam, a simple solution can be adopted for the default beam behaviour for scheme 3 and 4. When the UE can support enhanced default beam and the time offset between the DCI and at least one PDSCH transmission occasion among all PDSCHs is less than the threshold, the enhanced default TCI states are applied on the reception of all PDSCH transmission occasions. When applying the enhanced default TCI states for receiving PDSCH transmission occasions, the predefined mapping rule can be applied as in Section 5.1.2.1 in TS 38.214. when the UE cannot support enhanced default beam and the time offset between the DCI and at least one PDSCH transmission among all PDSCHs is less than the threshold, the Rel-15 default TCI state, i.e., the QCL assumption of the CORESET with the lowest controlResourceSetId in the latest slot, is applied to all PDSCH receptions.

Observation 1: Default QCL behaviours for scheme 3 and 4, stated at Clause 5.1.2.1 and Clause 5.1.5 in TS 38.214, are inconsistent each other.

Proposal 6: For Default beam for scheme 3 and 4,

- For UE supporting the enhanced default beam, when the time offset between the DCI and at least one PDSCH transmission occasion among all PDSCHs is less than the threshold, the enhanced default TCI states are applied on the reception of all PDSCH transmission occasions. The mapping between enhanced default TCI states and PDSCH transmission occasions follows the mapping specified in Section 5.1.2.1 in TS 38.214.

- For UE not supporting the enhanced default beam, when the time offset between the DCI and at least one PDSCH transmission occasion among all PDSCHs is less than the threshold, the Rel-15 default TCI state is applied on the reception of all PDSCH transmission occasions.

Other aspects for multi-TRP/Panel

- Issue 5. BWP switch in multi-DCI based NC-JT

Since the number of active BWPs for a UE is 1 per CC, the values of BWP indicators in the two different DCIs shall be identical for supporting NC-JT to allocate two different PDSCHs within the same serving cell. In the current specification 38.214 V16.1.0 [3], it is already clarified clearly that when the UE is scheduled with full/partially/non-overlapped PDSCHs in time and frequency domain, the UE is expected to be scheduled PDSCHs with the same active BWP and the same SCS. In other words, the current specification clearly says that it is up to gNB implementation to coordinate to ensure the same BWP for multi-DCI based multi-TRP operation. Otherwise, UE will follow the legacy operation from Rel-15, i.e. no critical issue on BWP switch with the current specifications.

Observation 2: In Rel-16, it is up to gNB implementation to coordinate each other to ensure the same BWP for scheduling two different PDSCHs by two different DCIs to the UE.

- Issue 6. Whether to allow simultaneous reception of PDCCHs with different QCL-TypeD

In the current specification 38.213 V16.1.0 [2], the UE is allowed to monitor PDCCH with a single QCL-TypeD based on the PDCCH prioritization rule when CORESETs having different QCL-TypeD are overlapped on the same OFDM symbol. Since the current specification allows simultaneous reception of PDSCHs associated with different QCL-TypeD, one may argue that the existing PDCCH prioritization rule should be revised to enable simultaneous reception of PDCCHs associated with different QCL-TypeD as well. Although such types of proposals may provide more flexible PDCCH allocation, e.g. for more resources for data transmission from link performance perspective, the actual benefit of such optimization is somewhat unclear in network perspective since the overall PDCCH transmission overhead of the network will not be changed. Therefore, it is preferred to have further discussions on this issue in the upcoming releases rather than to handle it in the CR phase without relevant investigations.

Proposal 7: Do not support simultaneous reception of PDCCHs with multiple different QCL-TypeD.

- Issue 7. Correction on CRS rate matching for multi-TRP

Agreement - Add a new RRC parameter [crs-RateMatch-PerCORESETPoolIndex] that indicates the UE does rate match PDSCH around configured CRS for each TRP - Adopt the following TP in R1-200XXXX for editor’s CR on TS 38.214: < Start of the text proposal > 5.1.4.2 PDSCH resource mapping with RE level granularity *** Unchanged text is omitted *** If the UE is configured by higher layer parameter PDCCH-Config with two different values of CORESETPoolIndex in ControlResourceSet and is also configured by the higher layer parameter LTE-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - If the UE is configured with [crs-RateMatch-PerCORESETPoolIndex], REs indicated by the CRS pattern(s) in lte-CRS-PatternList-r16 if the PDSCH is associated with CORESETPoolIndex = 0, or the CRS pattern(s) in lte-CRS-PatternListSecond-r16 if the PDSCH is associated with CORESETPoolIndex = 1CRSPatternList-CORESETPoolIndex for a UE supporting the capability of [separate-lte-CRS-ToMatchAround]; - Otherwise, REs indicated by CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 in ServingCellConfig for a UE not supporting the capability of [separate-lte-CRS-ToMatchAround]. < End of the text proposal >.

In RAN1#100-b-e, the above agreement was made but the endorsed editor CR for TS38.214 [6] has wrongly captured an RRC parameter in the agreement, lte-CRS-PatternList-r16. To fix the error, we propose the following TP:

Proposal 8: On CRS rate matching for multi-TRP, adopt the following TP in TS 38.214 V16.1.0:

5.1.4.2 PDSCH resource mapping with RE level granularity *** Unchanged text is omitted *** - If the UE is configured by higher layer parameter PDCCH-Config with two different values of CORESETPoolIndex in ControlResourceSet and is also configured by the higher layer parameter LTE-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - If the UE is configured with [crs-RateMatch-PerCORESETPoolIndex], REs indicated by the CRS pattern(s) in lte-CRS-PatternList-r16 if the PDSCH is associated with CORESETPoolIndex = 0, or the CRS pattern(s) in lte-CRS-PatternListSecond-r16 if PDSCH is associated with CORESETPoolIndex = 1,; - Otherwise, REs indicated by CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16, in ServingCellConfig. < End of the TP >

- Issue 8. Overlapping rule among multiple active SPS PDSCHs within a slot with multi-TRP operation

In current specification the association of SPS PDSCH to the TRPs is not clearly defined. In our understanding the SPS PDSCH with the activation DCI is associated with the CORESETPoolIndex value of the CORESET in which the activation DCI is received. The subsequent SPS PDSCHs without DCI should also follow the same association.

Proposal 9: For a SPS configuration, SPS PDSCH with or without DCI is associated with a TRP according to the value of CORESETPoolIndex in a CORESET in which the activation DCI of the SPS configuration is received.

In RAN1#99 an agreement was reached to allow reception of two overlapping DG PDSCHs each from a different TRP. UE may declare a capability to support such a feature.

Agreement For multi-DCI based multi-TRP, when PDCCHs schedule two PDSCHs/PUSCHs across TRPs, i.e. PDCCHs are associated with different values of CORESETpoolIndex, following operations are allowed: - PDCCH to PDSCH: For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i.

Rel-16 has also introduced support of multiple active SPS configuration in a cell. For a UE that operates in a cell with multiple active SPS configuration and Multi-DCI M-TRP, and declares a capability of receiving two overlapping DG PDSCH, reception of two overlapping SPS PDSCHs should also be supported if the two PDSCHs are associated with different CORESETPoolIndex values. In fact receiving two overlapping SPS PDSCHs may be even less of a challenge as the UE is aware of the PDSCH resources semi-statically, once the configurations have been activated.

Proposal 10: Extend the agreement in RAN1#99 to include two SPS PDSCHs which overlap in time domain and are associated to two different TRPs according to the value of CORESETPoolIndex.

Proposal 11: Adopt the following TP in TS 38.214 V16.1.0:

5.1 UE procedure for receiving the physical downlink shared channel --- start of TP --- For any SPS configuration, the corresponding SPS PDSCH with or without PDCCH is associated with a value of CORESETPoolIndex of the CORESET in which the activation DCI of the SPS configuration is received. If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are partially or fully overlapping in time and they are associated with the same value of CORESETPoolIndex, a UE is not required to receive a PDSCH among these PDSCHs other than one with the lowest configured sps-ConfigIndex. If UE declares a capability to receive two overlapping PDSCHs associated with different values of CORESETPoolIndex, UE is expected to receive two overlapping SPS PDSCHs in a given serving cell if they are associated with different values of CORESETPoolIndex. --- end of TP ---

UE capability for multi-TRP/Panel

In our view, multi-DCI based multi-TRP feature especially its our-of-order operation requires special consideration due to its burden on UE implementation. As well discussed in rel-16 eURLLC, such operation can considerably disrupt UE’s processing pipeline, so UE may have limited capability, such as the number of CCs, on the support of multi-DCI based multi-TRP. If we adopt per FS level capability for multi-DCI based multi-TRP, a fallback behavior is needed to limit the number of CCs. One example of such fallback is rel-15 cap#2, copied below from TS38.331 [5]:

ProcessingParameters ::=        SEQUENCE {

fallback                        ENUMERATED {sc, cap1-only},

differentTB-PerSlot             SEQUENCE {

upto1                           NumberOfCarriers                    OPTIONAL,

upto2                           NumberOfCarriers                    OPTIONAL,

upto4                           NumberOfCarriers                    OPTIONAL,

upto7                           NumberOfCarriers                    OPTIONAL

} OPTIONAL

}

NumberOfCarriers ::=    INTEGER (1..16)

In this case, a UE reports #CC on which it can perform this feature, and fallback behaviour copied below from TS38.306 [4] is applied when the number of configured CC’s is larger than the reported one.

pdsch-ProcessingType2 ---omitted-----. - fallback indicates whether the UE supports PDSCH processing capability 2 when the number of configured carriers is larger than numberOfCarriers for a reported value of differentTB-PerSlot. If , if fallback = 'cap1-only', UE supports only capability 1, in the band where the value is reported; ---omitted----.

Since rel-15 fallback behaviour for cap#2 is a bit inefficient, we need to modify fallback behaviour. In the modified fallback behaviour, a UE supports this feature only on 2Ncell,MTRP-Ncell number of serving cells where Ncell,MTRP is the number of cells a UE reported and Ncell is the number of configured cells. Basically, we count processing burden of a serving cell with this multi-TRP feature as double of other serving cells.

Instead of designing such complex fallback, a simple way is to have the capability signaling per FSPC level. Signaling per FSPC level provides highest flexibility on indicating the CC supporting multi-DCI based multi-TRP.

Proposal 12: Support UE capability signaling of multi-DCI based multi-TRP per FSPC level.

In the outcome of UE feature discussion at RAN1#100-b-e [7], support of full-/partial-/non-overlapping PDSCHs in frequency domain are indicated by separate components, captured in FG 16-2a, 16-2a-0, and 16-2a-1. The remaining issue is that which are the basic components for multi-DCI based multi-TRP and which are the optional. In our understanding, at least one component should be considered as basic since a UE not supporting any of the components cannot perform multi-TRP operation. Among those components, support of non-overlapping PDSCH requires the least UE complexity which best fits for the basic component. In this sense, we prefer to remove the bracket of component #4 in FG 16-2a, and remove that from FG 16-2a-0. Besides, UE needs different processing capabilities to receive partial- and full-overlapping PDSCHs so we prefer to put components for partial- and full-overlapping PDSCHs in separate FGs, e.g., partial-overlapping to FG 16-2a-0 and full-overlapping to FG 16-2a-1. Type of FGs 16-2a-0 and 16-2a-1 should be FSPC as mentioned in Proposal 12.

Proposal 13: On UE features regarding support of overlapping PDSCHs for multi-DCI based multi-TRP,

- Support non-overlapping PDSCHs in frequency domain as a basic component
(remove bracket of component #4 at FG 16-2a and remove component #1 from FG 16-2a-0)

- Support full-/partial-overlapping PDSCHs in frequency domain as optional components

- Move component #2 in FG 16-2a-1 to FG 16-2a-0

- Support the types of FG 16-2a-0 and 16-2a-1 as FSPC

At FG 16-2a-1, there’s a component (component #3) to indicate the maximum number of MIMO layers of each scheduled PDSCH whose necessity is under discussion. It seems that component 3 intends to capture the highlighted part of the following agreement.

Agreement For multi-DCI based multi-TRP/panel transmission, the total number of CWs in scheduled PDSCHs, each of which is scheduled by one PDCCH, is up to X and also the total number of MIMO layers of scheduled PDSCHs is up to reported UE MIMO capability, if resource allocation of PDSCHs are overlapped. - X=2 - FFS: X=3

In our understanding, the highlighted part indicates the existing Rel-15 capability for the maximum number of MIMO layers, maxNumberMIMO-LayersPDSCH [4], and not a new capability. Hence, there is no reason to introduce component 3 in FG 16-2a-1.

Proposal 14: Remove component #3 from FG 16-2a-1.

In multi-DCI based multi-TRP, it is FFS whether default value of R is needed or not as captured in FG 16-2a. Since we concluded to have separate FG for BD factor, as FG 16-2a-10, UE may not signal the FG even it can support multi-DCI based multi-TRP and in that case gNB cannot configure PDCCH appropriately to the UE. Hence, default value needs to be defined for the value of R, and most appropriate value is be one which means that the UE has the same capability for BD/CCE as in Rel-15.

Proposal 15: Define default value of R in FG 16-2a and/or FG 16-2a-10 as one.

The second component in FG 16-2a-4, ‘Support of two TDMed long PUCCHs in a slot,’ has a bracket and further discussion is needed. At RAN1#98, it is agreed to support TDMed long+long PUCCH transmission for HARQ-ACK as well as long+short / short+short PUCCH as follows.

Agreement For multi-DCI based multi-TRP transmission with separate ACK/NACK feedback - UE is allowed to transmit two TDMed long PUCCHs within a slot - UE is allowed to transmit TDMed short PUCCH and long PUCCH within a slot - UE is allowed to transmit TDMed short PUCCH and short PUCCH within a slot FFS whether/how to use PRI indication with the granularity of sub-slot for eMBB with M-TRP

While UE signals support of long+short PUCCH and short+short PUCCH by separate FGs, 4-2, 4-22, and 4-22a, there are no relevant signaling for long+long PUCCH. Hence, the aforementioned component for two TDMed long PUCCHs is needed and the bracket shall be removed.

Proposal 16: Remove the bracket of component #2 in FG 16-2a-4.

As aforementioned above, multi-DCI based multi-TRP feature should be signaled per FSPC level. As there is no need to signal the maximum number of CCs for multi-DCI based multi-TRP, FG 16-2a-9 in [7] needs to be removed.

Proposal 17: Remove FG 16-2a-9.

Conclusion

In this contribution, the following observation and proposals are given:

Proposal 1: Introduce new RRC parameters indicating UE to enable enhanced default beam for multi-DCI and single-DCI, respectively.

Proposal 2: On enhanced default beam for multi-DCI and single-DCI, adopt the following TP in TS 38.214 V16.1.0:

5.1.5 Antenna ports quasi co-location --- start of TP --- Independent of the configuration of tci-PresentInDCI and tci-PresentInDCI-ForFormat1_2 in RRC connected mode, if no TCI codepoints are mapped to two different TCI states and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If a UE configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, for both cases, when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and the UE is configured with [enableEnhDefaultBeamMDCI], the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. If the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states and the UE is configured with [enableEnhDefaultBeamSDCI], the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. --- end of TP ---

Proposal 3: On PDSCH default beam for single-PDCCH based NC-JT, adopt the following TP in TS 38.214 V16.1.0:

5.1.5 Antenna ports quasi co-location --- start of TP --- If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enabledDefaultBeamForCCS]: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell; - When tci-PresentInDCI is set to 'enabled' and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and if at least one TCI codepoint indicates two TCI states for the component carrier of scheduled PDSCH and the UE is configured with [enableEnhDefaultBeamSDCI], the UE may assume that the DM-RS ports of PDSCH of the component carrier are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. --- end of TP ---

Proposal 4: When AP-CSI-RS is not overlapped with any DL signals, apply default beam to receive AP-CSI-RS as

- Default beam of the CORESETPoolIndex associated with the triggering DCI, if enhanced default beam for M-DCI M-TRP is enabled

- First among the two default beams, if enhanced default beam for S-DCI M-TRP is enabled

- Rel-15 default beam, if enhanced default beam is not enabled

Note: The DL signal could be in the same CC, or different CC in the same band.

Proposal 5: When AP-CSI-RS is overlapped with DL signal in a same band,

- For multi-DCI based multi-TRP

  - When AP-CSI-RS is overlapped with a DL signal associated with the same CORESETPoolIndex, apply QCL assumption of the DL signal in receiving the AP-CSI-RS

  - When AP-CSI-RS is overlapped with a DL signal associated with different CORESETPoolIndex, apply default beam associated with the AP-CSI-RS

  - When AP-CSI-RS is overlapped with DL signal with no CORESETPoolIndex association, reuse Rel-15 rule

- For single-DCI based multi-TRP

  - When AP-CSI-RS is overlapped with a DL signal having two TCI states, apply the first indicated TCI state of the DL signal in receiving the AP-CSI-RS

  - When AP-CSI-RS is overlapped with a DL signal having single TCI state, reuse Rel-15 rule

Note: The DL signal could be in the same CC, or different CC in the same band.

Observation 1: Default QCL behaviours for scheme 3 and 4, stated at Clause 5.1.2.1 and Clause 5.1.5 in TS 38.214, are inconsistent each other.

Proposal 6: For Default beam for scheme 3 and 4,

- For UE supporting the enhanced default beam, when the time offset between the DCI and at least one PDSCH transmission occasion among all PDSCHs is less than the threshold, the enhanced default TCI states are applied on the reception of all PDSCH transmission occasions. The mapping between enhanced default TCI states and PDSCH transmission occasions follows the mapping specified in Section 5.1.2.1 in TS 38.214.

- For UE not supporting the enhanced default beam, when the time offset between the DCI and at least one PDSCH transmission occasion among all PDSCHs is less than the threshold, the Rel-15 default TCI state is applied on the reception of all PDSCH transmission occasions.

Observation 2: In Rel-16, it is up to gNB implementation to coordinate each other to ensure the same BWP for scheduling two different PDSCHs by two different DCIs to the UE.

Proposal 7: Do not support simultaneous reception of PDCCHs with multiple different QCL-TypeD.

Proposal 8: On CRS rate matching for multi-TRP, adopt the following TP in TS 38.214 V16.1.0:

5.1.4.2 PDSCH resource mapping with RE level granularity *** Unchanged text is omitted *** - If the UE is configured by higher layer parameter PDCCH-Config with two different values of CORESETPoolIndex in ControlResourceSet and is also configured by the higher layer parameter LTE-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - If the UE is configured with [crs-RateMatch-PerCORESETPoolIndex], REs indicated by the CRS pattern(s) in lte-CRS-PatternList-r16 if the PDSCH is associated with CORESETPoolIndex = 0, or the CRS pattern(s) in lte-CRS-PatternListSecond-r16 if PDSCH is associated with CORESETPoolIndex = 1,; - Otherwise, REs indicated by CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16, in ServingCellConfig. < End of the TP >

Proposal 9: For a SPS configuration, SPS PDSCH with or without DCI is associated with a TRP according to the value of CORESETPoolIndex in a CORESET in which the activation DCI of the SPS configuration is received.

Proposal 10: Extend the agreement in RAN1#99 to include two SPS PDSCHs which overlap in time domain and are associated to two different TRPs according to the value of CORESETPoolIndex.

Proposal 11: Adopt the following TP in TS 38.214 V16.1.0:

5.1 UE procedure for receiving the physical downlink shared channel --- start of TP --- For any SPS configuration, the corresponding SPS PDSCH with or without PDCCH is associated with a value of CORESETPoolIndex of the CORESET in which the activation DCI of the SPS configuration is received. If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are partially or fully overlapping in time and they are associated with the same value of CORESETPoolIndex, a UE is not required to receive a PDSCH among these PDSCHs other than one with the lowest configured sps-ConfigIndex. If UE declares a capability to receive two overlapping PDSCHs associated with different values of CORESETPoolIndex, UE is expected to receive two overlapping SPS PDSCHs in a given serving cell if they are associated with different values of CORESETPoolIndex. --- end of TP ---

Proposal 12: Support UE capability signaling of multi-DCI based multi-TRP per FSPC level.

Proposal 13: On UE features regarding support of overlapping PDSCHs for multi-DCI based multi-TRP,

- Support non-overlapping PDSCHs in frequency domain as a basic component
(remove bracket of component #4 at FG 16-2a and remove component #1 from FG 16-2a-0)

- Support full-/partial-overlapping PDSCHs in frequency domain as optional components

- Move component #2 in FG 16-2a-1 to FG 16-2a-0

- Support the types of FG 16-2a-0 and 16-2a-1 as FSPC

Proposal 14: Remove the component #3 from FG 16-2a-1.

Proposal 15: Define default value of R in FG 16-2a and/or FG 16-2a-10 as one.

Proposal 16: Remove the bracket of component #2 in FG 16-2a-4.

Proposal 17: Remove FG 16-2a-9.

References

- R2-2002378, “Guidelines for UE capability definitions,” RAN2

- 3GPP RAN1, TS 38.213 V16.1.0

- 3GPP RAN1, TS 38.214 V16.1.0

- 3GPP RAN2, TS 38.306 V15.8.0

- 3GPP RAN2, TS 38.331 V15.9.0

- R1-2003146, “Corrections on NR enhanced MIMO,” Nokia

- R1-2003073, “RAN1 UE features list for Rel-16 NR after RAN1#100bis-E,” AT&T/NTT DOCOMO, INC.