3GPP TSG RAN WG1 #101	R1-2003886

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	DL reference signals for NR Positioning

Document for:	Discussion and Decision

# Introduction

A new work item on “NR positioning support” was approved in RAN#83 and the following agreements were made in RAN1#100b-e [1].

Agreement:

- For the purpose of DL PRS processing capability, the duration of DL PRS symbols (K) in ms within any P msec window, is calculated by

  - Type 1 duration calculation with

Ks=Tsend-Tsstart

  - Type 2 duration calculation with

K=12μS

  - where

    - Type 1 or Type 2 is reported as UE capability,

    - S is the set of slots of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets (target and reference),

    - for Type 1, [Tsstart, Tsend]Tsstart, Tsend is the smallest interval in ms within slot s corresponding to an integer number of OFDM symbols of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s.

      - Interval[Tsstart, Tsend] considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets (target and reference).

Agreement:

- For UE DL PRS processing capability,

  - UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE

  - Additionally, UE reports new parameter, number of DL PRS resources that UE can process in a slot, which is reported per SCS per band.

    - Values: {1, 2, 4, 8, 12, 16, 32, 64}

- The following sets of values for N, T and B are supported

  - Values for N = {0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50} ms

  - Values for T = {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms

  - Values for maximum BW reported by UE = {5, 10, 20, 40, 50, 80, 100, 200, 400} MHz

Agreement:

The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS

Agreement:

UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time)

Agreement:

UE capability for DL PRS processing is defined assuming the case with configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than X%

- FFS: X

Agreement:

The following text proposal for TS 38.211 is agreed.

--------------- Begin of TP -------------------

7.4.1.7.3	Mapping to physical resources in a downlink PRS resource

-	the symbol l is not used by any SS/PBCH block used by the serving cell for downlink PRS transmitted from the serving cell or any SS/PBCH block from a non-serving cell indicated by the higher-layer parameter SSB-positionInBurst for downlink PRS transmitted from a the same non-serving cell;

--------------- End of TP -------------------

Agreement:

The following text proposal for TS 38.211 is agreed.

--------------- Begin of TP -------------------

7.4.1.7.4 Mapping to slot in a downlink PRS resource set

<Text is omitted>

For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil

Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS∈iTgapPRSi=0TrepPRS-1

<Text is omitted>

-	bi1 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS 2μTmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern where L∈2, 4, 6, 8, 16, 32 is the size of the bitmap;

-	bi2 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern;

<Text is omitted>

the periodicity TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 is given by the higher-layer parameter  dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16;

--------------- End of TP -------------------

Agreement:

The following text proposal for TS 38.211 is agreed.

--------------- Begin of TP -------------------

7.4.1.7.3	Mapping to physical resources in a downlink PRS resource

<Text is omitted>

and where

-	lstartPRS is the first symbol of the downlink PRS within a slot and given by the higher-layer parameter DL-PRS-ResourceSymbolOffset;

-	the size of the downlink PRS resource in the time domain LPRS∈2,4,6,12 is given by the higher-layer parameter DL-PRS-NumSymbols;

-	the comb size KcombPRS∈2, 4, 6,12 is given by the higher-layer parameter transmissionComb;, such that combination of {LPRS, KcombPRS} is one of {2, 2},{4, 2}, {6, 2}, {12, 2}, {4, 4}, {12, 4}, {6, 6}, {12, 6} and {12, 12};

-	the resource-element offset koffsetPRS∈0,1,…,KcombPRS-1 is given by the higher-layer parameter combOffset;

-	the quantity k' is given by Table 7.4.1.7.3-1.

--------------- End of TP -------------------

Conclusion:

- It is RAN1 understanding that the NR-TimingMeasQuality is the quality for time of arrival measurements

- NR-TimingMeasQuality is left up to UE implementation

- Notes:

  - No RAN1 specification changes are required.

  - NR-TimingMeasQuality measurement is also applicable for the reference timing used in RSTD measurements

In this contribution, we discuss the remaining issues for NR DL positioning.

# NR DL PRS design

## DL PRS processing capability

The value of maximum PRS bandwidth is upper bounded by the values of maximum channel bandwidth for each SCS as shown in Table 1.

Table 1: Maximum PRS bandwidth

| SCS (kHz) | Max channel bandwidth (number of RB) | Max channel bandwidth (MHz) | Max PRS bandwidth (number of RB) | Max PRS bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| 15 | 270 | 48.6 | 268 | 48.24 |
| 30 | 273 | 98.28 | 272 | 97.92 |
| 60 (FR1) | 135 | 97.20 | 134 | 96.48 |
| 60 (FR2) | 264 | 190.08 | 264 | 190.08 |
| 120 | 264 | 380.16 | 264 | 380.16 |

If a UE reports a single maximum BW not depending on SCS, scaling in the second FFS may be applied when the maximum allowed BW for certain SCS is smaller than that. However, this theoretical approach does not address SCS-specific processing differences in UE implementation. Hence, we propose that UE reports maximum PRS bandwidth for each SCS. Consequently, value of N and T are also reported per SCS. On top of this, the symbol duration should cover the union of all search windows given by expected RSTD and expected RSTD uncertainty from all TRPs on a positioning frequency layer. Even if two TRPs are synchronized, there is might be a cell phase synchronization error of no more than 3µs. Search window for neighbor cell could starts from or ends with the middle of a symbol for the serving cell, one extra symbol will be counted in the duration. Specifically, the reported value N in ms should include the impact of cell phase synchronization error between TRPs.

When UE report processing capability (N, T) for the maximum PRS bandwidth, network can configure smaller BW of PRS. In this case, the scaling in the second FFS can be applied to increase the value of N to interpret UE’s capability in this case. Theoretically speaking, the PRS processing burden is inversely proportional to the PRS bandwidth. However, there is additional constraint on UE implementation, which is that the FFT can only be non-contiguous number 128, 256, 512, 1024, 2048, and 4096. For a range of PRS bandwidth, UE will select one FFT size for processing, therefore the true processing burden will not linearly scale with BW. This can be acknowledged by introducing stepwise scaling as below when a UE reported (N0, T) for BWmax.. and if network configures PRS bandwidth BW.

N1=f(BWmax)f(BW) ∙N0,

where

fx=1,             if   BW1<x ≤BW0, 2,             if   BW2<x ≤BW1,4,             if   BW3<x ≤BW2,8,             if   BW4<x ≤BW3,16,             if     BW5<x ≤BW4,32,             if           0<x ≤BW5

The values of BWi, with i=0,1,⋯,5 is given by Table 2.

Table 2: PRS process capability scaling parameter

| PRS bandwidth | BW5(MHz) | BW4 (MHz) | BW3 (MHz) | BW2 (MHz) | BW1 (MHz) | BW0 (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| SCS=15 kHz | 1.44 | 3.6 | 7.2 | 15.12 | 30.24 | 48.24 |
| SCS=30 kHz | 2.88 | 7.2 | 14.4 | 30.24 | 60.48 | 97.92 |
| SCS=60 kHz (FR1) | 5.76 | 14.4 | 28.8 | 60.48 | 96.48 | NA |
| SCS=60 kHz (FR2) | 5.76 | 14.4 | 28.8 | 60.48 | 120.96 | 190.08 |
| SCS=120 kHz | 11.52 | 28.8 | 57.6 | 120.96 | 241.92 | 380.16 |

Alternatively, UE may report multiple PRS bandwidth values corresponding to indicate scaling boundaries. For example, a UE may report BW1, BW2, BW3 with BW1< BW2 < BW3. Then if the network configures a bandwidth BW such that BWi-1< BW ≤ BWi, then scaling of N should correspond to BWi.

Proposal 1: The UE DL PRS processing capability is not exactly scaled inversely proportional to DL PRS processing bandwidth.

- Option 1: UE reports the capability corresponding to maximum PRS bandwidth to be supported. Scaling rule in Table 2 is applied to interpret UE’s capability if network configures smaller BW.

- Option 2: UE reports multiple PRS bandwidth values to indicate scaling boundaries.

## DL PRS with multiple SCS

In RAN1 #97, it was agreed that 5G-NR numerology needs further study in relation to DL-PRS resource configuration. As part of the numerology, 5G-NR supports different sub-carrier spacing (SCS) in both FR1 and FR2. This can lead to different bandwidth parts (BWP) in the same cell having different SCS as well as neighbour cells having different SCS. For a UE connected to a certain cell and/or a certain BWP, this scenario can dictate that the neighbour cells/neighbour cell BWP it would need to hear may be configured with a different SCS, as given by 2µx15 kHz, with µ={-3, -2, -1, 1, 2, 3}. The sampling clock rate will determine the accuracy of the ToA multilateration based positioning techniques.

In some applications, the time taken to complete the positioning and the energy consumed in the process will be critical. In these cases for the higher SCS (faster sampling clock) UE, the full PRS patterns needs to be available within its own slot time from all the neighbour cells. With the different comb-N patterns suggested for adaptation so far, we would propose that the lower SCS cell should adapt a 2µ times denser pattern (in time density), on request from the higher SCS cell. For example, if a cell employs 30 kHz SCS and a comb-N PRS pattern, and then a group of UE’s in this cell request a time critical or low energy consuming PRS measurement from a neighbour cell or neighbour BWP with 15 kHz SCS, it should be possible to configure the neighbour cell PRS in a comb-N/2 pattern.

Proposal 2: When neighbour cells or BWPs employ different SCS and some of the UEs require time or energy critical PRS measurements from this cell or BWP, the lower SCS cell should be able to configure the PRS as a comb-N/µ pattern, where µ is the ratio between higher and lower SCS employed by the two respective cells/ BWPs.

# Conclusions

In this contribution, we discussed the DL reference signals design for NR positioning. Our proposals are summarized below.

Proposal 1: The UE DL PRS processing capability is not exactly scaled inversely proportional to DL PRS processing bandwidth.

- Option 1: UE reports the capability corresponding to maximum PRS bandwidth to be supported. Scaling rule in Table 2 is applied to interpret UE’s capability if network configures smaller BW.

- Option 2: UE reports multiple PRS bandwidth values to indicate scaling boundaries.

Proposal 2: When neighbour cells or BWPs employ different SCS and some of the UEs require time or energy critical PRS measurements from this cell or BWP, the lower SCS cell should be able to configure the PRS as a comb-N/µ pattern, where µ is the ratio between higher and lower SCS employed by the two respective cells/ BWPs.

# References

- RAN1#100b-e Chairman’s Notes.