3GPP TSG RAN WG1 #101                                R1-2003893

e-Meeting, May 25th – June 5th, 2020

Agenda Item:		       7.2.11.1

Source:			Samsung

Title:		     UE features for two-step RACH

Document for:		Discussion and Decision

# Introduction

This contribution discusses the remaining issues on UE features for 2step RACH.

# Feature Group 9-1

During the previous email discussion on UE features, we have submitted our comments for the initial version on the 2step RACH UE feature. A general comment shared by many companies is that the initial version is somehow too detail especially for feature group (FG) 9-1. With that understanding, the updated UE feature list provides two types of summary for the 2step RACH: 1) Alt.1 - Detailed feature group, 2) Alt. 2 - Simplified basic feature group

Alt.1 is still the “detailed” summary with reduction from the initial version, and Alt.2 is super simple summary which barely gives any details on what is needed for 2step RACH. That drives us to think about what is qualified to be listed in the UE feature. One of the most important symptoms is that it requires a UE to do something new rather than a UE is capable of doing. E.g., for 2step RACH, we think it might be good to capture these items requiring UE to have new behaviour than a Rel-15 UE. With above said, we think the Alt.1 might be more appropriate and a further reduction on the lists are needed.

Following this logic, we have the comments for the each item in FG 9-1 for 2step RACH as shown in the following table.

Table 1 – discussion on UE feature group 9-1 for 2step RACH

| Index | Listed UE feature in Alt.1 | New features compared to Rel-15 UE? | To be kept? |
| --- | --- | --- | --- |
| 1 | RACH type selection for CBRA according to SSB-based RSRP threshold | Yes. Rel-15 did not do RACH type selection. | Yes |
| 2 | msgA PRACH resource configuration including separately configured ROs not applicable to 4-step RO configuration and fully or partially shared ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration | Shared RO case will bring the impact on preamble determination. | Yes with change |
| 3 | msgA PUSCH resource (DMRS included) and waveform determination for 2-step CBRA - Supporting up to two msgA PUSCH configurations in an UL BWP | Yes. The msgA PUSCH configuration is new to Rel-15 UE. | Yes |
| 4 | Validation of MsgA PRACH and PUSCH | Yes, the PUSCH validation is new to Rel-15 UE. | Yes |
| 5 | Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH | Yes, this mapping between preamble and PUSCH is different from SSB-RO mapping for Rel-15 UE. | Yes |
| 6 | msgB monitoring and decoding for 2-step CBRA - (for UE in any RRC state) monitoring msgB PDCCH with CRC masked by msgB-RNTI in Type-1 CSS set, and decoding multi-cast msgB PDSCH carrying SuccessRAR, FallbackRAR and BI - (for RRC connected UE only) monitoring msgB PDCCH with CRC masked by C-RNTI in USS set, and decoding the unicast PDSCH carrying absolute TA MAC CE | Yes, Rel-15 UE only need to monitor the msg.2 with RA-RNTI or using C-RNTI. But Rel-16 2step RACH requires UE to monitor the msgB by C-RNTI and msgB-RNTI. | Yes |
| 7 | PUCCH transmission for HARQ-ACK feedback to a msgB | Yes, the PUCCH resource determination method is different from Rel-15. | Yes |
| 8 | Power control for msgA PRACH, msgA PUSCH and PUCCH carrying HARQ-ACK feedback to msgB | No. only with newly added the parameter name but no actual different. | No |

Proposal 1: Adopt the revised UE feature group in the appendix by change item 2) by removing “separately configured ROs not applicable to 4-step RO configuration and” and remove 8) in FG9-1 for 2step RACH Rel-16.

For the four separate feature group:

- [9-3] [Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA] [Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell]

As we commented before, the msgA PRACH is same as rel-15 PRACH, msgA PUSCH is same as rel-15 PUSCH (with the TA=0) which can be considered as PUSCHs in different CC belong to different TAG; so TA=0 is not a new thing for UE implementation, since TA=0 is listed in TA value range. Besides, from the WID of the 2step RACH, it clearly states that we will reuse rel-15 PRACH and rel-15 PUSCH, no new design of the channel; the so called channel structure did not make them as a new channel but just from transmission perspective. We did not agree the Consequence if the feature is not supported by the UE is “UE cannot transmit a MsgA and other UL transmissions in parallel across CCs in inter-band CA”, on the contrary, UE can do this without needing listing such feature group;

- [9-4]	[MsgA operation in a band combination including SUL]	[MsgA operations in a band combination including SUL]

It seems no need to have. Since if a UE support 9-1 and also 6-16, naturally should support msgA operation in SUL;

- [9-5]	[intra-slot msgA PUSCH FH with non-zero GP]	[intra-slot msgA PUSCH FH with non-zero GP]

Intra-slot hopping and guard period after PO are supported, but doesn’t mean guard between hops are supported. Since UE only need to pick one PO to transmit, so the impact of the guard after the PO and the impact of the guard in-middle of a PO are different;

- [9-6]	[up to X of msgBs per slot/within the msgB window]	[up to X of msgBs per slot/within the msgB window]

Wait for RAN2 further reply.

Proposal 2: remove FG9-3,9-4; adopt FG9-5 (or maybe put in FG9-1).

# Conclusion

In this contribution, considerations on 2step RACH UE feature are presented. In particular, the following are proposed:

Proposal 1: Adopt the revised UE feature group in the appendix by change item 2) by removing “separately configured ROs not applicable to 4-step RO configuration and” and remove 8) in FG9-1 for 2step RACH Rel-16.

Proposal 2: remove FG9-3,9-4; adopt FG9-5 (or maybe put in FG9-1).

# Reference

- TS38.213, “Physical layer procedures for control”, V16.1.0.

# Appendix- 2step RACH UE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - RACH type selection for CBRA according to SSB-based RSRP threshold - msgA PRACH resource configuration including fully or partially shared ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration - msgA PUSCH resource (DMRS included) and waveform determination for 2-step CBRA - Supporting up to two msgA PUSCH configurations in an UL BWP - Validation of MsgA PRACH and PUSCH - Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH - msgB monitoring and decoding for 2-step CBRA - (for UE in any RRC state) monitoring msgB PDCCH with CRC masked by msgB-RNTI in Type-1 CSS set, and decoding multi-cast msgB PDSCH carrying SuccessRAR, FallbackRAR and BI - (for RRC connected UE only) monitoring msgB PDCCH with CRC masked by C-RNTI in USS set, and decoding the unicast PDSCH carrying absolute TA MAC CE - PUCCH transmission for HARQ-ACK feedback to a msgB - Power control for msgA PRACH, msgA PUSCH and PUCCH carrying HARQ-ACK feedback to msgB |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-5] | [intra-slot msgA PUSCH FH with non-zero GP] | [intra-slot msgA PUSCH FH with non-zero GP] | TBD | Yes | N/A |  | [Per band] | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-6] | [up to X of msgBs per slot/within the msgB window] | [up to X of msgBs per slot/within the msgB window] | TBD | Yes | N/A |  | [Per band] | N/A | N/A | N/A |  | Optional with capability signalling |