3GPP TSG RAN WG1 #101-e	R1-2003900

# e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.11.9

Source:	Samsung

Title:	UE features for NR mobility enhancement

Document for:	Discussion and decision

- Introduction

In this contribution, the remaining UE feature issues for NR mobility enhancement are discussed based on the latest UE feature list [1].

- FG 21-2/21-2a/21-2b

| Index | Feature group | Components |
| --- | --- | --- |
| 21-2 | Basic UE power sharing for DAPS HO | Support of semi-static power sharing mode1 |
| 21-2a | Semi-static UL power sharing mode 2 for DAPS HO | Support of semi-static power sharing mode 2 |
| 21-2b | Dynamic UL power sharing for DAPS HO | Support of dynamic power sharing |

The UE feature for UL power sharing mode still has the possibility of further update depending on the corresponding update of MR-DC/CA FG 18-1/18-1a/18-1 which is the basis of FG 21-2/21-2a/21-2b.

In case of MR-DC/CA, Semi-static-mode2 is only applicable for synchronous NR-DC operation. It is our understanding that TS38.133 specifies maximum transmission timing difference (MTTD) and synchronous/asynchronous requirements in the context of DC/CA scenario not mobility scenario.

Since power sharing operation for DAPS-HO (section 15 of TS38.213) refers to the one for NR-DC (section 7.6.2 of TS38.213), RAN1 should conclude whether the synchronization requirement/condition applies to DAPS-HO scenario as well, i.e., between source and target MCG. The synchronization requirement/condition for NR-DC power sharing with Semi-static-mode2 is specified as following:

“The UE expects to be provided NR-DC-PC-mode = Semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].”

Proposal 1: Consider following alternatives for Semi-static-mode2 for DAPS-HO UL power sharing

- Alt.1: Semi-static-mode2 for DAPS-HO UL power sharing does not have constraints of synchronous DAPS-HO of which the requirement is to be specified in RAN4. No need for further clarification in NR UE feature.

- Alt. 2: Semi-static-mode2 for DAPS-HO UL power sharing is applicable only for synchronous DAPS-HO of which the requirement is to be specified in RAN4. Corresponding clarifications in both NR UE feature and section 15 of TS38.213 are needed.

- FG 21-2d

| Index | Feature group | Components |
| --- | --- | --- |
| [21-2d] | [UL transmission cancellation] | [Indicates support of cancelling UL transmission to the source cell] |

As discussed in the previous meeting, it is acceptable to remove this FG and keep it as a component of basic FG, e.g., FG 21-1a/21-1b.

Proposal 2: Remove FG 21-2d and keep it as a component of basic FG, e.g., FG 21-1a/21-1b.

- Conclusion

Proposal 1: Consider following alternatives for Semi-static-mode2 for DAPS-HO UL power sharing

- Alt.1: Semi-static-mode2 for DAPS-HO UL power sharing does not have constraints of synchronous DAPS-HO of which the requirement is to be specified in RAN4. No need for further clarification in NR UE feature.

- Alt. 2: Semi-static-mode2 for DAPS-HO UL power sharing is applicable only for synchronous DAPS-HO of which the requirement is to be specified in RAN4. Corresponding clarifications in both NR UE feature and section 15 of TS38.213 are needed.

Proposal 2: Remove FG 21-2d and keep it as a component of basic FG, e.g., FG 21-1a/21-1b.

Reference

[1] R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T, NTT DOCOMO

ANNEX: NR UE features for NR mobility enhancement [1]

| Index | Feature group | Components |
| --- | --- | --- |
| 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. |
| 21-1b | Inter-frequency DAPS HO | Support of inter-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. |
| 21-2 | Basic UE power sharing for DAPS HO | Support of semi-static power sharing mode1 |
| 21-2a | Semi-static UL power sharing mode 2 for DAPS HO | Support of semi-static power sharing mode 2 |
| 21-2b | Dynamic UL power sharing for DAPS HO | Support of dynamic power sharing |
| [21-2d] | [UL transmission cancellation] | [Indicates support of cancelling UL transmission to the source cell] |