3GPP TSG RAN WG1 #101	R1-2004044

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.4.5

Source: 	Fujitsu

Title:	Remaining issues on prioritization rule for NR V2X

Document for:	Discussion/Decision

Introduction

Until the last RAN1#100b meeting, some prioritization rules have been agreed for simultaneous sidelink and uplink transmissions. In this contribution, we share some further discussions on the remaining issues for the prioritization rule in NR V2X.

Discussion

In the RAN1#98b meeting, the following agreements have been achieved for the prioritization rule between the sidelink carrier and the uplink carrier [1].

Working assumption:

- For the power limited case in supporting simultaneous sidelink and uplink transmissions (SL carrier is different from UL carrier),

  - If sidelink transmission is prioritized over uplink transmission, the UE shall adjust the uplink transmission power before the start of the transmission such that its total transmission power does not exceed  on any overlapped portion. In this case, calculation of the adjustment to the uplink transmission power is not specified.

  - If uplink transmission is prioritized over sidelink transmission, the UE shall adjust the sidelink transmission power before the start of the transmission such that its total transmission power does not exceed  on any overlapped portion. In this case, calculation of the adjustment to the sidelink transmission power is not specified.

  - Total sidelink transmit power is the same in the symbols used for actual PSCCH/PSSCH transmissions in a slot in case of simultaneous transmission of sidelink and uplink

    - PSCCH/PSSCH transmissions can be dropped in some symbols when there are uplink transmissions with higher priority and the UE cannot keep the same sidelink transmission power in the symbols.

      - Selection of the dropped symbols is up to UE implementation where the dropped symbols should include the overlapping symbols.

  - If the simultaneous transmission of sidelink and uplink is beyond the UE capability, the one not prioritized can be dropped.

  - FFS: when to prioritize which transmission

  - FFS: how to address UE processing time

  - FFS: whether there is a case of dropping some symbols of uplink transmissions

  - Whether/how to address RF transient period is up to RAN4.

In Clause 7.5 (Prioritizations for transmission power reductions) of TS 38.213 [2], the prioritization rule has been defined for carrier aggregation of different uplink carriers.

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion  would exceed , where  is the linear value of  in transmission occasion  as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to  for that frequency range in every symbol of transmission occasion . When determining a total transmit power for serving cells in a frequency range in a symbol of transmission occasion , the UE does not include power for transmissions starting after the symbol of transmission occasion . The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot.

-	PRACH transmission on the PCell

-	PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information

-	PUCCH transmission with CSI or PUSCH transmission with CSI

-	PUSCH transmission without HARQ-ACK information or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell

-	SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell

The aforementioned NR V2X agreements have covered the case where the sidelink carrier is different from the uplink carriers. However, they have not covered the case where the uplink carrier conveying sidelink HARQ-ACK is different from the other uplink carriers. It has been agreed that sidelink HARQ-ACK can be conveyed by PUCCH or piggybacked by PUSCH. To differentiate from regular PUCCH, denote PUCCH conveying sidelink HARQ-ACK as SL-PUCCH hereafter. Similarly, PUSCH conveying both sidelink HARQ-ACK and UL-SCH is denoted as SL-PUSCH to distinguish from regular PUSCH. SL-PUCCH/SL-PUSCH is transmitted on an uplink carrier. There can be a case where simultaneous SL-PUCCH/SL-PUSCH and uplink transmissions are required and they are on different carriers. This case has not been covered by the existing agreements in NR V2X. For example, there can be simultaneous SL-PUCCH/SL-PUSCH and uplink transmissions, and SL-PUCCH/SL-PUSCH is transmitted on an uplink carrier which is different from the other uplink carriers. Two cases are also shown in Figure 1 and Figure 2 later.

Furthermore, the aforementioned case is not covered by NR Uu specifications either. There is a similar case in NR Uu where PUCCH, PRACH, PUSCH and SRS can be simultaneously transmitted on different carriers. As aforementioned, NR Uu has defined a prioritization rule for this case. Generally, the prioritization order (in descending order) is that PRACH > PUCCH > PUSCH > SRS. However, this prioritization rule does not apply to the case where SL-PUCCH/SL-PUSCH, PUCCH, PRACH, PUSCH and SRS can be simultaneously transmitted on different carriers due to the difference between SL-PUCCH/SL-PUSCH and regular PUCCH/PUSCH.

Based on the above analysis, the prioritization rule should be specified for simultaneous SL-PUCCH/SL-PUSCH and uplink transmissions on different carriers. Generally, the prioritization rule should follow what has been defined for simultaneous sidelink and uplink transmissions.

Figure 1. An example showing simultaneous SL-PUCCH and uplink transmissions in CA

A case is shown in Figure 1 where SL-PUCCH and other uplink transmissions are overlapped across different uplink carriers. As shown in Figure 1, the carriers will be divided into two groups where one group corresponds to SL-PUCCH and the other group corresponds to uplink. If SL-PUCCH is prioritized over uplink, the uplink power should be adjusted such that the total power does not exceed PCMAX. If uplink is prioritized over SL-PUCCH, the SL-PUCCH power should be adjusted such that the total power does not exceed PCMAX. To determine the prioritization order between SL-PUCCH and uplink, the priority of SL-PUCCH will be needed. For SL-PUCCH, the priority should be the same as that of the conveyed sidelink HARQ-ACK. Since sidelink HARQ-ACK is associated with a PSSCH, the priority of SL-PUCCH is actually determined by the priority field of PSCCH which schedules that PSSCH.

Proposal 1: For the power limited case in supporting simultaneous SL-PUCCH and uplink transmissions (SL-PUCCH denotes PUCCH conveying SL HARQ-ACK, and the SL-PUCCH carrier is different from the uplink carrier),

- If SL-PUCCH transmission is prioritized over uplink transmissions, the UE shall adjust the uplink transmission power such that the total transmission power does not exceed PCMAX on any overlapped portion.

- If uplink transmissions are prioritized over SL-PUCCH transmission, the UE shall adjust the SL-PUCCH transmission power such that the total transmission power does not exceed PCMAX on any overlapped portion.

Figure 2. An example showing simultaneous SL-PUSCH and uplink transmissions in CA

Another case is shown in Figure 2 where SL-PUSCH and uplink transmissions are performed simultaneously on different uplink carriers. The difference with the aforementioned SL-PUCCH case is that SL-PUSCH here carries both sidelink HARQ-ACK and UL-SCH. Therefore, SL-PUSCH cannot be simply treated as sidelink when comparing the priorities. In this case, the grouping does not necessarily use a carrier as the granularity. Instead, the grouping is performed between sidelink and uplink. As shown in Figure 2, one group (SL Group) includes sidelink HARQ-ACK in SL-PUSCH, and the other group (UL Group) includes UL-SCH in SL-PUSCH and uplink transmissions on the carriers other than Carrier #m. If SL Group is prioritized over UL Group, the sidelink power, i.e., the power of SL-PUSCH, should be firstly fulfilled, and the powers of the carriers other than Carrier #m will be adjusted to guarantee that the total transmission power does not exceed PCMAX. If UL Group is prioritized over SL Group, the power for uplink should be firstly satisfied. It should be noted that the “uplink” here should also include UL-SCH in SL-PUSCH. In this case, SL-PUSCH should be treated as if it were regular PUSCH conveying UL-SCH, and its power is determined together with those of the other uplink carriers by using the existing power reduction rule in NR Uu.

Proposal 2: For the power limited case in supporting simultaneous SL-PUSCH and uplink transmissions (SL-PUSCH denotes PUSCH conveying both SL HARQ-ACK and UL-SCH, and the SL-PUSCH carrier is different from the uplink carrier),

- If SL HARQ-ACK in SL-PUSCH is prioritized over the union of uplink transmissions and UL-SCH in SL-PUSCH, the UE shall adjust the uplink transmission power such that the total transmission power does not exceed PCMAX on any overlapped portion.

- If the union of uplink transmissions and UL-SCH in SL-PUSCH is prioritized over SL HARQ-ACK in SL-PUSCH, the UE shall apply the existing Uu power reduction rule to the union of uplink transmissions and SL-PUSCH by treating SL-PUSCH as PUSCH conveying UL-SCH only.

In the last RAN1#100b meeting, the following agreements have been achieved w.r.t. the prioritization rule for PUCCH carrying SL HARQ-ACK [3].

Agreements:

- When PUCCH carrying SL HARQ reporting overlaps with SL TX,

  - The one with a higher priority is transmitted.

    - The priority of PUCCH carrying SL HARQ reporting is the highest priority of the associated PSFCH

According to the agreements, the priority of PSFCH is required to determine the priority of PUCCH. As for the priority of PSFCH, it is the priority of the associated PSCCH/PSSCH. In the end, it is the priority indicated by the priority field in SCI. Hereafter, it is generally called “priority” for simplicity. However, there are some cases where PUCCH is transmitted but there is no priority indicated by SCI. At least for the following cases, the priority of PUCCH should be further clarified.

- Case 1: For configured grant, ACK is reported on PUCCH if the UE does not transmit PSSCH within a period.

- Case 2: For type-1 HARQ-ACK codebook, NACK is reported on PUCCH as the padding bit if no PSSCH is transmitted.

- Case 3: If PSSCH is not transmitted due to prioritization, NACK is reported on PUCCH.

For Case 1 and Case 2, there is no priority indicated by SCI since SL LCH is not generated at all. For both cases, the priority should be treated as the lowest SL priority. For Case 3, there is no priority indicated by SCI since SL LCH is dropped and SCI is not generated. In this case, the priority should be the logical channel priority but not the priority indicated by SCI.

Proposal 3: The priority of PUCCH carrying SL HARQ-ACK should be additionally defined for the following cases where there is no priority indicated by SCI.

- Case 1: For configured grant, ACK is reported on PUCCH if the UE does not transmit PSSCH within a period.

  - The priority of ACK is defined as the lowest SL priority.

- Case 2: For type-1 HARQ-ACK codebook, NACK is reported on PUCCH as the padding bit if no PSSCH is transmitted.

  - The priority of NACK is defined as the lowest SL priority.

- Case 3: If PSSCH is not transmitted due to prioritization, NACK is reported on PUCCH.

  - The priority of NACK is defined as the SL priority of the associated SL logical channel.

Conclusions

In this contribution, we share views on the remaining issues for the prioritization rule in NR V2X. The proposal has been highlighted as follows.

Proposal 1: For the power limited case in supporting simultaneous SL-PUCCH and uplink transmissions (SL-PUCCH denotes PUCCH conveying SL HARQ-ACK, and the SL-PUCCH carrier is different from the uplink carrier),

- If SL-PUCCH transmission is prioritized over uplink transmissions, the UE shall adjust the uplink transmission power such that the total transmission power does not exceed PCMAX on any overlapped portion.

- If uplink transmissions are prioritized over SL-PUCCH transmission, the UE shall adjust the SL-PUCCH transmission power such that the total transmission power does not exceed PCMAX on any overlapped portion.

Proposal 2: For the power limited case in supporting simultaneous SL-PUSCH and uplink transmissions (SL-PUSCH denotes PUSCH conveying both SL HARQ-ACK and UL-SCH, and the SL-PUSCH carrier is different from the uplink carrier),

- If SL HARQ-ACK in SL-PUSCH is prioritized over the union of uplink transmissions and UL-SCH in SL-PUSCH, the UE shall adjust the uplink transmission power such that the total transmission power does not exceed PCMAX on any overlapped portion.

- If the union of uplink transmissions and UL-SCH in SL-PUSCH is prioritized over SL HARQ-ACK in SL-PUSCH, the UE shall apply the existing Uu power reduction rule to the union of uplink transmissions and SL-PUSCH by treating SL-PUSCH as PUSCH conveying UL-SCH only.

Proposal 3: The priority of PUCCH carrying SL HARQ-ACK should be additionally defined for the following cases where there is no priority indicated by SCI.

- Case 1: For configured grant, ACK is reported on PUCCH if the UE does not transmit PSSCH within a period.

  - The priority of ACK is defined as the lowest SL priority.

- Case 2: For type-1 HARQ-ACK codebook, NACK is reported on PUCCH as the padding bit if no PSSCH is transmitted.

  - The priority of NACK is defined as the lowest SL priority.

- Case 3: If PSSCH is not transmitted due to prioritization, NACK is reported on PUCCH.

  - The priority of NACK is defined as the SL priority of the associated SL logical channel.

Reference

[1] 3GPP TSG RAN WG1 Meeting #98b, RAN1 Chairman’s Notes, Chongqing, China, Oct. 14th – 20th, 2019.

[2] 3GPP TS 38.213 V16.1.0 (2020-03).

[3] 3GPP TSG RAN WG1 Meeting #100b e-Meeting, RAN1 Chairman’s Notes, April 20th – 30th, 2020.