GPP TSG RAN WG1 #101		R1-2004057

e-Meeting, May 25th – June 5th, 2020

Source:	OPPO

Title:	Text proposals for UL Power Sharing for NR-DC

Agenda Item:	7.2.10.1

Document for:	Discussion and Decision

Introduction

In this contribution, we present our views on the remaining issues of UL power control for NR-DC and propose some text proposals to update the specification.

Discussion

RRC parameter alignment

There are some misalignments of RRC parameters in TS 38.213 and in TS 38.331. Thus we have the following proposal:

Proposal 1: Align the following RRC parameters in TS 38.213 with those in TS 38.331

- nrdc-PCmode-FR1-r16 -> nrdc-PC-mode-FR1-r16

- nrdc-PCmode-FR2-r16 -> nrdc-PC-mode-FR2-r16

The following gives the corresponding text proposal for Proposal 1

In TS 38.213 Section 7.6 Dual Connectivity CR R1-2003183 ### 7.6.2 NR-DC If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2-r16 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2-r16 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 for FR1 and/or by nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. If a UE is provided semi-static-mode1 fornrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16, or semi-static-mode2 fornrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively. If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 - if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power - otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively The UE expects to be provided semi-static-mode2 for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR2-r16 only for synchronous NR-DC operation [10, TS 38.133]. If the UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided dynamic for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16 - if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the linear values of UE transmission powers in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3] - if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a transmission power in slot i1 of the MCG or in slot i2 of the SCG as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively If a UE - is provided dynamic for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16, and - indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and - overlap with the transmission occasion on the SCG <omitted text>

Total transmission power for FR2

The total transmission  is not only for NR-DC operations in  FR1, but also for NR-DC operation in FR2. However, only the total transmission r NR-DC in FR1 is described in the current spec.

Proposal 2: In TS 38.213, clarify the total transmission power also for FR2.

The following gives the corresponding text proposal for Proposal 2.

In TS 38.213 Section 7.6 Dual Connectivity CR R1-2003183 ### 7.6.2 NR-DC If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2-r16 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2-r16 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 for FR1 and/or by nrdc-PCmode-FR2-r16 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. If a UE is provided semi-static-mode1 fornrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, or semi-static-mode2 fornrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or in FR2 as defined in [8-3, TS 38.101-3]. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively. If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 - if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power - otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively The UE expects to be provided semi-static-mode2 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR1-r16 only for synchronous NR-DC operation [10, TS 38.133]. If the UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided dynamic for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16 - if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the linear values of UE transmission powers in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or in FR2 as defined in [8-3, TS 38.101-3] - if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a transmission power in slot i1 of the MCG or in slot i2 of the SCG as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively <omitted text>

Remaining Issues for look-ahead operation

In the reply LS [4], RAN2 says as bellow

RAN2 would like to thank RAN1 for the LS on uplink power control for NR-NR Dual-Connectivity. RAN2 is still discussing the reply to RAN1 but has no consensus yet on introducing new inter-node signalling for T_offset. However, RAN2 would like to remind that it was agreed in Rel-15 that MN and SN are not required to comprehend each other’s UE configuration for MR-DC. Therefore, RAN1 making assumption that such comprehension is possible is not correct in RAN2 view.

Based on the above information, the value of Toffset cannot be determined based on {Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux,  Tproc,CSImux}.  Thus the current working assumption cannot be confirmed. There are two possible ways to address the issue

- Option 1: Not support look-ahead operation (without Toffset)

- Option 2: Toffset is fixed to be a large value to cover all possible configurations

As RAN2 clarified, MCG and SCG may not comprehend each other’s UE configuration for DC. MCG and SCG don’t exchange dynamic scheduling information. Thus NW cannot guarantee the following requirement

The UE does not expect to have transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

Thus, the current framework for look-ahead operation doesn't work based on RAN2 LS.

Proposal 3: Rel-16 does not support look-ahead operation with Toffset, but to support power sharing operation without Toffset (i.e., the current version of the first paragraph for dynamic power sharing)

In RAN1#100b-e meeting, there were lots of discussions on DCI format 2-3/2-0/PUCCH/SRS transmission, e.g.,

Issue 1:Handling TPC Commands in DCI format 2-3

FL proposal: When UE has an SCG UL transmission and an overlapping MCG UL transmission, then for adjusting the power of the MCG UL transmission, the UE does not expect DCI format 2-3 in PDCCH receptions with a last symbol that is earlier by less than or equal to T_offset from the first symbol of the transmission occasion on the SCG

Issue 2: Handling UL Transmission Cancelation on MCG

FL proposal: When UE has overlapping transmissions an SCG UL transmission and an overlapping MCG UL transmission,  The UE does not expect to receive a SFI that may impact on the transmission power of the MCG UL transmission that are provided by DCI format 2-0 in PDCCH receptions with a last symbol that is earlier by less than or equal to T_offset from the first symbol of the transmission occasion on the SCG;  For transmission power determination on SCG, the UE is not required to take into UL transmission cancellation that may impact on the transmission power of the MCG UL transmission that are provided by DCI format 2-4 in PDCCH receptions with a last symbol that is earlier by less than or equal to T_offset from the first symbol of the transmission occasion on the SCG;  It is up to UE whether to take into account UL skipping for CG-PUSCH or DG-PUSCH according to section 5.4.3.1.3 of TS 38.321

Issue 3: Power Determination for PUCCH and SRS Transmission

FL proposal: UE takes into account semi-statically configured MCG UL transmission in the periodic PUCCH (e.g., P-/activated SP-CSI) - If the higher layer configured UL transmission is triggered or cancelled due to the UE internal process (e.g. SR and CG-PUSCH), it is up to UE whether to take it into account for power determination.  If the higher layer configured UL transmission is cancelled due to a DCI indication and the UE detects such DCI in PDCCH receptions with a last symbol that is earlier by more than T_Offset from the first symbol of the transmission occasion of the SCG, it is up to UE whether consider the UL transmission for power determination

There are also some cases of RACH transmission need to be considered. For example, both contention-based and contention-free RACH transmission may be triggered by UE for beam failure recovery mechanism.

The main reason why these solutions are complicated and controversial is the introduce of Toffset. Thus if we adopt Proposal 3, all these cases can be addressed in a simpler approach (i.e., the current version of the first paragraph for dynamic power sharing).

Conclusion

In this contribution, we present our view on the current specification regarding NR-DC power sharing. The following proposals and the corresponding text proposals are provided.

Proposal 1: Align the following RRC parameters in TS 38.213 with those in TS 38.331

- nrdc-PCmode-FR1-r16 -> nrdc-PC-mode-FR1-r16

- nrdc-PCmode-FR2-r16 -> nrdc-PC-mode-FR2-r16

Proposal 2: In TS 38.213, clarify the total transmission power also for FR2.

Proposal 3: Rel-16 does not support look-ahead operation with Toffset, but to support power sharing scheme without Toffset (i.e., the current version of the first paragraph for dynamic power sharing)

References

- TS 38.213, NR; Physical layer procedures for control, V16.1.0

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification, v16.0.0

- R1-2003183, Corrections on NR-DC and on Cross-carrier Scheduling with Different Numerologies (CR)

- R1-2003262, LS reply on uplink power control for NR-NR Dual-Connectivity