3GPP TSG RAN WG1 #101 E-Meeting		R1-2004118

May 24th – June 5th, 2020

Source:	OPPO

Title:	Inter UE Tx prioritization and multiplexing

Agenda Item:	7.2.5.5

Document for:	Discussion and Decision

Introduction

In RAN1 #100-E, there are some remaining issues about UL cancelation [1][2][3]:

- Issue 3: Whether another UL transmission can be scheduled in the cancelled symbols that do not overlap with the resource indicated by UL CI

- Issue 4: Scheduling and cancellation at the same time

In this contribution, we will discuss about remaining issues for UL CI.

UL CI

  - Whether another UL transmission can be scheduled in the cancelled symbols that do not overlap with the resource indicated by UL CI

In RAN1#100b_e, there is a proposal made for this issue.

- In case of UL inter-UE prioritization, when a UL transmission from a UE is cancelled by UL CI, the UE is not expected to be scheduled with UL transmission in the cancelled symbols that do not overlap with the resource indicated by UL CI.

A UL transmission will be canceled if there is overlap resource between the resource of the transmission and the resource indicated by UL CI. If a UE cancels its UL transmission and there is a new UL transmission scheduled with the non-overlapped resource in the cancelled symbols, it may call a case of out-of-order. In current discussion, out-of-order is not supported. And it is easier if the new transmission is not allowed.

On the other side, this issue is considered in intra-UE case also, and the new transmission is not allowed. The same procedure could be used for inter-UE cases.

Proposal 1: Another UL transmission cannot be scheduled in the cancelled symbols that do not overlap with the resource indicated by UL CI.

  - Contradiction between Scheduling transmission and cancellation

In RAN1#100b_e, there are 3 options generated.

- Option 1:

  - UE does not expect to detect a DCI format 2_4 at a first PDCCH monitoring occasion indicating a set of time-frequency resources and detect a DCI format at a second PDCCH monitoring occasion scheduling a PUSCH/SRS transmission include resources indicated by the DCI format 2_4 if the second monitoring occasion does not start earlier than the first PDCCH monitoring occasion.

- Option 1-1

  - UE does not expect to detect a DCI format 2_4 at a first PDCCH monitoring occasion indicating a set of time-frequency resources and detect a DCI format at a second PDCCH monitoring occasion scheduling a PUSCH/SRS transmission to be cancelled by the DCI format 2_4 if the second monitoring occasion does not start earlier than the first PDCCH monitoring occasion

- Option 2:

  - UE does not expect to detect a DCI format 2_4 at a first PDCCH monitoring occasion indicating a set of time-frequency resources and detect a DCI format at a second PDCCH monitoring occasion scheduling a PUSCH/SRS transmission with low priority include resources indicated by the DCI format 2_4 if the second monitoring occasion does not start earlier than the first PDCCH monitoring occasion.

In RAN1#100b_e, the following agreement is generated.

Agreeement: (from conference call (4/28)):

- If both UL CI and intra-UE priority indicator are configured for a given UE, support a new RRC parameter to configure Behavior #1

  - Behaviour #1: UL CI is only applicable to the UL transmissions indicated/configured as low priority level

- When the RRC parameter is not provided to the UE, behaviour #2 is used

  - Behaviour #2: UL CI is applicable to UL transmission irrespective of its priority level

- Note: the RRC signaling details will be decided by RAN2

According to the agreement, there are two cases:

Case 1: if cancellation based on priority level is configured, UL transmission with high priority level could be transmitted on the resource indicated by UL CI. In this case, the resource indicated by UL grant and DCI format 2_4 could have overlapped set.

Case 2: If cancellation based on priority level is not configured, both high priority and low priority transmission are not allowed transmitted. In this case, UL grant is not allowed if the resource indicated by UL grant is overlapped with the resource indicated by UL CI.

Only option 1-1 can cover both case 1 and case 2, so option 1-1 is preferred. Option 1 does not allow case 1 and leads PDCCH transmission restriction and option 2 is not suitable for case 2.

Proposal 2: Option 1-1 is preferred due to it is feasible regardless cancellation based on priority level is configured or not.

Cancellation procedure for different type of UL transmission

In RAN1#100b_e, the following agreement is generated.

Agreeement: (from conference call (4/28)):

- If both UL CI and intra-UE priority indicator are configured for a given UE, support a new RRC parameter to configure Behavior #1

  - Behaviour #1: UL CI is only applicable to the UL transmissions indicated/configured as low priority level

- When the RRC parameter is not provided to the UE, behaviour #2 is used

  - Behaviour #2: UL CI is applicable to UL transmission irrespective of its priority level

- Note: the RRC signaling details will be decided by RAN2

UL CI is applicable to PUSCH and SRS. There are two types of PUSCH, CG-PUSCH and DG-PUSCH. For DG-PUSCH, UL transmission is controlled by gNB completely. So gNB can avoid inter-UE interference by UL CI and reasonable scheduling. However, for CG-PUSCH, PUSCH transmission is out of control of gNB, UL CI based on priority level for CG-PUSCH may not work well.

Case 1: UL CI is only applicable to CG-PUSCH with low priority.

If there is CG transmission with high priority and UL CI indicates the same resource as CG transmission, CG transmission will not be cancelled. And there may be another high priority transmission which triggers UL CI from another UE. Then inter-UE interference occurs. If there is no CG transmission or CG transmission with low priority, and UL CI indicates same resource, then inter-UE interference can be avoided.

Case 2: UL CI is applicable to CG-PUSCH irrespective of its priority level.

If there is CG transmission and UL CI indicates the same resource as CG transmission, CG transmission will be cancelled irrespective of its priority level, so inter-UE interference can be avoided.

So for CG-PUSCH, cancellation irrespective of priority level is preferred due to it can avoid inter-UE interference thoroughly. However, for DG-PUSCH, cancellation based on priority level is more feasible in some cases. So it is suggested that cancellation based on priority level is configured for CG-PUSCH and DG-PUSCH separately.

For SRS transmission, there is no priority for SRS, so cancellation indication is always valid.

Proposal 3: Cancellation based on priority level is configured for CG-PUSCH and DC-PUSCH separately. Proposal 4: For SRS transmission, cancellation indication is always valid.

Conclusions

In this contribution, we show our views on UL inter UE Tx prioritization and multiplexing with following observations and proposals:

Proposal 1: Another UL transmission cannot be scheduled in the cancelled symbols that do not overlap with the resource indicated by UL CI.

Proposal 2: Option 1-1 is preferred due to it is feasible regardless cancellation based on priority level is configured or not.

Proposal 3: Configuration on cancellation based on priority level is configured for CG-PUSCH and DC-PUSCH separately.

Proposal 4: For SRS transmission, cancellation indication is always valid.

References

- R1-2002937 Summary of [100b-e-NR-L1enh-URLLC-InterUE-01]

- R1-2002938 Summary of [100b-e-NR-L1enh-URLLC-InterUE-02]

- R1-2002939 Summary of [100b-e-NR-L1enh-URLLC-InterUE-03]