3GPP TSG RAN WG1 Meeting #101-e 		 		R1-2004140

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.11

Source: 	LG Electronics

Title: 	Discussion on UE features for NR CLI/RIM

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss on UE features for NR CLI/RIM. Especially, we explain the RAN2 agreement and RAN4 agreement regarding UE prioritization in case that FDMed reception is not supported.

- Discussion

FG 17-3 and FG 17-4

RAN2 has agreed to introduce following UE capabilities for CLI [1].

- cli-RSSI-Meas-r16

- cli-SRS-RSRP-Meas-r16

- cli-RSSI-FDM-DL-r16

- cli-SRS-RSRP-FDM-DL-r16

Observation 1:

- It is needed to keep FG 17-3 and FG 17-4

Also, in [1], the UE capability related RAN2 agreements were captured as below:

• FR1/FR2 diff should be “Yes” in general for all CLI capabilities.

• UE shall prioritize the DL transmission in case DL signal/channel and SRS RSRP resources are FDMed for UEs indicating the FDMed reception is not supported.

• UE shall prioritize the DL transmission in case DL signal/channel and CLI RSSI resources are FDMed for UEs indicating the FDMed reception is not supported.

But, RAN4 found that the RAN2 agreement is conflicted with RAN4 agreement. Then, in reply LS from RAN4 [2], RAN4 clarified the conflicting with RAN4 agreement regarding on UE prioritization in case that FDMed reception is not supported as below:

RAN4 finds the two highlighted bullets are conflicting with the RAN4 agreement to apply scheduling restriction for CLI measurement, which is defined in clause 9.7.4 of 38.133. Specifically, when UE indicates the FDMed reception is not supported, RAN4 assumes that UE shall prioritize CLI measurement, thus is not required to receive DL signals or channels that overlap with the CLI measurement time. RAN4 kindly asks RAN2 to take above information into account in the future specification work.

The intention of proposed notes in FG 17-3 and 17-4 is to reflect RAN4 assumption that UE shall prioritize CLI measurement in case DL signal/channel and CLI measurement resources are FDMed for UEs indicating the FDMed reception is not supported.

Observation 2:

- RAN4 clarified the confliction between RAN2 agreement and RAN4 agreement.

- The notes for FG 17-3 and FG 17-4 reflect RAN4 assumption.

Conclusion

Proposal 1: Keep FG 17-3 and FG 17-4 with note

| Index | Feature group | Components | Note |
| --- | --- | --- | --- |
| 17-3 | Simultaneous reception of DL signals/channels and CLI-RSSI measurement resource | Support simultaneous reception of DL signals/channels and CLI-RSSI measurement resource | [UE shall prioritize CLI-RSSI measurement when simultaneous reception of DL signals/channels and CLI-RSSI measurement resource is not supported.] |
| 17-4 | Simultaneous reception of DL signals/channels and SRS-RSRP measurement resource | Support simultaneous reception of DL signals/channels and SRS-RSRP measurement resource | [UE shall prioritize SRS-RSRP measurement when simultaneous reception of DL signals/channels and SRS-RSRP measurement resource is not supported.] |

Reference

- [1] R1-2000155, “LS on CLI measurements UE capabilities”, RAN2

- [2] R1-2001523, “reply LS on CLI measurement capability”, RAN4