3GPP TSG RAN WG1 #101		R1-2004181

e-Meeting, May 25th – June 5th, 2020

Agenda item:       8.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation assumptions for NR coverage enhancement evaluation

Document for:     Discussion and Decision

# 1	Introduction

In this contribution, we provide in detail the evaluation assumptions that have been used for link budget evaluation in our companion contributions [1][2].

# 2	General assumptions for link budget evaluation

## 2.1	FR1 scenarios

This subsection summarizes general assumptions for the link budget evaluation in our companion contribution [1] for the Rural and Urban scenarios in FR1. Parameters are presented in Table 1.

| Scenario parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Duplexing |  | TDD |  |  |  |  | FDD |  |  |
| Scenario |  | Urban/Rural |  |  |  |  | Rural |  |  |
| Carrier frequency |  | 4 GHz |  |  |  |  | 700MHz |  |  |
| SCS |  | 30 kHz |  |  |  |  | 15 kHz |  |  |
| BW |  | 100 MHz |  |  |  |  | 10 MHz |  |  |
| Channel parameters |  |  |  |  |  |  |  |  |  |
| Model |  | TDL-C as per IMT-2020 guidelines [3] |  |  |  |  |  |  |  |
| Delay spread |  | TDD |  |  |  |  | FDD |  |  |
|  |  | Urban |  | Rural |  |  | Rural |  |  |
|  |  | 363 ns |  | 37 ns |  |  | 37 ns |  |  |
| UE speed | Low speed | 3 Km/h |  | 3 Km/h |  |  | 3 Km/h |  |  |
|  | High speed | 30 Km/h |  | 120 Km/h |  |  | 120 Km/h |  |  |
| Propagation |  | Both NLOS and LOS |  |  |  |  |  |  |  |
| Power allocation parameters |  |  |  |  |  |  |  |  |  |
| Tx power gNB |  | TDD |  |  |  |  | FDD |  |  |
|  |  | 44 dBm |  |  |  |  | 49 dBm |  |  |
| Tx power UE |  | 23 dBm |  |  |  |  |  |  |  |
| Power allocation |  | DL | Uniformly allocated to CBW |  |  |  |  |  |  |
|  |  | UL | Concentrated on occupied BW |  |  |  |  |  |  |
| Antenna array configuration parameters |  |  |  |  |  |  |  |  |  |
| Antennas |  | As per IMT-2020 guidelines [3] |  |  |  |  |  |  |  |
|  |  | Elements [#E] |  |  |  |  | Ports [#P] |  |  |
|  |  | gNB | Urban |  | 62T64R |  | gNB | 2DL 2UL |  |
|  |  |  | Rural |  | 32T32R |  |  |  |  |
|  |  | UE | Urban |  | 2T4R |  | UE | 2DL 1UL |  |
|  |  |  | Rural |  | 2T2R |  |  |  |  |
| Beamforming Gain |  | 10log10#E / #P |  |  |  |  |  |  |  |
| Frame configuration parameters |  |  |  |  |  |  |  |  |  |
| Frame structure for TDD |  | 3D1S1U (2.5 ms periodicity) and 4D1S5U (5 ms periodicity) D:U=10:2 for S slot |  |  |  |  |  |  |  |
| Symbols per slot |  | 14 |  |  |  |  |  |  |  |
| DMRS |  | PxSCH |  | Urban |  | DL |  |  | UL |
|  |  |  |  |  |  | 1 symbol Type 1 - 1 layer 3dB power boost |  |  | 2 symbols Type 1 - 1 layer 3dB power boost |
|  |  |  |  | Rural |  | 2 symbols |  |  |  |
|  |  | PxCCH |  | Urban/Rural |  | DL/UL |  |  |  |
|  |  |  |  |  |  | According to NR specification |  |  |  |
| HARQ eMBB |  | No |  |  |  |  |  |  |  |
| HARQ VoIP |  | Yes [Up to 4 HARQ transmissions] |  |  |  |  |  |  |  |

Table 1. General assumptions for link budget evaluation in FR1.

## 2.2	FR2 scenarios

This subsection summarizes general assumptions for the link budget evaluation in our companion contribution [2] for the Urban, Suburban and Indoor scenarios in FR2. Parameters are presented in Table 2.

| Scenario parameters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Duplexing |  | TDD |  |  |  |  |  |  |  |  |  |
| Scenario |  | Urban |  |  |  | Suburban |  |  |  | Indoor |  |
| Carrier frequency |  | 30 GHz |  |  |  | 30 GHz |  |  |  | 30 GHz |  |
| SCS |  | 120 kHz |  |  |  | 120 kHz |  |  |  | 120 kHz |  |
| BW |  | 200 MHz |  |  |  | 200 MHz |  |  |  | 200 MHz |  |
| Channel parameters |  |  |  |  |  |  |  |  |  |  |  |
| Model |  | TDL-C for Urban/Suburban and TDL-A for Indoor as per IMT-2020 guidelines [3] |  |  |  |  |  |  |  |  |  |
| Delay spread |  | TDD |  |  |  |  |  |  |  |  |  |
|  |  | Urban |  |  | Suburban |  |  |  | Indoor |  |  |
|  |  | 263 ns |  |  | 37 ns |  |  |  | 26 ns |  |  |
| UE speed | Low speed | 3 Km/h |  |  | 3 Km/h |  |  |  | 3 Km/h |  |  |
|  | High speed | 30 Km/h |  |  | 120 Km/h |  |  |  | - |  |  |
| Propagation |  | Both NLOS and LOS |  |  |  |  |  |  |  |  |  |
| Power allocation parameters |  |  |  |  |  |  |  |  |  |  |  |
| Tx power gNB |  | Urban/Suburban |  |  |  |  |  |  | Indoor |  |  |
|  |  | 43 dBm |  |  |  |  |  |  | 26 dBm |  |  |
| Tx power UE |  | 23 dBm |  |  |  |  |  |  |  |  |  |
| Power allocation |  | DL | Uniformly allocated to CBW |  |  |  |  |  |  |  |  |
|  |  | UL | Concentrated on occupied BW |  |  |  |  |  |  |  |  |
| Antenna array configuration parameters |  |  |  |  |  |  |  |  |  |  |  |
| Antennas |  | As per IMT-2020 guidelines [3] |  |  |  |  |  |  |  |  |  |
|  |  | Elements [#E] |  |  |  |  |  |  | Ports [#P] |  |  |
|  |  | gNB | Urban |  |  |  | 128T128R |  | gNB | 2DL 2UL |  |
|  |  |  | Suburban |  |  |  | 128T128R |  |  |  |  |
|  |  | UE | Urban |  |  |  | 4T8R |  | UE | 2DL 1UL |  |
|  |  |  | Suburban |  |  |  | 4T8R |  |  |  |  |
| Beamforming Gain |  | 10log10#E / #P |  |  |  |  |  |  |  |  |  |
| Frame configuration parameters |  |  |  |  |  |  |  |  |  |  |  |
| Frame structure for TDD |  | 3D1S1U (0.625 ms periodicity) and 3D1S6U (1.25 ms periodicity) D:U=10:2 for S slot |  |  |  |  |  |  |  |  |  |
| Symbols per slot |  | 14 |  |  |  |  |  |  |  |  |  |
| DMRS |  | PxSCH |  | Urban |  |  |  | DL |  |  | UL |
|  |  |  |  |  |  |  |  | 1 symbol Type 1 - 1 layer 3dB power boost |  |  | 2 symbols Type 1 - 1 layer 3dB power boost |
|  |  |  |  | Rural |  |  |  | 2 symbols |  |  |  |
|  |  | PxCCH |  | Urban/Rural |  |  |  | DL/UL |  |  |  |
|  |  |  |  |  |  |  |  | According to NR specification |  |  |  |
| HARQ |  | No |  |  |  |  |  |  |  |  |  |

Table 2. General assumptions for link budget evaluation in FR2.

# 3	Detailed assumptions for each physical channel

## 3.1	FR1 shared channels

Table 3 shows the number of PRBs and MCS indices that are used for link budget evaluation in our companion contribution [1] for PDSCH in FR1. Similarly, Table 4 shows the number of PRBs and MCS indices that are used for link budget evaluation in [1] for PUSCH in FR2. It is worth mentioning that intra-slot frequency hopping is enabled for PUSCH. Given the number of PRBs, the lowest MCS index that satisfies target throughput is used for both PDSCH and PUSCH.

| Scenario | Frame structure | Target throughput | Number of PRBs | MCS index |
| --- | --- | --- | --- | --- |
| Rural FDD | - | 1Mbps | 52 | 4 |
| Rural TDD | 3D1S1U (10D:2G:2U) | 1Mbps | 273 | 0 |
|  | 4D1S5U (10D:2G:2U) | 1Mbps | 273 | 0 |
| Urban TDD | 3D1S1U (10D:2G:2U) | 10Mbps | 273 | 5 |
|  | 4D1S5U (10D:2G:2U) | 10Mbps | 273 | 7 |

Table 3. The considered PRBs and MCS indices that satisfy target throughput for PDSCH in FR1 scenarios.

| Scenario | Frame structure | Target throughput | Number of PRBs | MCS index |
| --- | --- | --- | --- | --- |
| Rural FDD | - | 100Kbps | 5 | 4 |
| Rural TDD | 3D1S1U (10D:2G:2U) | 100Kbps | 13 | 3 |
|  | 4D1S5U (10D:2G:2U) | 100Kbps | 13 | 0 |
| Urban TDD | 3D1S1U (10D:2G:2U) | 1Mbps | 55 | 7 |
|  | 4D1S5U (10D:2G:2U) | 1Mbps | 55 | 3 |

Table 4. The considered PRBs and MCS indices that satisfy target throughput for PUSCH in FR1 scenarios.

VoIP is also evaluated in FR1. In this case, we consider 4 PRBs as a baseline for both PDSCH and PUSCH, with TBS = 352 bits, including 16 bits of CRC. The valid MCSs for 4 PRBs and 352 bits TBS are 11 and 10 for PDSCH and PUSCH, respectively. It is worth mentioning that aggregation factor (AF) 4 is considered for both PDSCH and PUSCH in FDD, whereas AF 4 for PDSCH and AF 2 for PUSCH is considered for TDD deployments with 3D1S1U (10D:2G:2U) frame structure.

## 3.2	FR2 shared channels

Table 5 shows the number of PRBs and MCS indices that are used for link budget evaluation in our companion contribution [2] for PDSCH in FR2. Similarly, Table 6 shows the number of PRBs and MCS indices that are used for link budget evaluation in [2] for PUSCH in FR2. As mentioned above for FR1, intra-slot frequency hopping is enabled for PUSCH in FR2 and, given the number of PRBs, the lowest MCS index that satisfies target throughput is used for both PDSCH and PUSCH. Since VoIP is considered as second priority in FR2 as stated in the SID [2], we do not present any simulation assumption for VoIP in FR2, for the sake of compactness.

| Scenario | Frame structure | Target throughput | Number of PRBs | MCS index |
| --- | --- | --- | --- | --- |
| Urban | 3D1S1U (10D:2G:2U) | 25Mbps | 132 | 7 |
|  | 3D1S6U (10D:2G:2U) | 25Mbps | 132 | 10 |
| Suburban | 3D1S1U (10D:2G:2U) | 1Mbps | 132 | 0 |
| Indoor | 3D1S1U (10D:2G:2U) | 25Mbps | 132 | 7 |

Table 5. The considered PRBs and MCS indices that satisfy target throughput for PDSCH in FR2 scenarios.

| Scenario | Frame structure | Target throughput | Number of PRBs | MCS index |
| --- | --- | --- | --- | --- |
| Urban | 3D1S1U (10D:2G:2U) | 5Mbps | 37 | 10 |
|  | 3D1S6U (10D:2G:2U) | 5Mbps | 37 | 5 |
| Suburban | 3D1S1U (10D:2G:2U) | 50Kbps | 2 | 3 |
| Indoor | 3D1S1U (10D:2G:2U) | 5Mbps | 37 | 10 |

Table 6. The considered PRBs and MCS indices that satisfy target throughput for PUSCH in FR2 scenarios.

## 3.3	Control channels and DMRS

For PDCCH, we consider aggregation level 16 with 40 bits DCI and 24 bits CRC. A CORESET bandwidth of 48 PRBs and two OFDM symbols are used. The channel bandwidth for PDCCH is the same as for PDSCH.

For PUCCH, we consider both Format 1 with 2 bits UCI and Format 3 with 20 bits UCI. One and two PRBs are considered for Format 1 and Format 3, respectively. Intra-slot frequency hopping is enabled for both PUCCH formats.

Concerning DMRS, we consider configuration type 1 with 1 layer and 3.0 dB power boost for both DL and UL. For PDSCH, 2 OFDM symbols for DMRS are used in Rural and Suburban scenarios, where high-speed UEs (120 Km/h) are also considered.1 OFDM symbol is used for DMRS for PDSCH in the remaining scenarios. For PUSCH, 2 OFDM symbols are used for DMRS for all scenarios. Finally, for both PDCCH and PUCCH, DMRS configuration follows NR specification and no additional DMRS is considered for PUCCH.

# 4	Conclusion

In this contribution, we have presented the evaluation assumptions for the link budget study performed in [1][2]. Firstly, general assumptions for scenarios in FR1 and FR2 were summarized in two tables. Secondly, detailed channel configuration assumptions are provided for each physical channel for both FR1 and FR2.

# 5	References

- R1-2004178 Baseline coverage evaluation of UL and DL channels – FR1, Nokia/NSB.

- R1-2004179 Baseline coverage evaluation of UL and DL channels – FR2, Nokia/NSB.

- Report ITU-R M.2412-0, Guidelines for evaluation of radio interface technologies for IMT-2020, ITU, Oct. 2017.

- RP-193240, New SID on NR coverage enhancement, China Telecom.