3GPP TSG-RAN WG1 #101 								R1-2004241

e-Meeting, May 25th-June 5th, 2020

Source: 	Apple Inc.

Title:                     Discussions on NR-U UE features

Agenda item:       7.2.11.2

Document for:	Discussion and Decision

# 1 Introduction

This contribution discusses the UE capabilities for NR-U WI listed in [1].

# 2. Discussion

In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing

Proposal 1:

- The features listed with “FFS: Per band or Per UE” should be put “per band”

On FG 10-2b and 10-2e

We would like to add “PCell and PSCell” to the FG10-2b to scope the intended use case of this FG. For PSCell, reading MIB is still needed to acquire the SFN timing of the SCG (which maybe different from the MCG).  This modification is important to provide desirable flexibility so that UE that only supports NR-U deployment scenario A together with FG 10-23 (reportCGI for neighbour) does not necessarily support this feature. In other words, this modification gets rid of the overlapping part (i.e. MIB/SIB1 decoding) between FG 10-2b (PCell/PSCell) and FG 10-23 (neighbour cells).

| 10-2b | MIB reading on unlicensed cell | 1. MIB reading on unlicensed cell for PCell and PSCell |  |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| --- | --- | --- | --- | --- | --- |

For SIB-1 reading i.e. FG 10-2e, we propose to update the component to include “for PCell”. It should be noted that, according to TS 38.331 section 5.2.1, SIB-1 information for SCell, including PSCell, is provided by dedicated RRC signaling in SCell addition procedure or SCG addition procedure and therefore no need to read SIB1 for any SCell.

| 10-2e | SIB1 reception on unlicensed cell | 1. SIB1 reception on unlicensed cell for PCell |  |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| --- | --- | --- | --- | --- | --- |

Proposal 2:

- Modify the component of FG 10-2b to be “MIB reading on unlicensed cell for PCell and PSCell”

- Modify the component of FG 10-2e to be “SIB1 reception on unlicensed cell for PCell”

On FG 10-2c/10-2d

| 10-2c | SSB-based RLM [for dynamic channel access mode] | 1. SSB-based RLM with Q [for dynamic channel access mode] |  | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| --- | --- | --- | --- | --- | --- |
| 10-2d | SSB-based RLM [for semi-static channel access mode] | 1. SSB-based RLM with Q [for semi-static channel access mode] |  | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

As shown in Table above, the current FG 10-2c/10-2d only includes SSB-based RLM functionality. RAN1 needs to discuss how to handle other SSB-based functionalities, e.g. BFD (Beam Failure Detection) and CBD (Candidate Beam Detection) for NR-U operation, i.e. either adding them into 10-2c and 10-2d as components (Alt.1) or creating new FGs e.g. FG 10-2g/2h/2I/2J (Alt.2)

Proposal 3:

- RAN1 to discuss how to handle SSB-based BFD and CBD capabilities support by considering two alternatives below:

  - Alt.1: Adding SSB-based BFD/CBD with Q as additional components in FG 10-2c/2d.

    - FG-10-2c: SSB-based RLM/BFD/CBD with Q [for dynamic channel access mode]

    - FG-10-2d: SSB-based RLM/BFD/CBD with Q [for semi-static channel access mode]

  - Alt.2: Create four new FGs

    - FG 10-2g: SSB-based BFD with Q [for dynamic channel access mode]

    - FG 10-2h: SSB-based BFD with Q [for semi-static channel access mode]

    - FG 10-2I: SSB-based CBD with Q [for dynamic channel access mode]

    - FG 10-2J: SSB-based CBD with Q [for semi-static channel access mode]

Others

We also have two recommendations to complete the TBD aspects and align the specification terms.

Proposal 4:

- Adding the FG 4-5 as prerequisite feature groups for FG 10-23.

- Modify the “cat4” in FG 10-21a to “Type 1” to align with TS 37.213 specification.

# 3. Conclusion

Based on the discussions in the previous sections, we made the following proposal

Proposal 1:

- The features listed with “FFS: Per band or Per UE” should be put “per band”

Proposal 2:

- Modify the component of FG 10-2b to be “MIB reading on unlicensed cell for PCell and PSCell”

- Modify the component of FG 10-2e to be “SIB1 reception on unlicensed cell for PCell”

Proposal 3:

- RAN1 to discuss how to handle SSB-based BFD and CBD capabilities support by considering two alternatives below:

  - Alt.1: Adding SSB-based BFD/CBD with Q as additional components in FG 10-2c/2d.

    - FG-10-2c: SSB-based RLM/BFD/CBD with Q [for dynamic channel access mode]

    - FG-10-2d: SSB-based RLM/BFD/CBD with Q [for semi-static channel access mode]

  - Alt.2: Create four new FGs

    - FG 10-2g: SSB-based BFD with Q [for dynamic channel access mode]

    - FG 10-2h: SSB-based BFD with Q [for semi-static channel access mode]

    - FG 10-2I: SSB-based CBD with Q [for dynamic channel access mode]

    - FG 10-2J: SSB-based CBD with Q [for semi-static channel access mode]

Proposal 4:

- Adding the FG 4-5 as prerequisite feature groups for FG 10-23.

- Modify the “cat4” in FG 10-21a to “Type 1” to align with TS 37.213 specification.

# References

- R1-2003073	RAN1 UE features list for Rel-16 NR after RAN1#100bis-E AT&T, NTT DOCOMO, INC.