3GPP TSG RAN WG1 Meeting #101-e		R1-2004264

e-Meeting, May 25th – June 5th, 2020

Agenda item:       7.2.6.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance on Rel-16 CSI enhancements

Document for:     Discussion and Decision

# 1	Introduction

In this contribution we briefly discuss a modification to the definition of parameter K0 by introducing a minimum value for the number of nonzero coefficients reported per layer, when the value obtained by the existing formula is too small. We also discuss two text proposals to 38.214 intended to clarify the conditions on the PUSCH for which low-latency aperiodic CSI timing requirements apply and which UE capability component indicates the maximum number of CPUs, NCPU, supported for simultaneous CSI calculations.

# 2	On the minimum value of K0

In subclause 5.2.2.2.5 there is no lower limit specified to the value that parameter K0 can take. Therefore, given the definition

| K0=β2LM1 | (2) |
| --- | --- |

there exist possible codebook configurations for which the value of K0 is so small that

case 1. 	No coefficient amplitude/phase can be reported for some layer, i.e. K0<2 for reported rank ν≤2, or K0<ν  for rank 2<ν≤4

or

case 2. 	Not all the selected beams can be used in a layer, i.e. K0<L for rank ν≤2, or K0<νL2  for rank 2<ν≤4.

Both situations can arise for some parameter combinations when the number of subbands, NSB, in the CSI reporting band is small, namely for NSB≤4.

The last column of Table 1 shows the minimum value of K0 and the number of subbands for which it occurs, for all supported parameter combinations.

Table 1. Minimum value of K0 and number of subbands NSB for which it is achieved for all parameter combinations.

| paramCombination-r16 | L | pυ |  | β | min K0 (NSB) |
| --- | --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |  |
| 1 | 2 | ¼ | 1/8 | ¼ | 1 (NSB≤4) |
| 2 | 2 | ¼ | 1/8 | ½ | 2 (NSB≤4) |
| 3 | 4 | ¼ | 1/8 | ¼ | 2 (NSB≤4) |
| 4 | 4 | ¼ | 1/8 | ½ | 4 (NSB≤4) |
| 5 | 4 | ¼ | ¼ | ¾ | 6 (NSB≤4) |
| 6 | 4 | ½ | ¼ | ½ | 4 (NSB≤2) |
| 7 | 6 | ¼ | - | ½ | 6 (NSB≤4) |
| 8 | 6 | ¼ | - | ¾ | 9 (NSB≤4) |

Note that it is possible for a UE not to report any nonzero combination coefficients for some polarisations or beams, if it chooses to do so, however when case 1 above occurs, a UE is forced to do so regardless of a UE’s preferred PMI calculation. Besides, in case 1, some reported indicators become redundant: for example, if no amplitude/phase coefficients can be reported for a layer the bitmap, which is still reported, serves no useful purpose.

Similarly, a UE could decide, as a result of a PMI calculation, not to report any coefficient for certain beams in a layer. However, if this behaviour is forced by the CSI report configuration, like in case 2, because K0 is too small to report at least one nonzero coefficient for all L beams, effectively it is as if a lower value of L was configured.

The critical configurations described here are not typical and may be regarded as corner cases, however the issue can be easily fixed by introducing a minimum value of K0, for example by modifying the definition of K0 as follows

| K0=maxx,β2LM1 | (3) |
| --- | --- |

where x can be chosen from one the following alternatives:

1a. 	x=2, to ensure that a UE can report one NZC per polarisation for rank ν≤2

1b. 	x=max⁡{2,ν}, to ensure that a UE can report one NZC per polarisation for rank ν≤4

2a. 	x=L, to ensure that a UE can report one NZC for each selected beam for rank ν≤2

2b. 	x=L2⋅max⁡{2,ν}, to ensure that a UE can report one NZC for each selected beam for rank for ν≤4

# 3	PUSCH conditions for maximum CPU occupancy and low-latency timing requirement in A-CSI report

This is a TP to capture a shadow CR for Rel-16, corresponding to the CR in [4] for Rel-15.

In subclauses 5.2.1.6 and 5.4 of TS38.214, similar wording is used to describe the conditions of the PUSCH for which the maximum number of NCPU CPUs are occupied by an A-CSI report (Sec. 5.2.1.6) and the low latency timing requirements of Table 5.4-1 apply (Sec. 5.4).

In subclause 5.2.1.6 the sentence reads:

“if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both...”

whilst in subclause 5.4 the words used are:

“if the CSI is triggered without a PUSCH with either transport block or HARQ-ACK or both”

The correct interpretation, confirmed by the session notes in [1]-[3], is that the maximum CPU occupancy of an A-CSI report and the low-latency timing requirements apply when the PUSCH carries no transport block nor HARQ-ACK, i.e., it only carries a single A-CSI report.

However, the choice of words used to describe these conditions, “without... with either..., or...., or both”, may lead to a different interpretation. Namely that the maximum CPU occupancy of an A-CSI report and the low-latency timing requirements apply when the PUSCH carries only one of either HARQ-ACK or transport block, or none of them, i.e., it carries a single CSI report + HARQ-ACK, or a single CSI report + transport block, or only a single CSI report.

To remove this ambiguity, we suggest the following text proposals

TP to TS 38.214, Sec. 5.2.1.6: ## <Unchanged text is omitted> A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings.Processing of a CSI report occupies a number of CPUs for a number of symbols as follows: - OCPU=0 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured - OCPU=1 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured) - for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI', - if a CSI report is aperiodically triggered with a PUSCH with no transport block and no HARQ-ACK when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU, - otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement. ## <Unchanged text is omitted>

TP to TS 38.214, Sec. 5.4: ## <Unchanged text is omitted> Z, Z' and µ are defined as: Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as - (Z1,Z1') of the table 5.4-1 if the CSI is triggered with a PUSCH with no transport block and no HARQ-ACK when L = 0 CPUs are occupied (according to Clause 5.2.1.6) and the CSI to be transmitted is a single CSI and corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the reportQuantity is set to 'ssb-Index-SINR', or reportQuantity is set to 'cri-SINR', or - (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP', where Xµ is according to UE reported capability beamReportTiming and KBl is according to UE reported capability beamSwitchTiming as defined in [13, TS 38.306], or - (Z2,Z2') of table 5.4-2 otherwise. ## <Unchanged text is omitted>

# 4	Clarification on which UE capability component indicates NCPU

This is a TP to capture a shadow CR for Rel-16, corresponding to the CR in [5] for Rel-15.

In subclause 5.2.1.6 of TS38.214, the parameter NCPU is defined as the number of simultaneous CSI calculations supported by a UE. The text specifies that this parameter is indicated by the UE, but it is not clear which UE capability component indicates it and if the indication is given across all active CCs only or with finer granularity.

In fact, this paramer should be indicated by components simultaneousCSI-ReportsPerCC and simultaneousCSI-ReportsAllCC of FG 2-35 for a single CC or in case of CA. However, the name and description of these components refer to the number of simultaneous CSI reports rather then CSI calculations a UE can process. Because a single CSI report may require multiple CSI calculations, without a clarification a UE may under report its N_CPU capability as illustrated in the following example.

For a single CSI report configured with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI', and that is not a low-latency report, the number of simultaneous CSI calculations (i.e., occupied CPUs) equals the number of CSI-RS resources, Ks, in the resource set. Therefore, it is not clear if a UE indicating simultaneousCSI-ReportsPerCC=1 can support this CSI report if Ks>1.

The proposed text clarifies that the number of simultaneous CSI calculations supported by a UE is indicated by simultaneousCSI-ReportsPerCC for a single CC and by simultaneousCSI-ReportsAllCC across all CCs in case of carrier aggregation.

If a clarification is not added, a UE may report in simultaneousCSI-ReportsPerCC and simultaneousCSI-ReportsAllCC the maximum number of simultaneous CSI reports it can support instead of the maximum number of CPUs. Therefore, a gNB may not be able to schedule a CSI report that requires multiple simultaneous CSI calculations.

TP to TS 38.214, Sec. 5.2.1.6: The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports across all configured cells. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Subclause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings.Processing of a CSI report occupies a number of CPUs for a number of symbols as follows: - OCPU=0 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured - OCPU=1 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured) - for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI', - if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU, - otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement. ## <Unchanged text is omitted>

# 5	Conclusion

In this contribution, we propose the following modifications to TS38.214:

Text Proposal 1.	Modify the definition of  K0=maxx,β2LM1, by introducing a minimum value, x, from one of the following alternatives

1a. 	x=2, to ensure that a UE can report one NZC per polarisation for rank ν≤2

1b. 	x=max⁡{2,ν}, to ensure that a UE can report one NZC per polarisation for rank ν≤4

2a. 	x=L, to ensure that a UE can report one NZC for each selected beam for rank ν≤2

2b. 	x=L2⋅max⁡{2,ν}, to ensure that a UE can report one NZC for each selected beam for rank for ν≤4

Text Proposal 2.	[Mirror CR from Rel 15] Clarification on the number of occupied CPUs and delay requirements for CSI reports using PUSCH

Text Proposal 3.	[Mirror CR from Rel 15] Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU

# References

- 3GPP, “RAN1 Chairman's Notes RAN1#93,” Busan, Korea, May 2018

- Ad-hoc chair (Samsung), R1-1807785 “Chairman’s notes of AI 7.1.2 MIMO,” Busan, Korea, May 2018

- Ericsson, R1-1807810, “CSI reporting offline session notes Thursday morning,” Busan, Korea, May 2018

- Nokia/NSB, R1-2004261, “Clarification on the number of occupied CPUs and delay requirements for CSI reports using PUSCH,” RAN1#101-e, May-June 2020

- Nokia/NSN, R1-2004262, “Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU,” RAN1#101-e, May-June 2020