3GPP TSG RAN WG1 #101		                                    R1-2004272

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.5.5

Source:	InterDigital Inc.

Title:	Inter-UE prioritization/multiplexing

Document for:	Discussion and Decision

# Introduction

This contribution discusses remaining open issues for inter-UE prioritization/multiplexing, related to interaction between intra-UE prioritization and inter-UE cancellation. The summary of the discussion [1] includes the following proposals as basis for further discussion:

Proposal from [1]: - A DCI format 2_4 is only applicable to an uplink grant scheduling PUSCH/SRS if the ending symbol of the PDCCH carrying the UL grant is earlier than the first symbol of the PDCCH carrying DCI format 2_4. If the UE does not cancel a transmission in resources indicated by DCI format 2_4, the UE can receive an UL grant scheduling a transmission in any resources and transmit accordingly, if the ending symbol the PDCCH carrying UL grant is no earlier than the first symbol of the PDCCH carrying DCI format 2_4. - UE does not expect to receive a PDCCH carrying UL grant with its ending symbol no earlier than the first symbol of a PDCCH carrying DCI format 2_4 scheduling a transmission in resource colliding with the ones indicated by DCI formant 2_4 or in the cancelled symbols that do not overlap with the resource indicated by the DCI format 2_4, if the UE has to cancel another transmission in resource indicated by the DCI format 2_4 - FFS under what condition (if there is such a condition), that UE can be scheduled by an UL grant to transmit in any resource if the UE has to cancel a transmission in the resources indicated by the DCI format 2_4, and if the UL grant is received after the DCI format 2_4

# Discussion

The above proposals attempt to capture a consensus regarding the following issues:

- Override of UL CI by a subsequent uplink grant (first bullet)

- Whether network can schedule a PUSCH within the resources of a previously scheduled PUSCH cancelled by UL CI (second bullet)

The first bullet, if agreed, means that a UE can first receive an UL CI and second an uplink grant indicating PUSCH overlapping with the resources indicated by UL CI. In such case the UE would ignore the UL CI and transmit the PUSCH (as long as there was no other PUSCH cancelled). Use cases for this behaviour are to (1) ensure that the transmission to be protected by the UL CI is not itself cancelled by this UL CI; and (2) provide more network flexibility in case UL CI provides an indication multiple slots in advance. This behaviour seems reasonable, and the first bullet is acceptable.

The second bullet, if agreed without FFS, means that the reception of UL CI does not change the behaviour with respect to the restrictions applicable to PUSCH dynamic scheduling of R15. In other words, out-of-order PUSCH scheduling or overlap between two PUSCHs scheduling dynamically is not allowed even if one of the PUSCHs is cancelled by a CI. Again, this approach seems reasonable and follows the same principle as an agreement taken at the last meeting:

Agreements: l UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions is not affected by UL CI.

During RAN1#100-b there were discussions on whether there should be exceptions for scenarios in which e.g. the UL CI would be received a long time in advance of the second grant. However, this may add some complexity to UE implementation and the necessity of this is questionable. The network anyway has to operate with the same scheduling restriction when UL CI is not configured. It seems difficult to justify why operation with UL CI, whose main purpose is inter-UE cancellation, should optimize intra-UE operation.

Observation: There is no need to lift scheduling restrictions specified between two PUSCHs scheduled dynamically, when UL CI is received.

On the other hand, the above-mentioned scheduling restrictions are specified between two PUSCHs scheduling dynamically, but not when at least one of the PUSCHs is scheduled by configured grant. The latest proposal does not make this distinction and needs to be modified to avoid inadvertently including the case where at least one of the PUSCHs is scheduled by configured grant.

Proposal 1: agree on the following (same as first bullet of Proposal in section 2 of [1]):

- A DCI format 2_4 is only applicable to an uplink grant scheduling PUSCH/SRS if the ending symbol of the PDCCH carrying the UL grant is earlier than the first symbol of the PDCCH carrying DCI format 2_4. If the UE does not cancel a transmission in resources indicated by DCI format 2_4, the UE can receive an UL grant scheduling a transmission in any resources and transmit accordingly, if the ending symbol the PDCCH carrying UL grant is no earlier than the first symbol of the PDCCH carrying DCI format 2_4.

Proposal 2: agree on the following (modification to second bullet of Proposal in section section 2 of [1]):

- UE does not expect to receive a PDCCH carrying UL grant with its ending symbol no earlier than the first symbol of a PDCCH carrying DCI format 2_4 scheduling a transmission in resource colliding with the ones indicated by DCI formant 2_4 or in the cancelled symbols that do not overlap with the resource indicated by the DCI format 2_4, if the UE has to cancel another transmission scheduled dynamically in resource indicated by the DCI format 2_4

FFS under what condition (if there is such a condition), that UE can be scheduled by an UL grant to transmit in any resource if the UE has to cancel a transmission in the resources indicated by the DCI format 2_4, and if the UL grant is received after the DCI format 2_4

# Conclusion

This contribution proposes the following:

Observation: There is no need to lift scheduling restrictions specified between two PUSCHs scheduled dynamically, when UL CI is received.

Proposal 1: agree on the following [same as first bullet of Proposal in section 2 of [1]]:

- A DCI format 2_4 is only applicable to an uplink grant scheduling PUSCH/SRS if the ending symbol of the PDCCH carrying the UL grant is earlier than the first symbol of the PDCCH carrying DCI format 2_4. If the UE does not cancel a transmission in resources indicated by DCI format 2_4, the UE can receive an UL grant scheduling a transmission in any resources and transmit accordingly, if the ending symbol the PDCCH carrying UL grant is no earlier than the first symbol of the PDCCH carrying DCI format 2_4.

Proposal 2: agree on the following [modification to second bullet of Proposal in section section 2 of [1]]:

- UE does not expect to receive a PDCCH carrying UL grant with its ending symbol no earlier than the first symbol of a PDCCH carrying DCI format 2_4 scheduling a transmission in resource colliding with the ones indicated by DCI formant 2_4 or in the cancelled symbols that do not overlap with the resource indicated by the DCI format 2_4, if the UE has to cancel another transmission scheduled dynamically in resource indicated by the DCI format 2_4

FFS under what condition (if there is such a condition), that UE can be scheduled by an UL grant to transmit in any resource if the UE has to cancel a transmission in the resources indicated by the DCI format 2_4, and if the UL grant is received after the DCI format 2_4

# References

R1-2002937, “Summary of [100b-e-NR-L1enh-URLLC-InterUE-01]”, Vivo.

TS38.213, “NR; Physical layer procedures for control”, v16.1.0.

# Appendix

The following agreements were made during RAN1 meeting #96bis:

Working assumption:

- PDCCH is used for UL cancellation indication

  - The Working assumption can be revisited if the DCI for cancellation indication only carry very small number of information bits, e.g. 1 bit.

Agreements:

- Upon detecting an UL cancellation indication, at least stop without resuming is supported

  - FFS whether and how to support stop with resume

Agreements:

- Further discuss which UL transmissions that can potentially be cancelled by the UL cancellation indication, including

  - Dynamic scheduled UL transmissions, including PUSCH, PUCCH, SRS

  - Semi-persistent UL transmissions, including PUSCH, PUCCH, SRS

  - Periodic UL transmissions, including configured grant PUSCH, PUCCH, SRS

  - PRACH

Agreements:

- Further discuss, aiming for down-selection, the group common DCI and UE-specific DCI for UL cancellation indication

  - For group common DCI (different from Rel-15 SFI)

    - UE is configured to monitor a group common DCI which indicates the time/frequency region on which the UL cancellation indication applies

  - For UE specific-DCI

    - When applicable, UE is configured to monitor a second UL grant for the same TB as an earlier PUSCH indicating UL cancellation before the end of the earlier PUSCH transmission. In this case, the UE follows the UL cancellation indication.

The following agreements were made during RAN1 meeting #97:

Agreements:

- Support at least group common DCI for cancellation indication

  - FFS whether or not to additionally support UE-specific DCI for cancellation indication

Agreements:

To down-select from the following options for enhanced power control

- Option 1: Indication of open-loop parameter sets by DCI

- For DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI without using SRI is applied to the scheduled transmission

- FFS At least for single active CG-PUSCH, an open-loop parameter set is indicated to the UE by a UE-specific field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

- FFS For a UE, the open-loop parameter sets for DG-PUSCH and CG-PUSCH may be same or different

- Option 2: Indication of TPC with increased range by DCI

- For DG-PUSCH, a TPC with increased range is indicated to the UE by the TPC field in scheduling DCI

- FFS At least for single active CG-PUSCH (and potentially also for DG-PUSCH), a TPC with increased range is indicated to the UE by a UE-specific TPC field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

- At least for DG-PUSCH, for a UE, the number of TPC entries (4 or 8) and power adjustment value for each entry is higher layer configured

- FFS For a UE, the TPC configuration for DG-PUSCH and CG-PUSCH may be same or different

- Option 3:

- For DG-PUSCH, use either the solution from option 1 or option 2 for DG-PUSCH as above

    - To down-select from option 1 and 2

- FFS At least for single active CG-PUSCH, UE derives the transmissions power based on the time/frequency resource indicated by a group common DCI

    - If a CG-PUSCH transmission overlaps with the indicated time/frequency resource, UE use one open-loop parameter set with higher power for the transmission

    - If a CG-PUSCH transmission does NOT overlap with the indicated time/frequency resource, UE use another open-loop parameter set with lower power for the transmission

    - FFS for the case of multiple active CG-PUSCH

    - Note: some companies have concern that this was not captured in the TR as one potential solutions

The following agreements were made during RAN1 meeting #98:

Agreements:

- Reuse the existing methods for search space configuration to support UL CI monitoring

  - FFS possible restrictions

  - Note: this means both symbol level and slot level monitoring periodicities are possible from specification perspective

Agreements:

- The UE DCI size budget is not increased by UL CI monitoring

- Further discuss methods to reduce the UE monitoring for UL CI, e.g.

  - The number of aggregation levels and/or candidates for the UL CI monitoring should be limited

  - Conditions for eMBB UE UL CI monitoring:

    - For UL transmission with associated PDCCH,

      - Option 1: UE starts UL CI monitoring after the PDCCH is decoded

      - Option 2: UE monitors UL CI at least at the latest monitoring occasion ending no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - For UL transmission without associated PDCCH, UE monitors UL CI at least at the latest monitoring occasion that ends no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - Other conditions?

  - Others?

- FFS the enhancement of UE capability (number of non-overlapping CCE and/or blind decodes) for UL CI monitoring

Agreements:

- Upon detecting an UL cancelation indication, for the transmission of UL signal/channels, “stop with resuming” is not supported

  - Except:

    - SRS can still be transmitted on the non-cancelled symbols (conditioned on if SRS can be pre-empted)

    - FFS for the PUSCH repetition (Rel-15 & Rel-16) case

    - FFS for the PUCCH repetition case (conditioned on if PUCCH can be pre-empted)

  - FFS whether another PUSCH can be scheduled in non-pre-empted resource

  - FFS impact (e.g. phase continuity issue) to a different carrier due to UL cancelation

Agreements:

- The following UL channel/signals can be cancelled by UL cancelation indication

  - PUSCH (including DG-, CG- and SP-)

  - FFS for SRS

  - FFS for PUCCH

    - Option 1: PUCCH (all types) can be cancelled

    - Option 2: Some PUCCH can be cancelled, e.g. PUCCH carrying CSI

    - Option 3: PUCCH cannot be cancelled

  - FFS for PRACH (preamble and/or MSG 3 PUSCH)

Agreements:

- The UE processing time requirement for UL cancelation indication based on N2 defined in Rel-15 UE cap#2 is supported

  - FFS whether the processing time requirement for UL cancelation indication larger than N2 as defined in Rel-15 UE cap#2 can also be supported as an UE capability

  - FFS whether the processing time requirement for UL cancelation indication shorter than N2 as defined in Rel-15 UE cap#2 as can also be supported an UE capability

Agreements:

- For a DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI using a separate field than SRI is supported.

  - FFS number of bits for the indication

The following agreements were made during RAN1 meeting #98bis:

Agreements:

- Regarding UL CI monitoring, support the following:

  - A new RNTI (e.g. CI-RNTI) is used for UL CI

  - FFS: Monitoring periodicity larger than [5] slot is not supported for UL CI

  - The aggregation level(s) and the number of PDCCH candidates configured by RRC

    - FFS possible restrictions, e.g., the ones associated with SFI

  - The DCI payload size for UL CI  is configured by RRC

    - FFS possible values

Agreements:

- SRS can be cancelled by UL CI

- PUCCH cannot be cancelled by UL CI

- RACH related UL transmissions cannot be cancelled by UL CI, including MSG 1/3 in case of 4-step RACH, MSG A in case of 2-step RACH.

R1-1911660

Agreements:

- Cross-carrier UL cancelation indication is supported using the same way as Rel-15 SFI/DL PI

  - The indication field position in DCI for each cross-carrier indicated serving cell is configured by RRC

Agreements:

- Different UE processing time capability for UL CI (i.e. shorter or longer than T_proc2 for cap#2 UE) is not considered in Rel-16

  - d2,1=0 also when DMRS and UL-SCH (for the PUSCH to be cancelled) are multiplexed in the 1st symbol

Agreements:

- In case of PUSCH repetitions, UL CI is applied to each repetition individually (actual repetition in case of Rel-16 PUSCH repetition) that overlaps with the resource (in time and frequency) indicated by UL CI.

Agreements:

  - The reference time region where a detected UL CI is applicable is determined by the following:

      - The reference time region starts from X symbols after the ending symbol of the PDCCH CORESET carrying the UL CI, where X is at least equal to the minimum processing time for UL cancelation

        - FFS X can be configured to be larger than the minimum processing time for UL cancelation

      - The duration of the reference time region is configured by RRC

        - FFS Possible values (e.g. 2OS, 4OS, 7OS, 14OS, 28OS?)

      - FFS DL symbols are excluded from the reference time region

Agreements:

    - The reference frequency region where a detected UL CI is applicable is configured by RRC

R1-1911688

Agreements:

Support the following for UL CI

  - Each UL cancelation indicator per serving cell has a RRC configurable field size of  X bits

      - One value of X is 14

      - FFS other values (e.g. X can be N (N>0) times of 7)

  - The time domain granularity for the reference time region is configured by RRC

      - FFS the possible values  (e.g. the time region can be divided into [1],[2],[4],[7],[14],…portions)

      - FFS valid configurations according to the duration of the time reference region

  - The frequency domain granularity is determined based on the configured time domain granularity and the configured bit field size of each indicator

  - The time and frequency resource for cancellation is jointly indicated by a 2D-bitmap (i.e. similar as DL PI) over the time and frequency partitions within the reference region

      - FFS dynamic 2D-bitmap

R1-1911712

Agreements:

- For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set

  - Introduce one new RRC parameter that contains one additional P0-PUSCH-Set per SRI

  - The one bit indication is present in the UL grant when the above new RRC parameter is configured

  - If present, the one bit in the DCI is used to switch between the P0 value from the existing P0-PUSCH-AlphaSet and the P0 value from the newly configured P0-PUSCH-Set

Conclusion:

- No enhancement for CG-PUSCH power control in Rel-16 for inter-UE multiplexing

The following agreements were made during RAN1 meeting #99:

R1-1912035	Summary#1 of UL inter UE Tx prioritization/multiplexing	vivo

Agreements:

- There is no enhancement to PDCCH monitoring capability (number of BD and non-overlapping CCEs) specifically for UL CI monitoring purpose

Agreements:

- The maximum monitoring periodicity for UL CI is [5] slots

Agreements:

- Up to X BDs can be configured for UL CI

  - FFS per UL CI monitoring occasion or per span

  - The value of X is to be concluded during this week

  - Note: UE is not expected to be configured with search space configuration for UL CI with AL and number of candidates exceeding X BDs

Agreements:

- The maximum size for dci-PayloadSize-forCI is 126

Agreements:

- Possible values for RRC parameter timedurationforCI can be:

  - If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion

    - At least the same as the configured UL CI monitoring periodicity

      - FFS whether or not to additionally support multiple of UL CI monitoring periodicity

  - Otherwise (i.e., >1 monitoring occasion within 1 slot when 1-slot is the configured UL CI monitoring periodicity)

    - {2, 4, 7, [14]} OS, which SCS is used when determine the time duration

      - SCS for the DL BWP carrying UL CI

    - FFS The UE is not expected to be configured with a time duration for CI less than the time different (in symbols) between any adjacent monitoring occasions in a slot

R1-1913368

Agreements:

  - Possible values (16 values) for RRC parameter CI-PayloadSize are

    - {[1],2,4,[5],7,8,[10],14,16,[20],[25],28,32,[35],56,112}

  - timeGranularityforCI is defined as number of partitions within the time region, and possible values are

    - {1,2,4,7,14,28}

  - The configured value of CI-PayloadSize shall be a multiple integer of the configured value of timeGranularityforCI

Agreements:

  - The frequency region for UL CI is derived by the following

    - A RIV indication configured by RRC within value range of (0..37949) (i.e. the same way as IE “locationAndBandwidth” for BWP configuration ), the configuration is per serving cell specific

      - The reference point is derived based on the RRC parameter offsetToCarrier (existing parameter, same way as BWP configuration)

    - A reference SCS (no RRC configuration) for a serving cell (to handle the case where a UE is configured with multiple BWPs using different SCSs on the serving cell),

      - Use the SCS for the DL BWP carrying UL CI as the reference SCS

Agreements:

- Support per serving cell configuration for the following parameters

- CI-PayloadSize

- timedurationforCI

- timeGranularityforCI

- frequencyRegionforCI

Agreements:

- If a serving cell is configured with SUL, each UL carrier (SUL and non-SUL) can be configured with different positionInDCI.

Agreements:

- The DL symbols indicated by tdd-UL-DL-ConfigurationCommon are excluded from the reference time region for UL CI

  - The partition of reference time region is done after excluding the DL symbols

  - The symbols used for SSB are also excluded

Agreements:

- Clarification of 2D-bitmap

  - 2D-bitmap is to use X bits for bitmap indication over a time/frequency region with M partitions in time and N partitions in frequency, and X=M x N

Agreements:

Regarding “FFS whether or not to additionally support multiple of UL CI monitoring periodicity”

- If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion, no additionally support that the time duration to be multiple of UL CI monitoring periodicity

R1-1913527	Summary#3 of UL inter UE Tx prioritization/multiplexing	vivo

Agreement

To determine the P0 value in case SRI is not configured in the DCI

- Option 1A: The open-loop power control parameter set indication field in the DCI can be configurable to be 1 or 2bits

  - P0-PUSCH-Set can provide up to two P0 values

    - UE uses the P0 values according to open loop power control indication field in DCI

    - UE use P0 from P0-PUSCH-AlphaSet when

      - open-loop power control parameter set indication field is 1bit and “0” is indicated, or

      - open-loop power control parameter set indication field is 2bits and “00” is indicated

  - Open-loop power control parameter set indication field can be separately configurable for DCI format 0_1 and DCI format 0_2

    - If open-loop power control parameter set indication field is not present for a DCI format, use P0 from P0-PUSCH-AlphaSet

  - A single configuration of P0-PUSCH-Set applies to both DCI format 0_1 and DCI format 0_2

The following agreements were made during RAN1 meeting #100e:

R1-2000801	Summary of UL inter UE Tx prioritization for URLLC	vivo

R1-2001180	Summary#2 of UL inter UE Tx prioritization for URLLC	vivo

[100e-NR-L1enh_URLLC-Inter_UE-01] – Xueming (vivo)

Email discussion/approval on RRC parameter related issues (issues 1 to 5 as in R1-2000801) by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001290	Summary of email discussion [100e-NR-L1enh_URLLC-Inter_UE-01]	vivo

Decision: As per email decision posted on Feb. 28th,

Agreements:

- Confirm that 14OS can be configured for timedurationforCI (when 1-slot is the configured UL CI monitoring periodicity with more than one monitoring occasions within 1 slot)

- The possible values for CI-PayloadSize, are {1,2,4,5,7,8,10,14,16,20, 28,32,35,42,56,112}

The following TP is endorsed

------------------------------------ Start of TP for 38.213 --------------------------------------------

11.2A	Cancellation indication

< Unchanged parts are omitted >

For a group of symbols,  bits from each set of bits have a one-to-one mapping with  groups of PRBs where each of the first  groups includes  PRBs and each of the remaining  groups includes  PRBs. A UE determines a first PRB index as  and a number of contiguous RBs as  from frequencyRegionforCI that indicates an offset  and a length  as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB that indicates  for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

< Unchanged parts are omitted >

------------------------------------ End of TP for 38.213 --------------------------------------------

Decision: As per email decision posted on Feb. 29th,

Agreements:

- The maximum UL CI monitoring periodicity is 10 slots.

- Up to X BDs can be configured per UL CI monitoring occasion, X to be decided between X=1 or X=2 in RAN1#100bis.

No RAN1 spec impact - RRC parameter update only.

[100e-NR-L1enh_URLLC-Inter_UE-02] – Xueming (vivo)

Email discussion/approval on UE behavior related issues (issues #1, #3, #5) as in R1-2000801) by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001291	Summary of email discussion [100e-NR-L1enh_URLLC-Inter_UE-02]	vivo

Decision: As per email decision posted on Mar. 3rd,

Agreements:

- UE derives the RUR start based on “logical time” (i.e. assuming DL timing difference is 0 and TA=0) and the actual cancellation symbol based on “actual time” (i.e. assuming actual DL timing difference, actual TA)

- A new RRC parameter delta_offset d having possible values {0, 1, 2} OFDM symbols is introduced, update the spec as the following

For the serving cell, the UE determines the first symbol of the symbols to be the first symbol that is after + d from the end of a PDCCH reception where the UE detects the DCI format 2_4.corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming with being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell.

- Clarify the following by a RAN1 spec update (see below)

  - UE is not expected to cancel the transmission of SRS or PUSCH before the first symbol that is T_proc,2 after the end of the reception of the last symbol of the PDCCH carrying the ULCI including the effect of the timing advance.

38.213 Text proposal (maybe further refined by spec editor)

An indication by a DCI format 2_4 for a serving cell is applicable to PUSCH or SRS transmissions on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer parameter [xxxx]. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. UE is not expected to cancel the transmission of SRS or PUSCH before the first symbol that is Tproc,2 after the end of the reception of the last symbol of the PDCCH carrying the ULCI including the effect of the timing advance.

Decision: As per email decision posted on Mar. 3rd,

No conclusion on issues related to the interaction between intra-UE UL prioritization/multiplexing and inter-UE UL cancellation.

Postponed to RAN1#100bis

Last issue: Whether another UL transmission can be scheduled in the cancelled symbols that do not overlap with the resource indicated by UL CI. Also postponed to RAN1#100bis

[100e-NR-L1enh_URLLC-Inter_UE-03] – Xueming (vivo)

Email discussion/approval on UE behavior related issues (issues #6, #7, #8, #9) as in R1-2000801) by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001292	Summary of email discussion [100e-NR-L1enh_URLLC-Inter_UE-03]	vivo

Decision: As per email decision posted on Feb. 28th,

Agreements:

- UE performs the UL cancellation based on any detected UL CI, no additional specification for the case of overlapping reference time region for multiple UL CI occasions.

- A cancelled PUSCH transmission by a UE is counted towards the number of PUSCH that a UE can support per slot

Decision: As per email decision posted on Feb. 28th, the following  is taken as a conclusion with the understanding there is no spec impact

Conclusion:

- It is possible for a UE to indicate both  pa-PhaseDiscontinuityImpacts  (i.e. 6-23) and the support of UL CI for intra-band UL CA

For a UE indicates a capability to cancel overlapping PUSCHs on different intra-band serving cells (if any), and the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE cancels the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to.

The following agreements were made during RAN1 meeting #100-bis-e:

Agreements:

l  UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions is not affected by UL CI.

l  In case of UL inter-UE prioritization, when a UL transmission from a UE is cancelled by UL CI, the UE is not expected to be scheduled with UL transmission in the cancelled symbols that do not overlap with the resource indicated by UL CI.

Update from conference call (4/28):

Agreeement:

- If both UL CI and intra-UE priority indicator are configured for a given UE, support a new RRC parameter to configure Behavior #1

  - Behaviour #1: UL CI is only applicable to the UL transmissions indicated/configured as low priority level

- When the RRC parameter is not provided to the UE, behaviour #2 is used

  - Behaviour #2: UL CI is applicable to UL transmission irrespective of its priority level

- Note: the RRC signaling details will be decided by RAN2

Agreements:

To adopt the following TP for 38.213

11.2A Cancellation indication =====omitted text ====== For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCIa number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated bytdd-UL-DL-ConfigurationCommon, from the time durationprovided by timeDurationforCI in timeFrequencyRegion if the configured UL CI monitoring periodicity is 1 slot with more than one monitoring occasions. Otherwise, the time duration is equal tothe PDCCH monitoring periodicity provided by the value ofmonitoringSlotPeriodicityAndOffset, as described in Clause 10.1. - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion =====omitted text ======

Agreements:

Up to X BDs can be configured per UL CI monitoring occasion

- For ULCI monitoring occasion determination, search space sets start at a same OFDM symbol correspond to a same monitoring occasion

- X=1

Agreements:

- Adopt option 3 and the corresponding TP as below

  - UE uses the smallest SCS configuration between the SCS configurations of the PDCCH for DCI format 2_4 detection and  the SCS configurations in scs-SpecificCarrierList of UL carrier to determine the RUR starting symbol.

  - UE uses the smallest SCS configurations in scs-SpecificCarrierList of UL carrier to determine offset d.