3GPP TSG RAN WG1 #100bis-e	         	 R1- 2004275

e-Meeting, May 25th – June 5th, 2020

Agenda item:		7.2.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining Issues on Channel Access Procedures for NR-U

Document for:		Discussion and Decision

# 1	Introduction

NR-U study item discussed and studied several aspects related to channel access procedures, including LBT mechanisms, LBT options for different DL and UL channels, receiver assisted LBT, directional LBT, etc. Multiple options and design principles for channel access procedure have agreed and been captured into TR 38.899 [1]. The latest approved WID is in RP-191575.

The related work was concluded for the most part in RAN1#99. In this contribution we discuss the few remaining open issues related to configured grants for NR Unlicensed.

# 2. Open Issues and Corrections

Issue #1: Applicability of semi-static channel access (FBE) when Wi-Fi is present

During the NR-U Study Item, it was identified that FBE can be useful when LBE nodes are not present. The related agreement is as follows:

Agreement:

- It has been identified that FBE operation for the scenario where it is guaranteed that LBE nodes are absent on a long term basis (e.g., by level of regulation) and FBE gNBs are synchronized can achieve the following.

  - Ability to use frequency reuse factor 1

  - Lower complexity for channel access due to lack of necessity to perform random backoff

- FFS requirement of synchronization accuracy

- FFS specification impact

- Note: This does not imply that LBE does not have benefits in similar scenarios although there are differences between the two modes of operation

- Note: FBE may also have some disadvantages compared to other modes of operation such as LBE, e.g., a fixed overhead for idle time during a frame.

The related point in the WID is:

For FBE, channel access mechanism in line with agreements from the NR-U study item (TR 38.889, Section 7.2.1.3.1). FBE is intended for environments where the absence of Wi-Fi is guaranteed (e.g., by level of regulations, private premises policies, etc. Further, the targeted scenario is limited to a single NR-U network (i.e. single operator) in the operating band and geographic area. Specification work to be performed by RAN1.

Based on the discussion in the SI, it is clear that FBE is expected to work well only in absence of LBE nodes. However, there is no need to restrict the scenarios in which FBE can be deployed with RAN1 specifications. For example, even if the regulations allow for use of other technologies on a given frequency band and geographic region, the owner of a facility may choose not to deploy any nodes other than FBE in certain space. Therefore, we see that the description in 37.213 should be revised as described in the text proposal below:

-------- Beginning of Text Proposal (TS 37.213) ------------

## 4.3	Channel access procedures for semi-static channel occupancy

If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1.

---------- End of Text Proposal -------------

Proposal 1: remove the limitation in 37.213 on semi-static channel access being applicable only in absence of other technologies.

Issue #2: Applicability of CP extension to Aperiodic SRS

In RAN1#99, the details of using CP extension prior to UL transmission were agreed, e.g.:

Agreement:

For fallback DL assignments scheduling UL transmissions (e.g. PUCCH):

- LBT type, and the length of the CP extension are jointly encoded in the DL assignment and signaled with 2 bits.

- The supported combinations of LBT type and length of the CP extension are hardcoded in RAN1 specs

Related clarification to 38.211 were agreed in RAN1#100-e. However, it was left open whether to support CP extension also with SRS. SRS transmissions in NR can be triggered dynamically (i.e. aperiodic SRS), using a DL assignment (DCI 1_1, 1_2), a UL grant (0_1, 0_2), or DCI format 2_3. Furthermore, periodic SRS configuration is also supported (bot (periodic or semi-persistent SRS).

We note that the in the case of Aperiodic SRS, the triggering DCIs 1_1 and 0_1 already include the “ChannelAccess-CPext-CAPC”-field, and hence it becomes straight forward to apply CP extension to SRS as well. The other DCIs capable of triggering SRS do not presently support indication of CP extension. The latest version of 38.212 and 38.214 seem to already support this behavior, and hence the only necessary specification change is in 38.211. A corresponding TP is embedded below.

Proposal 2 When Aperiodic SRS is triggered with a DCI (0_1, 1_1) that also includes indication of CP extension, the CP extension applies to SRS as well.

------------- Beginning of Text Proposal, 38.211 ------------

In case of cyclic prefix extension of the first OFDM symbol l allocated for PUSCH, SRS or PUCCH transmission, the time-continuous signal sext(p,μ)t for the interval tstart,lμ-Text≤t<tstart,lμ preceding the first OFDM symbol for PUSCH, SRS or PUCCH is given by

sext(p,μ)t=sl(p,μ)t

where t<0 refers to the signal in the previous subframe and

-	for dynamically scheduled PUSCH, SRS and PUCCH transmissions

Text=minmaxText',0, Tsymb,(l-1)mod7∙2μμ

Text'=k=1CiTsymb,  l-kmod 7∙2μ μ-Δi

------------- End of Text Proposal ------------

Issue #3: Channel Access for periodic signals / channels

Channel access for periodic signals and channels, such as periodic/SPS CSI or SRS, has been little discussed. It is nevertheless necessary to agree on the type of channel access used for them.

When periodic UL transmissions occur outside of a gNB COT, they should naturally make use of Type 1 UL channel access, aka Cat4 LBT. On the other hand, when periodic UL transmission fall within a gNB COT, Cat4 LBT is not necessary, but the UE can instead use Type 2 (A/B/C) UL channel access.

According to the current version of 37.213, Section 4.2.1.0.0, after detecting DCI 2_0 and determining that a UL transmission (either dynamically scheduled or configured) falls within the gNB’s COT, the UE may convert from Type 1 channel access procedure to Type 2A. However, Type 2A channel access assumes that the gap from the end of a DL transmission until the start of the (periodic) UL transmission is at least 25 us, which is not always the case. Therefore, the description should be generalized so that also UL Type 2B and 2C channel access procedures are allowed, when the related side conditions are satisfied.

Proposal 3 : When a UL transmission scheduled with Type 1 UL channel access (either dynamically scheduled or configured) falls within a gNB COT, the UE may switch to either Type 2A, 2B, or 2C channel access, when the related conditions (duration of the gap and duration of the UL transmission) are met.

-------- Beginning of Text Proposal, TS 37.213 ------------

============================<<unchanged text omitted>>===============================

##### 4.2.1.0.0	Channel access procedures upon detection of a common DCI

If a UE detects 'UL duration and offset' field in DCI Format 1C as described in clause 5.3.3.1.4 of [5], the following are applicable:

-	If the 'UL duration and offset' field indicates an 'UL offset'  and an 'UL duration'  for subframe , then the scheduled UE may use channel access procedures Type 2 for transmissions in subframes  where , irrespective of the channel access Type signalled in the UL grant for those subframes, if the end of UE transmission occurs in or before subframe .

-	If the 'UL duration and offset' field indicates an 'UL offset'  and an 'UL duration'  for subframe  and the 'COT sharing indication for AUL' field is set to '1', then a UE configured with autonomous UL may use channel access procedures Type 2 for autonomous UL transmissions assuming any priority class in subframes  where , if the end of UE autonomous UL transmission occurs in or before subframe  and the autonomous UL transmission between  and  shall be contiguous.

-	If the 'UL duration and offset' field indicates an 'UL offset'  and an 'UL duration'  for subframe  and the 'COT sharing indication for AUL' field is set to '0', then a UE configured with autonomous UL shall not transmit autonomous UL in subframes  where .

If a UE determines the duration in time domain and the location in frequency domain of a remaining channel occupancy initiated by the gNB from a DCI format 2_0 as described in clause 11.1.1 of [7], the following is applicable:

-	The UE may switch from Type 1 channel access procedures as described in clause 4.2.1.1 to Type 2A, Type 2B, and Type 2C channel access procedures as described in clauses 4.2.1.2.1, 4.2.1.2.2, and 4.2.1.2.3, respectively,  for its corresponding UL transmissions within the determined duration in time and location in frequency domain of the remaining channel occupancy, when the conditions in clause 4.2.1.0.3 are met. In this case, if the UL transmissions are PUSCH transmissions on configured resources, the UE may assume any priority class for the channel occupancy shared with the gNB.

============================<<unchanged text omitted>>===============================

-------- End of Text Proposal ------------

Issue #4 Type 2C Channel Access for consecutive UL transmissions

Channel access for consecutive UL transmissions was discussed during RAN1#100bis-e, and a few related clarifications were agreed. However, one issue that was not concluded relates to use of Type 2C UL channel access procedures.

As has been agreed, Type 2C UL channel access procedures can be applied when the duration of the UL transmission does not exceed 0.584 ms. We note that use of Type 2C channel access procedure is beneficial especially for transmission of time-critical control information. Therefore, the chances of using Type 2C channel access should be maximized.

For stand-alone transmissions it is of course not an issue to schedule the UE directly with Type 2C UL channel access, but for consecutive transmissions this is not practical. Since the maximum duration of a transmission following Type 2C LBT is limited, one must use Type 2B channel access instead, or alternatively leave a gap between the consecutive transmissions. However, the best option would be to always allow for the transmission according to Type 2C channel access for a duration of up to 0.58 ms, even if Type 2B was indicated to the UE.

Proposal 4: A UE is allowed to transmit according to Type 2C UL channel access for up to 0.584 ms also when type 2B channel access has been indicated in the DCI.

The related text proposal is captured below:

-------- Beginning of Text Proposal ------------

============================<<unchanged text omitted>>===============================

##### 4.2.1.0.1	Channel access procedures for consecutive UL transmission(s)

For contiguous UL transmission(s), the following are applicable:

-	If a UE is scheduled to transmit a set of UL transmissions including PUSCH using a UL grant , and if the UE cannot access the channel for a transmission in the set prior to the last transmission according to one of Type 1, Type 2, or Type 2A UL channel access procedures, the UE shall attempt to transmit the next transmission according to the channel access type indicated in the UL grant. Otherwise, if the UE cannot access the channel for a transmission in the set prior to the last transmission according to Type 2B UL channel access procedure, the UE shall attempt to transmit the next transmission according to Type 2A UL channel access procedure. If a UE cannot access the channel for transmission according to a Type 2B UL channel access procedure, the UE may access the channel according to Type 2C UL channel access procedure.

-    If a UE is scheduled by a gNB to transmit a set of UL transmissions including PUSCH using a UL grant, the UE shall not apply a CP extension for the remaining UL transmissions in the set after the first UL transmission after accessing the channel.

-	If a UE is scheduled to transmit a set of  consecutive UL transmissions without gaps including PUSCH  using one or more UL grant(s) and the UE transmits one of the scheduled UL transmissions in the set after accessing the channel according to one of Type 1, Type 2, Type 2A, Type 2B or Type 2C UL channel access procedures, the UE may continue transmission of the remaining UL transmissions in the set, if any.

-    If a UE is configured to transmit a set of consecutive PUSCH transmissions on resources configured by the gNB, the time domain resource configuration defines multiple transmission occasions, and if the UE cannot access the channel according to Type 1 UL channel access procedure for transmitting in a transmission occasion prior to the last transmission occasion, the UE shall attempt to transmit in the next transmission occasion according to Type 1 UL channel access procedure. If the UE transmits in one of the multiple transmission occasions after accessing the channel according to Type 1 UL channel access procedure, the UE may continue transmission in the remaining transmission occasions in the set, wherein each transmission occasion starts at the starting symbol of a configured grant PUSCH within the duration of the COT.

-	A UE is not expected to be indicated with different channel access types for any consecutive UL transmissions without gaps in between the transmissions, except if Type 2B or Type 2C UL channel access procedures are identified for the first of the consecutive UL transmissions.

============================<<unchanged text omitted>>===============================

-------- End of Text Proposal ------------

# 3. Conclusions

In this contribution we point out a couple of critical missing aspects related to channel access in NR-Unlicensed. The related proposals are:

Proposal 1: Remove the limitation in 37.213 on semi-static channel access being applicable only in absence of other technologies.

Proposal 2 When Aperiodic SRS is triggered with a DCI (0_1, 1_1) that also includes indication of CP extension, the CP extension applies to SRS as well.

Proposal 3 : When a UL transmission scheduled with Type 1 UL channel access (either dynamically scheduled or configured) falls within a gNB COT, the UE may switch to either Type 2A, 2B, or 2C channel access, when the related conditions (duration of the gap and duration of the UL transmission) are met.

Proposal 4: A UE is allowed to transmit according to Type 2C UL channel access for up to 0.584 ms also when type 2B channel access has been indicated in the DCI.

# References

- TR 38.899 Study on NR-based access to unlicensed spectrum