3GPP TSG-RAN WG1 Meeting #101	R1- 2004347

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.1.1

Source:	Ericsson

Title:	Channel Structure Related Corrections For 2-Step RACH

Document for:	Decision

# 1	Introduction

This contribution addresses corrections for channel structure aspects of 2-step RACH, including

- PRACH occasions not associated with SSB are invalid ROs

- Length of MsgA PUSCH excludes guard period

- Clarification on the frequency order of PUSCH occasions and corrections on the start PRB of PUSCH occasion which should be described in 38.213 instead of being treated as resource allocation type 1,

- PUSCH is not transmitted when requirement for a time gap between PRACH and PUSCH for the MsgA is not met at least for licensed band and support gapless MsgA in NR-U as is already specified in current spec.

# 2	Discussion

## 2.1 	PRACH occasions not associated with SSB

In RAN1 #100bis meeting, following observations were agreed showing that ROs not mapped to SSB will not be used for PRACH transmissions in 2-step RACH.

Agreements:

- Observation: The RO validation rule defined in TS 38.213 section 8.1 and the rule for “PRACH occasions not associated with SS/PBCH blocks after an integer number of association periods, if any, are not used for PRACH transmissions” apply to the RACH occasions configured by both 4-step RACH and 2-step RACH.

Then it was discussed whether a clarification TP is needed to say that the ROs not associated SSB will be treated as invalid ROs to make sure they’re not considered in the preamble to PRU mapping procedure, which was summarized in [1].

Since the 2-step ROs and 4-step ROs may be shared and we cannot say part of the RO is valid for 4-step RACH while part of the RO is invalid for 2-step RACH, if a clarification TP is needed, it would be preferred to specify that an RO not used for PRACH transmission will be an invalid RO no matter if it’s for 2-step RACH or 4-step RACH. Thus, we have following proposal.

Further discuss the necessity of a TP to clarify that the ROs not associated SSB will be invalid ROs. If a TP is needed, the rules should be applied to ROs for both 2-step RACH and 4-step RACH.

## 2.2 	Clarification on the guard period between hops

In RAN1 #100bis meeting, following conclusion were made in the initial discussion regarding the PUSCH data symbol to RE mapping.

Conclusion

- Continue the discussion in this meeting based on the TP#2 in the appendix with potential updates, to clarify that the guard period between frequency hops is not used for PUSCH transmission.

  - It does not preclude the case that the TP is not approved

Then in the 2nd phase, no agreements were reached on the TP to clarify that guard OFDM symbols will not be considered in the mapping in 38.211 section 6.3.1.6, as summarized in [2].

Based on current text, especially the yellow shaded text below, the mapping is only on the PRBs and OFDM symbols allocated to the PUSCH which means the guard is excluded in the mapping, and the mapping is done in frequency first, and in time 2nd.

The start OFDM symbol index is specified in 38.214, where without frequency hopping it will be the start symbol in the initial resource allocation, but when frequency hopping is enabled, the start symbol of the 2nd hop may not be the symbol right after the last symbol of the 1st hop since there could be some gap.

38.211 V16.1.0: 6.3.1.6 Mapping to virtual resource blocks For each of the antenna ports used for transmission of the PUSCH, the block of complex-valued symbols shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with to resource elements (k',l)p,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission, and - the corresponding resource elements in the corresponding physical resource blocks are not used for transmission of the associated DM-RS, PT-RS, or DM-RS intended for other co-scheduled UEs as described in clause 6.4.1.1.3 The mapping to resource elements (k',l)p,μ allocated for PUSCH according to [6, TS 38.214] shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index , with the starting position given by [6, TS 38.214]. -------------------------------------------------------------------------------------------------------------------- 38.214 V16.1.0: 6.3.1 Frequency hopping for PUSCH repetition Type A … In case of intra-slot frequency hopping, the starting RB in each hop is given by: , where i=0 and i=1 are the first hop and the second hop respectively, and is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. The number of symbols in the first hop is given by , the number of symbols in the second hop is given by , where NsymbPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot. … 38.331 CR R2-2004288: startSymbolAndLengthMsgA-PO An index giving valid combinations of start symbol, length and mapping type as start and length indicator (SLIV) for the first msgA PUSCH occasion, for RRC_CONNECTED UEs in non-initial BWP as described in TS 38.214 [19] clause 6.1.2. The network configures the field so that the allocation does not cross the slot boundary. The number of occupied symbols excludes the guard period. If the field is absent, the UE shall use the value in msgA-PUSCH-TimeDomainAllocation (see TS 38.213 [13], clause 8.1A). msgA-PUSCH-TimeDomainAllocation Indicates a combination of start symbol and length and PUSCH mapping type from the TDRA table (PUSCH-TimeDomainResourceAllocationList if provided in PUSCH-ConfigCommon, or in PUSCH-Config, or else the default Table 6.1.2.1.1-2 in 38.214 [19] is used if msgA-PUSCH-TimeDomainAllocation is not provided in PUSCH-ConfigCommon or in PUSCH-Config).

According to discussions above, the only thing related to guard period between 2 hops of MsgA PUSCH that may require clarification in the specification is that the length of symbols determined by msgA-PUSCH-TimeDomainAllocation and the L symbols defined in the default time domain resource allocation tables should also exclude the guard symbols between 2 hops if frequency hopping for MsgA PUSCH is used, similar to the text when defining startSymbolAndLengthMsgA-PO.

Clarify in 38.214 that the length of symbols determined by msgA-PUSCH-TimeDomainAllocation should also exclude the guard symbols between 2 hops in case of frequency hopping for MsgA PUSCH, according to TP 1.

-----------------------------------------start of TP1 for 38.214 section 6.1.2.1---------------------------------------

#### 6.1.2.1	Resource allocation in time domain

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, and the number of repetitions (if numberofrepetitions is present in the resource allocation table) to be applied in the PUSCH transmission. The number of occupied symbols indicated by SLIV or allocated by L excludes the guard period when applicable.

--------------------------------------unchanged text omitted------------------------------------

------------------------------------------------------end of TP1------------------------------------------------------------

## 2.3	PUSCH frequency resource index ordering in case of frequency hopping

When frequency hopping is enabled for MsgA PUSCH, when different hops selected to determine the frequency resource index for the mapping from preambles to PRU, the mapping result may be different.

As an example, 2 ROs correspond to one PO set with 2 POs and intra-slot hopping is enabled. When the POs have same ordering in frequency domain as illustrated in Figure 1 below, no matter if hop1 or hop2 is selected for the preamble to PRU mapping, the mapping result will be the same.

Figure 1. POs in same order in frequency domain for 2 hops

But when the POs have different ordering in frequency domain on 2 hops, e.g. when the set of POs in the 1st hop are in the middle of the BWP while  is the indicated frequency offset for the 2nd hop, as illustrated in Figure 2 below, selecting hop1 or hop2 for the preamble to PRU mapping will have different results since the preambles are mapped to POs in the order of frequency first according to the agreements above.

Figure 2. POs in different order in frequency domain for 2 hops

To address this issue, the following conclusion has been made in RAN1 #100bis meeting to determine the frequency resource order when frequency hopping is applied. And whether a TP is needed is to be discussed in this meeting.

Conclusion:

- The ordering of the PUSCH occasions follows the “lowest PUSCH occasion” in the definition of frequencyStartMsgA-PUSCH, i.e. the start PRB in the resource allocation for a MsgA PUSCH, regardless the frequency hopping for the PUSCH transmission is enabled or not.

- FFS if TP is needed to clarify that the mapping order is based on the first hop when frequency hopping is enabled.

The conclusion means the frequency order of PO is always the PO order in the 1st hop since frequencyStartMsgA-PUSCH determines the lowest PO in the 1st hop when frequency hopping is enabled.

frequencyStartMsgA-PUSCH Offset of lowest PUSCH occasion in frequency domain with respect to PRB 0 (see TS 38.213 [13], clause 8.1A).

But in current the specification text, the conclusion is not captured, and it is still not clear the order is based on the 2nd hop or the 1st hop.

In the yellow shared text, the term “frequency resource index”, which was actually only used for ordering FDMed PRACH occasions in 38.213 in release 15, that is now used for FDMed POs only tells the order of POs in frequency domain, it is not clear whether the fid=0 is for the PO with a PRB determined by frequencyStartMsgA-PUSCH or should be for the PO with lowest PRB among all PUSCH occasions even when frequency hopping is enabled. If the understanding is the latter, then in Figure 2, the PO2 will have fid=0 which is against the conclusion we’ve made.

Each consecutive number of Npreamble preamble indexes from valid PRACH occasions in a PRACH slot - first, in increasing order of preamble indexes within a single PRACH occasion - second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions - third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot are mapped to a valid PUSCH occasion and the associated DMRS resource - first, in increasing order of frequency resource indexes fid for frequency multiplexed PUSCH occasions ….

To make it clear in the standard that the PUSCH occasion with zero frequency resource index is always the PO with lowest PRB defined by frequencyStartMsgA-PUSCH, TP2 is provided.

Furthermore, unlike msg3 PUSCH or a normal PUSCH, the frequency domain resource allocation for MsgA PUSCH is only described in 38.213 instead of being described in 38.214. But in current spec in clause 6.3.1 of 38.214, the start PRB of a PUSCH transmission (including MsgA PUSCH) supporting intra-slot frequency hopping is assumed to be described in Clause 6.1.2.2.2 (resource allocation type 1) of 38.214, which is wrong for MsgA PUSCH, so a TP3 is needed to correct the error regarding the start PRB definition for MsgA PUSCH.

Capture the conclusion in RAN1 spec. to make it clear on the frequency resource index values, according to text proposal TP2 for 38.213 and make it correct that the start PRB of MsgA PUSCH is described in 38.213 according to TP3 for 38.214.

-----------------------------------------start of TP2 for 38.213 section 8.1A-----------------------------------------

## 8.1A	PUSCH for Type-2 random access procedure

--------------------------------------unchanged text omitted------------------------------------

Each consecutive number of Npreamble preamble indexes from valid PRACH occasions in a PRACH slot

-	first, in increasing order of preamble indexes within a single PRACH occasion

-	second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

are mapped to a valid PUSCH occasion and the associated DMRS resource

-	first, in increasing order of frequency resource indexes fid for frequency multiplexed PUSCH occasions, where fid=0 for the PUSCH occasion with the lowest PRB defined by frequencyStartMsgA-PUSCH when frequency hopping in a slot is enabled by msgA-intraSlotFrequencyHopping

-	second, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211]

-	third, in increasing order of time resource indexes tid for time multiplexed PUSCH occasions within a PUSCH slot

-	fourth, in increasing order of indexes for Ns PUSCH slots

where Npreamble=ceilTpreambleTPUSCH, Tpreamble is a total number of valid PRACH occasions per association pattern period multiplied by the number of preambles per valid PRACH occasion provided by msgA-PUSCH-PreambleGroup, and TPUSCH is a total number of valid PUSCH occasions per PUSCH configuration per association pattern period multiplied by the number of DMRS resource indexes per valid PUSCH occasion provided by Configuration.

--------------------------------------unchanged text omitted------------------------------------

------------------------------------------------------end of TP2--------------------------------------------------------------

-----------------------------------------start of TP3 for 38.214 section 6.3.1-----------------------------------------

### 6.3.1	Frequency hopping for PUSCH repetition Type A

--------------------------------------unchanged text omitted------------------------------------

For a MsgA PUSCH the frequency offset is provided by the higher layer parameter as described in [6, TS 38.213.

In case of intra-slot frequency hopping, the starting RB in each hop is given by:

,

where i=0 and i=1 are the first hop and the second hop respectively, and  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) or as calculated from the resource assignment for MsgA PUSCH (described in 38.213) and is the frequency offset in RBs between the two frequency hops. The number of symbols in the first hop is given by , the number of symbols in the second hop is given by , where NsymbPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot.

--------------------------------------unchanged text omitted------------------------------------

------------------------------------------------------end of TP3-------------------------------------------------------------

## 2.4 	Gap requirement between MsgA preamble and PUSCH

In the RAN1 #99 meeting, the following agreement was made on the gap requirement between a MsgA preamble and MsgA PUSCH that applies only to licensed band operation.

Agreements:

- The minimum transmission gap between the end of msgA PRACH and the beginning of msgA PUSCH (guard time excluded) is no less than Ngap symbols, as specified in TS 38.213, i.e., 2 or 4 symbols depending on the SCS

  - This is not applied for NR-U

  - Note: This is aligned with Rel-15

In CBRA, the issue is that the current agreed MsgA PUSCH configurations and the preamble to PRU mapping methods may provide a MsgA that has a gap that does not meet the requirement above. In this case, the UE may need to select a 4-step RA type or select a different PRACH resource or discard the MsgA transmission.

In CFRA, the network may need to allocate a 2-step CFRA resource to meet the gap requirement in licensed operation for the UEs in the MsgA PUSCH configuration in dedicated signaling. But RAN2 has agreed to use full signaling for MsgA PUSCH configurations for CFRA similar to CBRA, so it’s also possible that a MsgA resource that has a gap does not meet the requirement may be assigned to a UE in CFRA of 2-step RA type.

According to current MsgA PUSCH configuration and preamble to PRU mapping, UE may determine a MsgA that does not meet the gap requirement between the MsgA preamble and MsgA PUSCH for operation in licensed band.

In this case, to switch to a 4-step RA type if configured is a possible solution. Another way is to allow preamble only MsgA transmission, which can be applied no matter whether 4-step RA is configured or not and is preferred so that the RA type selection can still be the same as agreed and as described in the MAC specification. If 4-step RA is configured, and when the network receives a preamble only MsgA, it can either fall back or do nothing so that UE will make another reattempt of 2-step RA. If 4-step RA is not supported, UE will only rely on the reattempt of MsgA in 2-step RA.

Based on the discussions and observations above, we have following proposal:

If the MsgA resource the UE determines does not meet the gap requirement between MsgA preamble and PUSCH, the PUSCH is not transmitted, according to text proposal TP4.

Another issue raised in the UE capability discussions in NR-U topic is whether a minimum gap should also be required for unlicensed operation.

Actually when this gap requirement in NR 2-step RACH work item in RAN1 was discussed, this requirement was only allowed for licensed band considering that an additional LBT operation can be avoided if there is less than a 16 us gap between the PRACH and PUSCH parts of MsgA. If a zero symbol gap is supported instead of the 2 or 4 symbol gap as for licensed bands, then there exist PRACH configurations that would fulfill the less than 16 us requirement, thus avoiding the additional LBT operation. This is beneficial in terms of reduced access latency.

Support of a zero-symbol gap (N = 0) between the PRACH and PUSCH parts of MsgA can eliminate an extra LBT operation for several PRACH formats for NR-U.

So, it is preferred that UEs in 2-step RACH in unlicensed spectrum support no gap (N = 0) between MsgA preamble and MsgA PUSCH. Based on currently text in 38.213, zero gap is already required since the spec says nothing about NR-U, which means the logical interpretation is that N=0 for NR-U, but it is better to be clear in the specifications so that there is no guessing. Also, it’s better to write the spec in terms of “operation with shared spectrum channel access” rather than “for operation without shared spectrum channel access”. So some text update is needed as provided also in TP4.

The presently specified zero-symbol gap (N = 0) requirement between the PRACH and PUSCH parts of MsgA, i.e. gap-less MsgA, is not changed for 2-step RA for operation with shared spectrum channel access, but the operation is clarified according to text proposal TP4.

---------------------------------------start of TP4 for 38.213 section 8.1A-----------------------------------------

## 8.1A	PUSCH for Type-2 random access procedure

For a Type-2 random access procedure, a UE transmits a PUSCH, when applicable, after transmitting a PRACH. The UE encodes a transport block provided for the PUSCH transmission using redundancy version number 0. For operation without shared spectrum channel access, tThe PUSCH transmission is after the PRACH transmission by at least N symbols where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP. For operation with shared spectrum channel access, the UE assumes N = 0.

A UE does not transmit a PUSCH in a PUSCH occasion if the PUSCH occasion associated with a DMRS resource is not mapped to a preamble of valid PRACH occasions or if the associated PRACH preamble is not transmitted as described in Clause 7.5 or Clause 11.1 or if the time gap between the PUSCH occasion and corresponding PRACH occasion is less than N symbols for operation without shared spectrum channel access. A UE can transmit a PRACH preamble in a valid PRACH occasion if the PRACH preamble is not mapped to a valid PUSCH occasion.

------------------------------------------unchanged text omitted------------------------------------------

--------------------------------------------------------end of TP4------------------------------------------------------------

# Conclusion

In the previous sections we made the following observations:

Observation 1	According to current MsgA PUSCH configuration and preamble to PRU mapping, UE may determine a MsgA that does not meet the gap requirement between the MsgA preamble and MsgA PUSCH for operation in licensed band.

Observation 2	Support of a zero-symbol gap (N = 0) between the PRACH and PUSCH parts of MsgA can eliminate an extra LBT operation for several PRACH formats for NR-U.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Further discuss the necessity of a TP to clarify that the ROs not associated SSB will be invalid ROs. If a TP is needed, the rules should be applied to ROs for both 2-step RACH and 4-step RACH.

Proposal 2	Clarify in 38.214 that the length of symbols determined by msgA-PUSCH-TimeDomainAllocation should also exclude the guard symbols between 2 hops in case of frequency hopping for MsgA PUSCH, according to TP 1.

Proposal 3	Capture the conclusion in RAN1 spec. to make it clear on the frequency resource index values, according to text proposal TP2 for 38.213 and make it correct that the start PRB of MsgA PUSCH is described in 38.213 according to TP3 for 38.214.

Proposal 4	If the MsgA resource the UE determines does not meet the gap requirement between MsgA preamble and PUSCH, the PUSCH is not transmitted, according to text proposal TP4.

Proposal 5	The presently specified zero-symbol gap (N = 0) requirement between the PRACH and PUSCH parts of MsgA, i.e. gap-less MsgA, is not changed for 2-step RA for operation with shared spectrum channel access, but the operation is clarified according to text proposal TP4.

# References

R1-2003019, “Text proposals of PRACH validation rule”, Moderator (ZTE), RAN1 #100bis-e meeting, April 2020.

R1-2003023, “Text proposal of clarification on the guard period between hops”, Moderator (ZTE), RAN1 #100bis-e meeting, April 2020.