3GPP TSG RAN WG1 Meeting RAN1#101	R1-2004372

e-Meeting, May 25th – June 5th, 2020

Source:	Motorola Mobility, Lenovo

Title:	Maintenance on MU-CSI Enhancements

Agenda Item:	7.2.6.1

Document for:	Discussion and Decision

# Introduction

At RAN1#98bis an agreement was reached for Rel-16 Type II CBSR process. In this contribution, we address an issue that hinders the functionality of CBSR and is a faulty function in the specification which can impact network performance and thus needs to be corrected. We propose a solution that resolves this issue.

# CBSR Related Issues

## Current Agreement

In TS 38.214 [1], CBSR for NR eType-II is specified in Section 5.2.2.2.5, as follows

----------------------------Beginning of text for TS38.214 -----------------------------------------------------------

The bitmap parameter n1-n2-codebookSubsetRestriction-r16 forms the bit sequence B=B1B2 and configures the vector group indices g(k) as in clause 5.2.2.2.3. Bits b2k,2(N1x2+x1+1)b2k,2(N1x2+x1) indicate the maximum allowed average amplitude, γi+pL (p=0,1), with i∈{0,1,…,L-1}, of the coefficients associated with the vector in group g(k) indexed by x1,x2, where the maximum amplitudes are given in Table 5.2.2.2.5-6 and the average coefficient amplitude is restricted as follows

1f=0Mυ-1kl,i+pL,f(3)f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL

for l=1,…,υ, and p=0,1. A UE that does not report the parameter amplitudeSubsetRestriction='supported' in its capability signaling is not expected to be configured with b2k,2(N1x2+x1+1)b2k,2(N1x2+x1)=01 or 10.

Table 5.2.2.2.5-6: Maximum allowed average coefficient amplitudes for restricted vectors

| Bit b2k,2(N1x2+x1+1)b2k,2(N1x2+x1) | Maximum Average Coefficient Amplitude γi+pL |
| --- | --- |
| 00 | 0 |
| 01 | 1/4 |
| 10 | 1/2 |
| 11 | 1 |

----------------------------End of text for TS38.214 -----------------------------------------------------------

## Discussion on CBSR for NR eType-II

In RAN1#98bis [2], an agreement was reached to support soft CBSR with sum-power constraint on the delay-domain coefficients, as follows

1f=0Mυ-1kl,i+pL,f(3)f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL                (1)

One can rewrite the CBSR check equation in (1) as follows

f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL.f=0Mυ-1kl,i+pL,f3              (2)

Note that (2) restricts the sum power of the coefficients of beam i with the product of γi+pL and f=0Mυ-1kl,i+pL,f3, leading the power constraint to be scalable from γi+pL up to γi+pL.Mv. This causes a serious issue in the CBSR process, since it would be parametrized by the number of non-zero coefficients in the transformed delay domain. This leads to anomalous behavior in the CBSR process, since the left-hand side in (1) may not always be proportional with the wideband beam power. The following example illustrates the underlying issue with the current CBSR check equation in (1), where two different parameter designs of a restricted beam are provided, where both designs have negligible difference in total power, yet showing substantial irregularity in the behavior of the CBSR check equation.

Example 1:

Setup. Rank-1 codebook with Mv=4, and beam i* is restricted with γi*=1/2

Design 1., p1,p1=1,  k1,i*,03=1,  p1,i*,02=1 and  k1,i*,13=k1,i*,23=k1,i*,33=0

i.e., W2(i*,:)=1   0   0   0.

Obviously, this beam violates the CBSR check equation in (1), since 1>12. However, the UE can alter the codebook design to pass the CBSR check, as follows

Design 2. p1,p1=1,  k1,i*,03=1,  p1,i*,02=1 and  k1,i*,13=k1,i*,23=1,  p1,i*,12=p1,i*,22=182, and  k1,i*,33=0

i.e., W2'(i*,:)=1  182  182   0.

Here, Design 2 passes the CBSR check in equation (1), since 130384<12, although the change in the power of beam i* is negligible. Not only would the issue result in random behavior of CBSR, it also opens a backdoor for UE implementation to manipulate the CBSR process via converting one or more zero-valued coefficients within a restricted beam to non-zero coefficients with the minimum amplitude value to satisfy the CBSR threshold, without actually reducing the restricted beam power. Note that such manipulative design would be beneficial to the UE since CBSR restricts the individual codebook of the UE in favor of reducing interference at other users, and thereby can adversely impact the network performance. In essence, the role of the 3GPP specifications is to prohibit such UE implementations that may have detrimental effect on other users in the network and on network performance. In Figure 1 we use system simulations to plot the per sub-band power of a beam whenever there is only one non-zero coefficient with amplitude larger than 1/2, similar to Design 1, and compare it with the beam power under Design 2, where two zero-valued coefficients are transformed to non-zero coefficients with amplitude 1/82 each. Assuming a CBSR threshold value of 1/2, the beam under Design 2 would pass the CBSR check in (1), whereas the beam under Design 1 would not pass the CBSR check, although it has less power than that of Design 2 for each sub-band. This implies the current CBSR equation in (1) is not robust. More details on simulation assumptions can be found in the Appendix.

The current CBSR check equation is not robust and can be bypassed via manipulating the codebook design at the UE.

## Proposal for CBSR Equation

As shown in Example 1, the issue with the CBSR check in (1) for eType-II is tied to the normalization factor f=0Mυ-1kl,i+pL,f3. Alternatively, the power of the restricted beam should be constrained with the power of another beam in the codebook, e.g., the unrestricted beam with the maximum wideband power. For instance, the following CBSR equation can be used

1maxp∈0,1maxi'ϵLf=0Mυ-1kl,i'+pL,f(3)pl,p1pl,i'+pL,f22f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL        (4)

where L is the set of indices of the selected L beams that do not include beams from the restricted beam set g(k), which is defined in TS 38.214, Sec. 5.2.2.2.3 [1].

Figure 1: Beam i* power per sub-band using both designs in Example 1

The proposed CBSR check equation has the following characteristics:

- It resembles the CBSR behavior in Rel. 15 Type-II codebook, in the sense that the amplitude restriction is with respect to the largest wideband amplitude value of an unrestricted beam, which has value one.

- It prohibits UE implementations that could bypass restricting a beam power via adding dummy coefficients with low amplitude. The proposed solution provides an intuitive and accurate behavior for CBSR checking.

- Applicable to frequency domain codebooks where the index f is replaced with f'∈1,…,Nsb. Hence, the proposed solution is expected to be forward-compatible with codebook designs in future releases.

Modify the CBSR check equation to be normalized with the power of the strongest selected beam that does not belong to the restricted set.

Consider the following text proposal for TS 38.214

---------------------------- Start of proposed TP for TS38.214 -----------------------------------------------------------

5.2.2.2.5	Enhanced Type II Codebook

--- Unchanged text omitted ---------

The bitmap parameter n1-n2-codebookSubsetRestriction-r16 forms the bit sequence B=B1B2 and configures the vector group indices g(k) as in clause 5.2.2.2.3. Bits b2k,2(N1x2+x1+1)b2k,2(N1x2+x1) indicate the maximum allowed average amplitude, γi+pL (p=0,1), with i∈{0,1,…,L-1}, of the coefficients associated with the vector in group g(k) indexed by x1,x2, where the maximum amplitudes are given in Table 5.2.2.2.5-6 and the average coefficient amplitude is restricted as follows

1f=0Mυ-1kl,i+pL,f(3)f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL

1maxp∈0,1maxi'ϵLf=0Mυ-1kl,i'+pL,f(3)pl,p1pl,i'+pL,f22f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL

for l=1,…,υ, and p=0,1. L⊆0,1,…, L-1 is the set of indices of the selected beams that are not associated with any of the sets of group indices g(k) for k=0,1,2,3 described in 5.2.2.3. A UE that does not report the parameter amplitudeSubsetRestriction='supported' in its capability signaling is not expected to be configured with b2k,2(N1x2+x1+1)b2k,2(N1x2+x1)=01 or 10.

--- Unchanged text omitted ---------

----------------------------End of proposed TP for TS38.214 -----------------------------------------------------------

# Conclusion

Based on the discussion in this contribution we make the following observations:

- The current CBSR check equation is not robust and can be bypassed via manipulating the codebook design at the UE. It is a faulty function in the specification which can adversely impact network performance and thus needs to be corrected.

- Modify the CBSR check equation to be normalized with the power of the strongest selected beam that does not belong to the restricted set.

Our proposal is as follows:

- Consider the text proposal for TS38.214.

# References

| [1] | 3GPP NR TS 38.214, “Physical layer procedures for data,” Mar. 2020. |
| --- | --- |
| [2] | “RAN1#98bis Chairman's Notes,” Chongqing, CN, Oct. 14-20, 2019. |

# Appendix

| Modulation | Up to 256 QAM |
| --- | --- |
| Coding on PDSCH | LDPC |
| Numerology | 15KHz 14 OFDM symbol slot and 52 PRBs |
| Frequency band | 4 GHz |
| Simulation bandwidth | 10 MHz |
| Transmission scheme | Closed SU/MU-MIMO adaptation |
| Scenario | Dense Urban (Micro layer only) |
| UE antenna height and gain | TR36.873 |
| Channel model | 38.901 UMa channel model B |
| Inter-site distance | 200 m. |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Type II coedebook parameters | (L, p0, β) = (4, 1/4, 1/4) |
| PRBs bundling per SB | 1 PRB |
| MU dimension | Up to 12 layers |
| SU dimension | 1/2 layers |
| Codeword (CW)-to-layer mapping | Single codeword |
| CSI feedback | PMI, CQI: sub-band based every 5 slots; 4 slot delay |
| Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
| ACK/NACK delay | The next available UL slot |
| Re-transmission delay | The next available DL slot after receiving NACK |
| Antenna configuration at TRxP | (M,N,P,Mg,Ng) =(8,1,2,1,1), (dH, dV)=(0.5, 0.8)λ |
| Antenna configuration at UE | (M,N,P,Mg,Ng) = (1,1,2,1,1), (dH, dV)=(0.5, N/A)λ |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Non-ideal |
| UE Noise Figure | 9 dB |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Handover margin (dB) | 1 dB |
| TRxP total transmit power | 41 dBm |