3GPP TSG RAN WG1 #101			R1-2004387

e-Meeting, May 25th – June 5th, 2020

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on sidelink physical layer procedure

Agenda Item:	7.2.4.5

Document for: 	Discussion and Decision

- Introduction

At RAN1#100bis-e meeting [1], we had many discussions via email and many agreements were reached for 5G V2X with NR-SL WID. There are a few remaining issues to make RAN1 specifications completed. In this contribution, we share our views on SL physical layer procedure for 5G V2X with NR-SL.

- Discussions

  - HARQ feedback

    - PSFCH occasion alignment in a resource pool

38.213 ## 16.3 UE procedure for reporting HARQ-ACK on sidelink […] A UE can be provided, by periodPSFCHresource, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled. […] If a UE receives a PSSCH in a resource pool and a ZYX field in a SCI format 0_2 scheduling the PSSCH reception indicates to the UE to report HARQ-ACK information for the PSSCH reception [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception. […]

As the above description from specification, a PSFCH transmission occasion is set per periodPSFCHresource slots. HARQ-ACK corresponding to a PSSCH reception is transmitted on a PSFCH at a first PSFCH occasion after MinTimeGapPSFCH slots from the PSSCH reception, in the resource pool. Time-domain resource of PSFCH slot is determined based on (pre-)configuration. Once PSFCH occasion is determined, there is no ambiguity among UEs, of PSSCH-PSFCH association in a resource pool.

However, to determine PSFCH occasion, parameters periodPSFCHresource and MinTimeGapPSFCH are not sufficient. The following two aspects are unclear in the current specification.

- Which slot is the initial PSFCH slot in a period of a resource pool?

For periodic feature, e.g. configured grant, two parameters are necessary: 1) starting point (offset) and 2) periodicity. For PSFCH occasion, 1) starting point is used to determine the initial PSFCH slot in a period of a resource pool, and from the initial PSFCH slot, occasions occur at equal time-interval of 2) periodicity. periodPSFCHresource is the periodicity for PSFCH occasions, while the initial PSFCH slot in a period of a resource pool has not discussed yet. To fix PSFCH occasion in a period of a resource pool, the initial PSFCH slot should clearly be defined. Fig. 1 is the illustration of this issue. Note that PSFCH slot means slot with PSFCH occasion.

Fig. 1: PSFCH occasion ambiguity due to no definition of the initial PSFCH slot in a period of a resource pool.

The following options can be considered to address this issue. It is noted that, PSFCH occasion is determined based on logical slots, i.e. slots in a resource pool. The initial PSFCH slot should be defined based on logical slots as well.

Option A-1: 0-th slot in a period of a resource pool is the initial PSFCH slot.

Option A-2: (periodPSFCHresource - 1)-th slot in a period of a resource pool is the initial PSFCH slot.

Option A-3: MinTimeGapPSFCH-th slot in a period of a resource pool is the initial PSFCH slot.

We slightly prefer option A-1 and option A-2. Firstly, system works by using any option. Secondly, in option A-3, the rule is a little strange when periodPSFCHresource < MinTimeGapPSFCH. Between option A-1 and option A-2, there seems to be no difference from technical perspective. For better structure, option A-2 is slightly preferred to option A-1. It is noted that, this proposal may be applied to the first period of a resource pool (e.g. resource pool in DFN0), which is dependent on the following discussion B.

Fig. 2: Initial PSFCH occasion in a period of a resource pool
(periodPSFCHresource = 4, MinTimeGapPSFCH = 2).

Observation 1:

- For PSFCH occasions, there is an ambiguity due to no definition of the initial PSFCH slot in a period of a resource pool while the periodicity is defined as higher layer parameter periodPSFCHresource.

Proposal 1:

- (periodPSFCHresource - 1)-th slot in a (or the first) period of a resource pool is the initial PSFCH slot in the period.

- How to handle PSFCH slot across resource pool periods?

PSFCH occasions in a period of a resource pool is fixed by agreeing the above proposal. However, further discussion is necessary to clarify PSFCH slot determination across periods of a resource pool. For example, let us assume that the number of slots in a period of a resource pool is not a multiple of periodPSFCHresource N as illustrated in Fig. 3. In this case, the following options can be discussed:

Option B-1: The initial PSFCH slot in a period of a resource pool is the same among periods.

- PSSCH resources at the last few slots after the last PSFCH occasion in a period of a resource pool is NOT associated with any PSFCH slot.

- PSSCH resources at the last few slots after the last PSFCH occasion in a period of a resource pool is associated with the first PSFCH slot in the next period. More PSFCH resources are prepared in the PSFCH occasion to ensure PSSCH-PSFCH associations.

Option B-2: The initial PSFCH slot in the next period of a resource pool is determined based on the current period.

Option B-3: No. of slots in a period of a resource pool shall be a multiple of periodPSFCHresource N.

Based on the above options, our preferred option is either option B-1-a or option B-3.

- In option B-1-a, the last few slots of each resource pool do not have corresponding PSFCH resources. ‘Smart UE’ can use the resources for e.g. bling retransmissions, broadcast transmissions. ‘Unsmart’ UE can set the resources to out of resource selection window.

- In option B-1-b, more RAN1 works are necessary to handle associations between PSCCH/PSSCH and PSFCH when PSFCH resource amount are not sufficient. Further RAN1 work is not desirable.

- In option B-2, if this determination is applied, the initial PSFCH slot in a period could be different between period #p and period #p+1 as Fig. 3. UEs have misalignment for PSFCH occasions unless UEs know the period index of the resource pool.

- In option B-3, associations between PSCCH/PSSCH and PSFCH are well-structured while this is a restriction of (pre-)configuration. Spec and UE implementation can be easy.

Fig. 3: PSFCH occasion ambiguity across periods of a resource pool.

Observation 2:

- PSFCH occasion across periods of a resource pool is unclear in the current specification/agreements.

Proposal 2:

- Either of the following is supported:

  - The initial PSFCH slot in a period of a resource pool is the same among periods. PSSCH resources at the last few slots after the last PSFCH occasion in a period of a resource pool is not associated with any PSFCH slot.

  - No. of slots in a period of a resource pool shall be a multiple of periodPSFCHresource N.

    - HARQ feedback indication in SCI

Agreements: One SCI format (referred to as 2nd SCI format A) is defined as follows: - This format includes Zone ID and Communication range requirement. - This format is used when the following HARQ operations are in use - HARQ-ACK information includes only NACK - FFS: No HARQ feedback Agreements: One SCI format (referred to as 2nd SCI format B) is defined as follows: - This format does not include Zone ID or Communication range requirement. - This format is used when the following HARQ operations are in use - No HARQ feedback - HARQ-ACK information includes ACK or NACK - FFS: how to determine M_ID in the equation for the PSFCH resource index - Option 1: Based on L1 ID(s) - Option 2: An explicit indication in SCI - FFS: HARQ-ACK information includes only NACK Agreements:: Down-select one out of the following for the indication of HARQ feedback enable/disable: - Option 1: This indication is conveyed in the 1st SCI. - Option 2: This indication is conveyed in the 2nd SCI. - Option 2-1: This indication is present both in 2nd SCI format A and B. - Option 2-2: This indication is present in 2nd SCI format B but not in 2nd SCI format A.

For HARQ feedback indication in SCI, there are three remaining issues.

- Whether 2nd-stage SCI format A (SCI format 2-B in spec.) supports no HARQ feedback or not

This SCI format includes Zone ID field and Communication range requirement field. These fields are 16 bits in total. The payload size of SCI format 2-B is larger than that of SCI format 2-A. Therefore, UE that intends blind retransmissions or broadcast transmissions can/should use SCI format 2-A, which provides better decoding performance. We do not see motivation to support no HARQ feedback in this SCI format.

- Whether 2nd-stage SCI format B (SCI format 2-A in spec.) supports groupcast option 1 or not

This SCI format does not include Zone ID field and Communication range requirement field. Groupcast option 1 is beneficial only when Zone ID and Communication range requirement are available. We do not see motivation to support groupcast option 1 in this SCI format.

- Which stage indicates HARQ feedback enable/disable

We believe that 2nd-stage SCI should indicate HARQ feedback enable/disable. We do not see strong motivation to use 1st-stage SCI. One possibility is, that ACK reception means the reserved resource is released. RAN1 had relative agreements in previous meeting. However, details have not been discussed and how to handle will be up to UE implementation. In addition, UE that supports the MCS table used for the PSSCH transmission can decode 2nd-stage SCI. The release mechanism will work in many cases. 1st-stage SCI with the indication has little advantage while decoding performance of 1st-stage SCI is degraded. Note that in SCI format 2-B, HARQ feedback is always enabled, i.e. no additional field is needed for the indication.

Observation 3:

- No HARQ feedback with SCI format 2-B (w/ zone ID and communication range req.) has no motivation.

- Groupcast option 1 with SCI format 2-A (w/o zone ID and communication range req.) has no motivation.

- Indication of HARQ feedback enable/disable in the 1st SCI has little motivation while 1st-stage SCI performance is degraded.

Proposal 3:

- SCI format 2-B is used with HARQ feedback enable and with groupcast option 1.

- SCI format 2-A indicates HARQ feedback enable/disable, where groupcast option 1 is not allowed.

    - Group/sequence hopping for PSFCH

Note that, this section is the same as one in our contribution for structure AI [2] since which AI is more appropriate for this discussion is unclear for us.

Regarding PSFCH sequence generation, the current specification describes the mechanism as follows.

38.211 #### 8.3.4.2 PSFCH format 0 ##### 8.3.4.2.1 Sequence generation The sequence xn shall be generated according to xn=ru,vα,δn n=0,1,…,NscRB-1 where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions: - mcs is given by clause 16.3 of [5, TS 38.213]; - m0 is given by clause 16.3 of [5, TS 38.213]; - l is the OFDM symbol number in the PSFCH transmission where l=0 corresponds to the first OFDM symbol of the PSFCH transmission; - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213] #### 6.3.2.2 Sequence and cyclic shift hopping PUCCH formats 0, 1, 3, and 4 use sequences ru,vα,δ(n) given by clause 5.2.2 with δ=0 where the sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α depends on the cyclic shift hopping in clause 6.3.2.2.2. ##### 6.3.2.2.1 Group and sequence hopping The sequence group u=fgh+fss mod 30 and the sequence number v within the group depends on the higher-layer parameter pucch-GroupHopping: - if pucch-GroupHopping equals 'neither' where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell. - if pucch-GroupHopping equals 'enable' where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell. - if pucch-GroupHopping equals 'disable' where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with cinit=25nID30+nID mod 30 where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell. The frequency hopping index nhop=0 if intra-slot frequency hopping is disabled by the higher-layer parameter intraSlotFrequencyHopping. If frequency hopping is enabled by the higher-layer parameter intraSlotFrequencyHopping, for the first hop and for the second hop. 38.331 SL-PSFCH-Config-r16 ::= SEQUENCE { sl-PSFCH-Period-r16 ENUMERATED {sl0, sl1, sl2, sl4} OPTIONAL, -- Need M sl-PSFCH-RB-Set-r16 BIT STRING (SIZE (275)) OPTIONAL, -- Need M sl-NumMuxCS-Pair-r16 ENUMERATED {n1, n2, n3, n4, n6} OPTIONAL, -- Need M sl-MinTimeGapPSFCH-r16 ENUMERATED {sl2, sl3} OPTIONAL, -- Need M sl-PSFCH-HopID-r16 INTEGER (0..1023) OPTIONAL, -- Need M ... }

At the previous e-meeting, definitions of m0 and mCS were agreed. That is, cyclic shift α for PSFCH sequence ru,vα,δ(n) is clearly defined. On the other hand, sequence group u and sequence number v seems not to be described for PSFCH. Further discussion and conclusion are necessary to have the same understanding of u and v among UEs.

Firstly, one higher layer parameter, sl-PSFCH-HopID-r16, is included in the current 38.331 to determine hopping ID of PSFCH sequence, based on the previous agreements. However, how to use this parameter is not explained in the current 38.211. This parameter is applied instead of hoppingId for the PUCCH sequence generation. Section 8.3.4.2.1 should mention the replacement as an additional ‘exception’.

Secondly, the PUCCH sequence generation uses one higher layer parameter, pucch-GroupHopping, to determine whether or not group hopping or sequence hopping is applied. Sequence group u and sequence number v are dependent on the parameter. Meanwhile, there is no higher layer parameter in SL-PSFCH-Config-r16 to replace pucch-GroupHopping with, and there is no clear definition of group hopping/sequence hopping for PSFCH sequence. It is noted that, three types of the hopping mechanisms: pucch-GroupHopping = neither, enable, disable.

- neither: Any hopping is not applied. The fixed u based on sl-PSFCH-HopID-r16 is used. v=0.

- enable: Group hopping is applied. u is determined based on sl-PSFCH-HopID-r16 and ns,fμ. v=0.

- disable: This is the same as neither case. v is not used for sequence of length less than 36.

Among these options, we prefer to apply neither always. Any additional higher layer parameter to replace pucch-GroupHopping is not needed for SL. Performance gain of group hopping has not been identified in this WI, and easier option (i.e. the fixed u) is preferable in CR phase.

Observation 4:

- Sequence group u and sequence number v for PSFCH sequence are unclear.

  - Performance gain of sequence/group hopping has not identified so far.

Proposal 4:

- Clarify that sl-PSFCH-HopID-r16 is used instead of hoppingId for PUCCH.

- PSFCH sequence is generated as pucch-GroupHopping = neither.

- Apply the following TP for TS 38.211.

#### 8.3.4.2 PSFCH format 0 ##### 8.3.4.2.1 Sequence generation The sequence xn shall be generated according to xn=ru,vα,δn n=0,1,…,NscRB-1 where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions: - mcs is given by clause 16.3 of [5, TS 38.213]; - m0 is given by clause 16.3 of [5, TS 38.213]; - l is the OFDM symbol number in the PSFCH transmission where l=0 corresponds to the first OFDM symbol of the PSFCH transmission; - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213]; - hoppingId is replaced with higher layer parameter sl-PSFCH-HopID-r16; - pucch-GroupHopping is assumed to be neither regardless of provided higher layer parameter pucch-GroupHopping if any

  - Channel collision

    - SL/UL prioritization

Agreements: For prioritization between PSFCH and UL TX, - The priority of PSFCH TX is the highest priority of the associated PSCCH/PSSCH - When the overlapping UL TX other than PUCCH carrying SL HARQ reporting, - when UL TX is associated with a DCI indicating “high” in “priority field” or configured with “high priority” by higher layers (i.e., URLLC case) - If SL-threshold for URLLC case is configured, LTE rule is used (i.e., UL TX is down-prioritized if the priority value of SL-TX is smaller than SL-threshold, otherwise prioritized) - Otherwise, UL TX is prioritized - Otherwise, LTE rule is used with another SL-threshold configured for non-URLLC case - Additionally, PRACH and PUSCH scheduled by RAR UL grant are always prioritized. Agreements: For prioritization between S-SSB and UL TX, - The priority of S-SSB is equal to the (pre-)configured priority introduced for in-device coexistence. - when UL TX is associated with a DCI indicating “high” in “priority field” or configured with “high priority” by higher layers (i.e., URLLC case) - If SL-threshold for URLLC case is configured, LTE rule is used (i.e., UL TX is down-prioritized if the priority value of SL-TX is smaller than SL-threshold, otherwise prioritized) - Otherwise, UL TX is prioritized - Otherwise, LTE rule is used with another SL-threshold configured for non-URLLC case - Additionally, PRACH and PUSCH scheduled by RAR UL grant are always prioritized. Agreements: - When PUCCH carrying SL HARQ reporting overlaps with SL TX, - The one with a higher priority is transmitted. - The priority of PUCCH carrying SL HARQ reporting is the highest priority of the associated PSFCH Agreements: - (Working assumption) For handling the case where more than one SL and UL transmissions overlap, adopt the following principle - For more than one SL transmissions overlapping with a UL transmission, the highest priority of SL transmissions is used for the prioritization. - For more than one UL transmissions overlapping with a SL transmission, the highest priority of UL transmissions is used for the prioritization. - FFS details

At the last meeting, many agreements on SL/UL prioritization were reached. Meanwhile, there is a remaining issue; FFS of the last agreements. Details of SL/UL collision handling are not concluded. Note that in NR Rel-15, many UL overlapping issues were/are identified. Some of them are solved but others are unsolved, which are conclusions in RAN1. 5G V2X WI should avoid this unsolved situation as much as possible.

- Highest priority determination

For more than one UL transmissions overlapping a SL transmission, the highest priority of UL transmissions is used for the prioritization, based on the agreements. However, ‘the highest priority’ is unclear. Priority of SR transmission is higher layer priority of the associated UL SCH, which is defined with several levels. Meanwhile, priority of DL HARQ-ACK transmission is PHY layer priority, which is just high or low. For example, the UL transmissions are SR transmission with priority value = 4 and DL HARQ-ACK transmission with high priority, which priority is higher? In other words, which prioritization rule should be used is not defined. This issue is not only multiple channels but also UL channel with information of multiple types, e.g. PUCCH with DL HARQ-ACK and SR.

One possible solution for the issue is that association with higher layer priority and PHY layer priority ONLY for this purpose. For example, PHY layer priority = 1 means higher layer priority = 1, and PHY layer priority = 0 means higher layer priority = 5. The highest priority can be determined by this association, and SL/UL prioritization is applied based on the highest priority. Prioritization mechanism can be that associated with the UL channel with the highest priority.

Observation 5:

- ‘Highest priority’ among UL transmissions overlapping a SL transmission is unclear since higher layer priority and PHY layer priority are defined with different granularities.

- Priority of UL transmission with multiple information types that is overlapping a SL transmission is unclear for the same reason.

Proposal 5:

- Define associations between PHY layer priority and higher layer priority ONLY for the purpose to determine UL channel priority to be used for SL/UL prioritization..

- Processing order

In NR-Uu, multiple UL channels could be overlapped each other. When overlapping happened, UL multiplexing/cancellation is applied based on priority/channel/UCI-type/etc. This is rule a.

When a SL channel is overlapped with a UL channel, prioritization is applied as the above agreements. Channel with lower priority is dropped or transmitted with reduced power. This is rule b.

Each rule is defined in the spec., while the above two situations could occur at the same time. In this case, which rule is done firstly? The processing order should be clarified. The following options can be considered:

- Option B-1: Firstly, rule a is applied. Then, rule b.

Let us use two examples as illustrated in Fig. 4. In the 1st case, one SL channel is overlapped with two PUCCHs that are overlapped each other. This option handles the PUCCH overlapping firstly. Now the new PUCCH is generated to multiplex the HARQ-ACK and the SR. The PUCCH is overlapped with the SL channel, then this option apply SL/UL prioritization by using sl-PriorityThresholdULURLLC (this is dependent on discussion A in this section). Transmit power of the SL channel is reduced.

In the 2nd case, one SL channel is overlapped with one PUCCH that is overlapped with another PUCCH. After PUCCH collision handling, the new PUCCH is overlapped with the SL channel. Transmit power of the SL channel is reduced due to SL/UL prioritization.

Therefore, we believe that option B-1 works well in both cases.

- Option B-2: Firstly, rule b is applied. Then, rule a.

In the 1st case of the Fig 4, this option handles SL/UL prioritization firstly. Transmit power of the SL channel is reduced based on the priorities. After that, PUCCH multiplexing is applied. The result is the same as option B-1.

In the 2nd case of the Fig. 4, transmit power of the PUCCH with SR is reduced due to SL/UL prioritization. Then, PUCCH multiplexing is applied. New PUCCH resource contains the HARQ-ACK and the SR. After the processing, we can see that the new UL channel is overlapped with the SL channel. Here one question arises, transmit power of the PUCCH should be reduced based on the 1st process? However, the priority of the HARQ-ACK is higher and the priority value of the SL channel is not over sl-PriorityThresholdULURLLC. The result is a bit strange. Instead, transmit of the SL channel should be reduced by comparison between the priority and sl-PriorityThresholdULURLLC? It means that UE needs to apply SL/UL prioritization again. One additional step is introduced, which leads to high UE complexity.

Base on the above analysis, option B-1 is better mechanism in our understanding.

Example 1

Example 2

Fig. 4: Processing order of UL multiplexing/cancellation and SL/UL prioritization.

Observation 6:

- Processing order of UL multiplexing/cancellation and SL/UL prioritization is unclear in the current spec.

- If SL/UL prioritization is applied firstly and UL multiplexing/cancellation is done secondly, the result could require further process.

Proposal 6:

- When a UE has multiple UL channels that are overlapped each other, and the UE has SL channel that is overlapped with at least one of the UL channels,

  - The UE firstly processes UL multiplexing/cancellation and secondly does SL/UL prioritization.

- SL/UL prioritization group

For SL/UL prioritization, many overlapping patterns are possible. Channel group which SL/UL prioritization is applied to should be clarified; otherwise, the result of the SL/UL prioritization would be different among UEs. Especially, the different outcome among UEs is undesirable in the case that SL resource is allocated by gNB (i.e. RA mode 1). The group determination mechanism can be the same as that for PUCCH overlapping group. That is, any SL/UL channel which overlaps with at least one of the SL/UL channels in the group is included in the group. For example, Fig. 5 is treated as one group and SL/UL prioritization is applied to the group.

Fig. 5: Group for SL/UL prioritization.

Observation 7:

- Channel group to apply SL/UL prioritization is unclear in the current spec.

Proposal 7:

- SL/UL prioritization is applied to a group. Any SL/UL channel which overlaps with at least one of the SL/UL channels in the group is included in the group.

    - Collision handling between SL HARQ-ACK on PUCCH and Uu UCI on PUCCH/PUSCH

Conclusion: - No support of multiplexing of SL HARQ and Uu UCI on PUCCH or PUSCH in Rel-16 - Note: this reverts the agreements made during RAN1#98b email discussion

At the RAN1#99 meeting, the above conclusion was reached to avoid huge RAN1 work. However, UE behavior in collision case between a PUCCH with SL HARQ-ACK and another PUCCH/PUSCH with Uu UCI is not concluded yet. It is noted that, the current specification allows at least multiplexing of SL HARQ-ACK and CSI/SR on PUCCH/PUSCH. Clarification is necessary to avoid misalignment among companies. The following two options are possible.

- Option 1: Error case

- Option 2: Drop either

From better scheduling flexibility perspective, option 2 is our preference. Option 1 is too restrictive at least for URLLC-type traffic since collision avoidance by gNB scheduler is not valid solution to meet the latency requirement. Then, for the details of option 2, prioritization mechanism can be the same as that between PSFCH and UL TX. This mechanism can provide good flexibility. Which channel should be prioritized is dependent on services. In some case, Uu UCI is more important, while in other case, SL HARQ-ACK should be prioritized.

Observation 8:

- UE behavior in collision case between a PUCCH with SL HARQ-ACK and another PUCCH/PUSCH with Uu UCI is not concluded yet.

- It is too restrictive that the collision is treated as an error case.

Proposal 8:

- For collision between a PUCCH with SL HARQ-ACK and another PUCCH/PUSCH with Uu UCI

  - Either is dropped by using the same mechanism as that for collision between PSFCH and UL TX.

- Conclusion

In this contribution, we discussed the remaining issues on SL physical layer procedure. Observations/Proposals are summarized as following:

Observation 1:

- For PSFCH occasions, there is an ambiguity due to no definition of the initial PSFCH slot in a period of a resource pool while the periodicity is defined as higher layer parameter periodPSFCHresource.

Proposal 1:

- (periodPSFCHresource - 1)-th slot in a (or the first) period of a resource pool is the initial PSFCH slot in the period.

Observation 2:

- PSFCH occasion across periods of a resource pool is unclear in the current specification/agreements.

Proposal 2:

- Either of the following is supported:

  - The initial PSFCH slot in a period of a resource pool is the same among periods. PSSCH resources at the last few slots after the last PSFCH occasion in a period of a resource pool is not associated with any PSFCH slot.

  - No. of slots in a period of a resource pool shall be a multiple of periodPSFCHresource N.

Observation 3:

- No HARQ feedback with SCI format 2-B (w/ zone ID and communication range req.) has no motivation.

- Groupcast option 1 with SCI format 2-A (w/o zone ID and communication range req.) has no motivation.

- Indication of HARQ feedback enable/disable in the 1st SCI has little motivation while 1st-stage SCI performance is degraded.

Proposal 3:

- SCI format 2-B is used with HARQ feedback enable and with groupcast option 1.

- SCI format 2-A indicates HARQ feedback enable/disable, where groupcast option 1 is not allowed.

Observation 4:

- Sequence group u and sequence number v for PSFCH sequence are unclear.

  - Performance gain of sequence/group hopping has not identified so far.

Proposal 4:

- Clarify that sl-PSFCH-HopID-r16 is used instead of hoppingId for PUCCH.

- PSFCH sequence is generated as pucch-GroupHopping = neither.

- Apply the following TP for TS 38.211.

#### 8.3.4.2 PSFCH format 0 ##### 8.3.4.2.1 Sequence generation The sequence xn shall be generated according to xn=ru,vα,δn n=0,1,…,NscRB-1 where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions: - mcs is given by clause 16.3 of [5, TS 38.213]; - m0 is given by clause 16.3 of [5, TS 38.213]; - l is the OFDM symbol number in the PSFCH transmission where l=0 corresponds to the first OFDM symbol of the PSFCH transmission; - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213]; - hoppingId is replaced with higher layer parameter sl-PSFCH-HopID-r16; - pucch-GroupHopping is assumed to be neither regardless of provided higher layer parameter pucch-GroupHopping if any

Observation 5:

- ‘Highest priority’ among UL transmissions overlapping a SL transmission is unclear since higher layer priority and PHY layer priority are defined with different granularities.

- Priority of UL transmission with multiple information types that is overlapping a SL transmission is unclear for the same reason.

Proposal 5:

- Define associations between PHY layer priority and higher layer priority ONLY for the purpose to determine UL channel priority to be used for SL/UL prioritization..

Observation 6:

- Processing order of UL multiplexing/cancellation and SL/UL prioritization is unclear in the current spec.

- If SL/UL prioritization is applied firstly and UL multiplexing/cancellation is done secondly, the result could require further process.

Proposal 6:

- When a UE has multiple UL channels that are overlapped each other, and the UE has SL channel that is overlapped with at least one of the UL channels,

  - The UE firstly processes UL multiplexing/cancellation and secondly does SL/UL prioritization.

Observation 7:

- Channel group to apply SL/UL prioritization is unclear in the current spec.

Proposal 7:

- SL/UL prioritization is applied to a group. Any SL/UL channel which overlaps with at least one of the SL/UL channels in the group is included in the group.

Observation 8:

- UE behavior in collision case between a PUCCH with SL HARQ-ACK and another PUCCH/PUSCH with Uu UCI is not concluded yet.

- It is too restrictive that the collision is treated as an error case.

Proposal 8:

- For collision between a PUCCH with SL HARQ-ACK and another PUCCH/PUSCH with Uu UCI

  - Either is dropped by using the same mechanism as that for collision between PSFCH and UL TX.

References

- 3GPP RAN1#100bis-e meeting,	chairman’s notes

- R1-2004383	Remaining issues on sidelink physical layer structure	NTT DOCOMO, INC.