3GPP TSG RAN WG1 #101e 	R1- 2004460

May 25th – June 5th, 2020

Agenda item:	7.2.5.5

Source: 	Qualcomm

Title: 	Remaining issues on uplink inter-UE Tx multiplexing and prioritization

Document for:	Discussion and Decision

# 1	Introduction

In this document, the proposals for the following topics are presented. Each corresponding section also includes a reason for the proposed addition or modification.

| Section 2 | Earlier cancellation relaxation for ULCI |
| --- | --- |
| Section 3 | Transmissions on the cancelled symbols, multiple cancellations and simultaneous reception of scheduling DCI and ULCI |
| Section 4 | 2-D bit-map for ULCI |

End

# 2	Earlier cancellation relaxation for ULCI

As has been discussed for cancellation under intra-UE prioritization, enforcing an exact cancellation time incurs significant implementation complexity for the UE. Instead, if an uplink transmission needs to be cancelled, the UE should be allowed to start the cancellation even before the first indicated symbol.

Proposal: If a UE is indicated by DCI 2_4 to cancel a PUSCH transmission, the UE can cancel the transmission before the first indicated symbol group. In other words, the first indicated symbol using DCI format 2_4 for cancellation is the latest time that the UE should start the cancellation of PUSCH.

# 3	Transmissions on the cancelled symbols, multiple cancellations and simultaneous reception of scheduling DCI and ULCI

For handling collisions in the uplink, it was agreed that the UE is not expected to be scheduled with another uplink transmission over the symbols that were not overlapping but cancelled according to the following agreement from RAN1 #99:

Agreement:

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

- The UE is not expected to be scheduled to transmit in the non-overlapping canceled symbols

The same scheduling behavior is expected when an uplink channel is cancelled due to the reception of ULCI.

Further, in case the UE cancels a low priority channel due to a collision with a high priority channel, the UE should not expect the high priority channel to be cancelled due to ULCI. Finally, the UE should not be expected to receive a scheduling DCI and the ULCI to cancel the same scheduled channel at the same time.

To capture these exceptions, we propose the following addtions as follows:

Modified clause (Section 11.2A of TS 38.213)

An indication by a DCI format 2_4 for a serving cell is applicable to PUSCH or SRS transmissions on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell.

A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission, or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, or an SRS transmission on the serving cell if, respectively,

-	a group of symbols, from the TCI symbols, has a corresponding bit value of '1' in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and

-	a group of PRBs, from the BCI PRBs, has a corresponding bit value of '1' in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission,

where

-	the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that are in one or more groups of symbols having corresponding bit values of '1' in the DCI format 2_4;

-	the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of '1' in the DCI format 2_4.

The UE is not expected to be scheduled by a PDCCH with another PUSCH/PUCCH or SRS where the transmission of the PUSCH/PUCCH/SRS starts before the end of the time domain allocation of the cancelled PUSCH or overlaps with the cancelled symbol(s) of SRS.

The UE is not expected to detect a DCI format scheduling a PUSCH/SRS and a DCI format 2_4 that indicates the cancellation of the same PUSCH/SRS in the same monitoring occasion of a search space set or overlapping monitoring occasions of different search space sets.

If a UE reports the capability of [intra-UE prioritization], and a PUSCH/PUCCH/SRS is cancelled by a PUSCH/PUCCH of a larger priority index, the UE is not expected to detect a DCI format 2_4 cancelling the transmission of the prioritized PUSCH/PUCCH.

End

# 4	2D bitmap for ULCI

In RAN1 #99, it was agreed that the time domain granularity for ULCI is defined as number of partitions within the time region denoted by GCI, and that the number of symbols indicated by ULCI, denoted by TCI, shall exclude cell-specific semi-static downlink symbols. However, since the ULCI may need to be monitored with small periodicity, it may happen that the number of symbols covered by the ULCI after removing the semi-static DL symbols is smaller than GCI. In this case, the current 2D bitmap method will resolve in unnecessarily wasted bits. Instead, these bits could be reused to indicate a finer resolution in the frequency domain.

Consider for example, GCI=7, NCI=14 and TCI=5. The 2D mapping rule in the current spec will have 2 frequency-region in each symbol group, and 4 bits will be wasted. Instead, we could reuse the additional four bits to achieve a finer granularity in the indicate 3 frequency domain on the first four symbol groups, as shown in the following figure.

Fig 1. 2D big map for ULCI

Modified clause (Section 11.2A of TS 38.213)

For a serving cell having an associated field in DCI format 2_4, for the field denote by

-	NCI a number of bits provided by CI-PayloadSize

-	BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion

-	TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, provided by timeDurationforCI in timeFrequencyRegion

-	GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion

If GCI≤TCI,

- GCI sets of bits from the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols.

- For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs.

If GCI>TCI,

- TCI sets of bits from the NCI bits have a one-to-one mapping with the TCI symbols, where each of the first NCI-NCITCI⋅TBI  set of bits includes NBI=NCITCI bits, and each of the remaining TCI-NCI+NCITCI⋅TBI   sets include NBI=NCITCI bits

- For a symbol, the corresponding set of bits has a one-to-one mapping with NBI RB groups (RBGs) of PRBs, where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs.

A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

End