3GPP TSG RAN WG1 #100-e	R1-2004462

May 25th – June 5th, 2020

Agenda item:	7.2.6.1

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on CSI enhancement for MU-MIMO support

Document for:	Discussion/Decision

# Introduction

In this contribution, we raise the issues of confusing UE behavior in CSI measurement and propose solutions to resolve these confusing issues. We also discuss remaining issues in UE feature list.

# Size of intermediate set when N3>19

When the total number of PMI subbands is greater than 19, i.e., N3>19, a two-step FD bases selection was agreed in RAN1 #97 [1]. The first step is to select an intermediate set which is common to all layers, the size of the intermediate set is agreed as follows:

- For N3>19, IntS is window-based and fully parameterized with Minitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …,

  - The value  where  is higher-layer configured from two possible values

In RAN1 #98, it was further agreed that α is fixed to 2, but the M value above is yet to be defined. There might be two understandings:

- Alt1: the M value above is fixed to the value in low-rank (i.e., M1 and M2)

- Alt2: the M value above depends on the reported rank (i.e., Mν).

The current spec [2, 3] follow Alt2, but it may result in a smaller window size for high rank than the low rank.

- For instance, for rank-2, we have M2=10 and N3'=20 for rank-2, it means that total 20 FD bases selected across layers should fit in a window with size N3'=20. However, for rank-4, we may have M4=5 and N3'=10, there are still total 20 FD bases selected across layers, but window size N3'=10 seems too small.

In our view, considering that the total number of FD bases are comparable for low rank and high rank, and the FD bases selection may vary across different layers, so Alt1 is preferred as it yields a constant window size for both low and high rank.

Another benefit of rank-common intermediate set size is lower implementation complexity. With Alt2, the UE has to recalculate the intermediate set to perform FD bases selection for RI={3,4}, such operation is unnecessary and may lead to performance loss compared to Alt1 as the size of intermediate set resulted by Alt2 is smaller than the size of intermediate set resulted by Alt1.

Based on the discussion, we observe and propose.

Proposal 1: When N3>19, the size of the intermediate set is give by N3'=2M1 for RI={1,2,3,4}, where M1 is the number of FD bases selected for RI={1,2}.

The corresponding text proposal is as follows

---------------------------------------- Start of text proposal to Section 5.2.2.2.5 in TS 38.214 ---------------------------------------

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, are identified by Minitial (for N3>19) and n3,l l=1,…, υ where

Minitial∈-2M1υ+1,-2M1υ+2,…,0

n3,l=n3,l(0),…,n3,l(Mυ-1)

n3,lf∈0,1,…,N3-1

which are indicated by means of the indices i1,5 (for N3>19) and i1,6,l l=1,…, υ, where

i1,5∈0,1,…,2M1υ-1

i1,6,l∈0,1,…,N3-1Mυ-1-1N3≤190,1,…,2M1υ-1Mυ-1-1N3>19.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

For all values of N3, n3,l(0)=0 for l=1,…,υ. The nonzero elements of n3,l, identified by  n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ), for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using  Cx,y as defined in 5.2.2.2.3 and the algorithm:

s0=0

for f=1,…,Mυ-1

Find the largest x*∈Mυ-1-f,…,N3-1-f in Table 5.2.2.2.5-4 such that

i1,6,l -sf-1≥Cx*,Mυ-f

ef=Cx*,Mυ-f

sf=sf-1+ef

if N3≤19

n3,l(f)=N3-1-x*

else

nl(f)=2M1υ-1-x*

if nl(f)≤Minitial+2M1υ-1

n3,l(f)=nl(f)

else

n3,l(f)=nl(f)+(N3-2M1υ)

end if

end if

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

When n3,l and Minitial are known, i1,5 and i1,6,l (l=1,…,υ) are found as follows:

-	If  N3≤19, i1,5=0 and is not reported. i1,6,l=f=1Mυ-1CN3-1-n3,lf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4 and where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases.

-	If  N3>19, Minitial is indicated by i1,5, which is reported and given by

i1,5=MinitialMinitial=0 Minitial+2M1υMinitial<0

Only the nonzero indices n3,l(f)∈IntS, where IntS={Minitial+i mod N3, i=0,1,…,2M1υ-1}, are reported, where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases. Let

nl(f)=n3,lf n3,lf≤Minitial+2M1υ-1 n3,lf-(N3-2Mυ1)n3,lf>Minitial+N3- 1,

then i1,6,l=f=1Mυ-1C2Mυ1-1-nlf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-------------------------------------------------------------- End of text proposal --------------------------------------------------------------

---------------------------------------- Start of text proposal to Section 6.3.2.1.2 in TS 38.212 ------------------------------------------

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Table 6.3.2.1.2-1A: PMI of codebookType= typeII-r16

|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i2,3,1 | i2,3,2 | i2,3,3 | i2,3,4 | i1,5 | i1,6,1 | i1,6,2 | i1,6,3 | i1,6,4 | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3>19 | 4 | N/A | N/A | N/A | log2(2M1) | log22M1-1M1-1 | N/A | N/A | N/A | 3(KNZ,TOT-1) | 4(KNZ,TOT-1) | 2LM1 |
| Rank=2 N3>19 | 4 | 4 | N/A | N/A | log2(2M12) | log22M12-1M2-1 | log22M12-1M2-1 | N/A | N/A | 3(KNZ,TOT-2) | 4(KNZ,TOT-2) | 4LM2 |
| Rank=3 N3>19 | 4 | 4 | 4 | N/A | log2(2M13) | log22M13-1M3-1 | log22M13-1M3-1 | log22M13-1M3-1 | N/A | 3(KNZ,TOT-3) | 4(KNZ,TOT-3) | 6LM3 |
| Rank=4 N3>19 | 4 | 4 | 4 | 4 | log2(2M14) | log22M14-1M4-1 | log22M14-1M4-1 | log22M14-1M4-1 | log22M14-1M4-1 | 3(KNZ,TOT-4) | 4(KNZ,TOT-4) | 8LM4 |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Table 6.3.2.1.2-2A: PMI of codebookType= typeII-PortSelection-r16

|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i2,3,1 | i2,3,2 | i2,3,3 | i2,3,4 | i1,5 | i1,6,1 | i1,6,2 | i1,6,3 | i1,6,4 | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3>19 | 4 | N/A | N/A | N/A | log2(2M1) | log22M1-1M1-1 | N/A | N/A | N/A | 3(KNZ,TOT-1) | 4(KNZ,TOT-1) | 2LM1 |
| Rank=2 N3>19 | 4 | 4 | N/A | N/A | log2(2M12) | log22M12-1M2-1 | log22M12-1M2-1 | N/A | N/A | 3(KNZ,TOT-2) | 4(KNZ,TOT-2) | 4LM2 |
| Rank=3 N3>19 | 4 | 4 | 4 | N/A | log2(2M13) | log22M13-1M3-1 | log22M13-1M3-1 | log22M31-1M3-1 | N/A | 3(KNZ,TOT-3) | 4(KNZ,TOT-3) | 6LM3 |
| Rank=4 N3>19 | 4 | 4 | 4 | 4 | log2(2M14) | log22M14-1M4-1 | log22M14-1M4-1 | log22M14-1M4-1 | log22M14-1M4-1 | 3(KNZ,TOT-4) | 4(KNZ,TOT-4) | 8LM4 |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-------------------------------------------------------------- End of text proposal --------------------------------------------------------------

# Parameter combination for small number of subbands

In eType II and eType II port-selection codebook, the codebook parameters (number of spatial bases, number of FD bases and number of non-zero coefficients) are configured by the higher-layer parameter paramCombination-r16. There are 8 combinations in total wherein combo indices 1-6 are mandatory and combo indices 7-8 with L=6 are optional.

| paramCombination-r16 | L | pυ |  | β |
| --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 2 | ¼ | 1/8 | ¼ |
| 2 | 2 | ¼ | 1/8 | ½ |
| 3 | 4 | ¼ | 1/8 | ¼ |
| 4 | 4 | ¼ | 1/8 | ½ |
| 5 | 4 | ¼ | ¼ | ¾ |
| 6 | 4 | ½ | ¼ | ½ |
| 7 | 6 | ¼ | - | ½ |
| 8 | 6 | ¼ | - | ¾ |

The number of FD bases is given by Mν=N3R×pν. Among the 8 combinations, we can see that when the number subbands (i.e., N3R) are very small, there would be only one FD basis. Then, in this case, the number of non-zero coefficients would be small than 2L as K0=2LM1×β<2L with M1=1. This observation implies that there would be some spatial beams with all zero coefficients. In other words, configuration of L=4 would be degraded to L=3 or L=2 if M1=1 and β=1/2 or β=3/4. Such degradation of L would be detrimental to system performance.

To solve the degradation of L, one way is to set β=1 for the scenario with M1=1. The scenario with M1=1 happens when N3R≤2 as the largest value of p1 is ½. Based on the discussion, we observe and propose

Observation 1: When M1=1, there is a degradation in terms of number of spatial beams, which is detrimental to system performance. M1=1 happens when N3R≤2.

Proposal 2: For eType II and eType II port-selection, support β=1 if N3R≤2.

The corresponding text proposal is as follows.

---------------------------------------- Start of text proposal to Section 5.2.2.2.5 in TS 38.214 ---------------------------------------

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

The values of L, β and pυ are determined by the higher layer parameter paramCombination-r16, where the mapping is given in Table 5.2.2.2.5-1.

-	The UE is not expected to be configured with paramCombination-r16 equal to

-	3, 4, 5, 6, 7, or 8 when PCSI-RS=4,

-	7 or 8 when PCSI-RS<32

-	7 or 8 when higher layer parameter typeII-RI-Restriction-r16 is configured with ri=1 for any i>1.

-	7 or 8 when R=2.

-	When N3R≤2, β is set to 1.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-------------------------------------------------------------- End of text proposal --------------------------------------------------------------

# Conclusion

In this contribution, we discuss the remaining UE capability issues related to Type II CSI enhancement. Based on the discussion, we observe

Observation 1: When M1=1, there is a degradation in terms of number of spatial beams, which is detrimental to system performance. M1=1 happens when N3R≤2.

And we propose

Proposal 1: When N3>19, the size of the intermediate set is give by N3'=2M1 for RI={1,2,3,4}, where M1 is the number of FD bases selected for RI={1,2}.

Proposal 2: For eType II and eType II port-selection, support β=1 if  N3R≤2.

We propose adopting text proposals in Sections 2 and 3.

# References

- Chairman’s notes RAN1#97 Final

- TS38.212 v16.0.0

- TS38.214 v16.0.0