3GPP TSG RAN WG1 Meeting #101			R1-2004476

e-Meeting, May 25th – June 5th , 2020

Agenda item:	7.2.11

Source: 	Qualcomm Incorporated

Title: 	Discussion on Two-Step RACH UE Features

Document for:	Discussion and Decision

- Introduction

In RAN1#100bis-e, the following agreements [1] have been made for NR Rel-16 2-step RACH:

Agreement:

- FG9-1 is kept as a single FG for basic support of Rel-16 2 step RACH.

  - Components and capability signaling impacts are discussed in [100b-e-NR-UEFeatures-2Step-02]

Agreement:

- FG9-3 is kept as a single FG for “Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA” with bracket.

- FG9-4 is kept as a single FG for “MsgA operation in a band combination including SUL” with bracket.

- A new FG for [PUSCH transmission with frequency hopping with non-zero guard period] is added with bracket.

- A new FG for [max number msgB to be monitored/decoded per slot] is added with bracket.

Email discussions about remaining UE features continued after RAN1#100bis-e meeting. In this contribution, we share our views on the remaining UE features for NR Rel-16 two-step RACH, based on the summary of email discussion after RAN1#100bis-e meeting [2].

- NR_2step_RACH

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - RACH type selection for CBRA according to SSB-based RSRP threshold - msgA PRACH resource configuration including separately configured ROs not applicable to 4-step RO configuration and fully or partially shared ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration - msgA PUSCH resource (DMRS included) and waveform determination for 2-step CBRA - Supporting up to two msgA PUSCH configurations in an UL BWP - Validation of MsgA PRACH and PUSCH - Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH - msgB monitoring and decoding for 2-step CBRA - (for UE in any RRC state) monitoring msgB PDCCH with CRC masked by msgB-RNTI in Type-1 CSS set, and decoding multi-cast msgB PDSCH carrying SuccessRAR, FallbackRAR and BI - (for RRC connected UE only) monitoring msgB PDCCH with CRC masked by C-RNTI in USS set, and decoding the unicast PDSCH carrying absolute TA MAC CE - PUCCH transmission for HARQ-ACK feedback to a msgB - Power control for msgA PRACH, msgA PUSCH and PUCCH carrying HARQ-ACK feedback to msgB |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-3] | [Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA] | [Parallel MsgA and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell] | 9-1 TBD | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-4] | [MsgA operation in a band combination including SUL] | [MsgA operations in a band combination including SUL] | 9-1, 6-16 TBD | Yes | N/A | UE does not support msgA operations in a band combination including SUL | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-6] | [up to X of msgBs per slot/within the msgB window] | [up to X of msgBs per slot/within the msgB window] | TBD | Yes | N/A |  | [Per band] | N/A | N/A | N/A |  | Optional with capability signalling |

- Qualcomm’s Views

# Our views on the remaining issues of UE feature can be summarized as follows:

- FG 9-1

- We are fine to remove components 3b and 3c.

- We think it is necessary to keep the rest of the components for clarity/accuracy of UE implementation.

- FG 9-3

- We think it should be kept, since msgA is associated with a new channel structure (i.e. PRACH+ TX Gap+ PUSCH) in NR Rel-16.

- FG 9-4

  - It is OK to keep this FG.

- FG 9-5

- We don’t think it is needed.

- FG 9-6

- The type of this FG should be per band.

- It is OK to be kept.

# References

- R1-2002860, “Summary on email discussion [100b-e-NR-UEFeatures-2Step-01]”, 3GPP TSG RAN1#100bis-E.

- R1-2003197, “Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR_2step_RACH.” 3GPP TSG RAN1#100bis-E.

- R1-1913598, “RAN1 Agreements for Rel-16 2-step RACH,” Reno, USA, November 2019.