3GPP TSG RAN WG1 #101 e-Meeting	R1- 2004488

e-Meeting May 25th – June 5th, 2020

Agenda item:	7.2.11.1	3

Source: 	Qualcomm

Title: 	Discussion on UE features for others

Document for:	Discussion and Decision

# 1	Introduction

An important high priority issue is to discuss the applicability of all Rel-15 and Rel-16 features to NR-U. For example, whether mTRP is supported in NR-U, etc. In order to avoid an extended debate, we suggest agreeing that all features should be applicable to NR-U as a default, and possibly discuss only some exception cases.

At the same time, this does give rise to IODT concern, since all the Rel-15 and Rel-16 features will not be introduced at the same time in licensed and unlicensed. Therefore, even though as default all features should apply to unlicensed, a UE capability differentiation is still needed.

In order to introduce the capability differentiation with the smallest possible change in the structure of the capability signaling, we propose to convert most per-UE capability to per-band capability. It would need further discussion how to do the same for Rel-15 features.

Proposal 1:  Agree that all features should be applicable to NR-U as default, and possibly discuss only some exception cases.

Proposal 2:  Convert most per-UE capability to per-band capability and discuss only exception cases. It would need further discussion how to do the same for Rel-15 features.

As an additional topic, we would like to clarify whether or not FR1 and FR2 serving cells can be in the same TAG in UL CA. This clarification may impact both Rel-15 and Rel-16.

Proposal 3:  Clarify that FR1 cells and FR2 cells cannot be in the same TAG.

Furthermore, we propose new FGs for Rel. 15 NR. The proposed changes in 5-11c, 5-12c, 5-13d and 5-13g are to allow for scheduling 3 TBs in both UL and DL. Considering the (4,3) span pattern of FG 3-5b, these additions would allow for matching the number of TBs and the spans in each slot.

In addition, the proposed FG 11-3a-e would allow for capability signalling for the simultaneous use of CBG-based UL transmission and minimum processing capability 2.

Proposal 4:  Add Rel-15 FGs to allow for scheduling 3 TBs in both UL and DL in order to match the number of control spans per slot

Proposal 5:  Add Rel-15 FGs for the simultaneous use of CBG-based UL transmission and UE processing time capability 2.

# 2         Proposed Feature Groups

| 5-11c | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 PDSCH(s) for Msg. 4 is included |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-12c | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 5-13d | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a or 5-5b | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |
| 5-13g | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |

| 11-3a | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-5a or 5-5b | Yes | N/A |  | FS | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11-3b | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13 | Yes | N/A |  | FS | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| 11-3c | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13a | Yes | N/A |  | FS | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| 11-3d | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13c | Yes | N/A |  | FS | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| 11-3e | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13d | Yes | N/A |  | FS | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |

# 3	Conclusion

In this contribution, the following proposals were made:

Proposal 1:  Agree that all features should be applicable to NR-U as default, and possibly discuss only some exception cases.

Proposal 2:  Convert most per-UE capability to per-band capability and discuss only exception cases. It would need further discussion how to do the same for Rel-15 features.

Proposal 3:  Clarify that FR1 cells and FR2 cells cannot be in the same TAG.

Proposal 4:  Add Rel-15 FGs to allow for scheduling 3 TBs in both UL and DL in order to match the number of control spans per slot

Proposal 5:  Add Rel-15 FGs for the simultaneous use of CBG-based UL transmission and UE processing time capability 2.

# References

- R1-2002687, “Discussion on UE features”, Qualcomm, 3GPP TSG RAN1#100bis-e

- R1-2002860, “Summary on email discussion [100b-e-NR-UEFeatures-2Step-01]”, 3GPP TSG RAN1#100bis-e