3GPP TSG RAN WG1 Meeting #101-e	                                                                          R1-2004489

e-Meeting, May 25th – June 5th, 2020

Agenda item:    8.1.2

Source:              Qualcomm Incorporated

Title:                  Channel access mechanism for NR in 52.6 to 71GHz band

Document for:  Discussion and Decision

# Introduction

One of the two main objectives for SID [1,2] on supporting NR from 52.6 GHz to 71 GHz is:

- Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam-based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1].

  - Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

In this paper, we discuss a few aspects for channel access mechanism for NR system for operation in the frequencies of interest –with primary view towards unlicensed operation – dubbed here as ‘FR2x Unlicensed’ operation.  The areas covered include:

Observations on characteristics of FR2x unlicensed operation

Possible channel access modes for deployment in FR2x unlicensed operation

- Aspects of sensing for channel access

- Proposal to keep the scope of studies to intra-technology coexistence

# Discussion

## Current regulatory considerations for coexistence with other technologies

ETSI BRAN is revisiting regulations for the extended 60GHz band. At present operating modes are identified with consideration of different deployment use cases, which place limits on EIRP and EIRP density for each operating mode. For channel access mechanisms, it is recommended that an ‘Adequate Spectrum Sharing Mechanism’ needs to be implemented [4]. It is recognized that the current specifications for channel access, e.g. [5] for mode c1 below, for timelines of monitoring requirements may need revisiting.  An effort to identify appropriate requirements on the mechanisms for fair coexistence is ongoing.

Table 1  ECC Recommendations for operating modes for 57-71 GHz frequency bands [4]

| Mode | Power / Magnetic Field | Use Case restrictions | Spectrum Sharing Requirement |
| --- | --- | --- | --- |
| c1 [5] | 40 dBm EIRP, 23 dBm/MHz EIRP density | No fixed outdoor installations | ‘Adequate Spectrum Sharing Mechanism shall be implemented’ [4] Note: Under current spec. c1, currently requires LBT mechanisms [5], while c3 does not require sensing and LBT mechanisms [6] |
| c2 | 40 dBm EIRP, 23 dBm/MHz EIRP density and max. transmit power of 27 dBm |  |  |
| c3 [6] | 55 dBm EIRP, 38 dBm/MHz EIRP density and transmit antenna gain ≥ 30 dBi | Only fixed outdoor installations |  |

## Characteristics of FR2x unlicensed operation and simulation study of uncoordinated sharing

Typical use case for FR2x unlicensed deployment are envisioned to involve highly beamformed transmissions from nodes under strict EIRP limits. Further, given the propagation characteristics in these bands, the interference, even under demanding traffic scenarios, is expected to have very different characteristics.  This is borne out in the following simulation study of uncoordinated sharing of the spectrum in the 60 GHz band in an Indoor Office deployment under full buffer traffic:

We compare the full buffer cell and UE DL throughput performance between (1) Configuration 1: shared but uncoordinated co-channel operation between nodes of two operators, and (2) Configuration 2: that when the available bandwidth is partitioned equally and each operator is allocated one part each (Figure 1).

Each configuration is simulated with 16x8=128 element antenna configuration of the gNB per polarization and 2x2=4 and 4x4=16 antenna configurations at the UEs.

Figure 1 Uncoordinated sharing of 2 GHz bandwidth vs Bandwidth Partitioning among two operators

Figure 2 Performance of uncoordinated sharing (‘2 Operator’, Configuration 2)  compared to bandwidth partitioning (‘1 Operator’, Configuration 1) for full buffer traffic.

As shown in Figure 2, Most users experience high SINR and high throughputs under shared bandwidth (Configuration 2). Following observation can be made for majority of the users in the deployment:

Observation 1: FR2x unlicensed deployments are likely to have a lower likelihood of encountering unmanaged interference, making spectrum sharing a natural choice. Even under dense uncoordinated deployment under heavy traffic, large majority of users experience very good SINR and long-term throughput performance.

Figure 2 also shows that a non-negligible tail of users suffers under shared bandwidth compared to partitioned bandwidth scenario. The loss can be as high at 35% reduction in throughput for 5th percentile of users, for the case of 4 antennas at the UE. These tail users under uncoordinated sharing suffer from lower SINR conditions. Their performance compared to hard bandwidth partitioning indicates that the main contributor for their drop in performance is sustained unmanageable interference coming from nodes of the other operator.  Although all interference can be argued to be bursty interference, that will eventually go away, the ‘sustained’ nature of the interference is a relative concept. For use cases of services such as Industrial IOT in 60GHz that have lower latency requirements, a few tens of milliseconds of interference can count as sustained and cause packet failures.  A further important consideration is the occurrence of low likelihood but catastrophic scenarios. For example, due to circumstances arising from unplanned placements and unfavorable line-of-sight conditions, a mobile device may experience high unavoidable interference from ongoing transmissions from another device such as a smart television.

Therefore, the following observation can be made for a certain small but important tail of user experience.

Observation 2:  The interference, though less likely, can be damaging when present.

Yet, broadly, higher SINRs, lower observed interferer densities and instantaneous high data rates due to high bandwidths, all point to a link operation in a regime where the SINR vs reuse trade-off tilts in favor of higher reuse.

## Channel access modes for deployment

Channel access procedures for unlicensed operation have relied on channel sensing (also variously referred to as medium sensing, carrier sensing, listening, monitoring) as a primary mechanism for ensuring reliability and fair access. The classical channel sensing takes the form of Listen Before Talk (LBT). Channel sensing adds to overall control overhead as a minimum overhead is added before every burst of transmission.  Further, the associated procedures may incur a variable medium access reuse loss, in return for improvements in networkwide metrics such as link SINR quality and fairness.

Observations made in the previous section point to deployment models for FR2x with channel access mechanisms that do not always need to pay an overhead/reuse-loss penalty in the form of silencing and sensing for every transmission. Instead following deployment modes can be envisioned.

No-Sensing or No-LBT deployment:  Based on Observation 1, this deployment mode is envisioned to permit a truly uncoordinated operation. The risks of running into phenomena of Observation 2 can be reduced/eliminated by potentially stricter requirements than some of those imposed by regulatory bodies. Under absolute constraints on EIRP, duty cycle and directionality of transmission, it is conceivable for nodes to be free from requirement of sensing before transmission.

Long Term Sensing based deployment: For nodes with requirement of higher EIRPs and duty cycle, adequate mechanisms could be identified that improves coexistence by sensing the channel over a slower timescale than individual bursts of transmissions.

Short Term Sensing based deployment: Short term sensing, often specified in the form of LBT channel access procedure before every transmission burst, may be specified while being cognizant of Observation 1. Observation 1 indicates that design focus of the study could be to keep the triggers, the overhead and the reuse loss costs associated with such sensing to be a minimum.  The impact in Observation 2 can further be limited by imposing constraints on duration, power, and other parameters for a subset of transmissions such as control information. It could be envisioned for such transmission bursts to be sent in a contention exempt manner.

- Proposal 1: Investigate and identify conditions for deployment modes where No-LBT or No Sensing is viable.

- Proposal 2: Investigate and identify conditions for deployment modes where a long-term sensing for coexistence is viable.

- Proposal 3: LBT or Short-Term Sensing:  Identify conditions for deployment modes where short term medium sensing before a transmission needs to be considered for every transmission.

- Proposal 4: Contention Exempt Transmissions: Investigate and identify conditions where some transmissions can be permitted in a contention exempt manner, i.e. a sensing medium is not a requirement before transmission.

## Sensing for Channel Access

The goal of channel sensing for medium access in a shared spectrum channel is two-fold:

Protect the on-going transmissions from being interfered by the intended transmission

Protect the intended transmission from being interfered by the on-going transmission

In sub7 unlicensed operation, the goal is mostly achieved by initiating device Cat 4 LBT and responding device Cat 2 LBT. However, in FR2x band, due to heavy beamforming required to close the link, we must consider the following characteristics that distinguish it from the sub7 unlicensed operation: (1) high directionality of transmissions and resulting spatial interference suppression at the transmitter: a transmitter – connected to multiple links -  may not always create same level of interference at a given location when it is occupying the medium (2) high directionality of receiver and resulting interference suppression:  a receiver may reject heavy interference from certain directions and may amplify weak interference from some other directions. Sensing for channel access in FR2x therefore, cannot be decoupled from directionality and associated aspects below.

### Omnidirectional vs directional sensing

Figure 3 The interference footprint of transmission from contending node N and the (omnidirectional)  sensing footprints at thresholds T1 and T2  for the sensing unit at node N.

Sensing based channel access procedures are implicitly based on an ‘interference-reciprocity’ assumption, i.e. a node expected to cause more interference should back-off/silence more. For example, the energy detection threshold to consider the medium busy, decreases with increasing transmit power or EIRP.  In case of highly beamformed transmissions and receptions in FR2x, the transmission may be able to provide high spatial suppression of interference in many directions, and the reception may provide high spatial interference rejection.   The transmit beamforming also increases the directivity of interference caused.   In the light of this, the relationship between a sensing operation and beamformed transmission at a node can be discussed in terms of its ‘interference footprint’ -which roughly identifies the locations affected by the intended transmission,  and its ‘sensing footprint’ – which identifies the set of locations that are sensed strongly. This is depicted in Figure 3. Just as the interference footprint is a function of EIRP together with beamforming pattern for transmission, the sensing footprint is a function of the thresholds applied and the directionality of the sensing unit. As indicated in Figure 3, an omnidirectional sensing footprint at any threshold may mismatch with the interference footprint of highly beamformed directional transmission. This can contribute to (1) interference e.g. the victim device in the interference footprint but not in sensing footprint under threshold  T1  (2) over-silencing e.g. the aggressor device not in the interference footprint but in the sensing footprint under threshold T2.   If the direction/transmit beamforming in which the contending node intends to occupy the medium is known prior to contention, then a sensing cognizant of this, i.e. a directional sensing, may align better with the interference reciprocity principle than omnidirectional sensing.

Directional sensing can also incentivize a good neighbour behaviour. For a given EIRP, a node using a wider beam has higher interference footprint than a node deploying higher beamforming gain and a narrower beam. An omnidirectional sensing based back-off treats both the nodes in the same way.

Proposal 5:  Study the use of directional sensing – sensing with beamforming - towards any channel access mechanism requiring channel monitoring.

### Sensing beam and Tx-beam mismatch

The previous section argued that a directional sensing mechanism can provide better match between the interference footprint and sensing footprint. On the other hand, it cannot be assumed, a priori, that the sensing footprint of the contending node will match the interference footprint. The sensing beam shape and gain may be different from the eventually used transmit beam’s gain and directivity. For example, this can prominently happen in the following use cases depicted in Figure 4:

A gNB contends and senses the medium using a sensing beam B0, and on winning the medium under the channel access procedure, transmits a group of SSBs, each beamformed by a separate beam Bi , that cannot necessarily  be assumed to be QCL with respect to each other. (Figure 4: left)

A gNB wins the medium using channel access procedure with a sensing beam B0, and goes to serve multiple users in the same COT, using separate non-QCL beams Bi. (Figure 4: right)

Figure 4 Sensing for channel access, for a known sequence of SSB transmissions (left) or for a dynamic sequence of data transmissions to different UEs (right).

If channel access procedures are to permit such use cases, it is important to identify and specify criteria under which a sensing beam B0, can be used for transmission using beam B1

Proposal 6:  Study the criteria under which the sensing beams can be different from the transmit beam at the transmitter.

### Tx sensing and Tx-Rx mismatch

Figure 5 Sensing a beamformed transmission.

Figure 6  Understanding Tx Sensing-Rx mismatch: The simulations scatterplots show measured energy (received power in omnidirectional sensing)  from interfering downlink transmissions measured at gNB-UE pairs in 60 GHz and 6 GHz environments respectively in an Indoor Office Scenario: [left: 60GHz/2.16GHz BW, 40dBm EIRP, 16x8 Antennas/Pol@gNB, right: 6GHz/20MHz BW, 23 dBm TxPower, 4 Antennas@gNB, ].A common energy threshold of θ= -78 dBm is used for depiction.

It is well recognized that the sensing done at a contending transmitter does not necessarily represent the eventual interference impact to the receiver. The sensing at transmitter may underestimate (leading to a hidden node problem) or overestimate (leading to an exposed node problem) the interference experienced by the receiver. This phenomenon is amplified under higher beamforming gain expected in FR2x. Figure 5 and Figure 6 depict this. In Figure 5, the ongoing transmission from gNB A can be sensed at contending gNB N at a widely different level than that at the intended receiver of the contending gNB, V.  Figure 6 shows  simulation scatter plots of the measured energies (received power in omnidirectional sensing) from an ongoing DL transmission, measured at contending gNBs and their UEs for two scenarios. The first, left figure, shows a 60GHz/2.16GHz BW Indoor Office scenario with 16x8 Antennas/Pol at the gNBs for transmission and a peak EIRP constraint of 40 dBm. The figure on the right is a sub7 scenario in the same layout, with 6GHz/20MHz BW, 4 antennas at the gNB for transmission and 23 dBm Transmit Power.  In general, the sensed received energy at the gNB from an interferer correlates poorly with the level of interference caused to the UE by the same interferer.  Further this Tx sensing -Rx  mismatch  is much more pronounced in the 60GHz deployments.  Such mismatch can reduce the effectiveness of Tx-sensing and back-off/silencing procedures as means to trade-off SINR for medium reuse.

A note on the common energy threshold in the above comparison: The figures used to highlight the Tx-sensing Rx mismatch use a common energy detection threshold of -78 dBm. In 60GHz, with 2.16GHz bandwidth, this threshold is not practical due to higher noise floor levels from much wider channel bandwidth. As apparent from Figure 6-left, at least in these simulation scenarios for 60GHz, a higher, more practical value of the energy detection threshold may never get crossed in the sensing units, indicating even high levels of sensing and Rx mismatch.

Observation 3:  High Tx Sensing-Rx Mismatch: The interference observed by sensing unit at the transmitter may be considerably different than that eventually observed by the receiver.

Proposal 7:  Study and design channel access procedures and sensing guidelines that consider the prevalence of Tx Sensing-Rx mismatch.

## Focus of simulation studies and simulation methodology

We propose that the focus of systems studies as part of the study item can safely be limited to intra-technology (NR-U-NR-U) coexistence. There are considerable differences in operating bandwidths and channelization requirements between NR-U and other technologies for 60GHz.  The relative abundance of spectrum in the FR2x unlicensed operation permits mitigation techniques for unmanaged interference such as channel selection (dynamic frequency selection) as the principal methods to combat inter-technology coexistence issues.   Further, it is expected that the intra-technology tools and procedures designed and identified as part of this study/work item, can later be adapted to work well with inter-technology coexistence aspects too.

Proposal 8:  Restrict the focus of the simulation studies for FR2x unlicensed bands to NR-U based deployments only, identifying tools for intra-technology coexistence.

Drawing from agreements in RAN1#92 for the NR-U study item, the following common scenarios can be used for system level evaluations for this study.  Given regulatory limits on EIRP, power spectral densities and the propagation characteristics, indoor scenarios are emphasized.

Layouts:

Indoor with 2 Operators: Indoor Hotspot topology with allocation of half of the gNBs to each operator: Some of the common assumptions for this set up are summarised in Table 2 in the appendix

Outdoor with 2 Operators: Dense Urban Micro topology can be considered as an additional scenario of interest

Proposal 9:  Adopt key simulation layouts described in this section.

# Conclusions

The proposals and observations made in this contribution are summarized below.

Observation 1: FR2x unlicensed deployments are likely to have a lower likelihood of encountering unmanaged interference, making spectrum sharing a natural choice. Even under dense uncoordinated deployment under heavy traffic, large majority of users experience very good SINR and long-term throughput performance.

Observation 2:  The interference, though less likely, can be damaging when present.

- Proposal 1: Investigate and identify conditions for deployment modes where No-LBT or No Sensing is viable.

- Proposal 2: Investigate and identify conditions for deployment modes where a long-term sensing for coexistence is viable.

- Proposal 3: LBT or Short-Term Sensing:  Identify conditions for deployment modes where short term medium sensing before a transmission needs to be considered for every transmission.

- Proposal 4: Contention Exempt Transmissions: Investigate and identify conditions where some transmissions can be permitted in a contention exempt manner, i.e. a sensing medium is not a requirement before transmission.

Proposal 5:  Study the use of directional sensing – sensing with beamforming - towards any channel access mechanism requiring channel monitoring.

Proposal 6:  Study the criteria under which the sensing beams can be different from the transmit beam at the transmitter.

Observation 3:  High Tx Sensing-Rx Mismatch: The interference observed by sensing unit at the transmitter may be considerably different than that eventually observed by the receiver.

Proposal 7:  Study and design channel access procedures and sensing guidelines that consider the prevalence of Tx Sensing-Rx mismatch.

Proposal 8:  Restrict the focus of the simulation studies for FR2x unlicensed bands to NR-U based deployments only, identifying tools for intra-technology coexistence.

Proposal 9:  Adopt key simulation layouts described in this section.

# References

- RP-193259, Study on supporting NR from 52.6 GHz to 71 GHz.

- RP-193229, Extending current NR operation to 71GHz.

- Chairman’s Notes, RAN1#92, Agreements for NR-U study.

- CEPT ECC, ERC Recommendation 70-03 for Short Range Devices, June 2019.

- ETSI EN 302 567, Harmonised European standard for Multi-Gigabit radio equipment in 60GHz.

- ETSI EN 303 722, WDTS for Fixed Networks.

# Appendix: Simulation methodology

This simulations in the contribution show an Indoor Office scenario with 2 Operators, derived from 38.901.

Figure 7 Indoor Office Layout with 2 Operators

Table 2:  Possible common simulation assumptions. The bold items have been used in the simulations for this contribution

| Carrier Frequency | 60GHz |
| --- | --- |
| Channel Bandwidth | {500MHz, 2.16GHz} |
| Channel Model | InH Open office model in TR 38.901 Chapter 7.4.1 |
| BS Tx Power | {Adjusted for Antenna Configuration to hit Peak EIRP*} |
| UE Tx Power | {Adjusted for Antenna Configuration to hit EIRP 25 dBm*} |
| BS Antenna gain | 5 dBi |
| UE Antenna gain | 5 dBi |
| BS Noise Figure | 7 dB |
| UE Receiver Noise Figure | 13 dB |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS antenna Array configuration | (M, N, P, Mg, Ng) = {(16, 8, 2, 1, 1), (8, 8, 2, 1, 1)} dH = dV = 0.5 λ for 60 GHz |
| UE antenna Array configuration | (M, N, P, Mg, Ng) ={(2, 2, 2, 1, 1), (4, 4, 2, 1, 1), dH = dV = 0.5 λ for 60 GHz |
| Tx power of gNB,UE | *EIRP limit for 60GHz indoor is 40dBm. Specifying UE EIRP instead of Tx Power permits comparisons across antenna configurations. |