3GPP TSG RAN WG1 #101		     R1-2004562

e-Meeting, May 25th – June 5th, 2020, 2020

Source: 	Nokia, Nokia Shanghai Bell

Title:	On UE features for 5G V2X

Agenda: 	7.2.11.4

Document for:	Discussion and Decision

# Introduction

The first baseline for Rel-16 UE features has been created in RAN1#100bis-e [1]. In this contribution we present our further views regarding the UE features for 5G V2X.

# Discussion

The agreed baseline for discussion over email discussion is copied in the Annex for reference. Our proposals on selected open issues for the FGs are as follows:

- 15-3

  - Component 4: Confirm the component. Sensing is an integral part of mode 2 operation, and hence it needs to be supported by all UEs supporting mode 2, otherwise further specification impact is needed to allow proper operation of mode 2 without sensing.

  - Component 5: component can be removed as it is redundant. Component 1 already indicates support for transmission of PSSCH.

- 15-11: should be mandatory for all UEs supporting transmission of NR sidelink

- 15-24: OK to support this FG.

# Conclusion

In this contribution we presented our proposals on UE features for 5G V2X.

# References

[1] R1-2003073, “RAN1 UE features list for Rel-16 NR after RAN1#100bis-E,” AT&T, NTT DOCOMO, Apr. 2020

# Annex

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-1 | Receiving NR sidelink | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink HARQ processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table in FR1]. 5) UE supports PT-RS reception in FR2. FFS: 6) The UE can receive [Z] total number of soft channel bits in a slot. [7) minimum two receive antennas] 8) UE can receive using the subcarrier spacing [and CP length] [defined for a given band in R15 in RAN4] [as configured for NR UL] FFS: 9) CP length 10) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [11) UE can receive PSSCH with 256QAM in NR sidelink] 12) [for NR SL by preconfiguration] UE can receive using 30 kHz subcarrier spacing in FR1, FFS FR2 | None | FFS | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 8 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 FFS: details for component (10) Note: Component 12 is required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Component-2 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 Component-3 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 FFS: Component-6 candidate value set: {value1, value2, …} [Component-8 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}}] [Component-12 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}}] Candidate values for A are {value1, value2 …} | Optional with capability signaling. For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to [8] configured grants can be configured for a UE. Up to [C] sidelink HARQ processes are supported. 2) UE supports transmission based on the normal 64QAM MCS OFDM table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2. 6) UE can transmit using the subcarrier spacing [and CP length] it reports. FFS: 7) CP length 8) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [9) Support downlink pathloss based open loop power control] [10) SL HARQ feedback on Uu] |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. FFS: This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise Candidate values for C are {value1, value2 …} Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} FFS: whether to mandate an SCS. [Note: For component (6) the supported SCS should be the same as the Uu link in licensed carrier] | Optional with capability signalling FFS: For UE supports NR sidelink in licensed spectrum where gNB is defined, UE must indicate this FG is supported. Candidate values for C are {value1, value2 …} |
| 15-3 | Transmitting NR sidelink mode 2 | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu or preconfiguration. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. [4) UE can perform sensing and resource allocation operations.] [5) UE supports rank 1 PSSCH transmissions.] 6) UE can transmit using the subcarrier spacing [and CP length] it reports for FG 15-1. FFS: 7) CP length 8) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH 9) default SCS with pre-configuration: 30 kHz with normal CP: [operator managed] same as Rel. 15 Uu 10) UE can transmit using 30 kHz [and normal CP] subcarrier spacing in FR1, FFS FR2 [11) DL pathloss based open loop power control when mode 2 is configured by NR Uu] | 15-1 | FFS | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 This is the basic FG for sidelink [in ITS spectrum where gNB is not defined and optional FG for licensed spectrum where gNB is defined] [Note: Component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1] Note: Component 6 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 10 is required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 FFS: all details for component (11) | Optional with capability signalling For UE supports NR sidelink, [for UE supports NR sidelink in ITS spectrum where gNB is not defined, UE must indicate this FG is supported,] UE must indicate this FG is supported. Candidate values for B are {value1, value2 …} |
| 15-4 | Synchronization sources for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 4) UE can transmit or receive NR sidelink based on the synchronization to an gNB 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 4 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 6 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB [when operating in Mode 1] [and mode 2] (FFS: delete component 1) 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. [in mode 2]. 3) UE can process CBR and CR within the time it indicates [in mode 2] | 15-1 and at least one of 15-2 and 15-3 | Yes | FFS |  | Per band | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink FFS: details of components (1) [Note: component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1] Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling |
| 15-6 | Short-term time-scale TDM for in-device coexistence | Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | FFS whether a set of candidate values need to be defined for the time required for the inter-RAT conflict resolution | Optional with capability signalling |
| 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can be scheduled over NR Uu for LTE sidelink mode 3 transmission.. 2) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3. 3) UE can monitor DCI format 3_1 for LTE sidelink SPS grant. |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Component-2 candidate value set: {0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms } | Optional with capability signalling |
| 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu | 1) UE can be configured over NR Uu for LTE sidelink mode 4 operation |  | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-10 | 256QAM sidelink transmission | 1) UE can transmit PSSCH with 256QAM in NR sidelink | At least one of 15-2, 15-3 | Yes | Yes | UE supports QPSK, 16QAM, and 64 QAM for transmission only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive [N] PSFCH(s) resources in a slot. 3) UE can transmit [M] PSFCH(s) resources in a slot. [4) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1.] [FFS: move to 15-2 or new FG 15-11a] | At least one of 15-1, 15-2, 15-3 | FFS | FFS |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 4 is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. ALT 1) Candidate values for N are {5, [10,] 15, [20,] 25, [30,] 35, [40,] 45, 50 } ALT 2) Candidate values for N are {32, 64} Candidate values for M are {1, 4, [5,] 8, 16} |
| 15-12 | Low-spectral efficiency 64QAM MCS table | 1) UE can transmit or receive PSSCH with low-spectral efficiency 64QAM MCS table. | At least one of 15-1, 15-2, 15-3 | Yes | Yes | UE supports normal 64QAM MCS table and 256QAM MCS table only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-14 | Sidelink CSI report | 1) UE can transmit and receive sidelink CSI-RS with 1 [or 2 antenna] port(s). 2) UE supports RI and CQI feedback on sidelink. | 15-1 and at least one of 15-2 and 15-3 | No | FFS |  | [Per band] | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-15 | eNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an eNB. 2) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-16 | Simultaneous transmission of uplink and sidelink | 1) UE supports simultaneous transmission of NR uplink and NR sidelink (onin different carriersbands) in a band combination for which the UE indicated simultaneous sidelink and uplink support in a band combination. | At least one of 15-2 and 15-3 | Yes | No |  | Per band combination | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-18 | Support of rank 2 transmission | 1) UE additionally supports rank 2 PSSCH transmission | [At least one of 15-2 and 15-3] | FFS | FFS | UE supports rank 1 PSSCH transmission only. | Per band | N.A. | N.A. | N.A. | This FG is a WA | Optional with capability signalling |
| 15-19 | Support of rank 2 reception | 1) UE additionally supports rank 2 PSSCH reception | [15-1] | FFS | FFS | UE supports rank 1 PSSCH reception only. | Per band | N.A. | N.A. | N.A. | This FG is a WA. FFS: This is the basic FG for NR sidelink | [Optional with capability signalling] |
| 15-22 | Support of SL slot less than 14 consecutive symbols Support of fewer than 14 consecutive sidelink symbols in a slot | 1) UE additionally supports transmission/reception of SL slot configured with 7, 8, 9, 10, 11, 12, 13 consecutive symbols [and the corresponding DMRS patterns it reports.] [2) UE supports [some/all] applicable DMRS patterns for the number of consecutive Sl symbols it reports] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports SL only in a SL slot configured with 14 consecutive symbols. | Per band | N.A. | N/A | N.A. | FFS: This is the basic FG for NR sidelink [Note: For Component (1) the support of 12 symbols is mandatory for ECP] [The component-1 candidate value set can be DRMS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {[{12,2}, {12,1},] {11,4}, {11,3}, {11,2}, {10,4}, {10,3}, {10,2}, {9,2}, [{9,3},] {8,3}, {8,2}, {7,2}, {6,2}, {5,2}}] | Optional with capability signalling |
| 15-23 | Support of open loop SL power control and RSRP report | 1) Support sidelink pathloss based open loop power control and RSRP report in case of unicast [2) downlink pathloss based OLPC] | FFS | FFS | Yes |  | Per band | N.A. | N/A | N.A. | Working assumption: This FG is a basic UE FG [at least] for UEs supporting mode 1 FFS: all details for component (2) FFS: whether this is a basic FG also for UEs not supporting mode 1 | Optional with capability signalling |
| 15-24 | [Support of multiple synchronization references] | [1) UE can support sidelink reception using up to A synchronziaion references in a carrier/BWP] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports only a single synchronization reference in a carrier/BWP. | Per band | N.A. | N.A. | N.A. | Component-1 candidate value set: {1, 2, 3, 4} Note: RAN1 is still discussing whether this FG is needed | Optional with capability signalling |