3GPP TSG RAN WG1 #101		     R1-2004568

e-Meeting, May 25th – June 5th, 2020, 2020

Source: 	Nokia, Nokia Shanghai Bell

Title:	On UE features for MR-DC/CA

Agenda: 	7.2.11.10

Document for:	Discussion and Decision

# Introduction

The first baseline for Rel-16 UE features has been created in RAN1#100bis-e post-meeting email discussion [2]. In this contribution we present our further views regarding the UE features for MR-DC/CA.

# Discussion

18-1/1a/1b:

- 18-1: Otherwise complete but need RAN2 input before the pre-requisite for 18-1 can be completed. OK to keep the [] around the pre-requisite for now.

18-2/2a/2b/3/3a:

- 18-2/2a, remove [] around component 4: 4) dropping NR transmission when LTE and NR transmissions collide for Type 1 UE

- 18-3: 6-13 should not be a pre-requisite as that makes 8-2 a pre-requisite, and there would be no need for a UE to support the pair of 6-13 and 8-2 on the same band combination with which it supports dual Tx.

- 18-3a: Should be per UE, if should apply to all bands where the UE supports the pre-requisite features, no need for a per band combination signalling.

- 18-2a/2b/3/3a, [This FG is for synchronous EN-DC]; OK to keep it, and remove [], but no need to repeat it in 18-3a, as all these features require the LTE and NR carriers to be synced.

18-4/4a/4b

- 18-4/4a should be per UE, dormancy does not relate to RF implementation. While the battery saving gains may relate to RF and thus to band combinations, the BB power saving should be there regardless.

- 18-4/4a we would prefer no FR1/FR2 differentiation, but if that is needed to compromise and not have this per BC, we’d be willing to accept FRx differentiation.

- 18-4b: No need to separate this from 18-4, FG 18-4b should be deleted.

18-5/5a/5b/5c/5d

- 18-5/5b: Support keeping component 2 as it now stands. Should be per UE as the cross-carrier ability is a BB feature not RF feature. Could consider making the capability multi-level as “intra-FR1, Intra-FR2, cross-FR” as a compromise, but should avoid unnecessary per BC or per FS signalling

- 18-5a: should be per UE, FRx differentiation

- 18-5c/5d: OK to keep these, same comment on applicability as for 5/5b.

18-6/6a

- 18-6: Should be per UE as the cross-carrier ability is a BB feature not RF feature. Could consider making the capability multi-level as “intra-FR1, Intra-FR2, cross-FR” as a compromise, but should avoid unnecessary per BC or per FS signalling

- 18-6a: Should be per UE, FRx differentiation.

# Conclusion

In this contribution we presented our proposals on UE features for MR-DC/CA.

# References

- R1-2003073, “RAN1 UE features list for Rel-16 NR after RAN1#100bis-E,” AT&T, NTT DOCOMO, Apr. 2020

- R1-2003202 Summary on email discussion [100b-e-NR-UEFeatures-Remaining] MR-DC/CA enhancement, NTT DOCOMO, May 2020

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-1 | Basic UL power sharing for DC | Semi-static power sharing mode1 between MCG and SCG cells of same FR for NR dual connectivity. | [intra-FR DC if such FG is introduced by RAN2] | Yes | N/A |  | Per BC | N/A | N/A | N/A | Absence means intra-FR DC is not supported. | Optional with capability signalling |
|  | 18-1a | Semi-static UL power sharing mode 2 for DC | Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity. | 18-1 | Yes | N/A |  | Per BC | N/A | N/A | N/A | Semi-static power sharing mode 2 between MCG and SCG cells of same FR is applicable only for synchronous NR dual connectivity | Optional with capability signalling |
|  | 18-1b | Dynamic UL power sharing for DC | Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity. - T_offset | 18-1 | Yes | N/A |  | Per BC | N/A | N/A | N/A | 1) {short, long} | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-4 | SCell dormancy indication within active time | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 | 6-5 | Yes | N/A |  | FFS [Per UE or Per BC] | No | [Yes or N/A] | N/A |  | Optional with capability signalling |
|  | 18-4a | SCell dormancy indication outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | 6-5 and [19-1] | Yes | N/A |  | FFS [Per UE or Per BC] | No | [Yes or N/A] | N/A |  | Optional with capability signalling |
|  | [18-4b] | [Support of SCell dormancy indication without data scheduling within active time] | [Support of SCell dormancy indication without data scheduling within active time] | TBD | Yes | N/A |  | FFS [Per UE or Per BC] | No | [Yes or N/A] | N/A |  | Optional with capability signaling |
| 18. MR-DC/CA enhancement | 18-5 | DL cross-carrier scheduling with different SCS | 1. The UE supports DL cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} [2. Processing up to X unicast DCI scheduling for DL per scheduled CC ] X is based on pair of (scheduling CC SCS, scheduled CC SCS): X=[4] for (15,120), (15,60), (30,120), X=[2] for (15,30), (30,60), (60,120 kHz), X applies per span in a slot of scheduling CC | 6-5 and one of {6-9, 6-9a} | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | [Yes or N/A] | N/A | crossCarrierScheduling-OtherSCS Note: This applies also to the case where there is a single span in the slot for the scheduling CC. In case UE supports 3-5b, the limits apply for each span for FDD scheduling cell and TDD scheduling cell. | Optional with capability signalling |
|  | 18-5a | Default QCL assumption for cross-carrier scheduling | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier scheduling. | 6-10 and 18-5 | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per band] | N/A | [Yes or N/A] | N/A |  | Optional with capability signalling |
|  | 18-5b | UL cross-carrier scheduling with different SCS | 1. The UE supports UL cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in UL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} [2. Processing up to X unicast DCI scheduling for UL per scheduled CC ] X is based on pair of (scheduling CC SCS, scheduled CC SCS): X=[4] for (15,120), (15,60), (30,120), X=[2] for (15,30), (30,60), (60,120 kHz), X applies per span in a slot of scheduling CC | 6-6 and one of {6-9, 6-9a} | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | [Yes or N/A] | N/A | crossCarrierScheduling-OtherSCS Note: This applies also to the case where there is a single span in the slot for the scheduling CC. In case UE supports 3-5b, the limits apply for each span for FDD scheduling cell and TDD scheduling cell. | Optional with capability signalling |
|  | [18-5c] | [DL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | [DL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | 18-5a TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | [Yes or N/A] | N/A |  | [Optional with capability signaling] |
|  | [18-5d] | [UL cross-carrier scheduling with different SCS and PUSCH processing capability 2] | [UL cross-carrier scheduling with different SCS and PUSCH processing capability 2] | 18-5b TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | [Yes or N/A] | N/A |  | [Optional with capability signaling] |
| 18. MR-DC/CA enhancement | 18-6 | Cross-carrier A-CSI RS triggering with different SCS | Cross-carrier A-CSI RS triggering with different SCS | 2-33 and 6-5 | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS or Per BC] | N/A | [Yes or N/A] | N/A | 1) {PDCCH cell of lower SCS and A-CSI RS cell of higher SCS, PDCCH cell of higher SCS and A-CSI-RS of lower SCS, both} . | Optional with capability signalling |
|  | 18-6a | Default QCL assumption for cross-carrier A-CSI-RS triggering | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier A-CSI-RS triggering. | 6-5 | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per band or Per BC] | N/A | [Yes or N/A] | N/A |  | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-7 | CA with non-aligned frame boundaries | CA with non-aligned frame boundaries for inter-band CA | 6-5 | Yes | N/A |  | Per BC | N/A | N/A | N/A | Defines whether the UE supports carrier aggregation operation where the frame boundaries of the Pcell and the Scell are not aligned, while the slot boundaries are. | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | 6-7 | Yes | N/A |  | Per UE | No | No | N/A | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | [TBD] |
| 18. MR-DC/CA enhancement | 18-2 | Single UL TX operation for TDD PCell in EN-DC | TDM restriction to LTE TDD PCell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (only for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe not limited to the reference TDM pattern (only for type 1 UE) [4) dropping NR transmission when LTE and NR transmissions collide for Type 1 UE] | EN-DC | Yes | N/A |  | Per band combination | Applicable to TDD-TDD EN-DC only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to TDD PCell This FG is for synchronous EN-DC | Optional with capability signalling |
|  | 18-2a | Enhanced single UL TX operation for FDD Pcell EN-DC | TDM restriction to LTE FDD Pcell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (only for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe not limited to the reference TDM pattern (only for type 1 UE) [4) dropping NR transmission when LTE and NR transmissions collide for Type 1 UE] | [6-13] | Yes | N/A |  | Per band combination | Applicable to in FDD-LTE -NR EN-DC | Applicable to FR1 only |  | Enhancement to the R15 capability tdm-Pattern [This FG is for synchronous EN-DC] | Optional with capability signalling |
|  | 18-2b | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | 18-2 | Yes | N/A |  | Per band combination | N/A | N/A | N/A | [This FG is for synchronous EN-DC] | Optional with capability signaling |
|  | 18-3 | Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE) | TDM restriction to LTE FDD PCell in EN-DC for dual UL Tx operation when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (only for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe not limited to the reference TDM pattern (only for type 1 UE) | [6-13], EN-DC | Yes | N/A |  | Per band combination | Applicable to EN-DC with LTE FDD PCell only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a dual Tx UE [This FG is for synchronous EN-DC] | Optional with capability signalling |
|  | 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern | UE configured with tdm-patternConfig-r16 can be semi-statically configured with LTE UL transmissions in all UL subframes not limited to the reference tdm-pattern (only for type 1 UE) | One of {18-2, 18-2a, 18-3} | Yes | N/A |  | [Per band combination] | Applicable to EN-DC only | Applicable to FR1 only |  | [This FG is for synchronous EN-DC] | Optional with capability signaling |